# 3GPP TSG-RAN WG3 Meeting #121	R3-234120

# 21-25 August 2023, Toulouse, France

Agenda Item:	25.2

Source:	CATT

Title:	Discussion on XR LSin from SA2

Document for:	Discussion

Introduction

There are five LS received from SA2/RAN2. The impact of R3-233731 is discussed in another XR paper. R3-233732 and R3-233733 do not have RAN3 specification impact. In this paper, we analyses R3-233734 and R3-233735 and give our proposals.

Discussion

2.1 R3-233735: Non-homogeneous deployment of PDU Set based handling

5.37.5.x	Non-homogenous support of PDU set based handling in NG-RAN

By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [xx]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

SMF sends at least one PDU Set QoS parameter to the NG-RAN to activate PDU set QoS handling for a give QoS flow.

- RAN3 impact: SMF sends PDU Set QoS parameter to NG-RAN via PDU session resource setup/modify request

NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not.

- RAN3 impact: NG-RAN sends PDU set Resource Setup Response/Failed to Setup List in PDU session resource setup/modify response.

At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling.

- RAN3 impact: Target NG-RAN sends the indication to SMF to indicate whether target NG-RAN support PDU set based handling via path switch request.

Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking.

- RAN3 impact: None.

Proposal 1: To support Non-homogeneous deployment of PDU Set based handling, the following RAN3 impacts are identified:

- SMF sends PDU Set QoS parameter to NG-RAN via PDU session resource setup/modify request.

- NG-RAN sends PDU set Resource Setup Response/Failed to Setup List in PDU session resource setup/modify response.

- Target NG-RAN sends the indication to SMF to indicate whether target NG-RAN support PDU set based handling via path switch request.

Proposal 2: The RAN3 impacts of supporting Non-homogeneous deployment of PDU Set based handling are included in TP for TS38.413 (R3-234121)

2.2 R3-233734: LS Reply on Design of RTP Header Extension for PDU Set handling

For Q1, SA2 confirms that a single QoS flow can include both N6 packets marked with the new RTP header extension. It will not impact RAN3 specifications because N6 packet is unvisitable in RAN.

Furthermore, SA2 agreed that a single QoS Flow includes either PDUs with GTP-U header extensions for PDU Sets or PDUs without such extension, but not both. If the UPF receives a PDU that do not belong to a PDU Set based on Protocol Description for PDU Set identification, then the UPF still maps it to a PDU Set. The potential impact is that the periodic DL/UL data and/or data burst will not stable. There may some non-periodic data occurs in XR traffic. RAN should consider how to handle such non-periodic data.

Observation 1: RAN may need to consider how to handle the PDU set which has PDU that do not belong to the PDU set but mapped to it by UPF and cause e.g., non-periodic data occurs.

For Q2, SA2 would like to confirm with SA4 that whether the current RTP header can reflect which PDU belongs to data burst and EoDB, if not, a new RTP header extension can be defined. It seems that there is no RAN3 impact.

Based on the analyses above, the RAN3 impact is not very clear in this stage. We would like to wait for further RAN2/SA2 progress.

Proposal 3: RAN3 wait for RAN2/SA2 progress for RTP Header Extension for PDU Set handling.

Conclusions

Proposal 1: To support Non-homogeneous deployment of PDU Set based handling, the following RAN3 impacts are identified:

- SMF sends PDU Set QoS parameter to NG-RAN via PDU session resource setup/modify request.

- NG-RAN sends PDU set Resource Setup Response/Failed to Setup List in PDU session resource setup/modify response.

- Target NG-RAN sends the indication to SMF to indicate whether target NG-RAN support PDU set based handling via path switch request.

Proposal 2: The RAN3 impacts of supporting Non-homogeneous deployment of PDU Set based handling are included in TP for TS38.413 (R3-234121)

Observation 1: RAN may need to consider how to handle the PDU set which has PDU that do not belong to the PDU set but mapped to it by UPF and cause e.g., non-periodic data occurs.

Proposal 3: RAN3 wait for RAN2/SA2 progress for RTP Header Extension for PDU Set handling.

# 4. Reference

[1] R3-233734, LS Reply on Design of RTP Header Extension for PDU Set handling

[2] R3-233735, Non-homogeneous deployment of PDU Set based handling