3GPP TSG-RAN WG3 Meeting #121	R3-234128

Toulouse, France, 21 – 25 Aug, 2023

Agenda item:	11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on QoE/RVQoE measurement report continuity and RAN overload in NR-DC

Document for:	Discussion and Decision

# 1	Introduction

Under current RAN3 discussion with respect to the RVQoE configuration framework the below agreements were made in RAN3#119bis-e and RAN3#120 respectively:

The node that has initially configured a UE in NR-DC with an RVQoE configuration can modify and release the RVQoE configuration as long as this node serves the UE.

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

In the response, the non-RVQoE-configuring node:

- Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

- Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

- Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

Moreover, one of the main open points to be further discussed as outcome in RAN3#119bis-e was the maintenance of QoE configuration after SN release, mobility for NR-DC UE as well as change from NR-DC to single connectivity as indicated below:

FFS how to handle the maintenance of QoE configuration after SN release, after mobility for an NR-DC UE and after the change from NR-DC to single connectivity.

In light of the discussion in RAN3#119bis-e, it was further agreed that QoE measurement reporting for NR-DC will be considered in the following scenarios:

Consider the QoE measurement reporting for NR-DC in following scenarios:

SCG failure scenario.

SN release scenario.

RAN overload scenario.

Furthermore, in the latest RAN3#120 meeting, RAN3 agreed the following with respect to the above-mentioned QoE measurement reporting continuation scenarios:

In line with the Rel-17 principles, if the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RVQoE measurements.

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

Finally, RAN3 has been working on principles of RAN overload in NR-DC. So far, the below agreements have been reached from RAN3#119bis-e:

The node that currently receives the QoE reports via the Uu can send a request to the peer node, asking that the QoE reporting leg is switched to the peer node.

The leg switch for QoE reporting needs to be approved by both nodes serving the UE.

In this paper, we provide additional input and discussion on the above open issues under the present sub-agenda item.

# 2	Discussion

## 2.1	Remaining discussions for RVQoE

Based on the recent RAN3 agreements concerning RVQoE, in this paper we provide additional explanation with respect to the remaining open issues.

RAN3 in RAN3#119bis-e and RAN3#120 agreed the following:

The node that has initially configured a UE in NR-DC with an RVQoE configuration can modify and release the RVQoE configuration as long as this node serves the UE

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node.

In the response, the non-RVQoE-configuring node:

Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

So far, in RAN3, in order to keep the framework simple, it was agreed that the first RVQoE configuration is blindly configured at the UE. However, after the first RVQoE reports are received, then the node receiving the reports can determine which node provides the bearers for the application session (the node that receives the reports or the peer node). In the case of the peer node providing the bearers, it is evident that this node, that had not initially configured the UE, should be able to reconfigure the UE based on its own interest.

However, given the first agreement above, it is not clear what does the statement node that “serves” the UE mean. If the node that serves the UE means the node that provides the bearers for the application session, then the third agreement indicated above needs to be adjusted to at least give the opportunity to the non-RVQoE configuring node to directly reconfigure the UE since it is the node that “serves” the UE. Otherwise, the two agreements are contradicting one another.

Proposal 1: RAN3 needs to clarify what does the statement node “serves” the UE exactly mean.

Proposal 2: RAN3 to acknowledge that if the non-RVQoE configuring node is the consumer of the RVQoE reports, then this node can reconfigure the UE directly with its own interested RVQoE metrics.

Furthermore, it is currently not clear for the first blind RVQoE configuration if the configuring node indicates itself as the node that shall receive the reports. In that case, depending on the node role (i.e., MN or SN), different behaviour can be expected otherwise no differentiation is maintained between MN and SN, although MN is the master node for the UE. For instance, if SN is blindly configuring the UE with RVQoE configuration then SN has only one option to indicate to the UE that SN shall receive the reports. However, for the case of MN, MN shall have the flexibility when blindly configuring the UE with RVQoE configuration to indicate to the UE whether the reports shall be sent to MN or SN.

Proposal 3: When MN blindly configures the UE with RVQoE configuration, MN can indicate to the UE whether reports shall be sent to MN or SN.

Proposal 4: When SN blindly configures the UE with RVQoE configuration, SN can only indicate to the UE that reports can be sent to SN.

If the above proposals are agreed in RAN3, it can occur that the node that has not initially (i.e., blindly configured the UE with RVQoE configuration) may receive the RVQoE reports although it not providing the bearers for the application session. In that scenario, RAN3 shall have similar agreements as for the case that the RVQoE configuring node receives the RVQoE report and determines that the non-RVQoE configuring node provides the bearers for the application session.

Proposal 5: RAN3 to propose that when the non-RVQoE-configuring node receives an RVQoE report and determines that the peer node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the peer node.

## 2.2	QoE/RVQoE report continuity during SN release and SCG failure

As already agreed in RAN3#119bis-e, the SN release and SCG failure scenarios will be supported and further studied if QoE/RVQoE report continuity needs to be maintained. In the following we detail all the possible cases as follows:

For QoE report continuity we have the following cases:

Case a) MN configures the UE in NR-DC with QoE/RVQoE configuration and MN receives the report directly from the UE.

Case b) MN configures the UE in NR-DC with QoE/RVQoE configuration, but SN receives the report from the UE.

Case c) SN configures the UE in NR-DC with QoE/RVQoE configuration and SN receives the report directly from the UE.

Case d) SN configures the UE in NR-DC with QoE/RVQoE configuration, but MN receives the report from the UE.

For RVQoE report continuity we additionally distinguish:

- MN provides the bearers for the application session and is the consumer of RVQoE reports.

- SN provides the bearers for the application session and is the consumer of RVQoE reports.

For QoE report continuity, in the scenarios of SN release and/or SCG failure, for both cases b) and c) where SN was the node that was receiving the QoE measurement reports from the UE, the reporting leg can be switched from SN to MN for the particular QoE configuration. For SN release, the switching of the reporting leg can be sent to the UE via SN as a trigger of SN Release Request message or it can be sent by MN after the SN Release Request Acknowledge message is received. However, for the scenario of SCG failure, only MN can instruct the UE to switch the reporting leg as the SCG leg is not available. In the case c) where SN provides the QoE configuration to the UE, SN may forward the QoE configuration to MN.

For case a), if MN had configured the UE with QoE measurement configuration and was the node receiving the QoE measurement reports, then no consequences or changes are needed. Otherwise, for case d) where SN had configured the UE with QoE configuration, SN may forward the QoE configuration to MN, but no switching of the reporting leg is needed.

Observation 1a: In the scenario of SN release or SCG failure in case b) and c) where SN was the node that was receiving the QoE measurement reports from the UE, then reporting leg can be switched from SN to MN. If SN had provided the QoE configuration to the UE, then SN may forward the QoE configuration to MN.

Observation 1b: In the scenario of SN release or SCG failure in case a) and d) where MN was the node that was receiving the QoE measurement reports from the UE, then reporting leg switch is not needed from SN to MN. If SN had provided the QoE configuration to the UE, then SN may forward the QoE configuration to MN.

Regarding the RVQoE configuration in addition to the above, If MN provides the bearers for the application session, then MN will be the consumer of the RVQoE reports. In the scenario of SN release, if SN was the node that was receiving the RVQoE reports from the UE, then the reporting leg can be switched from SN to MN for the particular RVQoE configuration. Similarly, to QoE configuration, the switching of the reporting leg can be sent to the UE via SN as a trigger of SN Release Request message or it can be sent by MN after the SN Release Request Acknowledge message is received. Alternatively, if MN was the node directly receiving the RVQoE reports from the UE then no consequences or changes are needed.

Observation 2a: In the scenario of SN release and/or SCG failure in case that MN was the consumer of the RVQoE reports, switching of reporting leg from SN to MN in case that SN was receiving the RVQoE reports directly from the UE can assure RVQoE report continuity.

Observation 2b: In the scenario of SN release and/or SCG failure in case that MN was the consumer of the RVQoE reports, if MN was the node directly receiving the RVQoE reports from the UE, then no consequences or changes are needed.

On the other hand, if the SN provides the bearers for the application session, then SN is the consumer of the RVQoE reports. Thus, in the scenario of SN release/SGC failure, it is unclear how the application layer at the UE and the MN should behave.

Observation 3: In the scenario of SN release and/or SCG failure in the case that SN is the consumer of RVQoE reports, then MN’s behaviour is not clear.

From MN’s perspective, if MN is interested in keeping the RVQoE reports being sent by the UE, simply switching the reporting leg might not be sufficient. The main reason stems from the fact that if MN decides to take over the session and provide the bearers for the application session, then the consumer of the RVQoE reports will be switched from SN to MN. In that case, MN might want to reconfigure the RVQoE metrics towards the UE based on MN’s own interest. Alternatively, MN may decide that it does not want to take over the bearers for the application session and thus RVQoE reports are unnecessary. In that case MN should notify the application layer at the UE to stop sending RVQoE reports.

In any case, MN should send a reconfiguration message to the UE, indicating the application layer measurement configuration ID to notify the UE whether the release of the RVQoE configuration, an update of the RVQoE metrics or maintenance of the existing RVQoE configuration applies.

Proposal 6: In the scenario of SN release and/or SCG failure, in case that SN was the consumer of the RVQoE reports, MN can either take over the bearers for the application session and become the new consumer of the RVQoE reports or MN releases the RVQoE configuration.

Proposal 7: In the scenario of SN release and/or SCG failure, in case that SN was the consumer of the RVQoE reports, MN should send a reconfiguration message towards the UE indicating the application layer measurement configuration ID concerning the RVQoE configuration that it wants to release, update or maintain.

## 2.3	QoE/RVQoE report continuity during MCG failure

So far, RAN3 has acknowledged the issue of QoE/RVQoE report continuity with respect to SN release and SCG failure scenarios among others. However, a similar problem might occur in case that MCG failure occurs. In the following we detail the scenario..

In the case of MCG failure, if fast MCG recovery is activated, then once UE detects a failure on the MCG leg and if the SCG leg is available, the UE might inform SN for the MCG failure. SN in that case would forward the information to MN. MN’s behaviour would be to either instruct the UE to handover to another cell or release the UE.

Observation 4: In the scenario of MCG failure, if UE is configured with fast MCG recovery and SCG leg is available, UE can notify MN via SN about the failure. In that case MN can either instruct the UE to handover to another cell or release the UE.

In the case that MN decides to handover the UE to another cell that is controlled by a new target MN, then the new target MN if it contains an Xn connection with the source MN may retrieve the UE context from the source MN containing all the QoE/RVQoE configurations that source MN had initially configured the UE with. In that case the new target MN may decide to either keep the QoE/RVQoE configurations or release them.

If the target MN decides to keep the QoE/RVQoE configuration(s), the new target MN may take over the bearers for the application session. In that situation, the new target MN might want to reconfigure the RVQoE metrics towards the UE based on new target MN’s own interest.

Proposal 8: In the scenario of MCG failure, if UE is instructed by source MN to handover to a new cell controlled by a new target MN, then the new target MN can retrieve the UE context from the source MN and either take over the bearers for the application session and become the new consumer of the RVQoE reports or release the RVQoE configuration.

Proposal 9: In the scenario of MCG failure, the new target MN should send a reconfiguration message towards the UE indicating the application layer measurement configuration ID concerning the RVQoE configuration that it wants to release, update or maintain.

## 2.4	RAN overload handling in NR-DC

To deal with RAN overload, Rel-17 introduced the QoE Pause/Resume mechanism. If the network is overloaded, it can use ‘pauseReporting’ RRC indication to temporarily stop QoE reports from being sent from the UE to the network. Handling of the suspended reports relies on buffering in the UE Access Stratum, until the gNB resumes the QoE reporting.

In NR-DC, the suspension of the QoE reporting does not necessarily imply there is no leg to continue. Since there will be the possibility to report QoE measurement to the peer node (leg). Therefore, currently the network node may have the possibility to either pause the QoE reporting at the UE or try to offload all or a fraction of the QoE sessions to the peer node and command the UE to switch the reporting leg for those QoE sessions.

Observation 5: In NR-DC, the suspension of the QoE reporting does not necessarily imply there is no leg to continue and now the network node can either pause the QoE reports or try to offload all or a fraction of QoE session to peer node.

In that case, if the network node that is overloaded decides to switch the reporting leg for some of the QoE sessions, based on current RAN3 discussions and agreements with respect to switching of the reporting leg, the node that currently receives the QoE reports from the UE needs to send a request to the peer node asking that the reporting leg is switched to the peer node and the request needs to be approved by both nodes.

Since the reporting leg switch can be related to the RAN overload situation in NR-DC, then RAN3 has to acknowledge that the basic negotiation feature for QoE in NR-DC between MN and SN can also be supported for the pausing and/or offloading of QoE measurement configurations.

Proposal 10: RAN3 to acknowledge that negotiation between MN and SN for switching of the reporting leg relates to pausing and/or offloading QoE measurement configurations between nodes in NR-DC.

Observation 6: In NR-DC, in case that the node that receives the QoE reports from the UE is overloaded and wants to offload some of the QoE sessions to the peer node and switch the reporting leg of the UE for those sessions, it needs the approval of the peer node.

Given the above observations, for NR-DC in case that the network node that currently receives the reports from the UE is overloaded, it may send a request to the peer node for the offloading indicating the list of RRC IDs that correspond to the QoE sessions it wants to offload and potentially ask the peer node to establish the SRB for those particular QoE sessions to enable report continuity. The decision of the network node with respect to which QoE sessions to offload may be based on QoE measurement priorities received by OAM, such that QoE sessions with high priority are maintained in overload scenarios. The peer node may then reply with a list of RRC IDs it accepts or rejects or simply a flag indicating that all QoE sessions could be offloaded or not.

Based on the reply from the peer node, the UE could be instructed by the network node to switch the reporting leg for those RRC IDs for which the peer node accepted the offloading and additionally UE is instructed to pause the reporting for those RRC IDs for which the peer node did not accept the offloading. By adopting such a mechanism, both nodes have a saying when it comes to the switching of the reporting leg as simply sending the command to the UE without the peer’s node approval may lead to overload of the peer node as well.

Proposal 11: In case of NR-DC, if the network node that receives the QoE reports from the UE is overloaded, it may initiate the process of offloading QoE sessions to the peer node by indicating a list of RRC IDs of the sessions it wants to offload optionally indicating their priorities as well as an indication for the peer node to establish the corresponding SRB for the reporting for those QoE sessions.

Proposal 12: In case of NR-DC, if a network node receives a request from the peer node indicating that the peer node wants to offload a list of QoE sessions with the respective RRC IDs, the network node may accept or reject some or all of the QoE sessions and replies to the peer node with the decision.

Proposal 13: In case of NR-DC, if the network node that receives the QoE reports from the UE is overloaded and receives a reply from the peer node that concerns the initial request of offloading QoE sessions, then the network node shall send the command for switching the reporting leg to the UE for the QoE sessions the peer node agreed to be offloaded and send the pause indication to the UE for those QoE sessions the peer node did not accept.

Additionally, given that a network node in NR-DC is overloaded, while requesting the peer node for offloading some or all QoE sessions together with the respective RRC IDs, it may also indicate to the peer node the allowance such that in case the peer node does not accept the offloading for some QoE sessions it can simply send the pause indication for those QoE sessions with a respective RRC ID to the UE. In that way the overloaded node in NR-DC does not have to further contact the UE.

Proposal 14: In case of NR-DC, if the network node that receives the QoE reports from the UE is overloaded while asking the peer node to offload QoE sessions indicating their RRC IDs, it may also indicate allowance for the peer node to send the pause indication to the UE for those QoE sessions it did not accept to be offloaded, in order to relieve the burden of the overloaded node.

# 3	Conclusion

We have made the following observations and proposals:

Proposal 1: RAN3 needs to clarify what does the statement node “serves” the UE exactly mean.

Proposal 2: RAN3 to acknowledge that if the non-RVQoE configuring node is the consumer of the RVQoE reports, then this node can reconfigure the UE directly with its own interested RVQoE metrics.

Proposal 3: When MN blindly configures the UE with RVQoE configuration, MN can indicate to the UE whether reports shall be sent to MN or SN.

Proposal 4: When SN blindly configures the UE with RVQoE configuration, SN can only indicate to the UE that reports can be sent to SN.

Proposal 5: RAN3 to propose that when the non-RVQoE-configuring node receives an RVQoE report and determines that the peer node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the peer node.

Observation 1a: In the scenario of SN release or SCG failure in case b) and c) where SN was the node that was receiving the QoE measurement reports from the UE, then reporting leg can be switched from SN to MN. If SN had provided the QoE configuration to the UE, then SN may forward the QoE configuration to MN.

Observation 1b: In the scenario of SN release or SCG failure in case a) and d) where MN was the node that was receiving the QoE measurement reports from the UE, then reporting leg switch is not needed from SN to MN. If SN had provided the QoE configuration to the UE, then SN may forward the QoE configuration to MN.

Observation 2a: In the scenario of SN release and/or SCG failure in case that MN was the consumer of the RVQoE reports, switching of reporting leg from SN to MN in case that SN was receiving the RVQoE reports directly from the UE can assure RVQoE report continuity.

Observation 2b: In the scenario of SN release and/or SCG failure in case that MN was the consumer of the RVQoE reports, if MN was the node directly receiving the RVQoE reports from the UE, then no consequences or changes are needed.

Observation 3: In the scenario of SN release and/or SCG failure in the case that SN is the consumer of RVQoE reports, then MN’s behaviour is not clear.

Proposal 6: In the scenario of SN release and/or SCG failure, in case that SN was the consumer of the RVQoE reports, MN can either take over the bearers for the application session and become the new consumer of the RVQoE reports or MN releases the RVQoE configuration.

Proposal 7: In the scenario of SN release and/or SCG failure, in case that SN was the consumer of the RVQoE reports, MN should send a reconfiguration message towards the UE indicating the application layer measurement configuration ID concerning the RVQoE configuration that it wants to release, update or maintain.

Observation 4: In the scenario of MCG failure, if UE is configured with fast MCG recovery and SCG leg is available, UE can notify MN via SN about the failure. In that case MN can either instruct the UE to handover to another cell or release the UE.

Proposal 8: In the scenario of MCG failure, if UE is instructed by source MN to handover to a new cell controlled by a new target MN, then the new target MN can retrieve the UE context from the source MN and either take over the bearers for the application session and become the new consumer of the RVQoE reports or release the RVQoE configuration.

Proposal 9: In the scenario of MCG failure, the new target MN should send a reconfiguration message towards the UE indicating the application layer measurement configuration ID concerning the RVQoE configuration that it wants to release, update or maintain.

Observation 5: In NR-DC, the suspension of the QoE reporting does not necessarily imply there is no leg to continue and now the network node can either pause the QoE reports or try to offload all or a fraction of QoE session to peer node.

Proposal 10: RAN3 to acknowledge that negotiation between MN and SN for switching of the reporting leg relates to pausing and/or offloading QoE measurement configurations between nodes in NR-DC.

Observation 6: In NR-DC, in case that the node that receives the QoE reports from the UE is overloaded and wants to offload some of the QoE sessions to the peer node and switch the reporting leg of the UE for those sessions, it needs the approval of the peer node.

Proposal 11: In case of NR-DC, if the network node that receives the QoE reports from the UE is overloaded, it may initiate the process of offloading QoE sessions to the peer node by indicating a list of RRC IDs of the sessions it wants to offload optionally indicating their priorities as well as an indication for the peer node to establish the corresponding SRB for the reporting for those QoE sessions.

Proposal 12: In case of NR-DC, if a network node receives a request from the peer node indicating that the peer node wants to offload a list of QoE sessions with the respective RRC IDs, the network node may accept or reject some or all of the QoE sessions and replies to the peer node with the decision.

Proposal 13: In case of NR-DC, if the network node that receives the QoE reports from the UE is overloaded and receives a reply from the peer node that concerns the initial request of offloading QoE sessions, then the network node shall send the command for switching the reporting leg to the UE for the QoE sessions the peer node agreed to be offloaded and send the pause indication to the UE for those QoE sessions the peer node did not accept.

Proposal 14: In case of NR-DC, if the network node that receives the QoE reports from the UE is overloaded while asking the peer node to offload QoE sessions indicating their RRC IDs, it may also indicate allowance for the peer node to send the pause indication to the UE for those QoE sessions it did not accept to be offloaded, in order to relieve the burden of the overloaded node.