3GPP TSG-RAN WG3 Meeting #121	R3-234141

Toulouse, France, 21st– 25th August, 2023

Agenda item:	13.2

Source: 	Huawei

Title: 	(TP for NR_mobile_IAB BL CRs for TS 38.401/ 38.473) Support of mobility for mobile IAB

Document Type:	Other

1 Introduction

Last RAN3 meeting discussed the mobile IAB migration issues, and has achieved the following agreements [1]:

- WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

To be continued:

Whether the information on the DU’s CU can also be configured by the MT’s CU.

- During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU.

To be continued:

How to pass the UE XnAP ID and NCGI or gNB-ID in case the IAB node knows the information of the DU’s CU by other means than OAM, if agreed.

It is FFS whether both gNB-ID and CGI needs to be provided to the mIAB-DU’s CU

It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP.

It is FFS whether the gNB-ID of the MT´s CU is passed from the IAB node to the IAB DU´s CU via F1AP.

- To be continued

Option 1: Use existing procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU

Option 2: Use new procedure(s) for triggering F1 Setup and for reporting the outcome of the F1Setup back to the DU´s source CU

- The DU’s CU can initiate the Xn TM Management Procedure pertaining to an mIAB-MT even though it has never had an RRC connection with this IAB-MT.

- Include “No PDU Session Indication” in the NGAP HANDOVER REQUEST message.

WA: legacy NG HO Required message with “dummy” PDU information is used. It needs to be further discussed if any specification changes are needed to describe the use of such dummy information. If specification changes is needed, consider whether an explicit indication is worth adding.

- To be continued:

RAN3 to introduce NGAP inter-gNB information exchange procedure(s) for exchange of IAB TMM messages and IAB Resource Coordination messages

RAN3 to decide whether this procedure is generic, i.e., it can carry any information, or IAB-specific, i.e., limited to IAB-specific purposes.

- After mIAB-DU migration, the BH RLC and BAP routing configurations used in the non-F1 terminating topology for TMM with the mIAB-DU’s source CU may be released.

- To be continued:

Discussions on enhancements to legacy HO procedures to be taken at RAN3-121.

If no consensus on such enhancements, the following will be agreed:

For DU migration, the following baseline is enough for target CU being aware of the QoS profile for each UE traffic:

The target CU for mIAB-DU migration learns the traffic profile of the UE traffic from Handover Preparation procedures for individual UEs. No further enhancements are needed.

This contribution mainly focuses on the remaining issues of the procedure for the inter-donor migration for mobile IAB. We provide the TPs for TS 38.401 and TS 38.473 for capturing the mobile IAB migration related progress so far.

2 Discussion

2.1 Scenario and terminology definition for IAB-node mobility

We have agreed in In RAN3 #117b [4] that “mIAB-DU migration and mIAB-MT handover can be executed independently from each other” and in RAN3 119bis-e [2] that “Capture the mIAB-MT HO and mIAB-DU migration as separate procedures in TS 38.401”. In this section, we give the 2 basic scenarios based on the independently executed DU migration and MT handover, and give the relevant terminologies.

- MT handover, without DU migration.

In Rel-17, we already support this scenario for one step partial migration, and in Rel-18, such scenario needs to be enhanced to support consecutive partial migration, as shown in Figure 1. To describe the involved nodes in a general way, we provides some terminologies, which are also useful to be captured in the stage 2 procedures. For this scenario, CU1, CU2, and CU3 are defined as F1 terminating CU, source RRC terminating CU, and target RRC terminating CU, respectively.

Figure 1. Consecutive partial migration scenario

- DU migration, without MT handover.

IAB-DU may also migrate to a target F1 terminating CU without the MT handover (this does not mean that the DU migrates without the IAB-node mobility, instead, such DU migration can be performed before or after the MT migration when mobile IAB node moves). As shown in Figure 2, the mIAB-MT is consistently under CU2’s topology, and mIAB-DU migrates from CU1 to CU3. For this scenario, CU1, CU2, and CU3 are defined as source F1 terminating CU, RRC terminating CU, and target F1 terminating CU, respectively.

Figure 2 illustrates a general scenario. In one specific case, the CU2 can be same as CU1, i.e. the mIAB-DU and mIAB-MT are connected to same source IAB donor, then the mIAB-DU migrates to a target IAB donor while the IAB-MT keeps connection with the source donor. In another specific case, the CU2 can be same as CU3, i.e. the mIAB-DU migrates from a source donor to the target donor which serves the IAB-MT.

Figure 2. DU migration without MT handover scenario

It is worth noting that there may be more complicated scenarios, but can be implemented by combining the above 2. For example, if the target CU of mIAB-DU and mIAB-MT are different, and the DU migration and MT handover can be performed one by one, then such case is composed by the 2 scenarios.

In summary, RAN3 to design stage 2 procedures for the following two basic scenarios:

1) MT handover, without DU migration.

2) DU migration, without MT handover.

And for these 2 basic scenarios, we propose the following terminologies for the description:

Proposal 0: RAN3 introduce the following terminologies for mobile IAB:

1) For basic scenario 1 (MT handover without DU migration): F1 terminating CU, source RRC terminating CU, target RRC terminating CU;

2) For basic scenario 2 (DU migration without MT handover): source F1 terminating CU, RRC terminating CU, target F1 terminating CU.

2.2 Mobile IAB-node integration

Besides the IAB-node mobility scenarios described in Section 2.1, when the mobile IAB integrates to the network before any mobility happens, the DU and MT may also connect to different CUs, as shown by Figure 2.

Figure 2. DU and MT integrating to different CUs scenario

In the last meeting, it was agreed that “During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU”. As for how the mIAB-DU’s CU knows these information, it was marked as to be continued in the last meeting that “It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP”. We don’t think the solution in this FFS is reasonable. Since anyway the two CUs need to directly exchange messages (e.g., the IAB TMM messages), avoiding direct interaction between the CUs is not a proper reason to support the CU2-mIAB-CU1 indirect transfer path. And, in our view, if the mobile IAB-DU and co-located IAB-MT are allowed to integrate to two different CUs, the intention of de-coupled integration for mobile IAB is to avoid DU migration as much as possible, the network deployment can provide one candidate F1 terminating donor for the mobile IAB node in large area covered by multiple  gNBs which can provide radio link for the mobile IAB-MT, and should ensure that the DU’s CU and MT’s CU have Xn connectivity, otherwise, such de-coupled integration does not make sense.

Proposal 1: The network deployment should ensure that the CU which to be the F1 terminating CU of mobile IAB has Xn connectivity to the MT’s CU, if allow the mobile IAB-DU connects to a CU different from the MT’s CU.

It is worth noting that, no matter which way to provide the MT’s UE XnAP ID and gNB ID to the DU’s CU, these information are generated by CU2 and can be directly transmitted to CU1 via Xn interface. The issue to be further discussed is when and how CU2 knows it is not the F1 terminating donor and should provide the gNB ID of itself and UE XnAP ID of MT to the F1 terminating donor.

Proposal 2: If the mobile IAB-DU connects to a donor CU which is different from the co-located MT’s CU during integration, MT’s CU should know it will not serve as the F1 terminating donor, to provide its gNB ID and UE XnAP ID of mIAB-MT to the F1 terminating donor. FFS on when and how to let the MT’s CU know that.

We may noticed that during the mobile IAB-DU migration, the target CU can be selected by either the source CU or the OAM. So, for the integration case, besides the OAM based configuration, it is also possible to allow the MT’s CU provide the F1 terminating donor information to the mobile IAB-node, based on some configuration. This will also be helpful for the MT’s CU to aware of it will not serve as the F1 terminating CU, and generate the information including MT’s UE XnAP ID and its gNB ID for the F1 terminating CU. Therefore, it is proposed:

Proposal 3: During integration procedure, the mobile IAB node may obtain the F1 terminating donor information (including gNB ID, CU TNL address, etc) from the MT’s CU, based on pre-configuration at the MT’s CU.

2.3 Consecutive partial migration

2.3.1 mIAB-MT’s UE XnAP ID exchanges

RAN3 #119b [2] agreed that “For consecutive partial migration, after the mIAB-MT HO is completed, the mIAB-DU’s CU sends the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the mIAB-MT’s target CU, including the UE XnAP ID assigned to the mIAB-MT by the mIAB-MT’s target CU as the “Non-F1-Terminating IAB-donor UE XnAP ID””. The agreement indicates the UE XnAP ID assigned by the mIAB-MT’s target RRC terminating CU should be known to the F1 terminating CU. As for how the F1 terminating CU knows the information, since in RAN3 #119 [3] we have agreed that “The mIAB-MT’s source donor CU can send the info on the mIAB-MT’s target donor CU to the mIAB-DU’s donor CU after the completion of IAB-MT HO”, it is natural to also let the MT’s source CU send the UE XnAP ID of the mIAB-MT allocated by the target CU after the completion of mIAB-MT HO. It is proposed:

Proposal 4: For consecutive partial migration, the source RRC terminating CU sends the UE XnAP IDs of mobile IAB-MT (allocated by CU3 and CU1) to the F1-terminating CU, after the mobile IAB-MT’s successful HO.

On the other hand, CU3 may also need to know the MT’s UE XnAP ID in CU1 and the gNB ID of CU1 before CU1 initializes the IAB transport migration management procedure. If not, although CU3 can figure out the MT this message associates with based on the MT UE XnAP  ID in CU3 which is included in IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message from CU1, that does not meet the convention of Xn interaction in some degree. In Xn protocol, when the UE XnAP IDs of both sides are included in a message, that always means the CUs have exchanged messages associated to this UE before. For example, the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message includes the MT’s UE XnAP ID in both CUs, and before that, the CUs should have exchanged the handover preparation/SN addition/UE context retrieve procedure. If CU1 directly initializes the IAB transport migration management procedure to CU3 with the MT’s UE XnAP IDs of both side in case that CU3 have never exchanged any MT associated messages before, it is abnormal for CU3. Therefore, we propose that CU2 informs CU3 the MT’s UE XnAP ID in CU1 and the gNB ID of CU1:

Proposal 5: For consecutive partial migration, the source donor-CU of mobile IAB-MT may inform the target donor-CU of mobile IAB-MT: the UE XnAP ID of mobile IAB-MT generated by F1 terminating CU (CU1) + gNB ID of CU1.

2.3.2 mIAB-MT’s UE XnAP ID retaining

1) For the XnAP UE ID retaining on the mIAB-DU’s CU

To support the above information exchange on the UE XnAP ID, and the consequent IAB transport migration management procedure between CU1 and CU3, it requires the CU1, i.e., the F1-terminating donor-CU to retain the UE XnAP IDs of the IAB-MT.

There were some discussions about how to retain the UE XnAP ID, but have not reached consensus. The key reason is no consensus on whether the UE XnAP ID allocated for the mIAB-MT is per Xn interface or per CU. Some companies prefer the per Xn interface UE XnAP ID allocation for the IAB-MT and tend to think: After the mIAB-MT switches to another CU, the Xn interface between DU’s CU and MT’s source CU will not transfer the messages associated to the mIAB-MT, and thus DU’s CU may not retain the XnAP UE ID used for the Xn interface between DU’s CU and MT’s source CU.

In our understanding, although the UE XnAP ID of mIAB-MT can be allocated per Xn interface theoretically, it will be beneficial if we assume the UE XnAP ID of mIAB-MT is allocated per node for the mobile IAB scenarios. Otherwise, there may be some issues if using the UE XnAP ID transmitted between the CUs to identify the mobile IAB-MT in some cases involving more than two donor CUs. For example, if the mIAB-MT performs HO from CU2 to CU3, CU2 informs CU1(the F1 terminating donor CU) the mIAB-MT’s UE XnAP ID allocated by CU3, and then CU1 initializes the IAB transport migration management procedure to CU3. If CU3 allocates ID1 to the mIAB-MT as the UE XnAP IDs for the Xn interface between CU2 and CU3 during the mIAB-MT’s HO preparation procedure, and ID2 to the same mIAB-MT for the Xn interface between CU1 and CU3, the complexity for the whole consecutive partial migration will be increased from the following aspects:

- CU3 should allocate ID2 to the mIAB-MT, despite that it has allocated ID1 to the same mIAB-MT using the legacy HO procedure.

- CU3 should include ID2 to the message CU2, in addition to the ID1 which should be carried as the Target NG-RAN node UE XnAP ID in the HO preparation procedure. This is to allow the CU2 can inform the ID2 to CU1, since the CU1 cannot using ID1 as the non-F1 terminating UE XnAP ID in the IAB TMM message to CU3.

Observation 1: If the UE XnAP ID allocated for the mIAB-MT is per Xn interface, the complexity for the consecutive partial migration will be increased, especially for the target CU of the mIAB-MT.

Therefore, we suggest the UE XnAP ID allocated to the mIAB-MT should be unique per node and can be reused by the node for multiple Xn interfaces.

Moreover, considering that CU1 would initiate IAB transport migration management procedure to CU3 when the mIAB-MT is connected to another CU (i.e., CU2 and CU3), as long as the mIAB-DU is connected to CU1. The mIAB-DU’s CU should retain the mIAB-MT UE XnAP ID allocated by itself as long as the corresponding mIAB-DU connects to it, regardless which CU the mIAB-MT connects to.

And the mIAB-DU’s CU should retain the mIAB-MT UE XnAP ID allocated by the mIAB-MT’s CU until it is notified that the mIAB-MT has been handed over to another CU, because mIAB-DU’s CU will not interact with mIAB-MT’s pervious CU about the mIAB-MT associated messages any more.

In summary, it is proposed:

Proposal 6a: For consecutive partial migration, the F1-terminating donor-CU retains the UE XnAP IDs allocated for the mobile IAB-MT by itself as long as the corresponding mobile IAB-DU connects to this CU, and retains the UE XnAP ID allocated for the mIAB-MT by the mIAB-MT’s CU until it is notified that the mIAB-MT has been handed over to another CU.

2) For the XnAP UE ID retaining on the mIAB-MT’s CU

For this issue, things are simpler. The mIAB-MT’s CU retains mIAB-MT UE XnAP ID until the mIAB-MT switches to another CU. mIAB-MT’s source CU will not interact with another CU about the mIAB-MT associated messages any more after the mIAB-MT has left, and the mIAB-MT’s context will be released on mIAB-MT’s source CU. It is proposed:

Proposal 6b: For consecutive partial migration, the source donor CU of IAB-MT should retain the UE XnAP IDs allocated for the mobile IAB-MT as long as the mobile IAB-MT is connected.

2.4 Support of IAB-DU migration

2.4.1 mIAB-node IP address related issue

After the mIAB-node receiving the DU migration trigger with the notification of the target CU identity, the mIAB-node should also obtain the IP address of itself for the new F1 interface. The mIAB-node requests new IP address of itself from the RRC terminating donor by existing RRC signalling, e.g., via IABOtherInformation IE. It is up to the RRC terminating donor whether the IP address for the new F1 interface is newly allocated or is reusing the IP address of the old F1 interface. There will be no stage-3 impact since the existing signaling is enough. It is proposed:

Proposal 7: For DU migration triggered by the donor CU of mobile IAB-DU, the mobile IAB-node requests IP address for the new logical IAB-DU using the existing RRC signaling, after receiving the trigger of new F1 setup.

2.4.2 How the target F1 terminating CU be aware of the RRC terminating CU and IAB-MT

For the source F1 terminating CU triggered DU migration, the target F1 terminating CU is selected by the source F1 terminating CU. The source F1 terminating CU is also aware of the RRC terminating CU since there were some Xn TMM procedures between the two CUs. Therefore, it is straightforward that the source F1 terminating CU sends the IDs of RRC terminating CU and mIAB-MT to the target F1 terminating CU, for the triggering of Xn TMM procedure. Specifically, the ID of the mIAB-MT should be the UE XnAP ID allocated by the RRC terminating CU. And the target F1 terminating CU allocates UE XnAP ID to the mIAB-MT, after it received the UE XnAP ID of mIAB-MT allocated by the RRC terminating CU.

For the OAM triggered DU migration, just one more step is to be added. That is, the mIAB-node should first notify the source F1 terminating CU that the DU migration is to be performed, carrying the ID of the target F1 terminating CU. Then the source F1 terminating CU can inform the target F1 terminating CU the IDs of the RRC terminating CU and the mIAB-MT as illustrated above.

Some companies prefer to let the mobile IAB-DU informs the target F1 terminating CU these information, to avoid the interaction between source and target F1 terminating CUs. We don’t think such motivation is necessary. Similar as what we illustrate in section 2.2, anyway the source and target F1 terminating CUs need to exchange messages, e.g., the UE handover preparation messages. If there is no existing Xn interface between the source and target F1 terminating CUs currently, it is because there is no neighbour cells under the two CUs. But an Xn interface can be established if needed, since the source and target CUs are IP reachable with each other (at least an IP path exists in CU1-donor DU2-CU3). There is no problem the source F1 terminating CU can send these information to the target F1 terminating CU.

It is proposed:

Proposal 8a: For DU migration, the source F1 terminating CU can send the ID of RRC terminating CU and UE XnAP ID of mobile IAB-MT allocated by the RRC terminating CU, to the target F1 terminating CU.

Proposal 8b: For DU migration, the target F1 terminating CU allocates UE XnAP ID to the mobile IAB-MT, after it receiving the UE XnAP ID of mobile IAB-MT allocated by the RRC terminating CU.

2.4.3 DSCP/flow label related issue

In Rel-17, the F1 termianting CU and non-F1 terminating CU will negotiate on the value of DSCP/flow label for DL traffic in the IAB transport migration management procedure. For the DU migration scenario as illustrated in Figure 2, CU3 needs to know the rule before the target F1 setup at least for the F1-C and non-F1 traffic, since the DSCP/flow label is needed for the transmision of the TNL association messages and the F1 SETUP RESPONSE.

Since the target F1 traffic is transported via CU2 topology, it is reasonable the DSCP/flow label mapping rule is sent from CU2 to CU3. On the other hand, CU1 knows the DSCP/flow label mapping rule for the source F1 traffic, and the target F1 traffic is the migration of source F1 traffic without changing the QoS requirements. So it is also possible that CU1 informs CU3 the DSCP/flow label. It is proposed:

Proposal 9: For DU migration, the source F1 terminating CU or the RRC terminating CU informs target F1 terminating CU on the value of DSCP and/or flow label for F1-C and non-F1 traffic, before new F1 setup procedure.

2.4.4 Release of the BH configuration on the non-F1 term topology

It was concluded in the last meeting that the BH configurations used for the old F1 transmission “may” be released. Whether the BH configuration is released is up to the RRC terminating CU’s decision, but not to release the configuration does have some benefits.

In section 2.4.2, we mention that the target F1 terminating donor can initiate IAB TMM procedures to the RRC terminating donor. The RRC terminating CU determines the BH configuration in its topology. At this stage, the RRC terminating CU can decide to reuse the BH configuration which was used for the source F1 interface, because the UE’s traffic QoS requirement is unchanged during the mobile IAB-DU migration procedure. The BH configuration can be reused in mIAB-DU migration as long as the collocated mIAB-MT does not switch. This is benificial for the RRC terminating topology, because there is no need to first configure the resource for new F1 transmission and then release the resource for old F1 transmission. It is observed and proposed:

Observation 2: The BH configuration in the RRC terminating topology can be reused for transmitting the traffic of target logical DU, considering that the UE’s traffic QoS requirement is unchanged during the mobile IAB-DU migration procedure.

Proposal 10: After mobile IAB-DU migration, the BH configuration in the RRC terminating topology is kept and reused to forwarding traffic between the target logical IAB-DU and the target F1 terminating donor, until the co-located mobile IAB-MT is handed over.

2.4.5 The procedures for the F1 setup trigger and outcome report

In DU migration, the source CU triggers the target F1 setup via F1AP, and the source DU reports the F1 setup completion to the source CU also via F1AP. According to the conclusion of the last meeting, it is to be continued to discuss whether to use the existing or new F1AP procedures for these messages. Since such function is purely new, defining new procedure is clear and no harmful.

Some companies propose to reuse existing F1AP messages (e.g. the gNB-CU Configuration Update message to carry the new F1 setup indication) between the source F1 terminating donor CU and the source logical DU. But this is not a good choice, since all the existing F1AP procedures are defined for the configuration/management related to the F1 interface between the gNB-CU and the gNB-DU, not involving another F1 interface towards the new logical DU. For example, in TS 38.473, the following sentence is present for the gNB-CU Configuration Update procedure: “The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface.”  Thus, we prefer new F1AP procedures.

Observation 3: The existing F1AP procedures are defined for the configuration/management related to the F1 interface carrying this F1AP, not suitable for another F1 interface.

Considering the new F1 setup may be triggered by the OAM configuration on the mobile IAB-node, and if triggered by OAM, the source logical IAB-DU also should report the complete of new F1 setup to the source F1 terminating CU, it will be good if the new procedure can cover both cases. So, we suggest the F1 setup outcome report should be a class 2 procedure, and the F1 setup trigger procedure between the source F1 terminating CU and the source logical IAB-DU should also be class 2. It is proposed:

Proposal 11: Introducing two new class 2 F1AP procedures, for triggering F1 setup, and reporting the outcome of the F1 setup.

It needs to be clarified in DU migration that the default configuration should be permitted to be used for the UL messages before and during new F1 setup. According to the current BAP spec, when the IAB-node is configured with other BAP configurations than the default configuration via F1AP, the default configuration will not be used anymore by the BAP layer of IAB node. But for the UL messages related to the new F1 setup, e.g., the SCTP handshake and IPsec negotiation, the only BH resource can be used is the default configuration, since there is not any other item in the UL mapping is available for the traffic towards the target F1 terminating CU. It is proposed:

Proposal 12: Before new F1 setup, the default configuration including default BH RLC CH and default UL BAP routing ID will be used for the SCTP handshake and IPsec negotiation between the mobile IAB DU and the target F1 terminating donor CU. And it is up to RAN2 to ensure the default configuration be available for new F1 setup.

2.5 NG related issue

RAN3 agreed “For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.” But it is still need to be first analyzed whether the scenario of “without Xn” really makes sense. If two CUs are IP reachable for each other, there should be no obstables to establish an Xn application connection between the two CUs. RAN3 agreed that “RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available”. In all the scenarios including mobile IAB-node integration, consecutive partial migration and DU migration, all the CUs are IP reachable: at least the IP routes between the donor DU of the RRC terminating CU and the (source/target) F1 terminating CU are connected. No existing Xn interface is mainly because there is no neighbour cells under the two CUs before, but does not mean the Xn interface cannot be established.

Observation 4: In mobile IAB scenarios, the IP is reachable between the CUs involved. If there is no Xn between the CUs currently, an Xn interface can be established.

Then, the question is transformed to: Which one is more cost effective in new Xn interface establishment and NG transmission? In our view, the NG based transmission is much problematic. Different than the NG based handover where the UE handover preparation messages pass through the AMF just one round, the NG based IAB TMM and IAB resource coordination need to frequently disturb the AMF. For example, in DU migration, each time the target F1 terminating CU receives a UE’s handover request, it may initilize an IAB TMM procedure.

Observation 5: Different from the NG based HO, which only requires limited procedures between the AMF and NG-RAN node, the IAB TMM and IAB resource coordination message may be frequently forwarded via NG interface if the MT’s CU and DU’s CU has no Xn interface.

Furthermore, we also have the concern on the fact that the NG based IAB TMM/resource coordination may re-define the AMF’s function. The AMF is re-defined like a router for the user plane transmission between CUs. In NG based HO, although there are signalling exchanges between the source AMF and the target AMF, it is mainly for the UE context relocation and service relocation, and after the HO completes, the source AMF deletes the UE context and service. The AMF has never act as a simple router for the NG-RAN nodes, which can be used anytime when the two NG-RAN nodes cannot be directly connected.

Observation 6: The forward of some Xn interface signaling, e.g. IAB TMM messages, and IAB resource coordination messages, requires the AMF works as a router between MT’s CU and DU’s CU, such router-like function is for user plane, not control plane.

Therefore, in the MT/DU target CU selection stage, the target CU is better to have existing Xn connections to the current MT’s CU and DU’s CU. If no existing Xn connection, it would be better to set up an Xn interface but not resort to the NG interface. It is proposed:

Proposal 13: RAN3 not support the exchange of IAB TMM messages and IAB Resource Coordination messages via NG interface.

Proposal 14: To support the consecutive partial migration and the DU migration, the MT’s CU and DU’s CU should have Xn connection, or able to establish Xn connection if necessary.

2.6 Mobile IAB authorization result notification

In RAN3 #119bis-e [2], it was agreed to use the high level description “use it accordingly” as the procedure text in TS 38.413 to describe the action of the IAB-donor when receiving the results of mobile IAB authorization. The detailed action is up to implementation.

For the case that the mobile IAB-node is first authorized, the F1 terminating CU should be aware of the updated mobile IAB node authorization result when the result is changed to be not authorized. Some companies argued in the last meeting that the MT’s CU can directly release the BH resources when it receives the “not authorized” indication and does not inform the DU’s CU. But if the F1 terminating CU does not know the authorization result, it may still attempt to initialize the IAB TMM procedure again and again, which causes signalling overhead. In addition, if the F1 terminating CU knows the IAB-node is not authorized, it can timely and orderly handover the UEs served by the mobile IAB-node and release the F1 connection, which is friendly for the UEs. It is observed:

Observation 7: The release of BH resource in the non-F1 terminating topology does not mean the IAB node is not authorized, and the F1 terminating CU may still attempt initializing the IAB TMM procedure.

Observation 8: If the F1 terminating CU knows the IAB-node is not authorized, it can handover the UEs served by the IAB-node as soon as possible, and release the F1 connection towards the IAB-DU.

Considering the MT’s CU knows the latest authorization result from the AMF, the MT’s CU should inform DU’s CU the mobile IAB-node’s authorization result when the IAB-node is firstly integrated and when the authorization is changed. It is proposed:

Proposal 15: The F1 terminating donor should be aware of the mobile IAB node authorization result from the RRC terminating CU, if the mobile IAB-DU and the mobile IAB-MT connects to different donor CUs.

3 Conclusion

This paper mainly discusses the issues on the mobile IAB integration, consecutive partial migration, and DU migration procedures, the following observation and proposals are provided:

Issue of scenario for IAB-node mobility

Proposal 0: RAN3 introduce the following terminologies for mobile IAB:

1) For basic scenario 1 (MT handover without DU migration): F1 terminating CU, source RRC terminating CU, target RRC terminating CU;

2) For basic scenario 2 (DU migration without MT handover): source F1 terminating CU, RRC terminating CU, target F1 terminating CU.

Proposal 2: RAN3 to avoid or deprioritize the case for the collision of mIAB-DU migration and HO of mIAB-MT.

Issue of mobile IAB node integration

Proposal 1: The network deployment should ensure that the CU which to be the F1 terminating CU of mobile IAB has Xn connectivity to the MT’s CU, if allow the mobile IAB-DU connects to a CU different from the MT’s CU.

Proposal 2: If the mobile IAB-DU connects to a donor CU which is different from the co-located MT’s CU during integration, MT’s CU should know it will not serve as the F1 terminating donor, to provide its gNB ID and UE XnAP ID of mIAB-MT to the F1 terminating donor. FFS on when and how to let the MT’s CU know that.

Proposal 3: During integration procedure, the mobile IAB node may obtain the F1 terminating donor information (including gNB ID, CU TNL address, etc) from the MT’s CU, based on pre-configuration at the MT’s CU.

Issue of consecutive partial migration

Observation 1: If the UE XnAP ID allocated for the mIAB-MT is per Xn interface, the complexity for the consecutive partial migration will be increased, especially for the target CU of the mIAB-MT.

Proposal 4: For consecutive partial migration, the source RRC terminating CU sends the UE XnAP IDs of mobile IAB-MT (allocated by CU3 and CU1) to the F1-terminating CU, after the mobile IAB-MT’s successful HO.

Proposal 5: For consecutive partial migration, the source donor-CU of mobile IAB-MT may inform the target donor-CU of mobile IAB-MT: the UE XnAP ID of mobile IAB-MT generated by F1 terminating CU (CU1) + gNB ID of CU1.

Proposal 6a: For consecutive partial migration, the F1-terminating donor-CU retains the UE XnAP IDs allocated for the mobile IAB-MT by itself as long as the corresponding mobile IAB-DU connects to this CU, and retains the UE XnAP ID allocated for the mIAB-MT by the mIAB-MT’s CU until it is notified that the mIAB-MT has been handed over to another CU.

Proposal 6b: For consecutive partial migration, the source donor CU of IAB-MT should retain the UE XnAP IDs allocated for the mobile IAB-MT as long as the mobile IAB-MT is connected.

Issue of DU migration

Observation 2: The BH configuration in the RRC terminating topology can be reused for transmitting the traffic of target logical DU, considering that the UE’s traffic QoS requirement is unchanged during the mobile IAB-DU migration procedure.

Observation 3: The existing F1AP procedures are defined for the configuration/management related to the F1 interface carrying this F1AP, not suitable for another F1 interface.

Proposal 7: For DU migration triggered by the donor CU of mobile IAB-DU, the mobile IAB-node requests IP address for the new logical IAB-DU using the existing RRC signaling, after receiving the trigger of new F1 setup.

Proposal 8a: For DU migration, the source F1 terminating CU can send the ID of RRC terminating CU and UE XnAP ID of mobile IAB-MT allocated by the RRC terminating CU, to the target F1 terminating CU.

Proposal 8b: For DU migration, the target F1 terminating CU allocates UE XnAP ID to the mobile IAB-MT, after it receiving the UE XnAP ID of mobile IAB-MT allocated by the RRC terminating CU.

Proposal 9: For DU migration, the source F1 terminating CU or the RRC terminating CU informs target F1 terminating CU on the value of DSCP and/or flow label for F1-C and non-F1 traffic, before new F1 setup procedure.

Proposal 10: After mobile IAB-DU migration, the BH configuration in the RRC terminating topology is kept and reused to forwarding traffic between the target logical IAB-DU and the target F1 terminating donor, until the co-located mobile IAB-MT is handed over.

Proposal 11: Introducing two new class 2 F1AP procedures, for triggering F1 setup, and reporting the outcome of the F1 setup.

Proposal 12: Before new F1 setup, the default configuration including default BH RLC CH and default UL BAP routing ID will be used for the SCTP handshake and IPsec negotiation between the mobile IAB DU and the target F1 terminating donor CU. And it is up to RAN2 to ensure the default configuration be available for new F1 setup.

NG related Issue

Observation 4: In mobile IAB scenarios, the IP is reachable between the CUs involved. If there is no Xn between the CUs currently, an Xn interface can be established.

Observation 5: Different from the NG based HO, which only requires limited procedures between the AMF and NG-RAN node, the IAB TMM and IAB resource coordination message may be frequently forwarded via NG interface if the MT’s CU and DU’s CU has no Xn interface.

Observation 6: The forward of some Xn interface signaling, e.g. IAB TMM messages, and IAB resource coordination messages, requires the AMF works as a router between MT’s CU and DU’s CU, such router-like function is for user plane, not control plane.

Proposal 13: RAN3 not support the exchange of IAB TMM messages and IAB Resource Coordination messages via NG interface.

Proposal 14: To support the consecutive partial migration and the DU migration, the MT’s CU and DU’s CU should have Xn connection, or able to establish Xn connection if necessary.

Issue of mobile IAB authorization result notification

Observation 7: The release of BH resource in the non-F1 terminating topology does not mean the IAB node is not authorized, and the F1 terminating CU may still attempt initializing the IAB TMM procedure.

Observation 8: If the F1 terminating CU knows the IAB-node is not authorized, it can handover the UEs served by the IAB-node as soon as possible, and release the F1 connection towards the IAB-DU.

Proposal 15: The F1 terminating donor should be aware of the mobile IAB node authorization result from the RRC terminating CU, if the mobile IAB-DU and the mobile IAB-MT connects to different donor CUs.

It is also proposed to:

- Agree the TPs to TS 38.401 and TS 38.473 BL CRs in Annex 1 and 2.

4 Reference

- Chairman notes of 3GPP TSG-RAN WG3 meeting #120, Incheon, Korea, May, 2023

- Chairman notes of 3GPP TSG-RAN WG3 meeting #119b-e, online, April, 2023.

- Chairman notes of 3GPP TSG-RAN WG3 meeting #119, Athens, Greece, March, 2023

- Chairman notes of 3GPP TSG-RAN WG3 meeting #117b-e, online, October, 2022.

- Chairman notes of 3GPP TSG-RAN WG3 meeting #117-e, online, August, 2022.

Annex 1: Text Proposal for TS 38.401

START OF CHANGES

#### 8.17.3.X    Mobile IAB consecutive partial migration procedure

Figure 8.17.3.x-1 shows an example of the consecutive partial migration procedure where the source and target RRC-terminating IAB-donor-CUs are different from the F1-terminating IAB-donor-CU. In this procedure, the mobile IAB-MT is switched from the source RRC-terminating IAB-donor-CU to the target RRC-terminating IAB-donor-CU, while the corresponding mobile IAB-DU retains the F1 connection to the F1-terminating IAB-donor-CU. The F1-terminating IAB-donor can be same as or different from the source/target RRC terminating IAB-donor.

Figure 8.17.3.X-1: Mobile IAB inter-CU consecutive partial migration procedure

- The source RRC-terminating IAB-donor-CU receives the measurement report from the mobile IAB-MT.

- The source RRC-terminating IAB-donor-CU sends an Xn HANDOVER REQUEST message to the target RRC-terminating IAB-donor-CU. This message may include the mobile IAB-node’s TNL address information in the RRC container, and may include the gNB ID of F1-terminating-IAB-donor-CU and the UE XnAP  ID of the mobile IAB-MT assigned by F1-terminating-IAB-donor-CU.

The target RRC-terminating IAB-donor-CU provide configuration to the target parent node, to create the UE context for the mobile IAB-MT and to set up the bearers, which the mobile IAB-MT uses for its signaling, and, optionally, data traffic.

- The target RRC-terminating IAB-donor-CU performs admission control and provides the new RRC configuration as part of the HANDOVER REQUEST ACKNOWLEDGE message. The RRC configuration includes a BAP address for the mobile IAB-node in the target RRC-terminating IAB-donor-CU’s topology, a default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the target path. The RRC configuration may include the new TNL address(es) anchored at the target RRC-terminating IAB-donor-DU for the mobile node.

- The source RRC-terminating IAB-donor-CU sends the received RRCReconfiguration message to the mobile IAB-MT.

- The mobile IAB-MT performs a random access procedure at the target parent node.

- The mobile IAB-MT responds to the target RRC-terminating IAB-donor-CU with an RRCReconfigurationComplete message.

- The target RRC-terminating IAB-donor-CU sends UE CONTEXT RELEASE message to the source RRC-terminating IAB-donor-CU.

- The source RRC-terminating IAB-donor-CU may release BH RLC channels and BAP-sublayer routing entries on the source path between source parent IAB-node of the mobile IAB-node and the source RRC-terminating IAB-donor-DU.

- The source RRC-terminating IAB-donor-CU sends an IAB MT HO NOTIFICATION message to the F1-terminating IAB-donor-CU. This message includes the gNB ID of the target RRC-terminating IAB-donor-CU, and the UE XnAP IDs of the mobile IAB-MT assigned by the target RRC-terminating IAB-donor-CU and assigned by the F1-terminating IAB-donor-CU.

Editors’ NOTE: The message name of step 9 can be updated after stage 3 progress on XnAP specification.

- The F1-C connection between the mobile IAB-node and the F1-terminating IAB-donor-CU are switched to the target path using the new TNL address information of the mobile IAB-node. The mobile IAB-node may report the new TNL address information it wants to use for F1-U traffic to the F1-terminating IAB-donor-CU, via the gNB-DU CONFIGURATION UPDATE message.

- The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the target RRC-terminating IAB-donor-CU, to provide the context of the traffic to be offloaded. The message may include the new DL TNL address information necessary for the target RRC-terminating IAB-donor-CU to configure or modify DL mappings on the target RRC-terminating IAB-donor-DU.

- The target RRC-terminating IAB-donor-CU responds to the F1-terminating IAB-donor-CU with an IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message, to provide the mapping information for the traffic to be offloaded. The message includes the L2 info that is used in the target RRC-terminating IAB-donor-CU’s topology and necessary to configure the mobile IAB-node with the UL mappings of traffic indicated in step 11. The message includes the DSCP/IPv6 Flow Label values used to configure the DL mappings of traffic indicated in step 11.

NEXT CHANGE

#### 8.17.3.Y    Mobile IAB-DU migration procedure

Figure 8.17.3.Y-1 shows an example of the mobile IAB-DU inter-CU migration procedure where the source and target F1-terminating IAB-donor-CUs are different from the RRC-terminating IAB-donor-CU. In this procedure, the mobile IAB-DU is migrating from the source F1-terminating IAB-donor-CU to the target F1-terminating IAB-donor-CU, while the corresponding mobile IAB-MT retains the RRC connection to the RRC-terminating IAB-donor-CU. The RRC-terminating IAB-donor can be same as or different from the source/target F1-terminating IAB-donor.

Figure 8.17.3.Y-1: Mobile IAB-DU inter-CU migration procedure

- The mobile IAB-node decides to perform DU migration based on the OAM configuration, and sends an F1 MOBILE IAB-DU MIGRATION NOTIFICATION message to inform the source F1-terminating IAB-donor-CU by the source logical DU. The F1 message includes the gNB ID of the candidate target F1-terminating IAB-donor-CU.

- The source F1-terminating IAB-donor-CU sends an Xn MOBILE IAB-DU MIGRATION NOTIFICATION message to the target F1-terminating IAB-donor-CU to notify the gNB ID and the IAB-MT’s UE XnAP ID allocated by the RRC-terminating IAB-donor-CU, for the future IAB TRANSPORT MIGRATION MANAGEMENT process using.

- The source F1-terminating IAB-donor-CU sends a NEW F1 SETUP TRIGGER to the source logical DU for triggering the target logical DU to initialize the new F1-C setup procedure towards the target F1-terminating IAB-donor-CU. The F1 message includes the gNB ID of the target F1-terminating IAB-donor-CU.

NOTE: The mobile IAB-DU inter-CU migration can be triggered by the OAM configuration on the mobile IAB-node, or the source F1-terminating IAB-donor-CU. For the former case, there is no step 3; and for the latter case, there is no step 1.

Editor’s Notes: The message name of step 1-3 can be updated according to RAN3 progress on stage 3 design.

- The target logical DU sends an F1 SETUP REQUEST message to the target F1-terminating IAB-donor-CU. The message may include the IP address(es) used by the target logical DU, which may be reused by the source logical DU or acquired from the RRC-terminating IAB-donor-CU via RRC signalling.

- The target F1-terminating IAB-donor-CU responds the target logical DU an F1 SETUP RESPONSE message.

- The source logical DU reports to the source F1-terminating IAB-donor-CU the target DU’s F1-C is established, including the cell mapping of the cells under the source logical DU and the target logical DU.

Editor Note: FFS on whether the source F1-terminating IAB-donor-CU can assign UE’s target cell according to the source cell under the source logical DU and the cell mapping, without the UE’s measurement report.

- The source F1-terminating IAB-donor-CU sends Xn HANDOVER REQUEST messages for the UEs served by the source logical DU.

- The target F1-terminating IAB-donor-CU sends an Xn IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the RRC-terminating IAB-donor-CU, for requesting the inter-topology transmission for the UE traffic.

- The RRC-terminating IAB-donor-CU configures the backhaul resources on the mobile IAB’s parent nodes. It is up to the RRC-terminating IAB-donor-CU’s implementation whether the backhaul resources for the target logical DU’s F1 interface is newly allocated or reusing the backhaul resources for the source logical DU’s F1 interface.

- The RRC-terminating IAB-donor-CU sends an Xn IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message to the target F1-terminating IAB-donor-CU with the BAP information of the RRC-terminating IAB-donor-CU’s topology.

- The target F1-terminating IAB-donor-CU configures the target logical DU’s UL mapping by the F1 UE CONTEXT SETUP REQUEST message.

- The target logical DU responds the target F1-terminating IAB-donor-CU F1 UE CONTEXT SETUP RESPONSE message.

- The target F1-terminating IAB-donor-CU responds the source F1-terminating IAB-donor-CU the Xn HANDOVER RESPONSE messages for the UEs served by the source logical DU.

- UE handover execution. The UEs served by the source logical DU switch to the target F1-terminating IAB-donor-CU. After all the UEs’ handover completion, the source F1-terminating IAB-donor-CU may initiate the removal of the F1 interface of the source logical DU.

END OF CHANGES

Annex 2: Text Proposal for TS 38.473

START OF CHANGES

8.10	IAB Procedures

8.10.X	New F1 Setup Trigger

8.10.X.1	General

The purpose of the New F1 Setup Trigger procedure is to set up a new F1 interface between target IAB-donor-CU and target logical IAB-DU. This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for mobile IAB-nodes, where the term "gNB-DU" applies to mobile IAB-DU (source logical DU), and the term "gNB-CU" applies to source F1-terminating IAB-donor-CU.

8.10.X.2	Successful Operation

Figure 8.10.X.2-1: New F1 Trigger procedure: Successful Operation

The gNB-CU initiates the procedure by sending the NEW F1 SETUP TRIGGER message to the gNB-DU. The gNB-DU initiates the F1 setup to the gNB-CU indicated by the Target gNB-CU ID which is included in the NEW F1 SETUP TRIGGER message.

8.10.X.3	Abnormal Conditions

Not applicable.

NEXT CHANGE

8.10.Y	New F1 Setup Complete

8.10.Y.1	General

The purpose of the New F1 Setup Complete procedure is to report the completion of the new F1 interface setup between the target IAB-donor-CU and target logical IAB-DU. This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for mobile IAB-nodes, where the term "gNB-DU" applies to mobile IAB-DU (source logical DU), and the term "gNB-CU" applies to source F1-terminating IAB-donor-CU.

8.10.Y.2	Successful Operation

Figure 8.10.X.2-1: New F1 Trigger procedure: Successful Operation

The gNB-DU initiates the procedure by sending the NEW F1 SETUP COMPLETE message to the gNB-CU. The Activated Cells Mapping List is included to provide the information of the mapping of the cells activated by the target F1 terminating CU and the source F1 terminating CU.

8.10.Y.3	Abnormal Conditions

Not applicable.

NEXT CHANGE

9.2.9	IAB messages

UNCHANGED PART OMITTED

9.2.9.X1	NEW F1 SETUP TRIGGER

This message is sent by the gNB-CU to trigger the new F1 setup to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Target gNB ID | M |  | Global gNB ID 9.3.1.Y |  | YES | reject |
| Target gNB IP address | O |  | Transport Layer Address 9.3.2.3 |  | YES | ignore |
| Target SeGW IP address | O |  | Transport Layer Address 9.3.2.3 |  | YES | ignore |

9.2.9.X2	NEW F1 SETUP COMPLETE

This message is sent by the gNB-DU to provide the information of the mapping of the cells activated by the target F1 terminating CU and the source F1 terminating CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Activated Cells Mapping List |  | 0..1 |  |  | YES | ignore |
| >Activated Cells List Mapping Item IEs |  | 1.. <maxCellingNBDU> |  | List of activated cells | EACH | ignore |
| >> NR CGI for target logical DU | M |  | NR CGI 9.3.1.12 | The IDs of the activated cells under the target logical DU | - |  |
| >> NR CGI for source logical DU | M |  | NR CGI 9.3.1.12 | The IDs of the activated cells under the source logical DU | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

NEXT CHANGE

9.3.1	Radio Network Layer Related IEs

9.3.1.Y	Global gNB ID

This IE is used to globally identify a gNB (see TS 38.300 [6]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.1.14 |  |
| CHOICE gNB ID | M |  |  |  |
| >gNB ID |  |  |  |  |
| >>gNB ID | M |  | BIT STRING (SIZE(22..32)) | Equal to the leftmost bits of the NR Cell Identity IE contained in the NR CGI IE of each cell served by the gNB. |

END OF CHANGES