3GPP TSG-RAN WG3 Meeting #121	R3-234165

21 – 25 August 2023, Toulouse, France

Title: 	Discussion on L1/L2 based inter-cell mobility

Source: 	Lenovo

Agenda item:	14.2

Document Type:	Discussion and Approval

1. Introduction

The signalling support for LTM was discussed at the RAN3#120 with the following achievements. The summary of offline discussion could be found in [1].

RAN3 agrees option2 and supports for multiple messages for LTM candidate cell configuration. Proposal 3.6-1: For LTM without UP change case, propose to turn the following WF to an agreement: In case of CP UP separation, once CUCP receives LTM cell switch signaling from (source)DU , CU CP initiates E1 bearer context modification to the CU UP including DL tunnel ID per DRB for target cell, for data transmission. Proposal 3.6-2: For LTM with UP change, once the CU-CP receives LTM cell switch signaling from (source) DU, the CU-CP initiates E1 bearer context modification to the target CU UP including DL tunnel ID per DRB for target cell for data transmission. Notification of LTM triggering to gNB-CUs: WA: option 1 is adopted with only class 2 procedure. If more information to source DU in response message is needed, we go for option 2. E1 aspects for inter-DU LTM: For LTM with UP change, the CU-CP initiates E1 bearer context modification to the source CU-UP for retrieving the latest PDCP status at the source CU-UP and exchanging the data forwarding information to target CU-UP. For LTM with UP change after detecting the UE has accessed to the target cell, the CU-CP initializes Path switch procedure towards the core network. TA acquisition: CU can request RACH resources for early TA acquisition together with the LTM candidate cell configuration to a Candidate DU in the UE Context Setup Request or UE Context Modification Request messages (for inter-DU LMT and intra-DU LTM respectively). If the Candidate DU accepts the RACH resource request for early TA acquisition, the Candidate DU responds the CU with RACH configuration in the UE Context Setup Response or UE Context Modification Response messages. DDDS: For intra-DU LTM, DDDS from gNB-DU to CU-UP is not needed for those DRBs RLC is not re-established. For both intra-DU and inter-DU LTM, target gNB-DU sends initial DDDS using the new UL TEID, if new UL TEID being assigned, to CU-UP after target gNB-DU detects the UE access.

In this contribution, we will further discuss the remaining issues of LTM related to RAN3 impact.

2. Discussion

With the goal to reduce the mobility latency, it is beneficial for the CU to know that the cell switch has been triggered to the UE by the DU (or the source DU in the inter-DU scenario). At the RAN3#119 meeting, it was agreed the CU will start data transmission after LTM cell switch signalling from DU including the target cell ID. However, it is still FFS whether a new message or legacy message (e.g., UE Context Modification Required) is used.

At the RAN3#120 meeting, the group made the following working assumption:

- WA: option 1 is adopted with only class 2 procedure. If more information to source DU in response message is needed, we go for option 2.

Part A: Signalling design

From our point of view, defining a new message is preferrable. The new message is only used to inform that the cell switch has been triggered by the DU/source DU but not about changing the UE Context. Therefore, no more information is needed from the CU to the DU/source DU in a response message.

Proposal 1: Confirm the LTM Cell Change Notification message sent from the DU/source DU to the CU is used to indicate that the LTM has been triggered.

In the inter-DU LTM scenario, there is a need that the CU informs the source DU about the candidate target cell(s) at other target DU(s) which are successfully configured for the UE. This is because that it is the responsibility of the source DU to trigger the cell change to the UE.

At the RAN3#118 meeting, the DL RRC Message Transfer message was introduced to carry the RRCReconfiguration message with the LTM configuration for the UE, as well as the LTM candidate target cell(s) at other target DU(s) for the source DU. The benefit is that this option allows the CU to signal the LTM candidate target cell(s) information to both the UE and the source DU.

From our view, this option is not an issue for the intra-DU LTM scenario, because the source cell and candidate target cell(s) are located within the same DU. However, as described in TS 38.473, the purpose of the DL RRC Message Transfer procedure is to transfer an RRC message, which is transparent to the DU. Therefore, reusing the DL RRC Message Transfer message to signal the LTM candidate target cell(s) information at the target DU(s) to the source DU is not suitable for the inter-DU LTM scenario since it is against the current RAN3 framework.

Instead, it is possible to reuse the UE Context Modification Request message to signal the LTM candidate target cell(s) information to both the UE and the source DU for the intra-DU LTM and the inter-DU LTM scenarios. First, the UE Context Modification Request message includes the RRC-Container IE to transfer the RRCReconfiguration message with the LTM candidate target cell(s) information for the UE. Second, the UE Context Modification Request message includes the LTM candidate target cell(s) information at the target DU(s) for the source DU. Therefore, this option shows the benefit of using the same message for both UE and the source DU, which is not against the current RAN3 framework.

Proposal 2: For inter-DU LTM, the CU uses the UE Context Modification Request message to the source DU to indicate the target candidate cell(s) at other candidate DU(s).

Part B: HO collision

Since the LTM is triggered by the DU based on the L1 measurements while the traditional L3 based inter-cell mobility is triggered by the CU based on the L3 measurements, it is possible that both LTM command and L3 handover command are sent to UE if there is no CU-DU coordination on the L1/L2 handover and the L3 handover decision, which results in the handover collision. For example, if the LTM command is sent to the UE firstly and then the L3 handover command is sent to the UE during the LTM execution period (e.g., leaving the source cell but not connecting target cell), the L3 handover command cannot be performed rightly because of serving cell change. Therefore, the resource reserved by the target cell for the UE L3 handover are wasted.

At the RAN3#120 meeting, some proposals were discussed, e.g., priority-based solution, up to network implementation. However, before taking any decision, we prefer to discuss the use case of HO collision between the LTM and L3 HO first.

- Case 1: The DU receives the L3 HO command while the DU is about to trigger LTM switching the UE to a different target cell. In this case, the DU may need to decide whether to (a) send L3 HO command to the UE; (b) send LTM command to the UE and fail the L3 HO.

- Case 2: The CU receives the LTM notification from the DU after the CU has started preparing (inter-CU) L3 HO but has not triggered L3 HO. In this case, the CU may need to cancel the prepared L3 HO over Xn interface, or the CU may coordinate with the DU before the L3 HO preparation to avoid such collision.

- Case 3: The DU receives the L3 HO command while the DU has already triggered the LTM. In this case, the UE may have already switched to another cell, and the L3 HO is no longer applicable. Therefore, the DU may fail the L3 HO with necessary indication about the cause.

In addition, at the RAN2#121bis meeting, RAN2 already agreed [3]:

At RLF or LTM execution failure (for MCG), RAN2 intend to support fast recovery to a candidate cell by LTM execution. While configured with LTM candidate cells, the UE can also execute any L3 handover command sent by the network. R2 assumes that is could be up to the network to avoid any issue due to the race condition between LTM execution and RRC Reconfiguration (e.g. L3 HO cmd), e.g. avoid sending LTM switch cmd and L3 HO cmd in the same TB.

Based on the agreements, it is the network responsibility to avoid the race condition between LTM and L3 HO. Therefore, the priority-based mechanism should not be used.

Proposal 3: RAN3 considers the following three cases for HO collision discussion, and discusses possible solutions to avoid sending LTM and L3 HO command to UE simultaneously:

- Case 1: The DU receives the L3 HO command while the DU is about to trigger LTM switching the UE to a different target cell.

- Case 2: The CU receives the LTM notification from the DU after the CU has started preparing (inter-CU) L3 HO but has not triggered L3 HO.

- Case 3: The DU receives the L3 HO command while the DU has already triggered the LTM. In this case, the UE may have already switched to another cell, and the L3 HO is no longer applicable.

Part C: Candidate cell configuration index

There is also one specific issue: in case multiple candidate cell configurations are prepared for the UE, a unique identifier should be introduced to indicate which candidate cell configuration is referred to. In addition, RAN2 agrees the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. Since the candidate cell configurations could be from multiple candidate DUs, how does the unique candidate configuration index is generated to avoid the index conflict or index confusion? How do the source DU and the UE have common understanding of the unique candidate configuration index?

- Option 1: The CU assigns the unique index for each candidate cell configuration and sends the unique index associated with the candidate cell confiuration to the source DU.

- Option 2: The CU sends the candidate cell configurations obtained from multiple candidate DUs to the source DU. After that, the source DU assigns the unique index for each candidate cell configuration, and sends the unique index associated with the candidate cell configuration to the CU. The unique index associated with the candidate cell configuration will be sent to the UE via the RRCReconfiguration message.

- Option 3: The CU assigns an index range for each candidate DU, where the candidate DU assigns the unique index for each candidate cell configuration within the candidate DU. After that, the candidate DU sends the unique index associated with the candidate cell configuration to the CU. The unique index associated with the candidate cell configuration will be sent to the UE and the source DU.

Proposal 4: RAN3 should discuss the unique index used to indicate which candidate cell configuration is referred to.

Part D: TA acquisition

At the RAN3#119bis meeting, the group agreed to discuss how to specify how the TA value(s) is (are) transmitted from the candidate DU to the source DU through the CU. In addition, two options were proposed for discussion [2]:

- Option 1: The candidate DUs transmit the TA values of candidate cells to serving DU before LTM triggering and the serving DU maintains the TA values, e.g., the candidate DUs shall transmit the TA values to the serving DU by F1 interface through the CU.

- Option 2: After LTM is triggered, the serving DU requests the target DU to transmit the latest TA value which is maintained by itself before sending LTM command, e.g., the target DU shall transmit the requested TA value to the serving DU by F1 interface through the CU.

Since RAN2 still not reply to the LS, both options are not excluded from RAN3 point of view. However, no matter which option is adopted, we can agree that it’s the CU to request the candidate DU to provide the TA for RACH-less LTM, and then sends the TA to the source DU.

Proposal 5: The CU requests the candidate DU to provide the TA for RACH-less LTM, and sends the TA to the source DU.

Regarding early TA acquisition without RAR, after UE receives the PDCCH order from source cell, UE transmits preamble to the corresponding candidate cell based on the RACH resource for early TA acquisition. Once the candidate cell receives the preamble, the candidate cell will calculate TA value and transmit early TA value to source cell/DU via CU. If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell can determine the failure of early TA acquisition. The candidate cell/DU transmits the message and indicates this to the source cell (DU). This will enable the source cell to determine if it would like the UE to re-attempt Early TA acquisition and when so, source cell may request dedicated PRACH resources from the candidate cell (DU).

Proposal 6: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU).

Source cell (DU) may store early TA value list for some candidate cells. Source cell will transmit LTM command MAC CE to UE once the source cell determines to switch to a candidate cell which may be located in another DU. After candidate cell receives the RRC reconfiguration complete message from UE, the candidate cell may transmit indication to source cell. In subsequent-LTM, UE will continue keeping the LTM candidate configuration after completing cell switching. If source cell (DU) does not transfer the stored early TA value list, UE will be expected to get early TA value again based on RACH. Therefore, source cell may provide the early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Proposal 7: Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Part E: Beam alignment

In RACH-less procedure, beam alignment between the UE and the target cell should be completed for delivering the UL message to let the target cell know that the UE is coming. At the RAN2#122 meeting, RAN2 made the following agreements [2]:

- Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. - Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization.

According to the RAN2 agreements, the selected beam should be sent from the source DU to the candidate DU for the inter-DU LTM. RAN3 should discuss how to carry the selected beam from the source DU to the candidate DU through the CU.

- Option 1: The selected beam is included as a separate IE in the DU to CU RRC Information in the UE Context Modification Required message from the source DU to the CU, and then included in the CU to DU RRC Information in the UE Context Modification Request message from the CU to the candidate DU.

- Option 2: The selected is included outside the DU to CU RRC Information from the source DU to the CU, and outside the CU to DU RRC Information from the CU to the candidate DU.

From our point of view, the option 1 is better because it is straightforward and has little impact on the current specification.

Proposal 8: For RACH-less LTM, the selected beam is included as a separate IE in the DU to CU RRC Information in the UE Context Modification Required message from the source DU to the CU, and in the CU to DU RRC Information in the UE Context Modification Request message from the CU to the target DU.

Part F: Subsequent LTM

It was agreed in RAN2 that subsequent LTM is supported. Namely, after cell switching, the UE continues to keep candidate cell configuration for LTM. Specifically, after the UE switches from source cell#0 to a candidate cell#1 belonging to candidate DU#1, UE continues to keep a candidate cell#2 belonging to candidate DU#2. According to LTM, the candidate cell#1 (new source cell) should transmit LTM command to trigger cell switching to the candidate cell#2. Therefore, the candidate DU (candidate DU#1) with the new source cell (candidate cell#1) should be aware of candidate cell(s) the UE has stored.

Proposal 9: The candidate DU with the new source cell should be aware of the candidate cell(s) configuration the UE has stored.

Part G: L1 measurement

In the step 9 in the current BL CR, the lower layer measurement report is used to trigger the inter-cell mobility execution decision. However, it is not clear what is the lower layer measurement report. Since RAN1 and RAN2 already agreed to use the L1 measurement report, we propose to use the L1 measurement report for better understanding.

Proposal 10: L1 measurement report is used to trigger the inter-cell mobility execution decision.

For L1 measurements for LTM, according to the RAN2 agreements, the location of RS configuration for SSB-based measurements of candidate cells is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells. In addition, the reporting configuration is placed inside the ServingCellConfig of current serving cell(s).

At the RAN2#122 meeting, RAN2 made the following agreements [3]:

- For L1 measurements for LTM - The RS configuration is provided to the UE per LTM candidate cell. - RAN2 assumes that Each candidate DU needs to know the RS configuration of each candidate DUs in order to provide the LTM candidate configuration. - RAN2 assumes that The CU transmits to each C-DU the RS configuration of S-DU (if this is an LTM candidate cell) and/or other C-DUs, to generate the corresponding L1 configuration for LTM. - RAN2 assumes C-DU generates the RS configuration and send to the CU. The CU transmits to the Source DU the RS configuration per LTM candidate cell and the associated LTM candidate (when the CU receives LTM candidate configuration(s) from the C-DU). It is up to RAN3 whether the RS configuration is sent before (or at the same time of) the C-DU creates the LTM candidate configuration (and whether is semi-statis or UE associated). - The RS configuration and/or CSI resource configuration for measuring LTM candidate cells is included in the LTM-Config IE and is a separate configuration, e.g. outside of the LTM candidate configuration. - CSI reports for LTM candidates (neighbour cell reports for the purpose of LTM cell switch) are configured by the serving cell in an IE that is like CSI-ReportConfig for LTM within the ServingCellConfig since this is the cell in which the report is to be transmitted. - RAN2 assumes the following about CSI measurement reporting for LTM (final decision up to RAN1): - a. UE reports all measured LTM candidate cells in a single report; or - b. UE reports one or a subset of measured LTM candidate cell(s) in a report.

From the source DU side, the source DU needs to know the L1 measurement RS configuration to generate the right report configuration under the serving cell configuration. Since the CU has all the L1 measurement RS configurations of candidate cells, it is reasonable that the CU provides the L1 measurement RS configuration to the source DU. RAN3 should design the F1 signalling to provide the L1 measurement RS configuration of candidate cells.

Proposal 11: The source DU should obtain the L1 measurement RS configuration of candidate cells at candidate DUs from the CU.

From the candidate DU side, to support subsequent LTM, the candidate DU needs to know the L1 measurement RS configuration of candidate cells at other candidate DUs, to generate the right report configuration. Since the CU has all the L1 measurement RS configurations of candidate cells at other candidate DUs, it is reasonable that the CU provides the L1 measurement RS configuration of candidate cells at other candidate DUs to the candidate DU. RAN3 should design the F1 signalling to provide the L1 measurement RS configuration of candidate cells at other candidate DUs.

Proposal 12: The candidate DU should obtain the L1 measurement RS configuration of candidate cells at other candidate DUs from the CU.

In addition, since the cell at the source DU would be a candidate cell after the UE leaving the source DU and connecting to the candidate DU, the candidate DU needs to know the L1 measurement RS configuration of candidate cells at source DU, to generate the right report configuration. RAN3 should design the F1 signalling to provide the L1 measurement RS configuration of candidate cells at source DU.

Proposal 13: The candidate DU may obtain the L1 measurement RS configuration of candidate cells at source DU from the CU.

Part H: TCI state

In order to minimize the time needed to acquire the target beam after receiving the cell switch command, the network can provide the UE the candidate cell beam indication in the form of TCI states associated with the candidate cell.

During the preparation phase, a set/list of TCI states associated with the candidate cells should be configured to the UE. At the RAN2#121bis meeting, RAN2 made the following agreements:

- RAN2 assumes that the location of configurations of TCI states for the candidate cells (used before/at cell switch) is external to the ServingCellConfig(s) of current serving cells and external to the configuration of the LTM candidate cells (same location as RS configuration).

For TCI state activation for candidate cells, RAN1 made the following agreements [4]:

C. TCI state activation RAN1 has made the following agreement in RAN1#113: Agreement - For TCI state activation for candidate cell(s) before the cell switch command, - MAC CE is used and the details of MAC-CE for TCI state activation for LTM is up to RAN2 - Further study if PDCCH order for candidate cell(s) can be used RAN1 respectfully asks RAN2 to perform necessary specification work along with this agreement. E. RAN1#113 agreements, working assumptions and conclusions Agreement A UE can be indicated and activated a single joint TCI state or a pair of UL/DL TCI state in the cell switch command.

From the source DU side, the source DU needs to know the TCI states of the candidate cells. Since the CU has all the TCI states of candidate cells, it is reasonable that the CU provides the TCI states of candidate cells to the source DU. RAN3 should design the F1 signalling to provide the TCI states of candidate cells. This requirement is also confirmed by the RAN1 in [5]:

- Beam indication of target cell(s) RAN1 has made the following agreement in RAN1#112bis-e - Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported), - Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE - Alt. 2: By indicating Rel-17 TCI state index This agreement implies that the source cell must be provided with information so that the source cell can send a Rel-17 TCI state index of the target/candidate cell(s) to the UE even when source cell and target/candidate cell(s) belong to different DUs.

Therefore, we propose:

Proposal 14: The source DU should obtain the TCI states of candidate cells of candidate DUs from the CU.

From the candidate DU side, to support subsequent LTM, the candidate DU needs to know the TCI states of candidate cells at other candidate DUs. Since the CU has all the TCI states of candidate cells at other candidate DUs, it is reasonable that the CU provides the TCI states of candidate cells at other candidate DUs to the candidate DU. RAN3 should design the F1 signalling to provide the TCI states of candidate cells at other candidate DUs.

Proposal 15: The candidate DU should obtain the TCI states of candidate cells of other candidate DUs from the CU.

In addition, since the cell at the source DU would be a candidate cell after the UE leaving the source DU and connecting to the candidate DU, the candidate DU needs to know the TCI states of candidate cells at source DU. RAN3 should design the F1 signalling to provide the TCI states of candidate cells at source DU.

Proposal 16: The candidate DU may obtain the TCI states of candidate cells of source DU from the CU.

Part I: Reference configuration

For the delta configuration, RAN2 made the following agreements:

- Whether the Reference configuration is a complete configuration or not is up to the network implementation. - Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. - The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

From the RAN2 agreements, the reference configuration is managed separately and explicitly signalled, i.e., not automatically derived from any other configuration, e.g., current configuration. The reference configuration may be generated by the source DU or the CU, without the RAN2 final decision. However, no matter which node generates the reference configuration, the reference configuration should be transferred from the CU to the candidate DU. The candidate DU may take the reference configuration into account when generating the candidate cell configuration.

Since RAN3 agreed that the UE Context Setup procedure is reused for the LTM preparation for the inter-DU LTM, it is reasonable that the CU provides the reference configuration to the candidate DU using the UE Context Setup Request message.

Proposal 17: The CU provides a reference configuration to the candidate DU in the UE Context Setup Request message.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals.

Signalling design

Proposal 1: Confirm the LTM Cell Change Notification message sent from the DU/source DU to the CU is used to indicate that the LTM has been triggered.

Proposal 2: For inter-DU LTM, the CU uses the UE Context Modification Request message to the source DU to indicate the target candidate cell(s) at other candidate DU(s).

HO collision

Proposal 3: RAN3 considers the following three cases for HO collision discussion, and discusses possible solutions to avoid sending LTM and L3 HO command to UE simultaneously:

- Case 1: The DU receives the L3 HO command while the DU is about to trigger LTM switching the UE to a different target cell.

- Case 2: The CU receives the LTM notification from the DU after the CU has started preparing (inter-CU) L3 HO but has not triggered L3 HO.

- Case 3: The DU receives the L3 HO command while the DU has already triggered the LTM. In this case, the UE may have already switched to another cell, and the L3 HO is no longer applicable.

Candidate cell configuration index

Proposal 4: RAN3 should discuss the unique index used to indicate which candidate cell configuration is referred to.

TA acquisition

Proposal 5: The CU requests the candidate DU to provide the TA for RACH-less LTM, and sends the TA to the source DU.

Proposal 6: If the candidate cell (DU) does not receive preamble in the configured RO, the candidate cell determines the failure of TA acquisition and indicates this to the source cell (DU).

Proposal 7: Source cell may provide the stored early TA value(s) of candidate cell(s) to target cell when receiving successful handover confirmation from it (i.e., upon UE’s arrival on the target side).

Beam alignment

Proposal 8: For RACH-less LTM, the selected beam is included as a separate IE in the DU to CU RRC Information in the UE Context Modification Required message from the source DU to the CU, and in the CU to DU RRC Information in the UE Context Modification Request message from the CU to the target DU.

Subsequent LTM

Proposal 9: The candidate DU with the new source cell should be aware of the candidate cell(s) configuration the UE has stored.

L1 measurement

Proposal 10: L1 measurement report is used to trigger the inter-cell mobility execution decision.

Proposal 11: The source DU should obtain the L1 measurement RS configuration of candidate cells at candidate DUs from the CU.

Proposal 12: The candidate DU should obtain the L1 measurement RS configuration of candidate cells at other candidate DUs from the CU.

Proposal 13: The candidate DU may obtain the L1 measurement RS configuration of candidate cells at source DU from the CU.

TCI state

Proposal 14: The source DU should obtain the TCI states of candidate cells of candidate DUs from the CU.

Proposal 15: The candidate DU should obtain the TCI states of candidate cells of other candidate DUs from the CU.

Proposal 16: The candidate DU may obtain the TCI states of candidate cells of source DU from the CU.

Reference configuration

Proposal 17: The CU provides a reference configuration to the candidate DU in the UE Context Setup Request message.

4. Reference

- R3-233357, Summary of offline discussion on CB: # MobilityEnh1_L1L2Mobility, Huawei (moderator).

- R3-233719, LS on early TA and RACH-less, RAN2.

- R3-233720, LS on L1 measurements for LTM, RAN2.

- R3-233709, LS on beam application time, contents of cell switch command, TCI state activation and UE based TA measurement for LTM, RAN1.

- R3-232569, LS on beam indication of target cell(s) and time gap between a PDCCH order and the corresponding PRACH transmission for LTM, RAN1.