3GPP TSG-RAN WG3 Meeting #121	R3-234168

Toulouse, France, 21st – 25th August, 2023

Agenda Item:	13.1

Source: 	Lenovo

Title: 	Discussion on IAB-node de-authorization handling

Document for:	Discussion and decision

- Introduction

In RAN3 119, SA2 had sent an LS [1] to RAN3 and provided some questions to RAN3.

SA2 is currently discussing the MBSR (i.e., mobile IAB-node) authorization state changed from authorized to non-authorized (e.g., due to subscription data update for the MBSR). The AMF notifies the Mobile IAB-node authorization status via the N2 message. SA2 noticed that when the AMF indicates the mobile IAB node is authorized, the mobile IAB-node integrates into the network. The donor-gNB communicates with IAB-MT via RRC messages to setup the backhaul, F1-C connection (i.e., as specified in TS 38.401 clause 8.12). However, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, it’s not clear what is the gNB behaviour. SA2 would like to check whether this is a correct understanding: - The gNB attempts to perform handover of all the UEs served by the MBSR - When the step 1 completes, the gNB releases the F1-C Q1: is the understanding above correct? Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)?

Based on the inputs from SA2, in this paper, issues for mIAB-node de-authorization are studied further including the scenario where mIAB-MT and mIAB-DU connect to different IAB-donor-CUs.

- Discussion

As specified for IAB-node in TS 38.413 [2], the IAB-donor shall initiate actions to ensure that the IAB-node will not serve the UE(s).

38.413: ### 8.3.4 UE Context Modification ...... Upon receipt of the UE CONTEXT MODIFICATION REQUEST message the NG-RAN node shall - if supported, store the received IAB Authorization information in the UE context. If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve the UE(s).

In the same way, once the authorization state changed from authorized to non-authorized for a mIAB-node, the IAB-donor of the mIAB-node shall also initiate actions to ensure that the mIAB-node will not serve the UE(s). In this case, the IAB-donor hands over all the connected UE to other cells, or releases the UEs, and stop accepting incoming handover UE or connection UE to the mIAB-node.

Proposal 1: In case authorization state changed from authorized to non-authorized for a mIAB-node, IAB-donor hands over all the connected UE to other cells, or releases the UEs, and stop accepting incoming handover UE or connection UE to the mIAB-node.

After operations in proposal 1, the mIAB-node has no served UE(s). And whether to release the F1 interface between mIAB-node and the IAB-donor-CU need to be studied. Once the mIAB-node is required to be deregistered from the AMF, the IAB-donor-CU shall release F1 interface between mIAB-node and IAB-donor-CU. And in this case, since both mIAB-MT and mIAB-DU are released, there is no problem for the issue as raised by SA2 in Q2.

However, if the AMF only updates the authorization state from authorized to non-authorized temporary, the F1 interface may be maintained for the mIAB-node, which can save the latency for F1 setup and SCTP association once the mIAB-node is re-authorized.

Proposal 2: F1 interface between mIAB-node and IAB-donor is released when the AMF uses the Deregistration procedure to update the authorization state changes from authorized to not authorized.

Proposal 3: F1 interface between mIAB-node and IAB-donor may be maintained when the AMF uses the UE Configuration Updated procedure to update the authorization state changes from authorized to not authorized.

As agreed in RAN side, IAB-MT and IAB-DU can connect to different IAB-donor-CUs both for IAB-node and mIAB-node. In addition, the authorization state is informed from the AMF to IAB-donor-CU and the IAB-donor-CU stores it in the UE context, which means current only RRC-terminating IAB-donor-CU is aware of the authorization state of the IAB-node. However, once the authorization state from authorized to not authorized, it’s the F1-terminating IAB-donor-CU to initiate actions to ensure that the IAB-node will not serve the UE. Therefore, if the IAB-MT and IAB-DU connect to different IAB-donor-CUs, the F1-terminating IAB-donor-CU needs to be informed about authorization state change of IAB-node.

Proposal 4: If IAB-MT and IAB-DU connect to different IAB-donor-CUs, the F1-terminating IAB-donor-CU needs to be informed about authorization state change of IAB-node.

# Conclusion

This contribution aims to discuss IAB-node de-authorization issues. And following observations and proposals are concluded.

Proposal 1: In case authorization state changed from authorized to non-authorized for a mIAB-node, IAB-donor hands over all the connected UE to other cells, or releases the UEs, and stop accepting incoming handover UE or connection UE to the mIAB-node.

Proposal 2: F1 interface between mIAB-node and IAB-donor is released when the AMF uses the Deregistration procedure to update the authorization state changes from authorized to not authorized.

Proposal 3: F1 interface between mIAB-node and IAB-donor may be maintained when the AMF uses the UE Configuration Updated procedure to update the authorization state changes from authorized to not authorized.

Proposal 4: If IAB-MT and IAB-DU connect to different IAB-donor-CUs, the F1-terminating IAB-donor-CU needs to be informed about authorization state change of IAB-node.

# Reference

- S2-2308158. LS On IAB-node De-authorization handling. SA2.

- TS 38.413. NG Application Protocol (NGAP) (Release 17).