3GPP TSG-RAN WG3 Meeting #121	R3-234181

Toulouse, France, 21st – 25th August 2023

Agenda item:	 11.4

Source:		Lenovo

Title:		Discussion on QoE measurement during intra-5GC inter-RAT handover

Document for:		Discussion and Decision

- Introduction

- In RAN3#119bis-e meeting, it was agreed and FFSs:

WA: For HO from LTE/5GC to NR, there is no impacts to RAN3.

For HO from NR to LTE/5GC case, identify the impact on RAN3

- In RAN3#120 meeting, it was agreed that:

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

- This contribution discusses the potential solutions to support the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover.

- Discussion

- Solution for HO from NR to LTE/5GC

- For a same service type (e.g., service type = streaming), the OAM system or CN provides more than one QoE measurements configurations (i.e. application layer measurements). In this example, two QoE measurement configurations are provided and each one is uniquely identified by a QoE reference ID (i.e., QoE reference ID =1 and QoE reference ID =2). The OAM system or CN provides an inter-RAT QoE continuity indication of one of QoE measurements which is used to indicate which QoE measurement (i.e., application layer measurement) is continued and kept during inter-RAT handover from gNB to ng-eNB.

- The gNB provides UE an Application layer configuration (i.e. AppLayerMeasConfig IE) in the RRCReconfiguration message (there can be multiple configurations in the same message). For the QoE measurement, the gNB provides the inter-RAT QoE continuity indication to the UE as received from OAM system or CN. In this example, the QoE measurement with measConfigAppLayerId = 1 is provided the inter-RAT QoE continuity indication.

- Figure 1. Procedures of QoE measurement continuity for HO from NR to LTE/5GC

- The gNB decides to perform intra-system inter-RAT handover to the target ng-eNB. The source gNB only includes the QoE measurement configuration that is provided ‘IRAT continuity indication’ to the ng-eNB in the Handover Request message. In this example, the source gNB only provides the QoE measurement configuration with QoE reference ID =1 to the target ng-eNB. The source gNB also generates and provides LTE RRC QoE configuration of the QoE measurement configuration in the source eNB to target ng-eNB transparent container to the target ng-eNB. The LTE RRC QoE configuration is encoded in LTE RRC ASN.1 format.

- When receives the QoE measurement configuration, the target eNB generates or updates the LTE RRC QoE configuration and send it to the source gNB in the Handover Response message, and source gNB forwards it to the UE in the MobilityFromNRCommand message. In the LTE RRC QoE configuration, only service type = streaming is provided without any measConfigAppLayerId.

- When UE receives the LTE QoE configuration in the MobilityFromNRCommand message, the UE continues the QoE measurement with provided ‘IRAT continuity indication’ of the service type (which is provided in the step 2) and release the other QoE configurations of the service type. In this example, the UE keeps and continues the QoE measurement with measConfigAppLayerId = 1.

- The solution has no impact on LTE specification.

To support QoE measurement continuity for HO from NR to LTE/5GC without LTE spec impact:

- [Impact on TS 38.413] The OAM system or CN provides an inter-RAT QoE continuity indication of one of QoE measurements which is used to indicate which QoE measurement (i.e., application layer measurement) is continued and kept during inter-RAT handover from gNB to ng-eNB;

- [Impact on TS 38.331] For the QoE measurement, the gNB provides the inter-RAT QoE continuity indication to the UE as received from OAM system or CN.

- [Potential Impact on stage 2] The source gNB generates and provides LTE RRC QoE configuration of the QoE measurement configuration in the source eNB to target ng-eNB transparent container to the target ng-eNB. The LTE RRC QoE configuration is encoded in LTE RRC ASN.1 format.

- [Impact on TS 38.331] When UE receives the LTE QoE configuration in the MobilityFromNRCommand message, the UE continues the QoE measurement with provided ‘IRAT continuity indication’ of the service type and release the other QoE configurations of the service type.

- Solution for HO from LTE/5GC to NR

- For a same service type (e.g., service type = streaming), the OAM system or CN provides one QoE measurements configuration of the service type to the ng-eNB.

- Figure 2. Procedures of QoE measurement continuity for HO from LTE/5GC to NR

- The ng-eNB provides UE an application layer configuration (i.e. measConfigAppLayer IE) in the RRCConnectionReconfiguration message and corresponding service type is indicated to the UE.

- The ng-eNB decides to perform intra-system inter-RAT handover to the target gNB. The source ng-eNB provides QoE Configurations to the target gNB, e.g., provides the UE Application Layer Measurement Information IE of the QMC Configuration Information IE in XnAP Handover Request message. The source ng-eNB also provides an indication that whether the QoE measurement is already configured to UE in LTE.

- When receives the QoE measurement configurations, the target gNB identifies the QoE measurement is already configured to UE in LTE. The gNB allocates a measConfigAppLayerID to the on-going QoE measurement, and sends an on-going QoE measurement association information in handover command, to indicate that UE associates the on-going QoE measurement. The on-going QoE measurement association information may be implicitly indicated by including the measConfigAppLayerID of the service type.

- When UE receives the NR QoE configuration including the on-going QoE measurement association information and measConfigAppLayerID, the UE associates the on-going QoE measurement with the allocated measConfigLayerId and continue the QoE measurement after the inter-RAT handover.

To support QoE measurement continuity for HO from LTE/5GC to NR without LTE spec impact:

- [Already supported] The source ng-eNB provides QoE Configurations to the target gNB, e.g., provides the UE Application Layer Measurement Information IE of the QMC Configuration Information IE in XnAP Handover Request message. The source ng-eNB also provides an indication that whether the QoE measurement is already configured to UE in LTE.

- [Impact on TS 38.331] The gNB allocates a measConfigAppLayerID to the on-going QoE measurement and sends an on-going QoE measurement association information in RRC handover command, to indicate that UE associates the on-going QoE measurement.

- [Impact on TS 38.331] The UE associates the on-going QoE measurement with the allocated measConfigLayerId and continue the QoE measurement after the inter-RAT handover.

RAN3 responds a LS to capture the potential NR RRC impacts to support QoE measurement continuity for HO from NR to LTE/5GC and HO from LTE/5GC to NR.

3	Conclusion

This paper discusses the continuity of legacy QoE measurement job for streaming and MTSI service during intra-5GC inter-RAT handover process with the following proposals.

- To support QoE measurement continuity for HO from NR to LTE/5GC without LTE spec impact:

- [Impact on TS 38.413] The OAM system or CN provides an inter-RAT QoE continuity indication of one of QoE measurements which is used to indicate which QoE measurement (i.e., application layer measurement) is continued and kept during inter-RAT handover from gNB to ng-eNB;

- [Impact on TS 38.331] For the QoE measurement, the gNB provides the inter-RAT QoE continuity indication to the UE as received from OAM system or CN.

- [Potential Impact on stage 2] The source gNB generates and provides LTE RRC QoE configuration of the QoE measurement configuration in the source eNB to target ng-eNB transparent container to the target ng-eNB. The LTE RRC QoE configuration is encoded in LTE RRC ASN.1 format.

- [Impact on TS 38.331] When UE receives the LTE QoE configuration in the MobilityFromNRCommand message, the UE continues the QoE measurement with provided ‘IRAT continuity indication’ of the service type and release the other QoE configurations of the service type.

To support QoE measurement continuity for HO from LTE/5GC to NR without LTE spec impact:

- [Already supported] The source ng-eNB provides QoE Configurations to the target gNB, e.g., provides the UE Application Layer Measurement Information IE of the QMC Configuration Information IE in XnAP Handover Request message. The source ng-eNB also provides an indication that whether the QoE measurement is already configured to UE in LTE.

- [Impact on TS 38.331] The gNB allocates a measConfigAppLayerID to the on-going QoE measurement and sends an on-going QoE measurement association information in handover command, to indicate that UE associates the on-going QoE measurement.

- [Impact on TS 38.331] The UE associates the on-going QoE measurement with the allocated measConfigLayerId and continue the QoE measurement after the inter-RAT handover.

RAN3 responds a LS to capture the potential NR RRC impacts to support QoE measurement continuity for HO from NR to LTE/5GC and HO from LTE/5GC to NR.

A draft LS is provided in the Annex to respond incoming LS R3-231110.

Reference

[1] RP-221803. WID update for Enhancement on NR QoE. China Unicom.

[2] R2-2301902 RAN2#121 Report on LTE legacy, XR, QoE and MUSIM

[3] R3-231110 LS on Continuity of QoE measurements during intra-5GC inter-RAT HO (RAN2, Huawei)

3GPP TSG-RAN WG3 Meeting #121	R3-23xxxx

Toulouse, France, 21st – 25th August 2023

Title:	[Draft] Response LS on Continuity of QoE measurements during intra-5GC inter-RAT HO

Response to:	R2-2302020/R3-231110

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	RAN3

To:	RAN2, SA4

Cc:	-

Contact Person:

#### Name:	Mingzeng Dai

E-mail Address:	daimz4@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks RAN2 for the LS on Continuity of QoE measurements during intra-5GC inter-RAT HO. RAN3 agreed the following solutions to support Continuity of QoE measurements during intra-5GC inter-RAT HO.

To support QoE measurement continuity for HO from NR to LTE/5GC without LTE spec impact:

-	The OAM system or CN provides an inter-RAT QoE continuity indication of one of QoE measurements which is used to indicate which QoE measurement (i.e., application layer measurement) is continued and kept during inter-RAT handover from gNB to ng-eNB;

-	For the QoE measurement, the gNB provides the inter-RAT QoE continuity indication to the UE as received from OAM system or CN.

-	The source gNB generates and provides LTE RRC QoE configuration of the QoE measurement configuration in the source eNB to target ng-eNB transparent container to the target ng-eNB. The LTE RRC QoE configuration is encoded in LTE RRC ASN.1 format.

-	When UE receives the LTE QoE configuration in the MobilityFromNRCommand message, the UE continues the QoE measurement with provided ‘IRAT continuity indication’ of the service type and release the other QoE configurations of the service type

To support QoE measurement continuity for HO from LTE/5GC to NR without LTE spec impact:

-	The source ng-eNB provides QoE Configurations to the target gNB, e.g., provides the UE Application Layer Measurement Information IE of the QMC Configuration Information IE in XnAP Handover Request message. The source ng-eNB also provides an indication that whether the QoE measurement is already configured to UE in LTE.

-	The gNB allocates a measConfigAppLayerID to the on-going QoE measurement and sends an on-going QoE measurement association information in handover command, to indicate that UE associates the on-going QoE measurement.

-	The UE associates the on-going QoE measurement with the allocated measConfigLayerId and continue the QoE measurement after the inter-RAT handover.

2. Actions:

To RAN2 and SA4:

ACTION: 	RAN3 respectfully asks RAN2 and SA4 to take the RAN3 agreements into account.

3. Date of Next RAN2 Meetings:

3GPP RAN3#121bis	from 2023-10-09	to 2023-10-13		Xiamen, CN

3GPP RAN3#122	from 2023-11-13	to 2023-11-17		Chicago, US