3GPP TSG-RAN WG3 Meeting #121	R3-234184

Toulouse, France, 21st – 25th August 2023

Agenda item:	 15.2

Source:		Lenovo

Title:		Remaining issues of supporting MBS reception in RAN Sharing

Document for:		Discussion and Decision

- Introduction

This contribution discusses the remaining issue to support MBS reception in RAN sharing.

- Discussion

In case of MOCN sharing, FFS on how many MBS session context procedure instances should be triggered in gNB-DU to establish the related MBS session contexts:

-	Option 1: the gNB-CU sends multiple F1AP: BROADCAST CONTEXT SETUP REQUEST messages with different TMGIs and same MBS RAN Sharing efficiency Information.

-	Option 2: the gNB-CU sends in a single F1AP: BROADCAST CONTEXT SETUP/MODIFICATION REQUEST message includes a list of TMGIs and an MBS RAN Sharing efficiency Information

For option 1, the shared DU receives multiple signalling over F1 from the shared CU (signalling per PLMN) and the shared DU coordinates the multiple pieces of information to be build the MCCH.

For option 2, the shared CU coordinates the information from different PLMNs to build the common signalling to be sent over F1 which is used by the shared DU to build the common MCCH.

To be honest, both option 1 and option 2 can work well. We would prefer option 1 since it can also be applied to multiple cell-ID cases.

For MOCN sharing, the gNB-CU sends multiple F1AP: BROADCAST CONTEXT SETUP REQUEST messages with different TMGIs and the same associated session ID to the gNB-DU.

Regarding PDCP aspects for RAN sharing with multiple Cell ID broadcast, a WA was agreed:

WA: In case of RAN sharing with multiple Cell ID broadcast, the entity controlling the logical DUs decides which MRB-PDCP-ConfigBroadcast to provide on MCCH. Details are FFS.

There were two solutions have been discussed:

-	Option 1: OAM configure the MRB-PDCP-ConfigBroadcast of each MRB to CU or DU.

-	Option 2: DU applied the MRB-PDCP-ConfigBroadcast of the first CU or the second CU. FFS whether DU needs to provide the used MRB-PDCP-ConfigBroadcast to CU.

As specified in TS 38.331, the PDCP configuration of a broadcast MRB includes

MRB-PDCP-ConfigBroadcast-r17 ::=     SEQUENCE {

pdcp-SN-SizeDL-r17                   ENUMERATED {len12bits}                   OPTIONAL, -- Need S

headerCompression-r17                CHOICE {

notUsed                              NULL,

rohc                                 SEQUENCE {

maxCID-r17                           INTEGER (1..16)               DEFAULT 15,

profiles-r17                         SEQUENCE {

profile0x0000-r17                    BOOLEAN,

profile0x0001-r17                    BOOLEAN,

profile0x0002-r17                    BOOLEAN

}

}

},

t-Reordering-r17                     ENUMERATED {ms1, ms10, ms40, ms160, ms500, ms1000, ms1250, ms2750}    OPTIONAL -- Need S

}

t-Reordering

Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms1 corresponds to 1 ms, value ms10 corresponds to 10 ms, and so on.  When the field is absent the UE applies the value as specified in 9.1.1.7.

As observed from TS 38.331, the parameters in PDCP configuration for broadcast MRB are very limited. For example, the PDCP SN is fixed as 12 bits for all MRBs, and the default value of t-Reordering can be used. The only thing is ROHC profiles may be differently used for different gNB-CUs. Since there is no UL feedback, only three ROCH profile are specified for broadcast MRB. In this case, it would be better to gNB-CUs configures ROCH profiles by pre-configurations. Besides that, the QoS flow to MRB mapping are also needs to be pre-configured anyway.

The gNB-CUs apply the same PDCP configuration as well as MBS QoS flow to MRB mapping by using default value or by pre-configuration in case of multiple cell-ID sharing case.

There is also an FFS on how many F1-U tunnels should be established for multiple cell-ID broadcast case.

FFS on how many F1-U tunnels should be established for multiple cell-ID broadcast cases.

From our view, single F1-U tunnel is sufficient since anyway the same packets are sent to UE in the air interface. Similar with NG interface, it is proposed to include an indication that the gNB-DU has decided to not establish F1-U transport resource of an MRB to gNB-CU.

Single F1-U tunnel for an broadcast MRB is established for multiple cell-ID case.

The gNB-DU sends an indication that the gNB-DU has decided to not establish F1-U transport resource of an MRB to gNB-CU.

3	Conclusion

This contribution discusses the remaining issue to support MBS reception in RAN sharing. And we propose

- For MOCN sharing, the gNB-CU sends multiple F1AP: BROADCAST CONTEXT SETUP REQUEST messages with different TMGIs and the same associated session ID to the gNB-DU.

- The gNB-CUs apply the same PDCP configuration as well as MBS QoS flow to MRB mapping by using default value or by pre-configuration in case of multiple cell-ID sharing case.

- Single F1-U tunnel for an broadcast MRB is established for multiple cell-ID case.

- The gNB-DU sends an indication that the gNB-DU has decided to not establish F1-U transport resource of an MRB to gNB-CU.

Reference