3GPP TSG-RAN WG3 #121	R3-234207

Toulouse, France, 21st – 25th August 2023

Source:	CATT

Title:	(TP to 38.473 & 37.483) Discussion on E1AP and F1AP impacts

Agenda Item:	12.2.2.4

Document for:	Discussion and decision

- Introduction

This discussion paper focuses impacts on E1AP and F1AP.

- Discussion

  - AI/ML deployment in split architecture

We have agreed that for the use case of load prediction, the training/inference function locates at the gNB-CU-CP. For alignment we think other training/inference function, as well as monitoring function, should locate at the gNB-CU-CP as well.

Proposal 1: Model training, inference and monitoring function, if deployed in NG-RAN using split architecture, should be deployed in the gNB-CU-CP as of Rel-18.

  - UE traffic

Typically both the use case of mobility and the use case of energy saving requires the information on UE traffic:

- Mobility: Predicting future UE traffic requires current per-QoS-flow UE traffic information for the concerned UE as input.

- Energy saving: Predicting the change of Energy Cost requires per-QoS-flow UE traffic information for the UEs which are to be handed over as input.

Observation 1: Both the mobility use case and the energy saving use case requires per-QoS-flow UE traffic information as model input.

Proposal 2: E1AP should be enhanced to support that gNB-CU-CPs retrieves per-QoS-flow UE traffic information from gNB-CU-UPs. The baseline of metric can be “DL PDCP SDU Data Volume” and “UL PDCP SDU Data Volume” defined in section 5.1.3.6.2 of TS 28.552.

  - Energy Cost

It is clear in TS 28.310 that the energy cost is measured in the gNB-DU.

Proposal 3: F1AP should be enhanced to support that gNB-CU(-CP)s retrieves energy cost.

  - UE performance feedback

Currently there are three types of metrics in UE performance feedback as of R3-233535: throughput, packet delay and packet loss.

The cases on throughput and packet loss are simple:

- Both downlink throughput and uplink throughput are measured by the gNB-DU as defined in Section 5.1.1.3.1 and Section 5.1.1.3.3 of TS 28.552 respectively.

- Downlink packet loss is measured by the gNB-DU as defined in Section 4.2.1.5.1 of TS 38.314, whereas uplink packet loss is measured by the gNB-CU-UP as defined in Section 5.1.3.1.1 of TS 28.552.

The case is quite complex on packet delay as shown in Section 4.2.1.2.1 of TS 38.314. In the downlink direction it is defined as the sum of 4 items:

- Average delay DL air-interface, measured by the gNB-DU as defined in Section 5.1.1.1.1 of TS 28.552;

- Average delay in RLC sublayer, measured by the gNB-DU as defined in Section 5.1.3.3.3 of TS 28.552;

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 5.1.3.3.2 of TS 28.552;

- Average delay DL in CU-UP, measured by the gNB-CU-UP as defined in Section 5.1.3.3.1 of TS 28.552.

In the uplink direction it is defined as the sum of 5 items:

- UL PDCP packet average delay, measured by the UE as defined in Section 4.3.1.1 of TS 38.314;

- Average over-the-air interface packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.2 of TS 38.314;

- Average RLC packet delay, measured by the gNB-DU as defined in Section 4.2.1.2.3 of TS 38.314;

- Average delay on F1-U, measured by the gNB-CU-UP as defined in Section 5.1.3.3.2 of TS 28.552;

- Average PDCP re-ordering delay, measured by the gNB-CU-UP as defined in Section 4.2.1.2.4 of TS 38.314.

Nevertheless, it is not necessary to report every metric separately. The delay measured in gNB-DU can be combined into one single metric, and so can the delay measured in gNB-CU-UP.

As the summary, there are 5 metrics collected in the gNB-DU whereas 3 metrics collected in the gNB-CU-UP. The remaining one is collected by the UE, which is reported toward the gNB-CU-CP directly and thus has no impact on either E1AP or F1AP.

The granularity is not consistent between TS 28.552 and TS 38.314. For simplicity we propose that the granularity of throughput is per UE (aligned with the one in R3-233535), whereas the granularity of remaining metrics is per DRB.

Proposal 3: F1AP should be enhanced to support that gNB-CU(-CP)s retrieve 5 metrics for UE performance:
-	DL throughput per UE;
-	UL throughput per UE;
-	DL packet loss per DRB;
-	Average DL delay air-interface plus Average DL delay in gNB-DU per DRB;
-	Average UL delay air-interface plus Average UL delay in gNB-DU per DRB.

Proposal 4: E1AP should be enhanced to support that gNB-CU-CPs retrieve 3 metrics for UE performance:
-	UL packet loss per DRB;
-	Average delay F1-U plus Average DL delay in gNB-CU-UP per DRB;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP per DRB.

- Conclusion

Proposal 1: Model training, inference and monitoring function, if deployed in NG-RAN using split architecture, should be deployed in the gNB-CU-CP as of Rel-18.

Observation 1: Both the mobility use case and the energy saving use case requires per-QoS-flow UE traffic information as model input.

Proposal 2: E1AP should be enhanced to support that gNB-CU-CPs retrieves per-QoS-flow UE traffic information from gNB-CU-UPs. The baseline of metric can be “DL PDCP SDU Data Volume” and “UL PDCP SDU Data Volume” defined in section 5.1.3.6.2 of TS 28.552.

Proposal 3: F1AP should be enhanced to support that gNB-CU(-CP)s retrieves energy cost.

Proposal 4: F1AP should be enhanced to support that gNB-CU(-CP)s retrieve 5 metrics for UE performance:
-	DL throughput per UE;
-	UL throughput per UE;
-	DL packet loss per DRB;
-	Average DL delay air-interface plus Average DL delay in gNB-DU per DRB;
-	Average UL delay air-interface plus Average UL delay in gNB-DU per DRB.

Proposal 5: E1AP should be enhanced to support that gNB-CU-CPs retrieve 3 metrics for UE performance:
-	UL packet loss per DRB;
-	Average delay F1-U plus Average DL delay in gNB-CU-UP per DRB;
-	Average delay F1-U plus Average UL delay in gNB-CU-UP per DRB.

The TP is provided in the Annexes (the ASN.1 part is not attached).

- Annex 1: TP for TS 38.473

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| F1 Setup |  | F1 SETUP REQUEST |  | F1 SETUP RESPONSE |  | F1 SETUP FAILURE |  |
| gNB-DU Configuration Update |  | GNB-DU CONFIGURATION UPDATE |  | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-DU CONFIGURATION UPDATE FAILURE |  |
| gNB-CU Configuration Update |  | GNB-CU CONFIGURATION UPDATE |  | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU CONFIGURATION UPDATE FAILURE |  |
| UE Context Setup |  | UE CONTEXT SETUP REQUEST |  | UE CONTEXT SETUP RESPONSE |  | UE CONTEXT SETUP FAILURE |  |
| UE Context Release (gNB-CU initiated) |  | UE CONTEXT RELEASE COMMAND |  | UE CONTEXT RELEASE COMPLETE |  |  |  |
| UE Context Modification (gNB-CU initiated) |  | UE CONTEXT MODIFICATION REQUEST |  | UE CONTEXT MODIFICATION RESPONSE |  | UE CONTEXT MODIFICATION FAILURE |  |
| UE Context Modification Required (gNB-DU initiated) |  | UE CONTEXT MODIFICATION REQUIRED |  | UE CONTEXT MODIFICATION CONFIRM |  | UE CONTEXT MODIFICATION REFUSE |  |
| Write-Replace Warning |  | WRITE-REPLACE WARNING REQUEST |  | WRITE-REPLACE WARNING RESPONSE |  |  |  |
| PWS Cancel |  | PWS CANCEL REQUEST |  | PWS CANCEL RESPONSE |  |  |  |
| gNB-DU Resource Coordination |  | GNB-DU RESOURCE COORDINATION REQUEST |  | GNB-DU RESOURCE COORDINATION RESPONSE |  |  |  |
| F1 Removal |  | F1 REMOVAL REQUEST |  | F1 REMOVAL RESPONSE |  | F1 REMOVAL FAILURE |  |
| BAP Mapping Configuration |  | BAP MAPPING CONFIGURATION |  | BAP MAPPING CONFIGURATION ACKNOWLEDGE |  | BAP MAPPING CONFIGURATION FAILURE |  |
| GNB-DU Resource Configuration |  | GNB-DU RESOURCE CONFIGURATION |  | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE |  | GNB-DU RESOURCE CONFIGURATION FAILURE |  |
| IAB TNL Address Allocation |  | IAB TNL ADDRESS REQUEST |  | IAB TNL ADDRESS RESPONSE |  | IAB TNL ADDRESS FAILURE |  |
| IAB UP Configuration Update |  | IAB UP CONFIGURATION UPDATE REQUEST |  | IAB UP CONFIGURATION UPDATE RESPONSE |  | IAB UP CONFIGURATION UPDATE FAILURE |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| Positioning Measurement |  | POSITIONING MEASUREMENT REQUEST |  | POSITIONING MEASUREMENT RESPONSE |  | POSITIONING MEASUREMENT FAILURE |  |
| Positioning Information Exchange |  | POSITIONING INFORMATION REQUEST |  | POSITIONING INFORMATION RESPONSE |  | POSITIONING INFORMATION FAILURE |  |
| TRP Information Exchange |  | TRP INFORMATION REQUEST |  | TRP INFORMATION RESPONSE |  | TRP INFORMATION FAILURE |  |
| Positioning Activation |  | POSITIONING ACTIVATION REQUEST |  | POSITIONING ACTIVATION RESPONSE |  | POSITIONING ACTIVATION FAILURE |  |
| E-CID Measurement Initiation |  | E-CID MEASUREMENT INITIATION REQUEST |  | E-CID MEASUREMENT INITIATION RESPONSE |  | E-CID MEASUREMENT INITIATION FAILURE |  |
| Broadcast Context Setup |  | BROADCAST CONTEXT SETUP REQUEST |  | BROADCAST CONTEXT SETUP RESPONSE |  | BROADCAST CONTEXT SETUP FAILURE |  |
| Broadcast Context Release |  | BROADCAST CONTEXT RELEASE COMMAND |  | BROADCAST CONTEXT RELEASE COMPLETE |  |  |  |
| Broadcast Context Modification |  | BROADCAST CONTEXT MODIFICATION REQUEST |  | BROADCAST CONTEXT MODIFICATION RESPONSE |  | BROADCAST CONTEXT MODIFICATION FAILURE |  |
| Multicast Context Setup |  | MULTICAST CONTEXT SETUP REQUEST |  | MULTICAST CONTEXT SETUP RESPONSE |  | MULTICAST CONTEXT SETUP FAILURE |  |
| Multicast Context Release |  | MULTICAST CONTEXT RELEASE COMMAND |  | MULTICAST CONTEXT RELEASE COMPLETE |  |  |  |
| Multicast Context Modification |  | MULTICAST CONTEXT MODIFICATION REQUEST |  | MULTICAST CONTEXT MODIFICATION RESPONSE |  | MULTICAST CONTEXT MODIFICATION FAILURE |  |
| Multicast Distribution Setup |  | MULTICAST DISTRIBUTION SETUP REQUEST |  | MULTICAST DISTRIBUTION SETUP RESPONSE |  | MULTICAST DISTRIBUTION SETUP FAILURE |  |
| Multicast Distribution Release |  | MULTICAST DISTRIBUTION RELEASE COMMAND |  | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |  |  |
| PDC Measurement Initiation |  | PDC MEASUREMENT INITIATION REQUEST |  | PDC MEASUREMENT INITIATION RESPONSE |  | PDC MEASUREMENT INITIATION FAILURE |  |
| PRS Configuration Exchange |  | PRS CONFIGURATION REQUEST |  | PRS CONFIGURATION RESPONSE |  | PRS CONFIGURATION FAILURE |  |
| Measurement Preconfiguration |  | MEASUREMENT PRECONFIGURATION REQUIRED |  | MEASUREMENT PRECONFIGURATION CONFIRM |  | MEASUREMENT PRECONFIGURATION REFUSE |  |
| Data Collection Initiation |  | DATA COLLECTION REQUEST |  | DATA COLLECTION RESPONSE |  | DATA COLLECTION FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  | Message |  |
| --- | --- | --- | --- |
| Error Indication |  | ERROR INDICATION |  |
| UE Context Release Request (gNB-DU initiated) |  | UE CONTEXT RELEASE REQUEST |  |
| Initial UL RRC Message Transfer |  | INITIAL UL RRC MESSAGE TRANSFER |  |
| DL RRC Message Transfer |  | DL RRC MESSAGE TRANSFER |  |
| UL RRC Message Transfer |  | UL RRC MESSAGE TRANSFER |  |
| UE Inactivity Notification |  | UE INACTIVITY NOTIFICATION |  |
| System Information Delivery |  | SYSTEM INFORMATION DELIVERY COMMAND |  |
| Paging |  | PAGING |  |
| Notify |  | NOTIFY |  |
| PWS Restart Indication |  | PWS RESTART INDICATION |  |
| PWS Failure Indication |  | PWS FAILURE INDICATION |  |
| gNB-DU Status Indication |  | GNB-DU STATUS INDICATION |  |
| RRC Delivery Report |  | RRC DELIVERY REPORT |  |
| Network Access Rate Reduction |  | NETWORK ACCESS RATE REDUCTION |  |
| Trace Start |  | TRACE START |  |
| Deactivate Trace |  | DEACTIVATE TRACE |  |
| DU-CU Radio Information Transfer |  | DU-CU RADIO INFORMATION TRANSFER |  |
| CU-DU Radio Information Transfer |  | CU-DU RADIO INFORMATION TRANSFER |  |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |  |
| Access And Mobility Indication |  | ACCESS AND MOBILITY INDICATION |  |
| Reference Time Information Reporting Control |  | REFERENCE TIME INFORMATION REPORTING CONTROL |  |
| Reference Time Information Report |  | REFERENCE TIME INFORMATION REPORT |  |
| Access Success |  | ACCESS SUCCESS |  |
| Cell Traffic Trace |  | CELL TRAFFIC TRACE |  |
| Positioning Assistance Information Control |  | POSITIONING ASSISTANCE INFORMATION CONTROL |  |
| Positioning Assistance Information Feedback |  | POSITIONING ASSISTANCE INFORMATION FEEDBACK |  |
| Positioning Measurement Report |  | POSITIONING MEASUREMENT REPORT |  |
| Positioning Measurement Abort |  | POSITIONING MEASUREMENT ABORT |  |
| Positioning Measurement Failure Indication |  | POSITIONING MEASUREMENT FAILURE INDICATION |  |
| Positioning Measurement Update |  | POSITIONING MEASUREMENT UPDATE |  |
| Positioning Deactivation |  | POSITIONING DEACTIVATION |  |
| E-CID Measurement Failure Indication |  | E-CID MEASUREMENT FAILURE INDICATION |  |
| E-CID Measurement Report |  | E-CID MEASUREMENT REPORT |  |
| E-CID Measurement Termination |  | E-CID MEASUREMENT TERMINATION COMMAND |  |
| Positioning Information Update |  | POSITIONING INFORMATION UPDATE |  |
| Multicast Group Paging |  | MULTICAST GROUP PAGING |  |
| Broadcast Context Release Request |  | BROADCAST CONTEXT RELEASE REQUEST |  |
| Multicast Context Release Request |  | MULTICAST CONTEXT RELEASE REQUEST |  |
| PDC Measurement Report |  | PDC MEASUREMENT REPORT |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| PDC Measurement Failure Indication |  | PDC MEASUREMENT FAILURE INDICATION |  |
| PDC Measurement Termination |  | PDC MEASUREMENT TERMINATION COMMAND |  |
| Measurement Activation |  | MEASUREMENT ACTIVATION |  |
| QoE Information Transfer |  | QOE INFORMATION TRANSFER |  |
| Data Collection Reporting |  | DATA COLLECTION REPORT |  |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 8.2.a1	Data Collection Initiation

#### 8.2.a1.1	General

This procedure is used by the gNB-CU to request the collection of data to the gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.a1.2	Successful Operation

Figure 8.2.a1.2-1: Data Collection Initiation, successful operation

[FFS]

#### 8.2.a1.3	Unsuccessful Operation

Figure 8.2.a1.3-1: Data Collection Initiation, unsuccessful operation

[FFS]

#### 8.2.a1.4	Abnormal Conditions

[FFS]

### 8.2.a2	Data Collection Reporting

#### 8.2.a2.1	General

This procedure is initiated by the gNB-DU to report the result of data collection admitted by the gNB-DU following a successful Data Collection Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.a2.2	Successful Operation

Figure 8.2.a2.2-1: Data Collection Reporting, successful operation

[FFS]

#### 8.2.a2.3	Unsuccessful Operation

Not applicable.

#### 8.2.a2.4	Abnormal Conditions

Void

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.2.1	Interface Management messages

#### 9.1.3.y1	DATA COLLECTION REQUEST

This message is sent by the gNB-CU to the gNB-DU to initiate the requested data collection according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. 1st Bit = Average DL throughput per UE, 2nd Bit = Average UL throughput per UE, 3rd Bit = Average DL packet loss per DRB, 4th Bit = Average DL delay parts, 5th Bit = Average UL delay parts, 6th Bit = Energy Cost, Other bits shall be ignored by the gNB-DU. | YES | reject |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.1.3.y2	DATA COLLECTION RESPONSE

This message is sent by the gNB-DU to the gNB-CU to indicate that at least one of the requested data collection is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-DU. 6th Bit = Energy Cost. Other bits shall be ignored by the gNB-CU. |  |  |
| >>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |  |  |
| Per Cell Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | ignore |
| >Per Cell Measurement Initiation Result Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that the gNB-DU could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  |  |  |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates the object that failed to be initiated in the gNB-DU. 1st Bit = Average DL throughput per UE, 2nd Bit = Average UL throughput per UE, 3rd Bit = Average DL packet loss per DRB, 4th Bit = Average DL delay parts, 5th Bit = Average UL delay parts, Other bits shall be ignored by the gNB-CU. | – |  |
| >>>>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 128. |

#### 9.1.3.y3	DATA COLLECTION FAILURE

This message is sent by the gNB-DU to the gNB-CU to indicate that for none of the requested data collection can be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID (FFS on the name) | O |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.1.3.y4	DATA COLLECTION REPORT

This message is sent by the gNB-DU to the gNB-CU to report the results of the requested data collection.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| UE Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. <maxnoofUEReports> |  |  |  |  |
| >>UE Assistant Identifier (FFS on the name) | M |  | gNB-CU UE F1AP ID 9.3.1.4 |  |  |  |
| >>UE Performance | O |  | 9.3.1.z1 |  |  |  |
| Energy Cost | O |  | FFS | It represents the actual measurement of the Energy Cost. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE that can be served by a gNB-DU. Value is FFS. |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| /////////////////////////////////////////////////////skip unchanged rows///////////////////////////////////////////////////// |  |  |  |  |  |  |
| Data Collection Measurement ID To Setup List | O |  | 9.3.1.z2 |  | YES | ignore |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| /////////////////////////////////////////////////////skip unchanged rows///////////////////////////////////////////////////// |  |  |  |  |  |  |
| Data Collection Measurement ID To Setup List | O |  | 9.3.1.z2 |  | YES | ignore |
| Data Collection Measurement ID To Release List | O |  | 9.3.1.z3 |  | YES | ignore |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.3.1.z1	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.3.1.22 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.3.1.22 |  |
| UE Performance DRB List |  | 0..1 |  |  |
| >UE Performance DRB Item |  | 1..<maxnoofDRBs> |  |  |
| >>Average Packet Loss DL | O |  | INTEGER (0..1000000) | As defined in Section 4.2.1.5.1 of TS 38.314 [42]. |
| >>Average Delay part DL | O |  | INTEGER | Unit: 0.1ms. Includes both the delay of air interface and the delay within gNB-DU. |
| >>Average Delay part UL | O |  | INTEGER | Unit: 0.1ms. Includes both the delay of air interface and the delay within gNB-DU. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

#### 9.3.1.z2	Data Collection Measurement ID To Setup List

This IE indicates the UE is subject of the listed data collection measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Collection Measurement ID To Setup Item |  | 1..<maxnoofDataCollection> |  |  |
| >gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU |
| >gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU |

| Range bound | Explanation |
| --- | --- |
| maxnoofDataCollection | Maximum no. of association to a UE. Value is FFS. |

#### 9.3.1.z3	Data Collection Measurement ID To Release List

This IE indicates the UE is no longer subject of the listed data collection measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Collection Measurement ID To Release Item |  | 1..<maxnoofDataCollection> |  |  |
| >gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU |
| >gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU |

| Range bound | Explanation |
| --- | --- |
| maxnoofDataCollection | Maximum no. of association to a UE. Value is FFS. |

///////////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////////

- Annex 2: TP for TS 37.483

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| BC Bearer Context Setup |  | BC BEARER CONTEXT SETUP REQUEST |  | BC BEARER CONTEXT SETUP RESPONSE |  | BC BEARER CONTEXT SETUP FAILURE |  |
| BC Bearer Context Modification (gNB-CU-CP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUEST |  | BC BEARER CONTEXT MODIFICATION RESPONSE |  | BC BEARER CONTEXT MODIFICATION FAILURE |  |
| BC Bearer Context Modification Required (gNB-CU-UP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUIRED |  | BC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| BC Bearer Context Release (gNB-CU-CP initiated) |  | BC BEARER CONTEXT RELEASE COMMAND |  | BC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| MC Bearer Context Setup |  | MC BEARER CONTEXT SETUP REQUEST |  | MC BEARER CONTEXT SETUP RESPONSE |  | MC BEARER CONTEXT SETUP FAILURE |  |
| MC Bearer Context Modification (gNB-CU-CP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUEST |  | MC BEARER CONTEXT MODIFICATION RESPONSE |  | MC BEARER CONTEXT MODIFICATION FAILURE |  |
| MC Bearer Context Modification Required (gNB-CU-UP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUIRED |  | MC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| MC Bearer Context Release (gNB-CU-CP initiated) |  | MC BEARER CONTEXT RELEASE COMMAND |  | MC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Data Collection Initiation |  | DATA COLLECTION REQUEST |  | DATA COLLECTION RESPONSE |  | DATA COLLECTION FAILURE |  |

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |
| IAB PSK Notification |  |  | IAB PSK NOTIFICATION |  |  |
| BC Bearer Context Release (gNB-CU-UP initiated) |  |  | BC BEARER CONTEXT RELEASE REQUEST |  |  |
| BC Bearer Context Release (gNB-CU-UP initiated) |  |  | BC BEARER CONTEXT RELEASE REQUEST |  |  |
| Data Collection Reporting |  |  | DATA COLLECTION REPORT |  |  |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 8.2.a1	Data Collection Initiation

#### 8.2.a1.1	General

This procedure is used by a gNB-CU-CP to request the collection of data to a gNB-CU-UP.

The procedure uses non UE-associated signalling.

#### 8.2.a1.2	Successful Operation

Figure 8.2.a1.2-1: Data Collection Initiation, successful operation

[FFS]

#### 8.2.a1.3	Unsuccessful Operation

Figure 8.2.a1.3-1: Data Collection Initiation, unsuccessful operation

[FFS]

#### 8.2.a1.4	Abnormal Conditions

[FFS]

### 8.2.a2	Data Collection Reporting

#### 8.2.a2.1	General

This procedure is initiated by a gNB-CU-UP to report the result of data collection admitted by the gNB-CU-UP node following a successful Data Collection Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.a2.2	Successful Operation

Figure 8.2.a2.2-1: Data Collection Reporting, successful operation

[FFS]

#### 8.2.a2.3	Unsuccessful Operation

Not applicable.

#### 8.2.a2.4	Abnormal Conditions

Void

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.2.1	Interface Management messages

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.2.1.y1	DATA COLLECTION REQUEST

This message is sent by the gNB-CU-CP to the gNB-CU-UP to initiate the requested data collection according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID (FFS on the name) | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) (FFS on others) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. 1st Bit = Average UL packet loss per DRB, 2nd Bit = Average DL delay parts, 3rd Bit = Average UL delay parts, 4th Bit = DL PDCP SDU Data Volume, 5th Bit = UL PDCP SDU Data Volume. Other bits shall be ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

#### 9.2.1.y2	DATA COLLECTION RESPONSE

This message is sent by the gNB-CU-UP to the gNB-CU-CP to indicate that at least one of the requested data collection is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| Node Measurement Initiation Result |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | ignore |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(128)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-CU-UP. 1st Bit = Average UL packet loss per DRB, 2nd Bit = Average DL delay parts, 3rd Bit = Average UL delay parts, 4th Bit = DL PDCP SDU Data Volume, 5th Bit = UL PDCP SDU Data Volume. Other bits shall be ignored by the gNB-CU-CP. |  |  |
| >>Cause | M |  | 9.3.1.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxFailedMeasObjects | Maximum number of measurement objects that can fail per measurement. Value is 128. |

#### 9.2.1.y3	DATA COLLECTION FAILURE

This message is sent by the gNB-CU-UP to the gNB-CU-CP to indicate that for none of the requested data collection can be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID (FFS on the name) | O |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.y4	DATA COLLECTION REPORT

This message is sent by the gNB-CU-UP to the gNB-CU-CP to report the results of the requested data collection.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID (FFS on the name) | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| UE Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. <maxnoofUEReports> |  |  |  |  |
| >>UE Assistant Identifier (FFS on the name) | M |  | gNB-CU-CP UE E1AP ID 9.3.1.4 |  |  |  |
| >>UE Performance | O |  | 9.3.1.z1 |  |  |  |
| >>UE Data Volume | O |  | 9.3.1.z2 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE that can be served by a gNB-CU-UP. Value is FFS. |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.2.2	Bearer Context Management messages

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| /////////////////////////////////////////////////////skip unchanged rows///////////////////////////////////////////////////// |  |  |  |  |  |  |
| Data Collection Measurement ID To Setup List | O |  | 9.3.1.z3 |  | YES | ignore |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| /////////////////////////////////////////////////////skip unchanged rows///////////////////////////////////////////////////// |  |  |  |  |  |  |
| Data Collection Measurement ID To Setup List | O |  | 9.3.1.z3 |  | YES | ignore |
| Data Collection Measurement ID To Release List | O |  | 9.3.1.z4 |  | YES | ignore |

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

### 9.3.1	Radio Network Layer Related IEs

///////////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////////

#### 9.3.1.z1	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Performance DRB List |  | 0..1 |  |  |
| >UE Performance DRB Item |  | 1..<maxnoofDRBs> |  |  |
| >>Average Packet Loss UL | O |  | INTEGER (0..1000000) | Unit: one packet per million. |
| >>Average Delay part DL | O |  | INTEGER | Unit: 0.1ms. Includes both the delay of F1-U and the delay within gNB-CU-UP. |
| >>Average Delay part UL | O |  | INTEGER | Unit: 0.1ms. Includes both the delay of F1-U and the delay within gNB-CU-UP. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |

#### 9.3.1.z2	UE Data Volume

This IE indicates the UE data volume measurements metrics.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Data Volume Session List |  | 0..1 |  |  |
| >UE Data Volume Session Item |  | 1..<maxnoofPDUSessionResource> |  |  |
| >>PDU Session ID | M |  | 9.3.1.21 |  |
| >>UE Data Volume Flow List |  | 0..1 |  |  |
| >>>UE Data Volume Flow Item |  | 1..<maxnoofQoSFlows> |  |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.24 |  |
| >>>>DL PDCP SDU Data Volume | O |  | INTEGER | Representing the number of bits measured in Mbits (1MBits=1000*1000 bits) |
| >>>>UL PDCP SDU Data Volume | O |  | INTEGER | Representing the number of bits measured in Mbits (1MBits=1000*1000 bits) |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

#### 9.3.1.z3	Data Collection Measurement ID To Setup List

This IE indicates the UE is subject of the listed data collection measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Collection Measurement ID To Setup Item |  | 1..<maxnoofDataCollection> |  |  |
| >gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP |
| >gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP |

| Range bound | Explanation |
| --- | --- |
| maxnoofDataCollection | Maximum no. of association to a UE. Value is FFS. |

#### 9.3.1.z4	Data Collection Measurement ID To Release List

This IE indicates the UE is no longer subject of the listed data collection measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data Collection Measurement ID To Release Item |  | 1..<maxnoofDataCollection> |  |  |
| >gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP |
| >gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP |

| Range bound | Explanation |
| --- | --- |
| maxnoofDataCollection | Maximum no. of association to a UE. Value is FFS. |

///////////////////////////////////////////////////////////////////////////end///////////////////////////////////////////////////////////////////////////