3GPP TSG-RAN WG3 Meeting #121	R3-234217

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	23.2.3

Source:	Huawei

Title:	Discussion on Carrier Phase Positioning and Sidelink Positioning

Document for:	Approval

# 1	Introduction

This paper discusses our thinkings on Carrier Phase Positioning and Sidelink Positioning.

# 2	Discussion

## 2.1 Carrier Phase Positioning

The following agreements have been made by RAN1[1-2]:

Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting.

RAN1 has agreed to introduce new measurements UL RSCP for UL CPP positioning, which the UL RSCP can be reported by the TRP together with RTOA measurements and/or gNB Rx-Tx time difference measurements to the LMF. It is proposed to enhance the TRP Measurement Result in TS 38.455 and TS 48.473 to capture the carrier phase measurements to support UL CPP. Since RAN1 is still discussing the details, RAN3 can wait for more RAN1 progress for the detailed implementation.

Proposal 1: It is proposed to enhance the TRP Measurement Result in TS 38.455 and TS 38.473 to capture the carrier phase measurements to support UL CPP. Wait for RAN1 progress for the detailed implementation.

The following agreement was made by RAN1 [2-3]:

Agreement Adopt the following modifications on the agreements made in RAN1#112bis-e: To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows

To enable simultaneous measurements by gNBs, RAN1 introduces a time window for SRS transmission configuration and another window for SRS measurements. From our understanding, these two windows should be implemented in Positioning Information Request message and Measurement Request message respectively. For SRS measurements, according to RAN1 agreements, some legacy parameters can be reused, e.g. SFN initialization time, slot number, etc. Measurement Request may be enhanced to support the time window by introducing new parameters introduced by RAN1.  For SRS configuration, the parameters are similar but there is only RAN1 working assumption, we can wait more table RAN1 agreements.

Proposal 2: It is proposed to enhance NRPPa and F1AP to support the time window for SRS configuration and SRS measurements for UL CPP. Wait for RAN1 progress for the detailed implementation.

## 2.1 Sidelink Positioning

The following agreement was made during last meeting on sidelink that RSPP/SLPP QoS shall be provided from AMF to RAN [4]:

RSPP/SLPP transport QoS parameters shall be provided from AMF to the NG-RAN for scheduled resource allocation mode resource management.

In the BL CRs the RSPP Transport QoS parameters are still FFS. According to the latest TS 23.586, the QoS handling mechanism and PQI values for V2X and ProSe can be reused. Therefore, it is proposed to remove the FFS for RSPP transport QoS parameters in the running CRs.

### 5.7.3 Handling of RSPP transport QoS The V2X/ProSe layer handles the RSPP traffic as the V2X/ProSe application data packets for the QoS treatment. QoS handling mechanism of V2X as defined in clause 5.4 of TS 23.287 [6] or QoS handling mechanism of 5G ProSe as defined in clause 5.6.1 of TS 23.304 [7] is reused. PQI values as defined in TS 23.287 [6] and TS 23.304 [7] may be reused for RSPP transport QoS.

Proposal 3: It is proposed to remove FFS of RSPP/SLPP transport QoS parameters in the BL CRs.

# 3	Conclusion

This document proposes:

Proposal 1: It is proposed to enhance the TRP Measurement Result in TS 38.455 and TS 38.473 to capture the carrier phase measurements to support UL CPP. Wait for RAN1 progress for the detailed implementation.

Proposal 2: It is proposed to enhance NRPPa and F1AP to support the time window for SRS configuration and SRS measurements for UL CPP. Wait for RAN1 progress for the detailed implementation.

Proposal 3: It is proposed remove FFS of RSPP/SLPP transport QoS parameters in the BL CRs.

# 4	References

- RAN1-112 Chair’s Notes

- RAN1-112bis Chair’s Notes

- RAN1-113 Chair’s Notes

- RAN3-120 Chair’s Notes