3GPP TSG-RAN WG3 Meeting #121	R3-234219

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	(TP to BL CRs, 36.300, 36.423) Alignment with NR NTN

Source:	Huawei, Ericsson

Title:	18.2

Document for:	Approval

# 1	Introduction

Considering the progress on NTN Work Item is almost done, the following contribution proposed to align the IoT NTN BL CRs with NTN BL CRs.

# 2	Discussion

## 2.1 Stage 2: TS 38.300 vs TS 36.300

The CHO time-Based information Text Proposal missing in TS 36.300 BL CR should be captured with [7].

The UE location Verification, existing in subclause 16.14.6 of TS 38.300 AMF (Re-)Selection of the Stage2 NTN BL CR [2] should be added in subclause 23.21.7 of TS 36.300 MME(Re-)Selection by eNB in the Stage 2 IoT NTN BL CR [4].

The TP for 36.300 is propose in Annex A.

Proposal 1: Agreed the TP for TS 36.300 on UE verification (Annex A)

## 2.2 TS 38.413 vs TS 36.413

The CHO time-Based information Text Proposal missing in TS 36.413 BL CR should be captured with [7].

## 2.1 NTN IoT RAN2 Agreements

RAN2 had the following agreement (RAN2#120) and capture the Handover Window Start IE and the Handover Window Duration IE for LTE [8]:

1.	CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.: UE is allowed to perform HO only during T1 to T2.

2.	For eMTC NTN, introduce a location-based conditional reconfiguration trigger based on condEventD1 in NR, where the event will be satisfied if the distance between the UE and a first reference location (e.g. within the serving cell) is above a threshold, and the distance between the UE and a second reference location (e.g. within a neighbour cell) is below a threshold. (similar to condEventD1 in NR)

3.	For eMTC NTN, introduce event A4 based conditional trigger (similar to condEventA4 in NR).

4.	FFS whether time and location-based trigger conditions may be configured independently (i.e., without a jointly configured event A4 measurement condition) for eMTC NTN.

Also reelecting in RRC TS 36.331 running CR by [8]:

condEventT1-r18                                 SEQUENCE {

t1-Threshold-r18                 			INTEGER (0..549755813887),

duration-r18                     			INTEGER (1..6000)

We propose, then to remove the FFS in X2 and S1 by:

| Time Based Handover Information |  | 0..1 |  | This IE only applies to IoT NTN. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to t1-Threshold-r18 defined in TS 36.331 [16] | - |  |
| >Handover Window Duration | M |  | INTEGER (1..6000) | Corresponds to duration-r18 defined in TS 36.331 [16] |  |  |

If the companies are not confident on RAN2 running CRs, we could put an new Editorial note …

The change related to TS 36.413 is captured in [7].

Proposal 2: Agreed the TP for TS 36.423 on Time based IE(s) (Annex B & C)

# 3	Conclusion

Based on discussion above we ask RAN3 to discuss and agreed the following proposals:

Proposal 1: Agreed the TP for TS 36.300 on UE verification (Annex A)

Proposal 2: Agreed the TP for TS 36.413 and TS 36.423 on Time based IE(s) (Annex B)

# 4	References

- R3-233751,	NGAP BLCR on NTN Functionality

- R3-233784,	(BLCR to 38.300) Stage 2 BL CR for NR NTN

- R3-233799,	XnAP BLCR on NTN FunctionalityRAN3-120 Chair’s Notes

- R3-233771	(BLCR to 36.300) IoT NTN enhancements

- R3-233773	(BLCR) Support for IoT NTN enhancements

- R3-233774	(BLCR) X2AP CR on IoT NTN Functionality

- R3-234106 Time-Based HO and IoT NTN – Stage 2 Impacts, Ericsson, CATT, ESA, Huawei

- R2-2306065, Running CR - Introduction of IoT NTN enhancements

# 5	Annex A: TP for BL CR 36.300 NTN IoT

START CHANGE

### 23.21.7	MME(Re-)Selection by eNB

The eNB implements the NAS Node Selection Function specified in TS 36.410 [95].

For a RRC_CONNECTED UE, the eNB ensures that the BL UE or the UE in enhanced coverage connects to an MME that serves the country in which the UE is located. If the eNB detects that a BL UE or a UE in enhanced coverage is in a different country to that served by the serving MME, it should perform an S1 handover to change to an appropriate MME or initiate an UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

For an RRC_CONNECTED NB-IoT UE, the eNB ensures that the NB-IoT UE connects to an MME that serves the country in which the UE is located. If the eNB detects that the UE is in a different country to that served by the serving MME, it should initiate a UE Context Release Request procedure towards the serving MME (in which case the MME may decide to detach the UE).

For the purpose of selecting an appropriate MME, the EPS may verify the UE location according to TS 23.401 [17] and TS 36.305 [51] after the UE has attached to the network.

NOTE:	UE location verification for MME selection should not be necessary if NTN cell(s) do not extend across countries.

END CHANGE

# 6	Annex B: TP for BL CR 36.423 NTN IoT

START CHANGE

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – |  |
| >UE Security Capabilities | M |  | 9.2.29 |  | – |  |
| >AS Security Information | M |  | 9.2.30 |  | – |  |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – |  |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – |  |
| >E-RABs To Be Setup List |  | 1 |  |  | – |  |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoofBearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – |  |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – |  |
| >>>DL Forwarding | O |  | 9.2.5 |  | – |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – |  |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >>>DAPS Request Information | O |  | 9.2.154 |  | YES | ignore |
| >>>Ethernet Type | O |  | 9.2.157 |  | YES | ignore |
| >>>Source DL Forwarding IP Address | O |  | BIT STRING (1..160, ...) | Identifies the TNL address used by the source node for data forwarding. | YES | ignore |
| >>>Security Indication | O |  | 9.2.181 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes the HandoverPreparationInformation message as defined in subclause 10.2.2 of TS 36.331 [9], or the HandoverPreparationInformation-NB message as defined in 10.6.2 of TS 36.331 [9]. | – |  |
| >Handover Restriction List | O |  | 9.2.3 |  | – |  |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – |  |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | ignore |
| >EPC Handover Restriction List Container | O |  | 9.2.153 |  | YES | ignore |
| >Additional RRM Policy Index | O |  | 9.2.25a |  | YES | ignore |
| >NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.159 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| >UE Radio Capability ID | O |  | 9.2.171 |  | YES | reject |
| >IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | Includes the VisitedCellInfoList IE contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  | – |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB | – |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB | – |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  | – |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  | – |  |
| NR UE Security Capabilities | O |  | 9.2.107 |  | YES | ignore |
| UE Context Reference at the SgNB | O |  |  |  | YES | ignore |
| >Global en-gNB ID | M |  | 9.2.112 |  | – |  |
| >SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the SgNB. | – |  |
| Aerial UE subscription information | O |  | 9.2.129 |  | YES | ignore |
| Subscription Based UE Differentiation Information | O |  | 9.2.136 |  | YES | ignore |
| Conditional Handover Information Request | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | – |  |
| >New eNB UE X2AP ID | C-ifCHOmod |  | eNB UE X2AP ID 9.2.24 | Allocated at the target eNB | – |  |
| >New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the target eNB | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| >Conditonal Handover Time Based Information | O |  |  | This IE only applies to NTN. |  |  |
| >>Handover Window Start | M |  | INTEGER (0..549755813887) | Corresponds to t1-Threshold-r18 defined in TS 36.331 [16] | – |  |
| >>Handover Window Duration | M |  | INTEGER (1..6000 | Corresponds to duration-r18 defined in TS 36.331 [16] | – |  |
| NR V2X Services Authorized | O |  | 9.2.158 |  | YES | ignore |
| PC5 QoS Parameters | O |  | 9.2.160 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |

NEXT CHANGE

CandidateCellsToBeCancelledList ::= SEQUENCE (SIZE (1..maxnoofCellsinCHO)) OF ECGI

CHO-Probability ::= INTEGER (1..100)

CHO-HandoverWindowStart ::= INTEGER (0.. 549755813887)

CHO-HandoverWindowDuration ::= INTEGER (1..6000)

CHOinformation-AddReq ::= SEQUENCE {

source-eNB-ID				GlobalENB-ID,

source-eNB-UE-X2AP-ID		UE-X2AP-ID,

source-eNB-UE-X2AP-ID-Ext	UE-X2AP-ID-Extension											OPTIONAL,

cHO-EstimatedArrivalProbability		CHO-Probability													OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CHOinformation-AddReq-ExtIEs} }	OPTIONAL,

...

}

END CHANGE