3GPP TSG-RAN WG3 #121                                                                                              R3-234247

Toulouse, France, 21th Aug - 25th Aug 2023

Agenda Item:	15.2

Source:	CATT, CMCC,CBN

Title:	Discussion on efficient MBS reception in RAN sharing scenario

Document for:	Approval

# 1	Introduction

In last RAN3 meeting, some agreements are achieved on various aspects related to resource efficiency on MBS reception in RAN sharing scenario. In this contribution, we make further analysis based on the agreements and work assumption and provide corresponding observations and proposals.

- Discussion

2.1 Support of location dependent services

FFS on where to apply the RAN sharing optimization.

FFS on how DU determines whether RAN sharing enhancement applies to the corresponding cells, i.e. based on which information and based on which behavior.

For location dependent broadcast service, it was discussed in last RAN3 meeting.There was the aggrement made as below：

In case of location dependent broadcast services, the gNB deduces identical broadcast content from the MBS Associated Session ID and the MBS Service Area information provided by the participating 5GCs.

Companies have different understandings on the MBS session area in the above agreements.

Understanding 1: The MBS Service Areas which are from different PLMN must be exactly the same .i.e. if PLMN A decides to broadcast the MBS service in cell 1,2 and 3, PLMN B also broadcast in cell 1,2 and 3. The gNB deduces identical broadcast content from the MBS Service Area which is exactly the same and MBS Associated Session ID.

Undersatnding 2: The MBS Service Areas which are from different PLMN have the overlapping cells/TA. For example, cell 1,2,3,4 broadcast the same MBS content, and PLMN A decides to broadcast the MBS service in cell 1,2 and 3 while PLMN B decides to broadcast in cell 3 and 4. NG-RAN node deduces that PLMN A and PLMN B aimed to deliver the same broadcast content according to the overlapping cell 3.

According to clause 6.18 and 7.1.2 in TS23.247, AF decides the service area for location dependent service and sends to NEF/MBSF.

For a location dependent MBS session (see Clauses 6.2.3), the AF(s) that create the location dependent MBS sessions towards the participating PLMNs shall supply MBS Service Areas mapped to the same shared radio cells (but that may also be mapped to different non-shared radio cells).

For location dependent broadcast services, the shared NG-RAN is required to determine that the multiple broadcast MBS Sessions via different CNs deliver the same content for location-dependent MBS session with additionally considering the MBS Service Area.

If the service area provided by AF is cell ID list or TAI list, it is not possible that the same physical cells are allocated to broadcast different content by different PLMN. So, unless some cells belong to this service area are not shared, the cell list/TA list should be completely the same.

If the service area provided by AF is geographical area information, then in case one physical cell covers two different service areas (the cell is in the boundary of the service area), NEF/MBSF of each PLMN may have different decision on which service area this physical cell belongs. Different content may be broadcasted in the same physical cell from different PLMNs. However, this already exist in Rel-17, no need to further optimize.

Therefore, NG-RAN node deduces identical broadcast content based on whether there is overlapping cells/TAs among the areas identified by different area session ID, and the overlapping cells/TAs could share the radio resources.For CU/DU split scenario,CU would send the MBS area information to DU and then DU could decide to apply resource efficiency in the overlapping areas.

Proposal 1: When there is overlapping cells/TAs in the MBS areas from different PLMNs, NG-RAN node/gNB-DU deduces identical broadcast content based on whether there is overlapping cells/TAs among the areas identified by different area session ID, and the overlapping cells/TAs could share the radio resources.

2.2 Whether/How to introduce Associated Session ID  in E1AP

FFS on E1.

FFS on the definition of Associated Session ID.

In the last RAN3 meeting, there was the agreement made as below：

Add Associated Session ID IE in the same level of MBS Session ID IE in related NG/F1 messages.

However,it is still FFS whether Associated Session ID should be included in E1AP message.For multiple Cell-ID broadcast scenario,if both CU-CP and CU-UP are shared,it is not needed to provide associated session ID since PDCP entities are not shared.In case CU-UP is shared while CU-CP is not shared, CU-UP may receive multiple BC BEARER CONTEXT SETUP REQEUST messages from different CU-CPs with different MBS session ID,CU-UP should be aware that the different MBS sessions aimed at the same MBS service. Similar as in F1 interface, the most straightforward solution is to let CU-CP provide Associated session ID of each MBS session from different PLMNs to CU-UP.

For MOCN case,it is also needed for CU-CP to provide associated MBS session ID and then CU-UP could understand that MBS sessions with different MBS session ID established by different PLMNs are delievring the same MBS content.

In TS 23.003 clause 30.5, the Structure of Associated Session ID is described as follows :

The concept of Associated Session ID is defined in 3GPP TS 23.247 [140].

The Associated Session ID is used to enable NG-RAN to identify the multiple MBS sessions delivering the same content when AF creates multiple broadcast MBS Sessions via different Core Networks in network sharing scenarios.

An Associated Session ID may comprise a Source Specific IP Multicast Address (SSM) or a string. See clause 5.9.4.21.1 of 3GPP TS 29.571 [129] for the encoding of Associated Session ID in 5GC SBIs.

According to TS 29.571, Associated Session ID is a bit string, its length is not limited. In RAN3,we could just follow the definition of Associated Session ID in CT4.

Proposal 2-1: It is proposed to introduce Associated Session ID in BC BEARER CONTEXT SETUP REQEUST messages to let gNB-CU-UP be aware that different MBS sessions from different PLMN aimed at the same MBS service.

Proposal 2-2: The definition of Associated Session ID follows the decision in CT4.

In addition, gNB-CU-UP doesn’t have the information of cell/TA list, so for location dependent MBS service, CU-UP could not be aware that different MBS area session ID allocated by different PLMN are going to deliver the same MBS service.Consequently,CU-UP could not know whether a new PDCP entity should be established or a existing one could be reused.To resolve this issue, one possible solution is to introduce a new Associated MBS Area Session ID IE in MC BEARER CONTEXT SETUP REQUEST message.Since associated MBS area session ID could only be allocated by AF,an LS should be sent to SA2.

Proposal 2-3: Introduce the Associated MBS Area Session ID IE in BC BEARER CONTEXT SETUP REQEUST messages and send LS to SA2 on the requirement.

2.3 Information/procedure on establishment of shared NG-U transport resource

In the last RAN3 meeting there were agreements made as below：

Include in the MBS Session Setup or Modification Response Transfer IE an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session in case of unicast transport.

Define a new class 1 procedure to allow the gNB/CU-CP requesting the setup of shared NG-U resources during an ongoing broadcast MBS session for unicast transport.

And it is FFS whether the above agreement also applied to multicast transport as below

FFS whether an indication that the NG-RAN node has decided to not establish shared NG-U transport resources for the broadcast MBS session also applies to multicast transport.

For multicast transport, 5GC provides a source address and the NG-RAN node joins it.Currently, MB-SMF cannot detect whether a NG-RAN node decides to join IP multicast transport group or not.However,it seems there is no strong motivation to let MB-SMF be aware of the situation.On the other hand ,it may be good to align the case of IP multicast and IP unicast.We have a slight preference to also have it for IP multicast case for alignmen.

Proposal 3-1: We have a slight preference on including the indication of whether to establish NG-U transport resources or not for multicast transport.

For the new procedur, FFS on the name of the message

Option 1:Broadcast Session Modification Required/Confirm/failure

Option 2:Broadcast Distribution Request/Response/failure or Broadcast session transport request/response/failure

We have slight preference on option 2 since the only intention of the procedure is to establish NG-U tunnels. Neverthless, we are also open to another option. Similarly as multicast service, for broadcast, when setup a new MBS service, multiple tunnels for all MBS areas in the concerned NG-RAN node should be established..

Proposal 3-2: We have slight preference on option 2, i.e.introduce a new procedure with the name Broadcast Distribution Request/Response/Failure.

2.4 Decision on RAN sharing optimization and establishment of F1/NG-U tunnels

Decision on RAN sharing optimization between CU and DU

The CU-CP sends the MBS Service Area to DU and DU determines whether to apply RAN sharing optimization over the radio. FFS on the determination between CU and DU for MOCN case.

For multiple cell ID Broadcast scenario，gNB-CU-CP is not shared, so it is natural that gNB-DU determines whether to share radio resource based on Associated Cell ID. Then in case gNB-CU-CP is separate and gNB-CU-UP is shared, similarly,gNB-CU–UP determines whether to share PDCP entity based on Associated Cell ID. So for multiple cell-ID broadcast scenario,it should always be DU/CP-UP which make decision on radio resource/PDCP resource optimization.

Observation 1:For multiple cell-ID broadcast scenario,gNB-DU/gNB-CU-UP make decision on radio resource/PDCP resource optimization.

For MOCN scenario，gNB-CU can choose to send multiple F1AP setup/modification messages, or it is also sufficient for gNB-CU to send only one message including a list of TMGIs. If gNB-CU sends only one message, whenever a new PLMN join the MBS service, gNB-CU needs to send a modification message to update the TMGI list. Considering the number of modification messages that need to be sent, one message including a list of TMGIs does not achieve much optimization.Neverthless,we are open on the optimization.In E1 interface,since it is necessary to provide information per PLMN on whether the NG-U tunnel is established or not and NG-U DL TEID,it is not possible to use one E1 message for multiple PLMNs without further enhancement.

Observation 2: For MOCN scenario,it is possible to do further optimization on F1 interface by  using one message including a list of TMGI. However,this solution does not applied in E1 interface unless further more enhancement is introduced.

With all of the scenarios considered,we suggest that gNB-CU sends multiple F1AP/E1AP setup/modification messages in both multiple cell ID scenario and MOCN scenario as a baseline. In this case, gNB-DU determines whether to share radio resources, and shared gNB-CU-UP determines whether to share the PDCP entity.We could further discuss whether to do some enhancement for MOCN scenario via using one F1APmessage for multiple PLMNs

Proposal 4-1: As a baseline,the decision of radio resource sharing is made by DU, and the decision of PDCP layer sharing is made by shared gNB-CU-UP. Enhancement on MOCN scenario is not precluded.

Decision on the number of F1-U tunnels

FFS on how many F1-U tunnels should be established for multiple cell-ID broadcast cases.

For MOCN scenario,it is already agreed that one shared F1-U tunnel is used.For multiple cell-ID broadcast scenario, PDCP PDUs for the same MBS service could be delivered from different CU-UP to the same DU.The situation is very similar as what it is in NG-U interface for RAN sharing scenario i..e multiple NG-U tunnels for the same MBS service. Thereby, it seems also not necessary to always estaliblish F1-U towards all CU-UPs which belong to different PLMNs.

Then one question arise is which node makes decision on how many F1-U tunnels should be established.

Since sepatate CU-CP/CU-UP does not know whether the same broadcast MBS service is already established for other PLMNs or not, there is no way for CU-CP/CU-UP to make decision on the establishement of F1-U tunnel.While in DU side,it has the information on MBS service establishment of all CU-UPs,thereby it could decide whether to establish F1-U tunnel for one specific PLMN.So,one feasible way is that CU-UP always provides the uplink TEID of F1-U tunnel i.e. F1-U TNL CU and after CU-CP provides the information to DU via BROADCAST CONTEXT SETUP/MDOFICAION REQUEST message, DU provide information on whether the F1-U tunnel needs to be established in BROADCAST CONTEXT SETUP/MDOFICAION RESPONSE message. With that, it is proposed to introduce the new Establishment of F1-U Tunnel IE in BROADCAST CONTEXT SETUP/MODIFICATION RESPONSE message to indicate whether to establish F1-U tunnel.

Proposal 4-2: It is not necessary to always estaliblish F1-U towards all CU-UPs which belong to different PLMNs

Proposal 4-3: It is proposed to let gNB-DU make decision on whether to establish F1-U tunnel towards one specific CU-UP. And it is proposed to introduce the new F1-U not established IE in BROADCAST CONTEXT SETUP/MODIFICATION RESPONSE message to indicate gNB-CU not to establish F1-U tunnel.

Proposal 4-4:It is proposed to introduce F1-U not established IE in BC Bearer Context To Modify IE which in E1 BC BEARER CONTEXT MODIFCIATION REQUEST message to enable CU-CP to inform CU-UP whether F1 tunnel is established or not.

If it is agreed that it is not necessary to establish F1-U tunnel towards all CU-UPs for multiple cel- ID broadcast sceanrio,then similar as in NGAP,if one PLMN decides to release the MBS service while F1-U tunnel is only established for this PLMN,it is needed for Gnb-DU to trigger the establishment of F1-U towards another CU-UP.To support this function,it is needed to introduce a new procedure i.e. Broadcast Context Distribution procedure (name is FFS).

Proposal 4-5： It is proposed to introduce Broadcast Context Distribution procedure (name is FFS) which enables gNB-DU node to trigger the establishment of F1-U.

Decision on NG-U tunnels establishment.

For this bullet,there are serveral issues that need to  be discussed

Issue1: Which node makes decision on whether the NG-U tunnel should be established or not i.e. CU-CP or CU-UP

Issue 2: For multiple cell-ID broadcast scenario with separate CU-UP,how many NG-U tunnels should be established?

For the first issue, both of the possible options, i.e. CU makes decision and DU makes decision ,could work.However,considering tunnel management function is located in  gNB-CU-UP,we have a slight preference on letting gNB-CU-UP make decision itself. With this,it is required for gNB-CU-UP to provide the indication on whether the NG-U transport resources are established or not in the BC BEARER CONTEXT SETUP RESPONSE message.

For the second issue, similar as MOCN scenario,we think it is not necessary to eastablish NG-U for all PLMNs since F1-U tunnels are not mandatory for all CU-UPs as proposed in proposal 4-2.CU-UP could make the decision on whether to establish the NG-U tunnel based on whether F1-U tunnel is established,i.e.NG-U tunnel should always be established if F1-U tunnel is setup for this CU-UP,otherwise,it completely depends on CU-UP decision.

Proposal 4-6: We have slight preference on gNB-CU-UP making decision on establishment of NG-U tunnel. With that,it is proposed to introduce the Shared NG-U Not Established IE in BC Bearer Context To Setup Response IE which in the BC BEARER CONTEXT SETUP RESPONSE message to indicate to gNB-CU-CP that it is not needed to establish shared NG-U tunnel.

Proposal 4-7: It is not necessary to establish NG-U tunnel for all separate CU-UPs for multiple cell-ID broadcast scenario

2.5 The overall procedure for Broadcast MBS Session Setup

Based on the discussion above, we provide an overall Broadcast service setup procedure for multiple Cell-ID broadcast scenario.In MOCN scenario, this procedure could be simplified.Here, we suggest to first discuss a common procedure which applied to both of the scenarios.

The overall procedure for Broadcast MBS Session Setup (multiple cell-ID broadcast case)

Figure 1: The overall procedure for Broadcast MBS Session Setup

- The PLMN1 5GC starts the broadcast session by sending the NGAP Broadcast Session Setup Request message to the gNB-CU1-CP containing 5GC NG-U TNL 1, Associated Session ID and so on.

- The PLMN2 5GC sends the NGAP Broadcast Session Setup Request message to the gNB-CU1-CP containing 5GC NG-U TNL 2, Associated Session ID and so on.

  - The gNB-CU1-CP sets up the broadcast bearer context, providing 5GC NG-U TNL 1 and from the 5GC to the gNB-CU1-UP.

  - The gNB-CU2-CP sets up the broadcast bearer context,providing 5GC NG-U TNL 2 from the 5GC to the gNB-CU2-UP.

Note: For MOCN scenario and multiple cell-ID broadcast scenario with shared CU-UPscenario, Associated Session ID IE and a new Area Associated Session ID IE should be included in step 3-1/3-2 to enable CU-UP make decision on RAN sharing improvement in CU-UP. The Area Associated Session ID is used to associate MBS service areas which broadcast same MBS content.

  - The gNB-CU1-CP receives from the gNB-CU1-UP the NG-U unicast transport address.i.e. gNB NG-U TNL 1and F1-U TNL CU1-UP. gNB-CU1-UP would provide information on whether to establish NG-U tunnel information

  - The gNB-CU2-CP receives from the gNB-CU2-UP the NG-U unicast transport address.i.e. gNB NG-U TNL 2 and F1-U TNL CU2-UP.

Note :For MOCN and multiple cell-ID broadcast scenario with shared CU-UP scenario, gNB-CU2-UP would provide information on whether to establish NG-U tunnel information in step 4.In the meanwhile ,it could also provide the same F1-U TNL address for all PLMNs.

5-1/6-1	The gNB-CU1-CP establishes the Broadcast MBS Session Context at the shared gNB-DU, providing the F1-U TNL CU1, Associated Session ID. In the response message, DU inform CU-CP that F1-U tunnel is established and provide F1-U TNL DU1 to CU-CP.

5-1/6-1	The gNB-CU1-CP establishes the Broadcast MBS Session Context at the shared gNB-DU, providing the F1-U TNL CU1, Associated Session ID. In the response message, DU informs CU-CP that F1-U tunnel is not established.

7-1/8-1		The gNB-CU1-CP triggers BC Bearer Context Modification Request towards the gNB-CU1-UP to provide the F1-U TNL DU1.

7-2/8-2		The gNB-CU1-CP triggers BC Bearer Context Modification Request towards the gNB-CU1-UP to inform Gnb-CU-UP that F1-U tunnel is not established.Then in the response message,CU2-UP indicate not to establish NG-U tunnel.

9	The gNB-CU1 CP successfully terminates the NGAP broadcast Session Setup procedureand providing the NG-U unicast .i.e. gNB NG-U TNL 1 to the PLMN1 5GC.

10	The gNB-CU2 CP sends the NGAP Broadcast Session Setup Response message to5GC from PLMN2 indicating the shared NG-U tunnel is not established.

The overall procedure for Broadcast MBS Session release (multiple cell-ID broadcast case)

Figure 3 illustrates the overall procedure for Broadcast MBS Session Release for CP and UP not shared case.

Figure 2: The overall procedure for Broadcast MBS Session Release

Assumption: The NG-U tunnel and F1-U tunnel are only established for PLMN1.

1	The PLMN1 5GC sends the NGAP Broadcast Session Release Request message to the gNB-CU1-CP.

2	The gNB-CU1-CP sends the F1AP Broadcast Context Release Command message to the shared gNB-DU.

3	The shared gNB-DU triggers F1AP Broadcast Context Distribution Setup Request (name FFS) towards the gNB-CU2-CP to provide F1-U TNL DU2.

4	The gNB-CU2-CP triggers BC Bearer Context Modification Request towards the gNB-CU2-UP to provide the F1-U TNL DU2.

5	The gNB-CU2-UP decides to establish NG-U tunnel since F1-U tunnel is requested while there is no NG-U tunnel established for this MBS service.In the response message,NG-U Tunnel at gNB and F1-U tunnel at CU-UP are provided.Meanwhile,gNB-CU-UP indicate to gNB-CU-CP that NG-U tunnel needs to be established.

6	The gNB-CU2-CP triggers NGAP Broadcast Distribution Setup Response message (name FFS) towards the PLMN2 5GC to provide gNB NG-U TNL 2.

7	The PLMN2 5GC provides 5GC NG-U TNL 2 to gNB-CU2-CP by sending a NGAP Broadcast Distribution Setup Response message(name FFS).

8-9  gNB-CU-CP provide NG-U TNL at 5GC to gNB-CU-UP via BC Bearer Context Modification procedure.

10	The gNB-CU2-CP provides F1-U TNL CU2 to DU by sending a F1AP Broadcast Context Distribution Setup Response message(name FFS)

11	The shared gNB-DU successfully terminates F1AP Broadcast Context Release procedure.

12	The gNB-CU2-CP successfully terminates NGAP Broadcast Session Release procedure. The NG-RAN node shall stop broadcasting and release all MBS session resources associated with the broadcast session.

Proposal 5： It is proposed to discuss above procedures and have an agreement on it.

2.6 PDCP configurations for RAN sharing with multiple Cell ID broadcast

Open issue 1: Which option should be used by DU to decide the MRB-PDCP-ConfigBroadcast?

Option 1: OAM configure the MRB-PDCP-ConfigBroadcast of each MRB to CU or DU.

Option 2: DU applied the MRB-PDCP-ConfigBroadcast of the first CU or the second CU. FFS whether DU needs to provide the used MRB-PDCP-ConfigBroadcast to CU.

Any action in RAN3 needed on OAM based solution?

For MOCN scenario, since gNB-CU is shared, it is natural that the same MRB is allocated in the shared gNB-CU. However, for multiple cell-ID broadcast scenarios, it is difficult for different gNB-CUs to have the same flow to MRB mapping for the same MBS sessions since the gNB-CU from different PLMN are deployed separately. In previous RAN3 meeting, two solutions i.e.OAM configures the PDCP configuration and signaling based solution were proposed. Both of the above solutions can work, but configuration based solution is too staticit, and it is not workable to configure the PDCP configuration by OAM since PDCP configuration is per DRB which is dynamic. Therefore, we have a slight preference on signaling based solution.

Proposal 6-1：We have slight preference on option 2, i.e.we prefer the signaling based solution rather than OAM based solution for RAN sharing with multiple Cell ID broadcast.

Since the Multiple Cell ID scenario is quite similar with shared CU-UP scenario discussed in Rel-17, we think the mechanism for Rel-17 shared CU-UP scenario could be reused, i.e. CU-CP informs DU whether DU could use the available configruaiont or requested configuration. As to how the DU selects the configuration is used, we suggest that gNB-DU uses the flow to MRB mapping decided by the first gNB-CU which requests to establish the broadcast session and then informs to other gNB-CUs of the information.

As to whether DU needs to provide the used MRB-PDCP-ConfigBroadcast to CU, we think it is not necessary since there seems no subsequent bahaviour of CU-CP even it receives the used MRB-PDCP-ConfigBroadcast.

Figure 4 illustrates an examplified interaction of F1AP about the establishment of MRB in multiple cell-IDs broadcast scenario.

Figure 3: Establishment of MRB in multiple cell-IDs broadcast scenario

It is assumed that gNB-DU uses the flow to MRB mapping decided by the first gNB-CU:

1.	The shared gNB-CU1 sends BROADCAST CONTEXT SETUP REQUEST message to the shared gNB-DU, providing MBS session ID, MRB configuration, and the new Requested Action for Multiple Cell-IDs Broadcast IE which includes three values: apply available configuration, apply requested configuration,

Since gNB-CU1 is the first CU to setup the MBS context, gNB DU just follows the configuration from this CU.

2.	After the shared gNB-DU setup MRB successfully, the gNB-DU replies to the gNB-CU1 with BROADCAST CONTEXT SETUP RESPONSE.

3.	Then, the gNB-CU2 sends BROADCAST CONTEXT SETUP REQUEST message to the shared gNB-DU. And if the Requested Action for Multiple Cell-IDs Broadcast IE is set to “apply available configuration”, the gNB-DU will follow the previous PDCP configuration provided by the first gNB-CU.i e. gNB-CU1. And if the Requested Action for Multiple Cell-IDs Broadcast IE is set to “apply requested configuration”,gNB-DU will setup the another MRB using the PDCP configuration decided by the gNB-CU2.

4.	After the shared gNB-DU setup MRB successfully, the gNB-DU replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

5/6.	Same as 3/4 until gNB-DU completed the establishment of MRB providing by all shared gNB-CUs.

Note: If the first CU-CP decides to release the broadcast MBS service,there are two cases

Case 1: There are backup F1-U tunnels.In this case,DU determines which CU data to use and the PDCP configuration. (MCCH may update at this time).

Case 2: No other F1-U tunnels exist.In this case DU sends Broadcast Context Distribution Setup message(name FFS) to CU to establish F1-U tunnel.

Proposal 6-2: For Multiple cell-ID broadcast scenarios, it is proposed to reuse the mechanism for shared CU-UP. It is proposed to introduce the new Requested Action for Multiple Cell-IDs Broadcast IE in BROADCAST CONTEXT SETUP REQUEST.

Proposal 6-3: It is not necessary for gNB-DU to provide the current MRB configuration to the gNB-CU.

2.7 Other issues

MBS CU to DU RRC Information

In the Broadcast Context Setup procedure, the mtch-neighbourCell info to be exchanged between the gNB-CU and gNB-DU is optional, the tabular specified in TS 38.473 is shown as below:

#### 9.3.1.225	MBS CU to DU RRC Information

This IE indicates the MBS CU to DU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Broadcast Cell List | M |  |  |  |
| >MBS Broadcast Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>mtch-neighbourCell | O |  | OCTET STRING | Includes the mtch-neighbourCell contained in the MBS-SessionInfoList IE, as defined in TS 38.331[8] |
| MBS Broadcast MRB List |  | 1 |  |  |
| >MBS Broadcast MRB Item |  | 1 .. <maxnoofMRBs> |  |  |
| >>MRB ID | M |  | 9.3.1.224 |  |
| >>MRB PDCP Config Broadcast | M |  | OCTET STRING | Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331[8]. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofMRBs | Maximum no. MRBs allowed to be setup for one MBS session, the maximum value is 32. |

DU may receive more than 8 mtch-neighbourCell from multiple CU-CPs.in this case,it could depends on DU implementation how to broadcast in MCCH.

Proposal 7-1: In case the number of mtch-neighbourCell received from different CU-CPs exceeds 8,it depends on gNB-DU implementation on how to broadcast in MCCH.

OAM based solution

One controversial issue discussed in previous RAN3 meeting is whether to capture OAM based solution in RAN3 spec.

For OAM based solution,in 38.300,it seems there are only two places which mentions OAM based solution as below

NOTE:	Information about the availability of the N26 interface may be configured by OAM at the NG-RAN.

The NG-RAN node may determine the gNB ID length of the candidate gNB based on, e.g.OAM configuration or UE

So,it seems OAM based solution usually is not officially captured in the spec.If there is strong wish to capture in the spec,we could add a note in the stage 2 which describe the assocated session IE could also be configured by OAM.

As to whether to coordinate with SA2 on this issue,from our point of view,maybe it is not suitable to ask other working group to remove something from the spec.But it would be good to communite with SA2 on the worksplit to gurantee the alignment among different working groups.And the LS could be general rather than MBS specific.

Proposal 7-2: For OAM based solution,one possible way to capture in the stage 2 spec is to add a note in 38.300 to clarify that assocated session could also be configured by OAM.And it would also be helpful to discuss an LS to SA2 on the worksplit to avoid duplicated/inconsistent work between working groups .

# 3	Conclusion

Based on the discussion in section 2, we have the following proposals

Support of location dependent services

Proposal 1: When there is overlapping cells/TAs in the MBS areas from different PLMNs, NG-RAN node/gNB-DU deduces identical broadcast content based on whether there is overlapping cells/TAs among the areas identified by different area session ID, and the overlapping cells/TAs could share the radio resources.

Whether/How to introduce Associated Session ID  in E1AP

Proposal 2-1: It is proposed to introduce Associated Session ID in BC BEARER CONTEXT SETUP REQEUST messages to let gNB-CU-UP be aware that different MBS sessions from different PLMN aimed at the same MBS service.

Proposal 2-2: The definition of Associated Session ID follows the decision in CT4.

Information/procedure on establishment of shared NG-U transport resource

Proposal 3-1: We have a slight preference on including the indication of whether to establish NG-U transport resources or not for multicast transport.

Proposal 3-2: We have slight preference on option 2, i.e.introduce a new procedure with the name Broadcast Distribution Request/Response/Failure.

Decision on RAN sharing optimization

Observation 1:For multiple cell-ID broadcast scenario,gNB-DU/gNB-CU-UP make decision on radio resource/PDCP resource optimization.

Observation 2: For MOCN scenario,it is possible to do further optimization on F1 interface by  using one message including a list of TMGI. However,this solution does not applied in E1 interface unless further enhancement is introduced.

Proposal 4-1: As a baseline,the decision of radio resource sharing is made by DU, and the decision of PDCP layer sharing is made by shared gNB-CU-UP. Enhancement on MOCN scenario is not precluded.

Establishment of F1-U tunnel for multiple cell-ID broadcast scenario

Proposal 4-2: It is not necessary to always estaliblish F1-U towards all CU-UPs which belong to different PLMNs

Proposal 4-3: It is proposed to let gNB-DU make decision on whether to establish F1-U tunnel towards one specific CU-UP. And it is proposed to introduce the new F1-U not established IE in BROADCAST CONTEXT SETUP/MODIFICATION RESPONSE message to indicate gNB-CU not to establish F1-U tunnel.

Proposal 4-4:It is proposed to introduce F1-U not established IE in BC Bearer Context To Modify IE which in E1 BC BEARER CONTEXT MODIFCIATION REQUEST message to enable CU-CP to inform CU-UP whether F1 tunnel is established or not.

Proposal 4-5： It is proposed to introduce Broadcast Context Distribution procedure (name is FFS) which enables gNB-DU node to trigger the establishment of F1-U.

Establishment of NG-U tunnel

Proposal 4-6: We have slight preference on gNB-CU-UP making decision on establishment of NG-U tunnel. With that,it is proposed to introduce the Shared NG-U Not Established IE in BC Bearer Context To Setup Response IE which in the BC BEARER CONTEXT SETUP RESPONSE message to indicate to gNB-CU-CP that it is not needed to establish shared NG-U tunnel.

Proposal 4-7: It is not necessary to establish NG-U tunnel for all separate CU-UPs for multiple cell-ID broadcast scenario

The overall procedure for Broadcast MBS Session Setup

Proposal 5： It is proposed to discuss above procedures and have an agreement on it.

PDCP configurations for RAN sharing with multiple Cell ID broadcast

Proposal 6-1：We have slight preference on option 2, i.e.we prefer the signaling based solution rather than OAM based solution for RAN sharing with multiple Cell ID broadcast.

Proposal 6-2: For Multiple cell-ID broadcast scenarios, it is proposed to reuse the mechanism for shared CU-UP. It is proposed to introduce the new Requested Action for Multiple Cell-IDs Broadcast IE in BROADCAST CONTEXT SETUP REQUEST.

Proposal 6-3: It is not necessary for gNB-DU to provide the current MRB configuration to the gNB-CU.

Other issues

Proposal 7-1: In case the number of mtch-neighbourCell received from different CU-CPs exceeds 8,it sepends on gNB-DU implementation on how to broadcast in MCCH.

Proposal 7-2: For OAM based solution,one possible way to capture in the stage 2 spec is to add a note in 38.300 to clarify that assocated session could also be configured by OAM.And it would also be helpful to discuss an LS to SA2 on the worksplit to avoid duplicated/inconsistent work between working groups .

The TP for F1AP, E1AP, NGAP and 38.401 are provided in annex.

# 4	Reference

[1]	RAN3#120 Chair Notes

# 5	TP for BLCR on 38.401

7.7.x	Support of resource efficiency for RAN Sharing

7.7.x1	General

Editor’s Note:	TBD

7.7.x2	Support of resource efficiency for MOCN

F1-AP supports resource efficiency in RAN sharing for MOCN.

For one MBS session, gNB-CU initiates one Broadcast Context Setup procedure for multiple PLMNs to establish

MBS context in the shared gNB-DU.

7.7.x3	Support of resource efficiency for RAN Sharing with multiple cell-ID broadcast

F1AP supports resource efficiency in RAN Sharing with multiple cell-ID broadcast.

gNB-DUs sharing the same physical cell resources receive via F1-C information enabling identifying broadcast MBS sessions providing identical content.

Applying resource efficiency for RAN Sharing with multiple cell-ID broadcast

-	resolve different QoS requirements received from the participating 5GCs in an implementation specific way.

-    apply the MRB configuration from the first gNB-CU which requests the establishment of MBS context（To be updated based on the conclusion in this meeting）.

Editor’s Note:	Whether specific text for location dependent MBS sessions is necessary is FFS.

# 6	TP for BLCR on 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.17.1	Broadcast Session Setup

#### 8.17.1.1	General

The purpose of the Broadcast Session Setup procedure is to request the NG-RAN node to setup MBS session resources for a broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.1.2	Successful Operation

Figure 8.17.1.2-1: Broadcast Session Setup, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION SETUP REQUEST message to the NG-RAN node. If the NG-RAN node accepts all the MBS QoS flows in the MBS session at least in one of its cells, the NG-RAN node responds with the BROADCAST SESSION SETUP RESPONSE message.

If the MBS Service Area IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account as specified in TS 23.247 [44].

If the MBS Session FSA ID List IE is included in the BROADCAST SESSION SETUP REQUEST message, the NG-RAN node shall take it into account to determine cells/frequencies within the MBS service area to broadcast MBS session data as specified in TS 23.247 [44].

If the Associated Session ID IE is included in the BROADCAST SESSION SETUP REQUEST message the NG-RAN node shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.300 [8]. If the NG-RAN node decides to not establish shared NG-U resources towards the 5GC it shall include the Shared NG-U Not Established IE set to "not established" in the MBS Session Setup or Modification Response Transfer IE.

#### 8.17.1.3	Unsuccessful Operation

Figure 8.17.1.3-1: Broadcast Session Setup, unsuccessful operation.

If the NG-RAN node is not able to provide the requested MBS session resources for all the MBS QoS flows in the MBS session in any of its cells, it shall send the BROADCAST SESSION SETUP FAILURE message.

#### 8.17.1.4		Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.17.X	Broadcast Distribution Setup

#### 8.17.X.1	General

The purpose of the Broadcast Distribution Setup procedure is to assign NG-U resources for a broadcast MBS session. The procedure uses non-UE-associated signalling.

#### 8.17.X.2	Successful Operation

Figure 8.17.X.2-1: Broadcast Distribution Setup, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST DISTRIBUTION SETUP REQUEST message to the AMF. The AMF responds with a BROADCAST DISTRIBUTION SETUP RESPONSE message.

#### 8.17.X.3	Unsuccessful Operation

Figure 8.17.X.3-1: Broadcast Distribution Setup, unsuccessful operation.

In case the shared NG-U transport cannot be setup successfully, the AMF shall respond with the BROADCAST DISTRIBUTION SETUP FAILURE message to the NG-RAN node with an appropriate cause value.

#### 8.17.X.4	Abnormal Conditions

Void.

### 8.17.Y	Broadcast Distribution Release

#### 8.17.Y.1	General

The purpose of the Broadcast Distribution Release procedure is to enable the release of already established NG-U resources for a given broadcast MBS session, or for a given area session of the broadcast MBS session. The procedure uses non-UE-associated signalling.

#### 8.17.Y.2	Successful Operation

Figure 8.17.Y.2-1: Broadcast Distribution Release, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the BROADCAST DISTRIBUTION RELEASE REQUEST message, the AMF shall send the BROADCAST DISTRIBUTION RELEASE RESPONSE message after successfully removing the corresponding NG-U resource for the MBS session.

#### 8.17.Y.3		Unsuccessful Operation

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 9.2	Message Functional Definition and Content

#### 9.2.16.1	BROADCAST SESSION SETUP REQUEST

This message is sent by the AMF to establish MBS session resources for a broadcast MBS session.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.24 |  | YES | reject |
| MBS Service Area | M |  | 9.3.1.208 |  | YES | reject |
| MBS Session Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Session Setup or Modification Request Transfer IE specified in subclause 9.3.5.3 | YES | reject |
| Associated Session ID | O |  | 9.3.1.y |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.16.a	BROADCAST DISTRIBUTION SETUP REQUEST

This message is sent by the NG-RAN node to request the setup of the NG-U transport for a broadcast MBS session.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| MBS Distribution Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Request Transfer IE specified in subclause 9.3.5.7. | YES | reject |

#### 9.2.16.b	BROADCAST DISTRIBUTION SETUP RESPONSE

This message is sent by the AMF to confirm the setup of the NG-U transport.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| Broadcast Distribution Setup Response Transfer | M |  | OCTET STRING | Containing the Broadcast Distribution Setup Response Transfer IE specified in subclause 9.3.5.X | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.16.c	BROADCAST DISTRIBUTION SETUP FAILURE

This message is sent by the AMF to indicate that the setup of the NG-U transport was unsuccessful.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| MBS Distribution Setup Unsuccessful Transfer | M |  | OCTET STRING | Containing the MBS Distribution Setup Unsuccessful Transfer IE specified in subclause 9.3.5.9. | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.16.d	BROADCAST DISTRIBUTION RELEASE REQUEST

This message is sent by the NG-RAN node to request the release of the NG-U transport.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| MBS Distribution Release Request Transfer | M |  | OCTET STRING | Containing the MBS Distribution Release Request Transfer IE specified in subclause 9.3.5.10. | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.16.e	BROADCAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the AMF to confirm the release of the NG-U transport.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Area Session ID | O |  | 9.3.1.207 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.y	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | OCTET STRING | Editor’s Note: coding is FFS |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	MBS Assistance Information

This IE provides MBS Assistance Information as specified in TS 38.300 [8] and TS 23.247 [44].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Assistance Information | M |  | ENUMERATED (true, ...) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.2.X	Shared NG-U Broadcast TNL Information

This IE provides the shared NG user plane transport layer information per Area Session ID associated with an MBS session at the 5GC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP Broadcast Address | M |  | Transport Layer Address 9.3.2.4 |  |
| IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |
| GTP-TEID at 5GC | M |  | GTP-TEID 9.3.2.5 |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.5.X	Broadcast Distribution Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  |
| Shared NG-U Broadcast Information List |  | 1 |  |  |
| >Shared NG-U Broadcast Information Item |  | 0..<maxnoofMBSAreaSessionIDs> |  |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  |
| >>Shared NG-U Broadcast TNL Information | O |  | 9.3.2.X |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs allowed within one MBS session. Value is 256. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.211	MBS Session Setup Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup Request List |  | 1 |  |  | - |  |
| >MBS Session Setup Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS Assistance Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

#### 9.3.1.212	MBS Session Setup or Modify Request List

This IE provides information related to MBS sessions joined by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session Setup or Modify Request List |  | 1 |  |  | - |  |
| >MBS Session Setup or Modify Request Item |  | 1..<maxnoofMBSSessions> |  |  | - |  |
| >>MBS Session ID | M |  | 9.3.1.206 |  | - |  |
| >>MBS Area Session ID | O |  | 9.3.1.207 |  | - |  |
| >>Associated MBS QoS Flow Setup or Modify Request List |  | 0..1 |  |  | - |  |
| >>>Associated MBS QoS Flow Setup or Modify Request Item |  | 1..<maxnoofMBSQoSflows> |  |  | - |  |
| >>>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>>>Associated Unicast QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.51 |  | - |  |
| >>MBS QoS Flow To Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS Flow Identifiers of the MBS QoS Flows to be released. | - |  |
| >>MBS Assistance Information | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessions | Maximum no. of MBS sessions allowed within one PDU session. Value is 32. |
| maxnoofMBSQoSflows | Maximum no. of MBS QoS flows allowed within one MBS session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.5.5	MBS Session Setup or Modification Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session TNL Information NG-RAN | O |  | 9.3.2.17 |  | - |  |
| Shared NG-U Not Established | O |  | ENUMERATED (true, ...) | Editor’s Note: whether this IE is applicable for both, unicast and multicast NG-U transport or only for unicast transport is FFS | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

BroadcastSessionReleaseResponse,

BroadcastSessionSetupFailure,

BroadcastSessionSetupRequest,

BroadcastSessionSetupResponse,

BroadcastDistributionReleaseRequest,

BroadcastDistributionReleaseResponse,

BroadcastDistributionSetupFailure,

BroadcastDistributionSetupRequest,

BroadcastDistributionSetupResponse,

CellTrafficTrace,

ConnectionEstablishmentIndication,

DeactivateTrace,

DistributionReleaseRequest,

DistributionReleaseResponse,

DistributionSetupFailure,

DistributionSetupRequest,

DistributionSetupResponse,

////////////////////////////////No change/////////////////////////////////

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionModification,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

id-BroadcastDistributionSetup,

id-BroadcastDistributionRelease,

id-CellTrafficTrace,

id-ConnectionEstablishmentIndication,

id-DeactivateTrace,

id-DistributionSetup,

id-DistributionRelease,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

broadcastDistributionSetup		|

broadcastDistributionRelease	|

distributionSetup				|

distributionRelease				|

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

aMFConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFConfigurationUpdate

SUCCESSFUL OUTCOME		AMFConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	AMFConfigurationUpdateFailure

PROCEDURE CODE			id-AMFConfigurationUpdate

CRITICALITY				reject

}

aMFCPRelocationIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFCPRelocationIndication

PROCEDURE CODE			id-AMFCPRelocationIndication

CRITICALITY				reject

}

aMFStatusIndication NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		AMFStatusIndication

PROCEDURE CODE			id-AMFStatusIndication

CRITICALITY				ignore

}

broadcastSessionModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequest

SUCCESSFUL OUTCOME		BroadcastSessionModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationFailure

PROCEDURE CODE			id-BroadcastSessionModification

CRITICALITY				reject

}

broadcastSessionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastSessionReleaseResponse

PROCEDURE CODE			id-BroadcastSessionRelease

CRITICALITY				reject

}

broadcastSessionReleaseRequired NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequired

PROCEDURE CODE			id-BroadcastSessionReleaseRequired

CRITICALITY				reject

}

broadcastSessionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionSetupRequest

SUCCESSFUL OUTCOME		BroadcastSessionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionSetupFailure

PROCEDURE CODE			id-BroadcastSessionSetup

CRITICALITY				reject

}

broadcastDistributionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastDistributionSetupRequest

SUCCESSFUL OUTCOME		BroadcastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastDistributionSetupFailure

PROCEDURE CODE			id-BroadcastDistributionSetup

CRITICALITY				reject

}

broadcastDistributionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastDistributionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastDistributionReleaseResponse

PROCEDURE CODE			id-BroadcastDistributionRelease

CRITICALITY				reject

}

cellTrafficTrace NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-CellTrafficTrace

CRITICALITY				ignore

}

connectionEstablishmentIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ConnectionEstablishmentIndication

PROCEDURE CODE			id-ConnectionEstablishmentIndication

CRITICALITY				reject

}

deactivateTrace NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

distributionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DistributionSetupRequest

SUCCESSFUL OUTCOME		DistributionSetupResponse

UNSUCCESSFUL OUTCOME	DistributionSetupFailure

PROCEDURE CODE			id-DistributionSetup

CRITICALITY				reject

}

distributionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DistributionReleaseRequest

SUCCESSFUL OUTCOME		DistributionReleaseResponse

PROCEDURE CODE			id-DistributionRelease

CRITICALITY				reject

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AllowedNSSAI,

AMFName,

AMFSetID,

AMF-TNLAssociationSetupList,

AMF-TNLAssociationToAddList,

AMF-TNLAssociationToRemoveList,

AMF-TNLAssociationToUpdateList,

AMF-UE-NGAP-ID,

AssistanceDataForPaging,

AssociatedSessionID,

AuthenticatedIndication,

BroadcastCancelledAreaList,

BroadcastCompletedAreaList,

BroadcastDistributionSetupResponseTransfer,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-AMF-TNLAssociationToUpdateList,

id-AMFTrafficLoadReductionIndication,

id-AMF-UE-NGAP-ID,

id-AssistanceDataForPaging,

id-AssociatedSessionID,

id-AuthenticatedIndication,

id-BroadcastCancelledAreaList,

id-BroadcastCompletedAreaList,

id-BroadcastDistributionSetupResponseTransfer,

id-CancelAllWarningMessages,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MBS-DistributionSetupResponseTransfer,

id-MBS-DistributionSetupUnsuccessfulTransfer,

id-MBSSessionModificationFailureTransfer,

id-MBSSessionModificationRequestTransfer,

id-MBSSessionModificationResponseTransfer,

id-MBSSessionReleaseResponseTransfer,

id-MBSSessionSetupFailureTransfer,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- Broadcast Session Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION REQUEST

--

-- **************************************************************

BroadcastSessionModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationRequestIEs} },

...

}

BroadcastSessionModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea									CRITICALITY reject	TYPE MBS-ServiceArea									PRESENCE optional		}|

{ ID id-MBSSessionModificationRequestTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBSSessionSetupOrModRequestTransfer)		PRESENCE optional		}|

{ ID id-AssociatedSessionID							CRITICALITY ignore	TYPE AssociatedSessionID							PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE RESPONSE

--

-- **************************************************************

BroadcastSessionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseResponseIEs} },

...

}

BroadcastSessionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-MBSSessionReleaseResponseTransfer		CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBSSessionReleaseResponseTransfer)		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Distribution Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

BroadcastDistributionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionSetupRequestIEs} },

...

}

BroadcastDistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupRequestTransfer			CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupRequestTransfer)			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

BroadcastDistributionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionSetupResponseIEs} },

...

}

BroadcastDistributionSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-BroadcastDistributionSetupResponseTransfer	CRITICALITY reject	TYPE OCTET STRING (CONTAINING BroadcastDistributionSetupResponseTransfer)		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION SETUP FAILURE

--

-- **************************************************************

BroadcastDistributionSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionSetupFailureIEs} },

...

}

BroadcastDistributionSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionSetupUnsuccessfulTransfer	CRITICALITY ignore	TYPE OCTET STRING (CONTAINING MBS-DistributionSetupUnsuccessfulTransfer)	PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Distribution Release Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION RELEASE REQUEST

--

-- **************************************************************

BroadcastDistributionReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { BroadcastDistributionReleaseRequestIEs} },

...

}

BroadcastDistributionReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-MBS-DistributionReleaseRequestTransfer		CRITICALITY reject	TYPE OCTET STRING (CONTAINING MBS-DistributionReleaseRequestTransfer)		PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST DISTRIBUTION RELEASE RESPONSE

--

-- **************************************************************

BroadcastDistributionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastDistributionReleaseResponseIEs} },

...

}

BroadcastDistributionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-AssistanceInformation,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SgNB-UE-X2AP-ID,

id-SharedNGUNotEstablished,

id-S-NSSAI,

id-SONInformationReport,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

AssociatedQosFlowList ::= SEQUENCE (SIZE(1..maxnoofQosFlows)) OF AssociatedQosFlowItem

AssociatedQosFlowItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

qosFlowMappingIndication		ENUMERATED {ul, dl, ...}							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AssociatedQosFlowItem-ExtIEs} }	OPTIONAL,

...

}

AssociatedQosFlowItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetIndex	PRESENCE optional	},

...

}

AssociatedSessionID ::= OCTET STRING -- coding FFS

AuthenticatedIndication ::= ENUMERATED {true, ...}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

BroadcastPLMNItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-NPN-Support					CRITICALITY reject	EXTENSION NPN-Support				PRESENCE optional}|

{ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList 	PRESENCE optional}|

{ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION TAINSAGSupportList		PRESENCE optional},

...

}

BroadcastDistributionSetupResponseTransfer ::= SEQUENCE {

mBS-SessionID								MBS-SessionID,

sharedNGU-BroadcastInformationList		SharedNGU-BroadcastInformationList,

iE-Extensions		ProtocolExtensionContainer { {BroadcastDistributionSetupResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

BroadcastDistributionSetupResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

BluetoothMeasurementConfiguration ::= SEQUENCE {

bluetoothMeasConfig				BluetoothMeasConfig,

bluetoothMeasConfigNameList		BluetoothMeasConfigNameList											OPTIONAL,

bt-rssi							ENUMERATED {true, ...}												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BluetoothMeasurementConfiguration-ExtIEs } } 	OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBS-ActiveSessionInformation-TargettoSourceList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MBS-ActiveSessionInformation-TargettoSourceItem

MBS-ActiveSessionInformation-TargettoSourceItem ::= SEQUENCE {

mBS-SessionID 						MBS-SessionID,

mBS-DataForwardingResponseMRBList	MBS-DataForwardingResponseMRBList													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBS-ActiveSessionInformation-TargettoSourceItem-ExtIEs} }		OPTIONAL,

...

}

MBS-ActiveSessionInformation-TargettoSourceItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBS-AssistanceInformation ::= ENUMERATED {true, ...}

MBSSessionSetupOrModFailureTransfer ::= SEQUENCE {

cause						Cause,

criticalityDiagnostics		CriticalityDiagnostics		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionSetupOrModFailureTransfer-ExtIEs} }	OPTIONAL,

...

}

MBSSessionSetupOrModFailureTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupResponseList ::= SEQUENCE (SIZE(1.. maxnoofMBSSessions)) OF MBSSessionSetupResponseItem

MBSSessionSetupResponseItem ::= SEQUENCE {

mBS-SessionID			MBS-SessionID,

mBS-AreaSessionID		MBS-AreaSessionID													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { MBSSessionSetupResponseItem-ExtIEs} }	OPTIONAL,

...

}

MBSSessionSetupResponseItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupOrModRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MBSSessionSetupOrModRequestTransferIEs} },

...

}

MBSSessionSetupOrModRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	mandatory	}|

{ ID id-MBS-SessionFSAIDList				CRITICALITY ignore	TYPE MBS-SessionFSAIDList				PRESENCE	optional		},

...

}

MBS-SessionFSAIDList ::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-SessionFSAID

MBS-SessionFSAID ::= OCTET STRING (SIZE(3))

MBSSessionReleaseResponseTransfer ::= SEQUENCE {

mBS-SessionTNLInfoNGRAN		MBS-SessionTNLInfoNGRAN				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionReleaseResponseTransfer-ExtIEs} }	OPTIONAL,

...

}

MBSSessionReleaseResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

MBSSessionSetupOrModResponseTransfer ::= SEQUENCE {

mBS-SessionTNLInfoNGRAN 	MBS-SessionTNLInfoNGRAN														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MBSSessionSetupOrModResponseTransfer-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupOrModResponseTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedNGUNotEstablished				CRITICALITY ignore	EXTENSION SharedNGUNotEstablished			PRESENCE optional		},

...

}

MBS-SupportIndicator ::= ENUMERATED {

true,

...

}

MBS-SessionTNLInfo5GC ::= CHOICE {

locationindependent			SharedNGU-MulticastTNLInformation,

locationdependent			MBS-SessionTNLInfo5GCList,

choice-Extensions		ProtocolIE-SingleContainer { {MBS-SessionTNLInfo5GC-ExtIEs} }

}

MBS-SessionTNLInfo5GC-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MBSSessionSetupRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetupRequestItem

MBSSessionSetupRequestItem ::= SEQUENCE {

mBS-SessionID									MBS-SessionID,

mBS-AreaSessionID								MBS-AreaSessionID																	OPTIONAL,

associatedMBSQosFlowSetupRequestList			AssociatedMBSQosFlowSetupRequestList												OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { MBSSessionSetupRequestItem-ExtIEs} }		OPTIONAL,

...

}

MBSSessionSetupRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	TYPE MBS-AssistanceInformation		PRESENCE	optional		},

...

}

MBSSessionSetuporModifyRequestList ::= SEQUENCE (SIZE(1..maxnoofMBSSessions)) OF MBSSessionSetuporModifyRequestItem

MBSSessionSetuporModifyRequestItem ::= SEQUENCE {

mBS-SessionID										MBS-SessionID,

mBS-AreaSessionID									MBS-AreaSessionID									OPTIONAL,

associatedMBSQosFlowSetuporModifyRequestList		AssociatedMBSQosFlowSetuporModifyRequestList		OPTIONAL,

mBS-QosFlowToReleaseList 							QosFlowListWithCause								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{MBSSessionSetuporModifyRequestItem-ExtIEs}}			OPTIONAL,

...

}

MBSSessionSetuporModifyRequestItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-MBS-AssistanceInformation			CRITICALITY ignore	TYPE MBS-AssistanceInformation		PRESENCE	optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SharedNGUNotEstablished ::= ENUMERATED {

true,

...

}

SharedNGU-BroadcastInformationList ::= SEQUENCE (SIZE(0.. maxnoofMBSAreaSessionIDs)) OF SharedNGU-BroadcastInformationItem

SharedNGU-BroadcastInformationItem ::= SEQUENCE {

mBS-AreaSessionID							MBS-AreaSessionID					OPTIONAL,

sharedNGU-MulticastTNLInformation			SharedNGU-MulticastTNLInformation,

iE-Extensions					ProtocolExtensionContainer { { SharedNGU-BroadcastTNLInformationItem-ExtIEs} }	OPTIONAL,

...

}

SharedNGU-BroadcastTNLInformationItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SharedNGU-BroadcastTNLInformation ::= SEQUENCE {

iP-BroadcastAddress 		TransportLayerAddress,

iP-SourceAddress			TransportLayerAddress,

gTP-TEID						GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { {SharedNGU-BroadcastTNLInformation-ExtIEs} } 	OPTIONAL,

...

}

SharedNGU-BroadcastTNLInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionReleaseRequired			ProcedureCode ::= 75

id-BroadcastDistributionSetup 				ProcedureCode ::= XX

id-BroadcastDistributionRelease 			ProcedureCode ::= XX

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-AssociatedSessionID									ProtocolIE-ID ::= 998 -- to be allocated

id-SharedNGUNotEstablished								ProtocolIE-ID ::= 999 -- to be allocated

id-MBS-AssistanceInformation							ProtocolIE-ID ::= 1000 -- to be allocated

id-BroadcastDistributionSetupResponseTransfer			ProtocolIE-ID ::= XXX

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 7	TP for BLCR on 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.14	NR MBS Procedures

### 8.14.1	Broadcast Context Setup

#### 8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If the Associated Session ID IE is contained in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take this information into account to determine whether MBS session resource sharing is possible, as specified in TS 38.401 [4].

#### 8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

#### 8.14.1.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.14.X	Broadcast Context Distribution Setup

#### 8.14.X.1	General

The purpose of the Broadcast Context Distribution Setup procedure is to assign F1-U resources for a broadcast MBS session context. The procedure uses non-UE-associated signalling.

#### 8.17.X.2	Successful Operation

Figure 8.17.X.2-1: Broadcast Context Distribution Setup, successful operation.

The gNB-DU node initiates the procedure by sending a BROADCAST CONTEXT DISTRIBUTION SETUP REQUEST message to the gNB-CU. The gNB-CU responds with a BROADCAST CONTEXT DISTRIBUTION SETUP RESPONSE message.

#### 8.17.X.3	Unsuccessful Operation

Figure 8.17.X.3-1: Broadcast Context Distribution Setup, unsuccessful operation.

In case the F1-U transport cannot be setup successfully, the gNB-CU shall respond with the BROADCAST CONTEXT DISTRIBUTION SETUP FAILURE message to the gNB-CU with an appropriate cause value.

#### 8.17.X.4	Abnormal Conditions

Void.

### 8.14.Y	Broadcast Context Distribution Release

#### 8.14.Y.1	General

The purpose of the Broadcast Context Distribution Release procedure is to enable the release of already established F1-U resources for a given MBS session context. The procedure uses non-UE-associated signalling.

#### 8.14.Y.2	Successful Operation

Figure 8.17.Y.2-1: Broadcast Context Distribution Release, successful operation.

The gNB-DU initiates the procedure by sending a BROADCAST CONTEXT DISTRIBUTION RELEASE REQUEST message.

Upon receipt of the BROADCAST CONTEXT DISTRIBUTION RELEASE REQUEST message, the gNB-CU shall send the BROADCAST CONTEXT DISTRIBUTION RELEASE RESPONSE message after successfully removing the corresponding F1-U resource for the MBS session contex.

#### 8.14.Y.3		Unsuccessful Operation

Not applicable.

## 9.2	Message Functional Definition and Content

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.13	Broadcast Context Management messages

#### 9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Associated Session ID | O |  | 9.3.1.x |  | YES | ignore |
| Requested Action for Multiple Cell-IDs Broadcast | O |  | 9.3.1.y |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| F1-U Not Established | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.13.7	BROADCAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |
| F1-U Not Established | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.13.a	BROADCAST CONTEXT DISTRIBUTION SETUP REQUEST

This message is sent by the gNB-DU to request the setup of the F1-U transport for a broadcast MBS session context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast Context Distribution Setup Request List |  | 1 |  |  | YES | reject |
| > Broadcast Context Distribution Setup Request Item |  | 0..<maxnoofMBSAreaSessionIDs> |  |  | EACH | reject |
| >> MBS Area Session ID | O |  | 9.3.1.221 |  | YES | reject |
| >>F1-U Distribution Setup Request List |  | 0..1 |  |  | YES | reject |
| >>>F1-U Distribution Setup Request Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>>>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>>>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |

#### 9.2.13.b	BROADCAST CONTEXT DISTRIBUTION SETUP RESPONSE

This message is sent by the gNB-CU to confirm the setup of the F1-U transport.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast Context Distribution Setup Response List |  | 1 |  |  | YES | reject |
| > Broadcast Context Distribution Setup Response Item |  | 0..<maxnoofMBSAreaSessionIDs> |  |  | EACH | reject |
| >> MBS Area Session ID | O |  | 9.3.1.221 |  | YES | reject |
| >>F1-U Distribution Setup Response List |  | 0..1 |  |  | YES | reject |
| >>>F1-U Distribution Setup Response Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>>>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>>> BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.13.c	BROADCAST CONTEXT DISTRIBUTION SETUP FAILURE

This message is sent by the gNB-CU to indicate that the setup of the F1-U transport was unsuccessful.

Direction:gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.13.d	BROADCAST CONTEXT DISTRIBUTION RELEASE REQUEST

This message is sent by the gNB-DU to request the release of the F1-U transport.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.13.e	BROADCAST CONTEXT DISTRIBUTION RELEASE RESPONSE

This message is sent by the gNB-CU to confirm the release of the F1-U transport.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | FFS |  |

#### 9.3.1.y	Requested Action for Multiple Cell-IDs Broadcast

This IE provides information about the requested gNB-DU’s action with regards to a potentially available MRB PDCP configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested Action for Multiple Cell-IDs Broadcast | M |  | ENUMERATED (apply available configuration, apply requested configuration, ,apply available configuration if same as requested…) |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

BroadcastContextDistributionReleaseRequest,

BroadcastContextDistributionReleaseResponse,

BroadcastContextDistributionSetupFailure,

BroadcastContextDistributionSetupRequest,

BroadcastContextDistributionSetupResponse,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-BroadcastDistributionSetup,

id-BroadcastDistributionRelease,

id-MulticastGroupPaging,

id-MulticastContextSetup,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification|

broadcastContextDistributionSetup		|

broadcastContextDistributionRelease	|

multicastContextSetup			|

multicastContextRelease		|

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

broadcastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseRequest

PROCEDURE CODE			id-BroadcastContextReleaseRequest

CRITICALITY				reject

}

broadcastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextModificationRequest

SUCCESSFUL OUTCOME		BroadcastContextModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastContextModificationFailure

PROCEDURE CODE			id-BroadcastContextModification

CRITICALITY				reject

}

broadcastContextDistributionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextDistributionSetupRequest

SUCCESSFUL OUTCOME		BroadcastContextDistributionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastContextDistributionSetupFailure

PROCEDURE CODE			id-BroadcastContextDistributionSetup

CRITICALITY				reject

}

broadcastContextDistributionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextDistributionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastContextDistributionReleaseResponse

PROCEDURE CODE			id-BroadcastContextDistributionRelease

CRITICALITY				reject

}

multicastGroupPaging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastGroupPaging

PROCEDURE CODE			id-MulticastGroupPaging

CRITICALITY				ignore

}

multicastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextSetupRequest

SUCCESSFUL OUTCOME		MulticastContextSetupResponse

UNSUCCESSFUL OUTCOME	MulticastContextSetupFailure

PROCEDURE CODE			id-MulticastContextSetup

CRITICALITY				reject

}

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AssociatedSessionID,

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue

RequestedActionforMultipleCellIDsBroadcast

F1UNOTEstablished

BroadcastContextDistributionSetupRequestList

F1UDistributionSetupRequestList

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

FROM F1AP-Containers

id-AssociatedSessionID,

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-RequestedActionforMultipleCellIDsBroadcast,

id-F1UNOTEstablished

id-BroadcastContextDistributionSetupRequestList

id-F1UDistributionSetupRequestList

maxCellingNBDU,

maxnoofCandidateSpCells,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- BROADCAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP REQUEST

--

-- **************************************************************

BroadcastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupRequestIEs} },

...

}

BroadcastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE	GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID					PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea					CRITICALITY reject 	TYPE	MBS-ServiceArea					PRESENCE optional   }|

{ ID id-MBS-CUtoDURRCInformation		CRITICALITY reject	TYPE	MBS-CUtoDURRCInformation		PRESENCE mandatory	}|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI							PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	BroadcastMRBs-ToBeSetup-List	PRESENCE mandatory	}|

{ ID id-AssociatedSessionID							CRITICALITY ignore	TYPE AssociatedSessionID							PRESENCE optional		}|

{ ID id-RequestedActionforMultipleCellIDsBroadcast	CRITICALITY reject	TYPE	RequestedActionforMultipleCell-IDsBroadcast			PRESENCE optional   }

,

...

}

BroadcastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetup-ItemIEs} }

BroadcastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	BroadcastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

BroadcastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupResponseIEs} },

...

}

BroadcastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-Setup-List			CRITICALITY reject TYPE BroadcastMRBs-Setup-List				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetup-List	PRESENCE optional	}|

{ ID id-BroadcastAreaScope					CRITICALITY ignore TYPE BroadcastAreaScope						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics					PRESENCE optional	}|

{ ID id-F1UNOTEstablished					CRITICALITY ignore TYPE F1UNOTEstablished						PRESENCE optional   }

,

...

}

BroadcastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Setup-ItemIEs} }

BroadcastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetup-ItemIEs} }

BroadcastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Setup-Item			CRITICALITY reject	TYPE BroadcastMRBs-Setup-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeSetup-Item	CRITICALITY ignore	TYPE BroadcastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},	...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

BroadcastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationResponseIEs} },

...

}

BroadcastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory}|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory}|

{ ID id-BroadcastMRBs-SetupMod-List				CRITICALITY reject TYPE BroadcastMRBs-SetupMod-List					PRESENCE optional}|

{ ID id-BroadcastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetupMod-List	PRESENCE optional}|

{ ID id-BroadcastMRBs-Modified-List				CRITICALITY reject TYPE BroadcastMRBs-Modified-List					PRESENCE optional}|

{ ID id-BroadcastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeModified-List	PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-BroadcastAreaScope						CRITICALITY ignore TYPE BroadcastAreaScope								PRESENCE optional}|

{ ID id-F1UNOTEstablished						CRITICALITY ignore TYPE F1UNOTEstablished								PRESENCE optional   }

,

...

}

BroadcastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-SetupMod-ItemIEs} }

BroadcastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetupMod-ItemIEs} }

BroadcastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Modified-ItemIEs } }

BroadcastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeModified-ItemIEs} }

BroadcastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE BroadcastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE BroadcastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Modified-Item				CRITICALITY		reject	TYPE BroadcastMRBs-Modified-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE BroadcastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

BroadcastContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationFailureIEs} },

...

}

BroadcastContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

BroadcastContextDistributionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastContextDistributionSetupRequestIEs} },

...

}

BroadcastContextDistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-DU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-BroadcastContextDistributionSetupRequestList	CRITICALITY reject	TYPE BroadcastContextDistributionSetupRequestList	PRESENCE mandatory	},

...

}

BroadcastContextDistributionSetupRequestList ::= SEQUENCE (SIZE(0.. maxnoofMBSAreaSessionIDs)) OF ProtocolIE-SingleContainer { { BroadcastContextDistributionSetupRequestItem} }

BroadcastContextDistributionSetupRequestItem F1AP-PROTOCOL-IES ::= {

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id-F1UDistributionSetupRequestList				CRITICALITY reject	TYPE F1UDistributionSetupRequestList					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

BroadcastContextDistributionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastContextDistributionSetupResponseIEs} },

...

}

BroadcastContextDistributionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID								CRITICALITY reject	TYPE MBS-SessionID											PRESENCE mandatory	}|

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-CU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-DU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-BroadcastContextDistributionSetupResponseList	CRITICALITY reject	TYPE BroadcastContextDistributionSetupResponseList	PRESENCE mandatory	},

...

}

BroadcastContextDistributionSetupResponseList ::= SEQUENCE (SIZE(0.. maxnoofMBSAreaSessionIDs)) OF ProtocolIE-SingleContainer { { BroadcastContextDistributionSetupResponseItem} }

BroadcastContextDistributionSetupResponseItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MBS-AreaSessionID							CRITICALITY reject	TYPE MBS-AreaSessionID									PRESENCE optional		}|

{ ID id- F1UDistributionSetupResponseList				CRITICALITY reject	TYPE F1UDistributionSetupResponseList					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION SETUP FAILURE

--

-- **************************************************************

BroadcastContextDistributionSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastContextDistributionSetupFailureIEs} },

...

}

BroadcastContextDistributionSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-CU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-DU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION RELEASE REQUEST

--

-- **************************************************************

BroadcastContextDistributionReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { BroadcastContextDistributionReleaseRequestIEs} },

...

}

BroadcastContextDistributionReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-CU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-DU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT DISTRIBUTION RELEASE RESPONSE

--

-- **************************************************************

BroadcastContextDistributionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastContextDistributionReleaseResponseIEs} },

...

}

BroadcastContextDistributionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-CU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE gNB-DU-MBS-F1AP-ID														PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- A

AbortTransmission ::= CHOICE {

sRSResourceSetID		SRSResourceSetID,

releaseALL				NULL,

choice-extension		ProtocolIE-SingleContainer { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { AccessPointPosition-ExtIEs} } OPTIONAL

}

AccessPointPosition-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Activated-Cells-to-be-Updated-List ::= SEQUENCE (SIZE(1..maxnoofServedCellsIAB)) OF Activated-Cells-to-be-Updated-List-Item

Activated-Cells-to-be-Updated-List-Item ::=	SEQUENCE{

nRCGI								NRCGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info	IAB-DU-Cell-Resource-Configuration-Mode-Info,

iE-Extensions						ProtocolExtensionContainer { { Activated-Cells-to-be-Updated-List-Item-ExtIEs} } OPTIONAL

}

Activated-Cells-to-be-Updated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ActivationRequestType ::= ENUMERATED {activate, deactivate, ...}

ActiveULBWP  ::= SEQUENCE {

locationAndBandwidth		INTEGER (0..37949,...),

subcarrierSpacing           ENUMERATED {kHz15, kHz30, kHz60, kHz120,...},

cyclicPrefix				ENUMERATED {normal, extended},

txDirectCurrentLocation		INTEGER (0..3301,...),

shift7dot5kHz				ENUMERATED {true, ...} OPTIONAL,

sRSConfig					SRSConfig,

iE-Extensions					ProtocolExtensionContainer { { ActiveULBWP-ExtIEs} } OPTIONAL

}

ActiveULBWP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalDuplicationIndication ::= ENUMERATED {

three,

four,

...

}

AdditionalPath-List::= SEQUENCE (SIZE(1..maxnoofPath)) OF AdditionalPath-Item

AdditionalPath-Item ::=SEQUENCE {

relativePathDelay	RelativePathDelay,

pathQuality			TRPMeasurementQuality 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPath-Item-ExtIEs } }	OPTIONAL

}

AdditionalPath-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MultipleULAoA	CRITICALITY ignore EXTENSION MultipleULAoA		PRESENCE optional}|

{ ID id-pathPower		CRITICALITY ignore EXTENSION UL-SRS-RSRPP		PRESENCE optional},

...

}

ExtendedAdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPathExtended)) OF ExtendedAdditionalPathList-Item

ExtendedAdditionalPathList-Item ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TRPMeasurementQuality	OPTIONAL,

multipleULAoA		MultipleULAoA  			OPTIONAL,

pathPower			UL-SRS-RSRPP			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAdditionalPathList-Item-ExtIEs} } OPTIONAL,

...

}

ExtendedAdditionalPathList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalPDCPDuplicationTNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF AdditionalPDCPDuplicationTNL-Item

AdditionalPDCPDuplicationTNL-Item ::=SEQUENCE {

additionalPDCPDuplicationUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { AdditionalPDCPDuplicationTNL-ItemExtIEs } }	OPTIONAL,

...

}

AdditionalPDCPDuplicationTNL-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo		CRITICALITY ignore	EXTENSION BHInfo		PRESENCE optional	},

...

}

AdditionalSIBMessageList ::= SEQUENCE (SIZE(1..maxnoofAdditionalSIBs)) OF AdditionalSIBMessageList-Item

AdditionalSIBMessageList-Item ::= SEQUENCE {

additionalSIB			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { AdditionalSIBMessageList-Item-ExtIEs} } OPTIONAL

}

AdditionalSIBMessageList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AffectedCellsAndBeams-List ::= SEQUENCE (SIZE (1.. maxAffectedCells)) OF AffectedCellsAndBeams-Item

AffectedCellsAndBeams-Item::= SEQUENCE {

nRCGI					NRCGI,

affectedSSB-List		AffectedSSB-List OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AffectedCellsAndBeams-Item-ExtIEs} } OPTIONAL,

...

}

AffectedCellsAndBeams-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AffectedSSB-List::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF AffectedSSB-Item

AffectedSSB-Item::= SEQUENCE {

sSB-Index	INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { AffectedSSB-Item-ExtIEs} } OPTIONAL,

...

}

AffectedSSB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggressorCellList ::= SEQUENCE (SIZE(1..maxCellingNBDU)) OF AggressorCellList-Item

AggressorCellList-Item ::= SEQUENCE {

aggressorCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { AggressorCellList-Item-ExtIEs } }		OPTIONAL

}

AggressorCellList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggressorgNBSetID ::= SEQUENCE {

aggressorgNBSetID		GNBSetID,

iE-Extensions	ProtocolExtensionContainer { { AggressorgNBSetID-ExtIEs } }	OPTIONAL

}

AggressorgNBSetID-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AngleMeasurementQuality ::= SEQUENCE {

azimuthQuality	INTEGER(0..255),

zenithQuality	INTEGER(0..255) OPTIONAL,

resolution		ENUMERATED{deg0dot1,...},

iE-Extensions	ProtocolExtensionContainer { { AngleMeasurementQuality-ExtIEs } }	OPTIONAL

}

AngleMeasurementQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoofSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (1..3)

Associated-SCell-Item ::= SEQUENCE {

sCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Associated-SCell-ItemExtIEs } }	OPTIONAL

}

Associated-SCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= -- coding is FFS

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL

}

AvailablePLMNList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailableSNPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF AvailableSNPN-ID-List-Item

AvailableSNPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

availableNIDList			BroadcastNIDList,

iE-Extensions				ProtocolExtensionContainer { { AvailableSNPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

AvailableSNPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow  ::= INTEGER (0..4095, ...)

AreaScope ::= ENUMERATED {true, ...}

AoA-AssistanceInfo ::= SEQUENCE {

angleMeasurement			AngleMeasurementType,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AoA-AssistanceInfo-ExtIEs } } OPTIONAL,

...

}

AoA-AssistanceInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AngleMeasurementType ::= CHOICE {

expected-ULAoA		Expected-UL-AoA,

expected-ZoA		Expected-ZoA-only,

choice-extension ProtocolIE-SingleContainer { { AngleMeasurementType-ExtIEs } }

}

AngleMeasurementType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AppLayerBufferLevelList ::= OCTET STRING

ARP-ID ::= INTEGER (1..16, ...)

ARPLocationInformation ::= SEQUENCE (SIZE (1..maxnoARPs)) OF ARPLocationInformation-Item

ARPLocationInformation-Item ::= SEQUENCE {

aRP-ID				ARP-ID,

aRPLocationType		ARPLocationType,

iE-Extensions		ProtocolExtensionContainer { { ARPLocationInformation-ExtIEs} } OPTIONAL,

...

}

ARPLocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ARPLocationType ::= CHOICE {

aRPPositionRelativeGeodetic			RelativeGeodeticLocation,

aRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { ARPLocationType-ExtIEs } }

}

ARPLocationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

ReportingPeriodicity ::= ENUMERATED{ms500, ms1000, ms2000, ms5000, ms10000, ...}

RequestedActionforMultipleCellIDsBroadcast ::= ENUMERATED {

apply-available-configuration,

apply-requested-configuration,

apply-available-configuration-if-same-as-requested,

…

}

RequestedBandCombinationIndex ::= OCTET STRING

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

F1UNotEstablished ::= ENUMERATED {

true,

...

}

F1UDistributionSetupRequestItem ::= SEQUENCE {

mRB-ID							MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU		BCBearerContextF1U-TNLInfo	,

iE-Extensions					ProtocolExtensionContainer { { F1UDistributionSetupRequestItem-ExtIEs} },

...

}

F1UDistributionSetupRequestItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

F1UDistributionSetupResponseItem ::= SEQUENCE {

mRB-ID							MRB-ID,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo	,

iE-Extensions					ProtocolExtensionContainer { { F1UDistributionSetupResponseItem-ExtIEs} },

...

}

F1UDistributionSetupResponseItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-AssociatedSessionID								ProtocolIE-ID ::= XXX

id-RequestedActionforMultipleCellIDsBroadcast		ProtocolIE-ID ::= XXX

id-F1UNotEstablished								ProtocolIE-ID ::= XXX

id-BroadcastContextDistributionSetupRequestList		ProtocolIE-ID ::= XXX

id-BroadcastContextDistributionSetupResponseList	ProtocolIE-ID ::= XXX

id-F1UDistributionSetupRequestItem					ProtocolIE-ID ::= XXX

id-F1UDistributionSetupResponseItem					ProtocolIE-ID ::= XXX

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 8	TP for BLCR on 37.483

-----------Start of the Change----------

#### 8.6.1.1	BC Bearer Context Setup

##### 8.6.1.1.1	General

The purpose of the BC Bearer Context Setup procedure is to allow the gNB-CU-CP to establish MBS session resources for a broadcast MBS session in the gNB-CU-UP. The procedure uses MBS-associated signalling.

##### 8.6.1.1.2	Successful Operation

Figure 8.6.1.1.2-1: BC Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BC BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested MBS session resources, it replies to the gNB-CU-CP with the BC BEARER CONTEXT SETUP RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BC BEARER CONTEXT SETUP RESPONSE message, the result of all the requested resources in the following way:

-	A list of BC MRBs which are successfully established shall be included in the BC MRB Setup Response List IE;

-	A list of BC MRBs which failed to be established shall be included in the BC MRB Failed List IE;

-	For each established BC MRB, a list of MBS QoS Flows which are successfully established shall be included in the MBS QoS Flow Setup List IE;

-	For each established BC MRB, a list of MBS QoS Flows which failed to be established shall be included in the MBS QoS Flow Failed List IE.

When the gNB-CU-UP reports the unsuccessful establishment of a BC MRB or MBS QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Requested Action for Available Shared NG-U Termination IE in the BC Bearer Context To Setup IE in the BC BEARER CONTEXT SETUP REQUEST message is set to

-	"apply available configuration" and an appropriate Shared NG-U Termination is available, the gNB-CU-UP shall apply the radio bearer configuration of the Shared NG-U Termination, and indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of the Shared NG-U Termination, if the radio bearer configuration of the Shared NG-U Termination is different than the one requested by the gNB-CU-CP.

-	"apply requested configuration" the gNB-CU-UP shall make use of an available appropriate Shared NG-U Termination if the radio bearer configuration of the Shared NG-U Termination, is the same as the one requested by the gNB-CU-CP, otherwise allocate separate resources as requested by the gNB-CU-CP and indicate in the BC BEARER CONTEXT SETUP RESPONSE message within the Available BC MRB Configuration IE in the BC Bearer Context To Setup Response IE the radio bearer configuration of the Shared NG-U Termination.

-	"apply available configuration if same as requested" the gNB-CU-UP shall make use of an available appropriate Shared NG-U Termination only if the radio bearer configuration of the Shared NG-U Termination is the same as the one requested by the gNB-CU-CP and reply with BC BEARER CONTEXT SETUP RESPONSE message.

If the Associated Session ID IE and Associated MBS Area Session ID IE are contained in the BC BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, take this information into account to determine whether MBS session resource sharing is possible.

-----------Next Change----------

#### 9.2.5.1	MBS Messages for Broadcast

##### 9.2.5.1.1	BC BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup MBS session resources for a broadcast MBS session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.106 |  | YES | reject |
| Global MBS Session ID | M |  | 9.3.1.108 |  | YES | reject |
| BC Bearer Context To Setup | M |  | 9.3.3.26 |  | YES | reject |
| Associated Session ID | O |  | 9.3.1.X |  | YES | ignore |
| Associated MBS Area Session ID | O |  | 9.3.1.Y |  | YES | ignore |

### 9.3.1	Radio Network Layer Related IEs

-----------Next Change----------

#### 9.3.1.X	Associated Session ID

This IE is used to associate MBS Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated Session ID | M |  | FFS | Editor’s Note: coding is FFS |

#### 9.3.1.Y	Associated MBS Area Session ID

This IE is used to associate MBS Area Session IDs providing identical user data.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated MBS Area Session ID | M |  | FFS | Editor’s Note: coding is FFS |

### 9.3.3	Container and List IE definitions

-----------Next Change----------

#### 9.3.3.27	BC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm BC Bearer Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  | - | - |
| BC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  | - | - |
| >MRB ID | M |  | 9.3.1.16a |  | - | - |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.118 |  | - | - |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  | - | - |
| >MRB ID | M |  | 9.3.1.16a |  | - | - |
| >Cause | M |  | 9.3.1.2 |  | - | - |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 |  | - | - |
| Shared NG-U Not Established | O |  | ENUMERATED (true, ...) | Editor’s Note: whether this IE is applicable for both, unicast and multicast NG-U transport or only for unicast transport is FFS | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

#### 9.3.3.28	BC Bearer Context To Modify

This IE contains MBS session resource related information used to request BC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference |  | Semantics description |  |  | Criticality |  | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at 5GC To Setup or Modify | O |  | BC Bearer Context NG-U TNL Info at 5GC 9.3.1.112 |  |  |  |  | - |  | - |
| BC MRB To Setup List | O |  | BC MRB Setup Configuration 9.3.1.114 |  |  |  |  | - |  | - |
| BC MRB To Modify List |  | 0..<maxnoofMRBs> |  |  |  |  |  | - |  | - |
| >MRB ID | M |  | 9.3.1.16a |  |  |  |  | - |  | - |
| >BC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.119 |  |  |  |  | - |  | - |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 |  |  |  |  | - |  | - |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  |  |  |  | - |  | - |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 |  | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |  |  | - |  | - |
| BC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |  |  |  | - |  | - |
| >MRB ID | M |  | 9.3.1.16a |  |  |  |  | - |  | - |
| F1-U Not Established | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

MCBearerContextToModifyConfirm,

MBSMulticastF1UContextDescriptor,

GNB-CU-UP-MBS-Support-Info,

SDTContinueROHC,

MDTPLMNModificationList,

AssociatedSessionID,

AssociatedMBSAreaSessionID,

////////////////////////////////No change/////////////////////////////////

id-MBSMulticastF1UContextDescriptor,

id-gNB-CU-UP-MBS-Support-Info,

id-SDTContinueROHC,

id-ManagementBasedMDTPLMNModificationList,

id-AssociatedSessionID,

id-AssociatedMBSAreaSessionID,

////////////////////////////////No change/////////////////////////////////

-- **************************************************************

--

#### -- BC BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT SETUP REQUEST

--

-- **************************************************************

BCBearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextSetupRequestIEs } },

...

}

BCBearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GlobalMBSSessionID				CRITICALITY reject	TYPE 	GlobalMBSSessionID					PRESENCE mandatory	}|

{ ID id-BCBearerContextToSetup			CRITICALITY reject	TYPE 	BCBearerContextToSetup				PRESENCE mandatory	}|

{ ID id-AssociatedSessionID				CRITICALITY ignore	TYPE 	AssociatedSessionID					PRESENCE optional	}|

{ ID id-AssociatedMBSAreaSessionID		CRITICALITY ignore	TYPE 	AssociatedMBSAreaSessionID		PRESENCE optional	},

...

}

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

////////////////////////////////No change/////////////////////////////////

id-MCForwardingResourceRelease,

id-MCForwardingResourceReleaseIndication,

id-PDCP-COUNT-Reset,

id-MBSSessionAssociatedInfoNonSupportToSupport,

id-SharedNGUNotEstablished,

id-F1UNotEstablished,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

////////////////////////////////No change/////////////////////////////////

#### -- A

ActivityInformation	::=	CHOICE	{

dRB-Activity-List						DRB-Activity-List,

pDU-Session-Resource-Activity-List		PDU-Session-Resource-Activity-List,

uE-Activity								UE-Activity,

choice-extension						ProtocolIE-SingleContainer	{{ActivityInformation-ExtIEs}}

}

ActivityInformation-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

ActivityNotificationLevel	::=	ENUMERATED	{

drb,

pdu-session,

ue,

...

}

AdditionalHandoverInfo ::= ENUMERATED	{

discard-pdpc-SN,

...

}

AdditionalPDCPduplicationInformation	::=	ENUMERATED	{

three,

four,

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AveragingWindow  ::= INTEGER (0..4095, ...)

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParameterIndex		INTEGER(1..8,...),

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

AssociatedSessionID ::= OCTET STRING -- coding FFS

AssociatedMBSAreaSessionID ::= OCTET STRING -- coding FFS

////////////////////////////////No change/////////////////////////////////

##### -- BCBearerContextToSetupResponse

BCBearerContextToSetupResponse ::= SEQUENCE {

bcBearerContextNGU-TNLInfoatNGRAN			BCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

bcMRBSetupResponseList					BCMRBSetupResponseList,

bcMRBFailedList							BCMRBFailedList							OPTIONAL,

availableBCMRBConfig						BCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SharedNGUNotEstablished		CRITICALITY ignore	EXTENSIONSharedNGUNotEstablished		PRESENCE optional	},

...

}

////////////////////////////////No change/////////////////////////////////

##### -- BCBearerContextToModify

BCBearerContextToModify ::= SEQUENCE {

bcBearerContextNGU-TNLInfoat5GC		BCBearerContextNGU-TNLInfoat5GC	OPTIONAL,

bcMRBToSetupList						BCMRBSetupConfiguration			OPTIONAL,

bcMRBToModifyList					BCMRBModifyConfiguration			OPTIONAL,

bcMRBToRemoveList					BCMRBRemoveConfiguration			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

{ ID id-F1UNotEstablished		CRITICALITY ignore		EXTENSIONF1UNotEstablished				PRESENCE optional	},

...

}

////////////////////////////////No change/////////////////////////////////

ECGI-Support-Item ::= SEQUENCE {

eCGI	ECGI,

iE-Extensions				ProtocolExtensionContainer { { ECGI-Support-Item-ExtIEs } }	OPTIONAL

}

ECGI-Support-Item-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

SharedNGUNotEstablished ::= ENUMERATED {

true,

...

}

F1UNotEstablished ::= ENUMERATED {

true,

...

}

EUTRAN-QoS-Support-List ::= SEQUENCE (SIZE(1.. maxnoofEUTRANQOSParameters)) OF EUTRAN-QoS-Support-Item

EUTRAN-QoS-Support-Item ::= SEQUENCE {

eUTRAN-QoS	EUTRAN-QoS,

iE-Extensions			ProtocolExtensionContainer { { EUTRAN-QoS-Support-Item-ExtIEs } }	OPTIONAL

}

////////////////////////////////No change/////////////////////////////////

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

////////////////////////////////No change/////////////////////////////////

id-MCForwardingResourceResponse									ProtocolIE-ID ::= 181

id-MCForwardingResourceRelease									ProtocolIE-ID ::= 182

id-MCForwardingResourceReleaseIndication						ProtocolIE-ID ::= 183

id-PDCP-COUNT-Reset												ProtocolIE-ID ::= 184

id-MBSSessionAssociatedInfoNonSupportToSupport					ProtocolIE-ID ::= 185

id-AssociatedSessionID											ProtocolIE-ID ::= XXX

id-AssociatedMBSAreaSessionID									ProtocolIE-ID ::= XXX

id-SharedNGUNotEstablished										ProtocolIE-ID ::= XXX

id-F1UNotEstablished											ProtocolIE-ID ::= XXX

END

-- ASN1STOP

-----------End of the Change----------