3GPP TSG-RAN3 Meeting #121	R3-234250

# Toulouse, France, Aug 21st – 25th 2023

Agenda item:	11.4

Source: 	Xiaomi

Title: 	Discussion on R17 leftovers

Document for:	Discussion & decision

Introduction

In this contribution, we’d like to discuss the following topics based on the previous discussion and agreements:

- Buffer level threshold-based RVQoE reporting

- RVQoE information divarication over F1

- Inter-RAT QoE

Discussion

Buffer level threshold-based RVQoE reporting

A Reply LS [1] on buffer level threshold-based RVQoE reporting is received from SA4,

SA4 has further discussed this problem and came to an understanding that buffer level on its own is not an appropriate information for RAN to be able to assist the application due to a variety of reasons, e.g.

- A higher buffer level may help reduce the probability of a playback stall, but may also increase the playback latency, so it’s a trade-off that needs to be balanced by applications. Different proprietary application mechanisms attempting to strike this balance may result in different application behavior for same buffer level or same content.

- Also, different applications (e.g. low-latency live vs. on-demand video applications) have very different behaviour of maintaining the buffer level.

Hence based on this, SA4 would like to update the reply sent in S4-230684, and does not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer. SA4 is considering appropriate alternatives for this purpose. SA4 requests RAN2 and RAN3 to take this information in account, and that SA4 will keep RAN2 and RAN3 updated on progress in this direction.

The LS states that the buffer level may not be suitable for RAN optimization, and SA4 is considering appropriate alternatives. If the characteristic of the RVQoE information is not clear for us, it’s hard for RAN3 to discuss further on the threshold triggering, so we’d like to pending this discussion until SA4

Observation 1, SA4 informs RAN2/3 buffer level on its own is not an appropriate information for RAN and they’re considering appropriate alternatives.

Observation 2, it’s hard for RAN3 to further discuss threshold-based reporting if the actual measurement content is not clear.

Proposal 1, RAN3 agrees that the buffer level threshold-based RVQoE reporting is pending to SA4 progress.

RVQoE information divarication over F1

In last RAN3 meeting, we had the following agreements on DU participation:

Turn WA to agreement: A class-2 procedure is used for DU to deactivate the RVQoE reporting over F1AP.

UE associated or non-UE associated?

UE associated or non-UE associated signalling

There’re three deactivation levels had been discussed:

- Per cell level deactivation

- Per UE level deactivation

- Per QoE reference level deactivation

Which signalling is used should based on the DU’s needs. In our view, the RVQoE information is used for scheduling for all the UEs in the cell, the non-UE associated signalling should be used. And the needs for per UE level deactivation and per QoE reference level deactivation is not clear for us.

Proposal 2, non-UE associated signalling can be used to deactivate the RVQoE information transfer over F1AP for cell level scheduling.

Inter-RAT QoE

In last RAN3 meeting, we had the following agreements on Support QoE during Intra-5GC Inter-RAT

WA: For HO from LTE/5GC to NR, there is no impacts to RAN3.

For HO from NR to LTE/5GC case, identify the impact on RAN3.

In our understanding, besides the configuration content and structure, the main difference between LTE QoE and NR QoE are below:

- LTE QMC only support streaming and MTSI, NR QoE support more service types than LTE QoE.

- LTE QMC only support one QMC per UE, NR QoE support multiple QMC per UE.

If the UE hands over from LTE to NR, and if the UE is configured LTE QMC, the LTE QMC can be continued, but how UE continue the QMC after handover should be discussed in RAN2 scope, NR may assign new RRC ID for the LTE QMC. After UE switches to NR network, UE may perform handover among NR gNBs, it may also move back to LTE network, the Ng-eNB needs to select one of the QoE configuration to continue the measurement. Of course, Ng-eNB can only select the QoE configuration for the streaming service and MTSI service, but there may be multiple QoE configurations for the same service type, how the Ng-eNB select the one QMC to continue needs to be discussed. In our view, if there is one QMC configured in LTE network, after the UE handed over from LTE to NR, an indictor can be added to that QMC, to help QMC selection in the further handover from NR to LTE.

Proposal 3, RAN3 discuss how the Ng-eNB select the one QMC among multiple QMCs to continue the measurement in LTE, when UE performs handover from LTE to NR.

Proposal 4, an indictor can be added to the QMC that configured in LTE network to help QMC selection in the further handover from NR to LTE.

Conclusion

The following are the observations and proposals:

Buffer level threshold-based RVQoE reporting

Observation 1, SA4 informs RAN2/3 buffer level on its own is not an appropriate information for RAN and they’re considering appropriate alternatives.

Observation 2, it’s hard for RAN3 to further discuss threshold-based reporting if the actual measurement content is not clear.

Proposal 1, RAN3 agrees that the buffer level threshold-based RVQoE reporting is pending to SA4 progress.

RVQoE information divarication over F1

Proposal 2, non-UE associated signalling can be used to deactivate the RVQoE information transfer over F1AP for cell level scheduling.

Inter-RAT QoE

Proposal 3, RAN3 discuss how the Ng-eNB select the one QMC among multiple QMCs to continue the measurement in LTE, when UE performs handover from LTE to NR.

Proposal 4, an indictor can be added to the QMC that configured in LTE network to help QMC selection in the further handover from NR to LTE.

Reference

[1] S4-231119, Reply LS on buffer level threshold-based RVQoE reporting, SA4