3GPP TSG-RAN3 Meeting #121	R3-234251

# Toulouse, France, Aug 21st – 25th 2023

Agenda item:	13.5

Source:	Xiaomi

Title:	(TP for TS 38.423 support of de-authorization for mobile IAB) Discussion on authorization and additional ULI

Document for:	Discussion and Decision

1	Introduction

RAN3 had discussed the issues from SA2 for several meetings and has already support providing mobile TRP location information and additional ULI to the core network. And there are two leftover issues.

- Authorization and de-authorization

- Additional ULI (in case the MT and DU connects to different CUs)

In this contribution, we’d like to discuss the above issues including the LS by SA2 on IAB-node de-authorization handling [1].

2	Discussion

2.1	Authorization and de-authorization

For this chapter, we think the following issues need to be discussed:

- Issue1, whether to introduce mobile IAB indication in N2 INITIAL UE MESSAE (leftover from previous meeting)

- Issue 2, how to handle the de-authorization of mobile IAB-node (SA2 LS related)

  - Issue 2bis-1, whether and how to handle the coexistence issue for IAB authorization and mobile IAB authorization. (new issue)

  - Issue 2bis-2, gNB behavior for de-authorization (Q1 in the LS [1])

  - Issue 2bis-3, IAB-DU behavior after de-authorization (Q2 in the LS[1])

Issue 1, whether to introduce mobile IAB indication in N2 INITIAL UE MESSAE

RAN3 had discussed whether to introduce mobile IAB indication in the initial UE message during IAB-MT’s registration, and RAN3 have the following agreement in RAN3 119bis e-meeting.

Mobile IAB indication over NGAP:

The IAB-donor-CU selects an AMF that supports mobile IAB-node based on the mobile IAB-node indication received via Msg5.

RAN3 to decide whether to support NGAP indication to AMF for mobile IAB-authorization based on progress in SA2.

The main point of disagreement on introducing a mobile IAB-indication in NGAP is that IAB indication may be reused to obtain the mobile IAB indication.

According to the specification in TS 23.501[3], SA2 thinks a sperate indication IE for mobile IAB should be used

“For a MBSR node to operate as a MBSR, it provides a mobile IAB-indication to the IAB-donor-CU when the RRC connection is established as defined in TS 38.331 [28]. When the mobile IAB-indication is received, the IAB-donor-CU selects an AMF that supports IAB-node with mobility and includes the mobile IAB-indication in the N2 INITIAL UE MESSAGE as defined in TS 38.413 [34] so that the AMF can perform mobile IAB authorization as described in clause 5.35A.4.”

After some study on subscription information previsioning, we have the following observations,

- AMF will not always communicate with UDM for subscription information (the procedure is optional in TS 23.502), which indicates that AMF may not perform mobile IAB authorization without a specific indication.

- UDM will not provide everything to AMF (too much subscription information), the information provided is per request, which indicates that AMF may not get the authorized information without a specific indication in the request

Based on the above analysis, we think a specific mobile IAB indication is needed to be introduced in NGAP INITIAL UE MESSAE, if not, we need to notify SA2 to update their specification to align RAN3.

Observation 1, AMF may not perform mobile IAB authorization without a specific indication.

Observation 2, AMF may not get the authorized information without a specific indication in the request

Proposal 1, RAN3 either agree to introduce mobile IAB indication in NGAP INITIAL UE MESSAE or notify SA2 that RAN3 decide to reuse existing IAB-indication in NGAP INITIAL UE MESSAE to obtain mobile IAB authorization information.

Issue 2 how to handle the de-authorization of mobile IAB-node

Below is the content of SA2’s LS [1].

“SA2 is currently discussing the MBSR (i.e., mobile IAB-node) authorization state changed from authorized to non-authorized (e.g., due to subscription data update for the MBSR). The AMF notifies the Mobile IAB-node authorization status via the N2 message. SA2 noticed that when the AMF indicates the mobile IAB node is authorized, the mobile IAB-node integrates into the network. The donor-gNB communicates with IAB-MT via RRC messages to setup the backhaul, F1-C connection (i.e., as specified in TS 38.401 clause 8.12).

However, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, it’s not clear what is the gNB behaviour.

SA2 would like to check whether this is a correct understanding:

- The gNB attempts to perform handover of all the UEs served by the MBSR

- When the step 1 completes, the gNB releases the F1-C

Q1: is the understanding above correct?

Q2: Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)? ”

For question#1, when an MBSR is no longer authorized as indicated to the gNB by AMF by N2 message, the gNB attempts to perform handover of all the UEs served by the MBSR.

According to the text captured in NGAP BL CR, If the IAB Authorized IE is set to "not authorized" for an IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the IAB node will not serve any UE(s).

“If the Mobile IAB Authorized IE is set to "not authorized" for an mobile IAB-MT, the NG-RAN node shall, if supported, initiate actions to ensure that the mobile IAB node will not serve any UE(s).”

We’re wondering whether there is a case that gNB will receive both IAB authorization information and mobile IAB authorization, e.g. IAB is authorized but mobile IAB is not authorized, the case may happen if an operator’s communication vehicle can be authorized as a fixed IAB node when it stays immobile for a long time, and it can also be authorized as a mobile IAB.

Proposal 2, RAN3 discuss whether it is possible that both IAB authorization information and mobile IAB authorization information will be received by IAB-donor.

Proposal 3, RAN3 check with SA2 whether a mobile IAB-node can be authorized as IAB-node when it stays immobile for a long time.

Regarding the SA2’s question#1, if mobile IAB authorization indicates that the mobile IAB node is not authorized, the gNB should perform handover of all the UEs served by the mobile IAB-node and release the F1 connection between IAB-donor and IAB-DU.

However, in case the F1-termniating IAB-donor is different from the RRC-terminating IAB-donor, it should be the F1-terminating IAB-donor to trigger the UE handover and F1 release, and only RRC-terminating IAB-donor and mobile IAB-node can be aware of the authorization information from AMF.

Thus, the F1-terminating IAB-donor needs to be aware of the de-authorized information to perform the de-authorization action, e.g. handover UEs and release F1. Technically, there’re three options:

- option A, F1-terminating IAB-donor obtains de-authorized information from RRC-terminating IAB-donor via XnAP

- option B, F1-terminating IAB-donor obtains de-authorized information from IAB-DU via F1AP

- option C, F1-terminating IAB-donor obtains de-authorized information from AMF via NGAP

For option B, it may not be reliable if the authorized information is from IAB-DU (i.e. actually from IAB-MT), so option B is not preferred.

For option A, it’s straightforward if there’s Xn connection.

For option C, it can support both with and without Xn connection, but the AMF needs to know the gNB ID of F1-terminating IAB-donor and the cell provided by the co-located IAB-DU, it seems more spec impacts needed.

For the case with Xn connection, option A is perfect, and an existing procedure can be reused, i.e. IAB Transport Migration Modification.

For the case without Xn connection, maybe option C is suitable but we can wait for further progress on the migration procedure without Xn connection.

Observation 3, the IAB-MT can obtain the authorization information from AMF via NAS.

Observation 4, RRC-terminating IAB-donor can obtain the authorization information from AMF via NGAP

Observation 5, F1-termniating IAB-donor should be responsible for performing the de-authorization operation.

Proposal 4, F1-terminating IAB-donor needs to be informed about the de-authorized information from RRC-terminating IAB-donor in case of with Xn connection.

Proposal 5, reuse IAB Transport Migration Modification procedure to transfer the de-authorized information.

Proposal 6, agree the TP for TS 38.423 in the annex to support of mobile IAB de-authorization.

Proposal 7, how F1-terminating IAB-donor obtains the de-authorized information in case of without Xn connection is pending to the migration procedure for this case.

For question#2, i.e. “Whether and how is the Mobile IAB-DU indicated to refrain from setting up a F1-C connection to the donor CU immediately after the F1-C release (is there an indication in the F1-C release instructing the DU to not try again)? ”, in our view, Mobile IAB-MT will also receive authorization information via NAS message, so mobile IAB-DU will not try to trigger F1 connection again if the mobile IAB-MT is not authorized, we don’t see any issues here, IAB-node will act as normal UE if it’s not-authorized to be IAB-node, it will not trigger F1 connection, so no additional indication from IAB-donor is needed.

Observation 6, mobile IAB-node will act as normal UE if mobile IAB-MT receives the “not authorized” information from AMF via NAS.

Observation 7, the F1 connection from IAB-node will not be triggered if it acts as normal UE.

Proposal 8, reply SA2 the F1 connection from IAB-node will not be triggered if it acts as normal UE and no indication is needed from IAB-donor.

2.2	Additional ULI

Additional ULI of the IAB-MT’s serving is agreed to be provided along with ULI of the serving cell from the serving gNB of the UE, when the UE access to the network via the IAB-node.

In case the MT and DU connect to different CUs, the serving gNB of the UE (i.e. F1-terminating IAB-donor) is not aware of the ULI of IAB-MT’s serving cell, as the IAB-node is always aware of the ULI of IAB-MT’s serving cell, the co-located IAB-DU can provide the ULI of IAB-MT’s serving cell to F1-terminating IAB-donor via F1AP.

Proposal 9, the ULI of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU via F1AP message in case IAB-MT and IAB-DU are connected to different IAB-donors over F1AP message.

3	Conclusion

In this contribution, we discussed authorization related issues and how to provide additional ULI in case MT and DU connect to different CUs, and we had the following observations and proposals:

mobile IAB indication over NGAP

Observation 1, AMF may not perform mobile IAB authorization without a specific indication.

Observation 2, AMF may not get the authorized information without a specific indication in the request

Proposal 1, RAN3 either agree to introduce mobile IAB indication in NGAP INITIAL UE MESSAE or notify SA2 that RAN3 decide to reuse existing IAB-indication in NGAP INITIAL UE MESSAE to obtain mobile IAB authorization information.

authorization information from AMF

Proposal 2, RAN3 discuss whether it is possible that both IAB authorization information and mobile IAB authorization information will be received by IAB-donor.

Proposal 3, RAN3 check with SA2 whether a mobile IAB-node can be authorized as IAB-node when it stays immobile for a long time.

Observation 3, the IAB-MT can obtain the authorization information from AMF via NAS.

Observation 4, RRC-terminating IAB-donor can obtain the authorization information from AMF via NGAP

Observation 5, F1-termniating IAB-donor should be responsible for performing the de-authorization operation.

Proposal 4, F1-terminating IAB-donor needs to be informed about the de-authorized information from RRC-terminating IAB-donor in case of with Xn connection.

Proposal 5, reuse IAB Transport Migration Modification procedure to transfer the de-authorized information.

Proposal 6, agree the TP for TS 38.423 in the annex to support of mobile IAB de-authorization.

Proposal 7, how F1-terminating IAB-donor obtains the de-authorized information in case of without Xn connection is pending to the migration procedure for this case.

De-authorization

Observation 6, mobile IAB-node will act as normal UE if mobile IAB-MT receives the “not authorized” information from AMF via NAS.

Observation 7, the F1 connection from IAB-node will not be triggered if it acts as normal UE.

Proposal 8, reply SA2 the F1 connection from IAB-node will not be triggered if it acts as normal UE and no indication is needed from IAB-donor.

Additional ULI

Proposal 9, the ULI of IAB-MT’s serving cell is passed from IAB-DU to IAB-DU’s donor CU via F1AP message in case IAB-MT and IAB-DU are connected to different IAB-donors over F1AP message.

4	References

[1] S2-2308158 LS On IAB-node De-authorization handling, SA2

5	Annex A (TP to TS 38.423 support of mobile IAB De-authorization)

<stat of change>

### 8.5.3	IAB Transport Migration Modification

#### 8.5.3.1	General

The purpose of the IAB Transport Migration Modification procedure is to modify the backhaul information of the offloaded traffic in the topology of the non-F1-terminating IAB-donor of a boundary IAB-node. The procedure can also be used to release the resources under the non-F1-terminating IAB-donor used for serving the offloaded traffic. The procedure can also be used to release F1 connection between F1-terminating IAB-donor and mobile IAB-DU in case the mobile IAB-MT is not authorized

The procedure is applicable to inter-donor partial migration, inter-donor RLF recovery and inter-donor topology redundancy cases. The procedure is initiated by the non-F1-terminating IAB-donor of the boundary IAB-node.

The procedure uses UE-associated signalling.

#### 8.5.3.2	Successful Operation

Figure 8.5.3.2-1: IAB Transport Migration Modification, successful operation

The non-F1-terminating IAB-donor initiates the procedure by sending the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message to the F1-terminating IAB-donor. The F1-terminating IAB-donor responds with the IAB TRANSPORT MIGRATION MODIFICATION RESPONSE message.

If the Traffic Required To Be Modified List IE is contained in the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message, the F1-terminating IAB-donor shall update the backhaul information in non-F1-terminating topology for each traffic indicated in the list, and include the Traffic Required Modified List IE in the IAB TRANSPORT MIGRATION MODIFICATION RESPONSE message.

If the Traffic To Be Released Information IE is contained in the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message, the F1-terminating IAB-donor shall consider that all offloaded traffic will be released by the non-F1-terminating IAB-donor if the All Traffic Indication IE in the Traffic to Be Released Information IE is set to “true”, or that only the traffic indicated by the Traffic to Be Released Item IE will be released by the non-F1-terminating IAB-donor.

If the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message contains the Traffic To Be Released Information IE, the F1-terminating IAB-donor shall include the Traffic Released List IE in the IAB TRANSPORT MIGRATION MODIFICATION RESPONSE message.

If the IAB TNL Address To Be Added IE is contained in the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message, the F1-terminating IAB-donor shall allocate the TNL address(es) contained in this IE to the boundary IAB-node or the descendant IAB-nodes.

If the IAB TNL Address To Be Released IE is contained in the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message, the F1-terminating IAB-donor shall release the TNL address(es) contained in this IE for the boundary IAB-node or the descendant IAB-nodes.

If the IAB QoS Mapping Information IE is contained in the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message, the F1-terminating IAB-donor, shall, if supported, use it to set DSCP and/or IPv6 flow label fields for the downlink IP packets of the offloaded traffic.

If the Mobile IAB-node De-authorization IE is contained in the IAB TRANSPORT MIGRATION MODIFICATION REQUEST message, the F1-terminating IAB-donor, shall, if supported, initiate actions to ensure that the mobile IAB node will not serve any UE(s).

<next change>

#### 9.1.4.4	IAB TRANSPORT MIGRATION MODIFICATION REQUEST

This message is sent by a non-F1-terminating IAB-donor to an F1-terminating IAB-donor of a boundary IAB-node, for the purpose of modifying or releasing (e.g., for the purpose of revoking) the configuration for the migrated traffic of boundary IAB-node or descendant IAB-node.

Direction: non-F1-terminating IAB-donor  F1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-Terminating IAB-donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID, or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non-F1-Terminating IAB-donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID, or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Traffic Required To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Required To Be Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>Non-F1-terminating topology BH information | M |  | 9.2.2.83 |  | – |  |
| Traffic To Be Released Information | O |  | 9.2.2.84 |  | YES | reject |
| IAB TNL Address To Be Added | O |  | IAB TNL Address Response 9.2.2.86 |  | YES | reject |
| IAB TNL Address To Be Released List |  | 0..1 |  |  | YES | reject |
| >IAB TNL Address To Be Released Item |  | 1..<maxnoofTLAsIAB> |  |  | – |  |
| >>IAB TNL Address | M |  | 9.2.2.92 |  | – |  |
| Mobile IAB-node De-authorization | O |  | ENUMERATED (True, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor. The value is 1024. |
| maxnoofTLAsIAB | Maximum total no. of IPv4 address(es), IPv6 address(es) and IPv6 address prefix(es) that can be requested in one procedure execution. The value is 1024. |

9.1.4.5	IAB TRANSPORT MIGRATION MODIFICATION RESPONSE

This message is sent by the F1-terminating IAB-donor to the non-F1-terminating IAB-donor of a boundary IAB-node to acknowledge the update of configuration requested by the non-F1-terminating IAB-donor.

Direction: F1-terminating IAB-donor  non-F1-terminating IAB-donor.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| F1-Terminating IAB-donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Source NG-RAN node UE XnAP ID or to the M-NG-RAN node UE XnAP ID, or to the S-NG-RAN node UE XnAP ID. | YES | reject |
| Non-F1-Terminating IAB-donor UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | This IE refers to the Target NG-RAN node UE XnAP ID or to the S-NG-RAN node UE XnAP ID, or to the M-NG-RAN node UE XnAP ID. | YES | reject |
| Traffic Required Modified List |  | 0..1 |  |  | YES | reject |
| >Traffic Required Modified Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| Traffic Released List IE |  | 0..1 |  |  | YES | reject |
| >Traffic Released Item |  | 1 .. <maxnoofTrafficIndexEntries> |  |  | – |  |
| >>Traffic Index | M |  | 9.2.2.80 |  | – |  |
| >>BH Info List | O |  | 9.2.2.99 |  | – |  |
| Not Serving UE(FFS) |  |  | ENUMERATED (True, …) |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTrafficIndexEntries | Maximum no. of traffic offloaded to the non-F1-terminating IAB-donor. The value is 1024. |

<end of change>