3GPP TSG-RAN WG3 Meeting #121                                                                 R3-234259

Toulouse, France, Aug 21 – 25, 2023

Agenda item: 		13.2

Source: 		MITRE Corporation

Title: 	Discussion on XnAP and F1AP enhancements for mobile IAB

Document for:	Discussion and Agreement

# Introduction

In the previous TS-RAN WG3 meeting [1], following decisions were captured for mobile IAB:

WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

To be continued:

Whether the information on the DU’s CU can also be configured by the MT’s CU.

During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU.

To be continued:

How to pass the UE XnAP ID and NCGI or gNB-ID in case the IAB node knows the information of the DU’s CU by other means than OAM, if agreed.

It s FFS whether both gNB-ID and CGI needs to be provided to the mIAB-DU’s CU

It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP.

It is FFS whether the gNB-ID of the MT´s CU is passed from the IAB node to the IAB DU´s CU via F1AP.

WA: The mIAB-DU and mIAB-MT can integrate at different CUs. For this purpose, OAM can be used to configure the mIAB-DU with: a) the donor CU to connect to, and b) the parameters used by the mIAB-DU to establish TNL associations, IPSec tunnels and F1 connectivity to this donor CU.

To be continued:

Whether the information on the DU’s CU can also be configured by the MT’s CU.

During network integration where mIAB MT and the co-located mIAB-DU integrates to different donor CUs, mIAB-MT’s UE XnAP ID assigned by the MT´s CU and the gNB-ID of the MT´s CU shall be known to the mIAB-DU’s CU.

To be continued:

How to pass the UE XnAP ID and NCGI or gNB-ID in case the IAB node knows the information of the DU’s CU by other means than OAM, if agreed.

It s FFS whether both gNB-ID and CGI needs to be provided to the mIAB-DU’s CU

It is FFS whether the XnAP ID can be transparently passed from the mIAB-MT’s CU to the mIAB-MT via RRC, then from the co-located mIAB-DU to mIAB-DU’s CU via F1AP.

It is FFS whether the gNB-ID of the MT´s CU is passed from the IAB node to the IAB DU´s CU via F1AP.

Based on the previous meetings’ outcome, WI Rapporteur has proposed a “Workplan for Rel-18 mobile IAB” [2] for the current meeting. The goal of this submission is to further converge on solutions for the following issues, mentioned in that workplan:

- Passing of gNB-ID of MT’s CU and MT’s XnAP UE ID to DU’s CU

- (Full) DU migration

# Discussion

During its lifecycle, the mobile IAB node will experience IAB Integration (1), Inter-CU topology adaption for MT Migration (2) and Full DU Migration (3) as shown in Figure 1. In this diagram, the source and target nodes are abbreviated as SRC and TRG respectively. The nodes directly involved in MT migration are mIAB-MT; and RRC-terminating SRC MT-CU/DU and TRG MT-CU/DU. The nodes directly involved in DU migration are SRC mIAB-DU, TRG mIAB-DU; and F1-terminating SRC F1-CU and TRG F1-CU. The user data traffic flow between the User Equipment (UE) and Next Generation Core (NGC) at different stages is shown by the green lines.

The first two procedures have already been introduced in Release 17 [3] for one step partial migration and are being enhanced in Release 18. It may be noted that the second procedure consists of two phases: MT migration and F1 Transport migration. With the generalized node demarcations of Figure 1, subsequent partial migrations can be handled the same way as the initial partial migration. The third procedure i.e. the Full DU migration is currently under discussion in Release 18.

Figure 1: Overall sequence of IAB Integration, MT Migration and DU Migration

## Inter-CU topology adaptation

In this section, we’ll focus our discussion on the Inter-CU topology adaptation and passing of gNB-ID of MT’s CU and MT’s XnAP UE ID (referred as “New Info”) to DU’s CU.

In previous WG3 meeting [1], some companies proposed that the “New Info” is given to the mIAB-MT during Phase 1 and can be exchanged locally between mIAB-MT and the collocated mIAB-DU. This information can be optionally then passed on to the F1 terminating donor CU using GNB-DU CONFIGURATION UPDATE procedure. This is shown in Figure 1 with the red lines. Then the F1 terminating donor CU can establish a path with the target RRC terminating CU via IAB TRANSPORT MIGRATION MANAGEMENT. After this, the uplink and downlink traffic will flow via the target RRC terminating CU.

MT migration can be triggered anytime the mIAB-MT experiences difficult RF conditions. However, it may also be necessary to control the user data traffic flow for mIAB-DU while MT migration is still in progress. Otherwise, there is a possibility that the F1 terminating donor CU keeps sending the traffic to the old MT link (SRC MT-CU/DU), thus resulting in the loss and retransmission of that data. This is indicated by the broken green arrows in Figure 1.

Observation 1:  It may be necessary to control the downlink user data traffic flow while MT migration is still in progress.

Figure 2 details the modified MT migration procedure. We have introduced the modification (red lines) so that the user data flow control can be achieved with the IAB TRANSPORT MIGRATION MODIFICATION procedure.

Figure 2: Modified MT Migration procedure

Proposal 1: During MT migration, the user data flow control should be achieved with the IAB TRANSPORT MIGRATION MODIFICATION procedure.

The flow control can be applied both at the beginning and the ending of MT migration. We observe that at the ending, the gNBID of target MT-CU and XnAP UE ID of MT, referred as the “New Info” are already available at source MT-CU. Hence using IAB TRANSPORT MIGRATION MODIFICATION procedure at the ending of MT migration, the “New Info” can also be exchanged directly between non F1 terminating CU and F1 terminating CU.

Observation 2: With the IAB TRANSPORT MIGRATION MODIFICATION procedure during MT migration, the “New Info” can also be provided to the F1 terminating CU.

The description of modified procedure is given below:

- mIAB-MT sends MeasurementReport to source RRC terminating CU, indicating the need for MT migration.

- Source RRC terminating CU sends IAB TRANSPORT MODIFCATION REQUEST to F1 terminating CU, requesting suspension of the user data. F1 terminating CU confirms with IAB TRANSPORT MODIFCATION RESPONSE.

- Source RRC terminating CU sends HANDOVER REQUEST to target RRC terminating CU. The latter provides “New Info” (gNBID and XnAP UE ID) in the HANDOVER REQUEST ACKNOWLEDGE.

- mIAB-MT receives the target CU info in the RRCReconfiguration message.

- mIAB-MT connects with the target RRC terminating CU using RandomAccessProcedure.

- mIAB-MT confirms handover with RRCReconfigurationComplete.

- Target RRC terminating CU requests release of UE context at source CU.

- Now that the MT handover is complete, source RRC terminating CU informs source F1 terminating CU to get ready for resuming the user data. At that same time, it provides “New Info” also.

- Source F1 terminating CU establishes link with the target RRC terminating CU via IAB TRANSPORT MIGRATION MANAGEMENT.

In the previous discussion, we have considered the two possibilities:

- “New Info” is exchanged over XnAP during the MT migration

- “New Info” is exchanged over F1AP after the MT migration

The advantage of a) is that it also takes care of flow control during the MT migration.

Observation 3: The advantage of passing “New Info” over XnAP is that it also takes care of flow control during the MT migration.

Proposal 2: IAB TRANSPORT MIGRATION MODIFICATION procedure during MT migration may be used to exchange the gNBID of target MT-CU and XnAP UE ID of MT between non F1 terminating CU and F1 terminating CU.

If IAB TRANSPORT MIGRATION MODIFICATION procedure is kept optional during the MT migration stage, the “New Info” can be provided to the F1 terminating CU after the MT migration. But then the mIAB-MT may have to be signaled specifically for this follow-through.

Observation 4: If both XnAP and F1AP options are considered as optional for exchanging “New Info”, RRCReconfiguration message will need modification to indicate to the mIAB-MT that it needs to do a GNB-DU CONFIGURATION UPDATE procedure after the MT migration completes.

Proposal 3: If both XnAP and F1AP options are acceptable options for exchanging the gNBID of target MT-CU and XnAP UE ID of MT, RRCReconfiguration message should be modified to indicate to the mIAB-MT that it needs to do a GNB-DU CONFIGURATION UPDATE procedure after the MT migration completes.

## Full DU Migration

Since this is a new procedure in Release 18, we propose defining new signaling messages for the F1AP and XnAP. Our proposed procedure for the successful case is shown in Figure 3. It also takes care of the flow control while DU migration is in progress. In this diagram, the proposed new messages are shown in the red.

Figure 3: Successful case of Full DU Migration procedure

Observation 5: Full DU migration is a new procedure for Release 18 and needs new signaling messages for F1AP and XnAP.

The description of the successful full migration procedure is given below:

- When Full migration is triggered, the target F1-CU’s configuration is known to the source F1-CU e.g. configured by the OAM or CN, based on the individual node load reports and links availability.

- Source F1-CU indicates the migration start to target F1-CU with IAB F1 MIGRATION MANAGEMENT REQUEST. It also indicates the configuration for the RRC terminating CU in the same message.

- Source F1-CU asks the source logical mIAB-DU to add the second (target) logical mIAB-DU and provides the configuration for the target F1-CU in IAB DU MODIFICATION REQUEST.

- Target logical mIAB-DU sends F1 SETUP REQUEST to the target F1 CU. Target F1-CU establishes the new F1.

- Source logical mIAB-DU indicates the successful addition to the source F1-CU.

- Target F1-CU establishes path with the RRC terminating CU using IAB TRANSPORT MIGRATION MANAGEMENT procedure.

- Target F1-CU requests mIAB to delete the old logical DU.

- Target mIAB-DU confirms the deletion of the old logical DU.

- Target F1-CU informs the source F1-CU that migration is complete with IAB F1 MIGRATION MANAGEMENT RESPONSE. At this point, the latter may also terminate any intermediate donor CU connections (e.g. consecutive partial migration case).

- Path is switched from NGC to the target F1 CU.

NOTE: If any of the steps fail, procedure is aborted using IAB F1 MIGRATION MANAGEMENT REJECT.

Proposal 4: To support full DU migration, we propose the following new messages for XnAP:

- IAB F1 MIGRATION MANAGEMENT REQUEST/RESPONSE/REJECT, used for F1-DU migration handshaking between source and target F1 terminating CUs.

We also propose the following new messages for F1AP:

- IAB DU MODIFICATION REQUEST/RESPONSE, used for addition or deletion of logical mIAB-DU.

# Conclusions

In this contribution, we explored two possibilities to exchange the gNBID of target MT-CU and XnAP UE ID of MT (“New Info”) between non F1 terminating CU and F1 terminating CU.

Observation 1:  It may be necessary to control the downlink user data traffic flow while MT migration is still in progress.

Observation 2: With the IAB TRANSPORT MIGRATION MODIFICATION procedure during MT migration, the “New Info” can also be provided to the F1 terminating CU.

Observation 3: The advantage of passing “New Info” over XnAP is that it also takes care of flow control during the MT migration.

Observation 4: If both XnAP and F1AP options are considered as optional for exchanging “New Info”, RRCReconfiguration message will need modification to indicate to the mIAB-MT that it needs to do a GNB-DU CONFIGURATION UPDATE procedure after the MT migration completes.

Observation 5: Full DU migration is a new procedure for Release 18 and needs new signaling messages for F1AP and XnAP.

Proposal 1: During MT migration, the user data flow control should be achieved with the IAB TRANSPORT MIGRATION MODIFICATION procedure.

Proposal 2: IAB TRANSPORT MIGRATION MODIFICATION procedure during MT migration may be used to exchange the gNBID of target MT-CU and XnAP UE ID of MT between non F1 terminating CU and F1 terminating CU.

Proposal 3: If both XnAP and F1AP options are acceptable options for exchanging the gNBID of target MT-CU and XnAP UE ID of MT, RRCReconfiguration message should be modified to indicate to the mIAB-MT that it needs to do a GNB-DU CONFIGURATION UPDATE procedure after the MT migration completes.

Proposal 4: To support full DU migration, we propose the following new messages for XnAP:

- IAB F1 MIGRATION MANAGEMENT REQUEST/RESPONSE/REJECT, used for F1-DU migration handshaking between source and target F1 terminating CUs.

We also propose the following new messages for F1AP:

- IAB DU MODIFICATION REQUEST/RESPONSE, used for addition or deletion of logical mIAB-DU.

# References

- RAN-WG3#120 Chair Notes

- R3-233889 “Workplan for Rel-18 mobile IAB”

- 3GPP TS 38.401 “NG-RAN Architecture description”