# 3GPP TSG-RAN WG3 Meeting #121	R3-234267

# 21-25 August 2023, Toulouse, France

Agenda Item:	20.2

Source:	Ericsson

Title:	(TP to TS 38.401 BL CR): Correction of MT-SDT DL Data Notification

Document for:	Other

# Discussion

- The gNB-CU-CP configures the gNB-CU-UP during the Bearer context setup or modification (step 1) with a data volume threshold.

- The DL Data Notification can indicate the request of SDT termination.

# TP to TS 38.401 BL CR

>>>>>>START OF CHANGE<<<<<<<

### 8.18.x	MT-SDT

The procedure for mobile terminated small data transmission in RRC Inactive is shown in Figure 8.18.x-1.

Figure 8.18.x-1: Mobile Terminated Small Data Transmission in RRC Inactive state.

1. During the setup or modification of the bearer context as specified in 8.9.2, the gNB-CU-CP requests the gNB-CU-UP to provide MT-SDT information. The gNB-CU-CP can also configure the gNB-CU-UP with a SDT data volume threshold.

2a-0.	The gNB-CU-UP receives DL data for the UE in RRC Inactive on NG-U interface.

2a-1.	The gNB-CU-UP sends DL DATA NOTIFICATION message to the gNB-CU-CP. If determining that DL data packets are only mapped to SDT bearers, as requested in step 1, the gNB-CU-UP includes the MT-SDT information in the DL DATA NOTIFICATION message.

NOTE: In case of subsequent DL Data, the gNB-CU-UP indicates to gNB-CU-CP if the buffered DL data size exceeds the data volume threshold, if configured during step 1, and the gNB-CU-CP takes decision to terminate the SDT transaction.

2b. The gNB-CU-CP receives DL NAS signalling over NGAP.

3.	After 2a or 2b, the gNB-CU-CP sends PAGING message to the gNB-DU. The MT-SDT information may be included in the PAGING message.

4.	The gNB-DU sends the Paging message to the UE. In case the MT-SDT information is received in step 3, the gNB-DU includes the MT-SDT indicator in the Paging message.

5. If the UE has been successfully reached, it initiates the RRC connection resume procedure as described in 8.6.2 or 8.9.6.2, or initiates the SDT procedure as described in 8.18.1 or from step 9 in 8.18.2 or from step 1 in 8.18.3 with the following difference:

- In case SDT procedure is initiated, the UE may indicate the RRC Resume Request is for MT-SDT, which may be without UL data.

------------End of the Change-----------------