# 3GPP TSG-RAN WG3 Meeting #121	R3-234268

# 21-25 August 2023, Toulouse, France

Agenda Item:	20.2

Source:	Ericsson

Title:	(TP to TS 38.300 BL CR): Miscellaneous fixes

Document for:	Other

# Discussion

- Remove the EN on steps 3/4/5

- Addition of the specific ResumeCause indicated by UE in step 4/5

# TP to TS 38.300 BL CR

<<<<<<<<<<<<<<<<<<<<<<<< Begin of changes >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

## 18.X	MT-SDT with/without UE context relocation

The overall procedure for MT-SDT procedure with/without UE context relocation is illustrated in the figure 18.X-1.

Figure 18.X-1. MT-SDT with/without UE context relocation

1.  DL user data and/or DL NAS signalling are received at the Last Serving gNB for the UE in RRC_INACTIVE state.

2.  The Last Serving gNB may send MT-SDT information to the neighbour gNBs within the RNA, via XnAP RAN PAGING message.

3.  The gNB that receives MT-SDT information within the RNA, takes into account the MT-SDT information received in the XnAP RAN PAGING message to decide whether to trigger MT-SDT Paging. The gNB which ultimately reaches the UE via the Uu Paging becomes the Receiving gNB.

4/5.  The UE may decide to initiate MT-SDT procedure and sends an RRCResumeRequest message with a specific ResumeCause for MT-SDT to the Receiving gNB.

6. The Receiving gNB identifies the Last Serving gNB using the I-RNTI and retrieves the UE context by means of XnAP Retrieve UE Context procedure.

7.  The following steps are the same as Figure 18.2-1/18.3-1 from step 3, except that the first SDT user data and/or NAS signalling is DL SDT data and/or DL SDT NAS signalling.

NOTE: In case DL non-SDT data or DL non-SDT signalling has arrived between step 2 and step 7, the Last Serving gNB should relocate the UE context to the Receiving gNB and forward the received data to the Receiving gNB.

<<<<<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>>>>>>>>