3GPP RAN WG3 Meeting #120	R3-234269

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	23.2.1

Source:	Ericsson

Title:		Discussion on RAN3 impacts to support SL Positioning, CPP and other topics with TP for CPP support

Document for:	Discussion

# Introduction

In this paper we discuss the RAN3 impacts of the following topics:

- SL Positioning

- CPP

- PRS and SRS BW aggregation.

# Discussion

## Sidelink Positioning

In last meeting, we have discussed in our contribution [1] that LMF  needs to be involved in the SL Positioning signalling to communicate with the measuring UE in case of network-controlled resource allocation. In fact, the gNB must configure the source UE for SL-PRS transmission based on some recommendation from LMF (e.g., SL PRS characteristics for pre-configuring SL-PRS by the serving gNB) and the LMF should signal the SL-configuration to the other measuring UEs (e.g. RSU). In that sense, the signalling flow could be almost similar to e.g., UL-AoA or multi-RTT, where the serving gNB configures and provides the SL PRS configuration instead of UL-SRS to LMF after getting a new request from LMF via NRPPa POSITIONING INFORMATION REQUEST message. An overall call-flow with re-using existing NRRPA/F1AP procedures is presented below:

Figure 1: Signaling flow of SL-related information over NRPPA and F1AP to support scheme 1 SL positioning.

The details of the UE’s SL configuration by gNB (e.g., SL BWP, UE SL active BW configuration, BW configuration of SL resource pool, other SL parameters, etc.) are up to RAN1/RAN2. RAN3 can take this signalling as basis, with the F1/NRPPA impacts, and send LS to RAN2 to prompt for SL PRS configuration parameters.

Observation 1: For Sidelink Positioning, LMF should be able to deliver SL Positioning information to measuring UEs in coverage after gNB configuration

Proposal 1: Take proposed NRPPA/F1AP design as baseline for signalling and send LS to RAN2 to prompt for SL PRS configuration parameters

A draft LS to RAN2/RAN1 is provided in Annex 1.

## Carrier Phase Positioning

In the context of Rel-18 CPP, simultaneous transmission of UL SRS by a target UE and a PRU have been introduced by RAN1 in order to correct the TRP phase offsets. Since PRU is a UE with known location, measurements of the simultaneously transmitted UL SRS from a target UE and a PRU by a same TRP can be used to cancel the phase offsets introduced in the TRP. Below we can find the relevant RAN1 agreement son CPP and their potential impacts to RAN3 highlighted:

RAN1#112bis-e Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE RAN1#112bis-e Agreement Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. RAN#113 Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows

We understand from the RAN1 agreements that to support CPP, the LMF needs to be able to

- request the gNB to configure SRS configuration and activate its transmission time at the same time window as the PRU. This can be done by enhancing the messages defined in NRPPA and F1AP Positioning Information Exchange procedure;

- request the gNB to report simultaneous measurements for carrier phase positioning, i.e., UL RSCP (Reference signal carrier phase), based on the indicated SRS configuration that maps to a configured UL SRS transmitting UE. This can be done by enhancing the messages defined in NRPPA and F1AP Measurement procedure.

Observation 2: To support CPP simultaneous measurements of target UE and PRU, new enhancements are needed in the F1AP/NRPPA Positioning Information Exchange and Measurement procedures.

More specifically, for the SRS configuration phase, the LMF can indicate an Activation time IE in the POSITIONING INFORMATION REQUEST message, which can be a time window for simultaneous SRS transmission start time, duration of such a window and/or the periodicity of such a window, which are FFS by RAN1.

The gNB indicates to LMF the success of activating/triggering UL SRS transmissions in the POSITIONING INFORMATION RESPONSE. The existing SFN Initialisation Time IE can be re-used. A new time window IE can also be introduced (FFS).

Afterwards, the LMF sends in the NRPPA MEASUREMENT REQUEST message to request simultaneous measurements reporting. The LMF can indicate the request for simultaneous measurements (e.g., simultaneous UL CPP measurements for UL-RTOA and/or gNB Rx-Tx measurements) reporting as part of the Measurement Characteristics Request Indicator IE (9.2.81).  As stated by RAN1, the time window for reporting UL CPP can be indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time, already defined in the MEASUREMENT REQUEST message. FFS is an Activation time IE should be included in the message.

In case of simultaneous measurements for CPP are successful, the gNB includes a new UL CPP IE in the TRP Measurement Result IE (9.2.37). The reported CPP measurement contains the UL RSCP information.  (encoding FFS – pending RAN4).

Similar enhancements are needed over F1AP equivalent positioning messages.

Proposal 2: RAN3 to discuss the following changes to support CPP configuration and measurement reporting:

- Include a new Activation time IE in the POSITIONING INFORMATION REQUEST message. The gNB will configure UE and TRP to transmit UL SRS during the same time window indicated in the Activation time IE. FFS if the Activation time window is needed in the POSITIONING INFORMATION RESPONSE, or if the existing SFN Initialisation Time IE can be re-used.

- Include a new bit for simultaneous (e.g., UL RSCP with UL-RTOA and/or gNB Rx-Tx) measurements for CPP in the Measurement Characteristics Request Indicator IE (9.2.81) in the MEASUREMENT REQUEST message.

- Include a new UL CPP IE in the TRP Measurement Result IE (9.2.37), which contains the UL RSCP information (encoding FFS – pending RAN4).

- Adopt similar changes in corresponding F1AP messages and IEs

A TP to NRPPA BL CR is provided in Annex 2.

## PRS and SRS BW aggregation

RAN1 have made the following agreements related to Bandwidth aggregation for positioning measurements:

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. - FFS whether the link is for all TRPs or per TRP basis - FFS whether the link is per PRS resource set basis or per PRS resource basis.

Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked - SRS resources are per BWP per carrier configuration - FFS whether the link is per SRS resource set basis or per SRS resource basis. Agreement - Support LMF-initiated and UE-initiated on-demand PRS request for PRS bandwidth aggregation - FFS details - Support preconfigured on-demand PRS across PFLs for PRS bandwidth aggregations - FFS details Agreement From RAN1 perspective, support UE performs PRS measurement across multiple aggregated PFLs in RRC_CONNECTED, RRC_INACTIVE and RRC_IDLE state. Agreement Support joint measurement and report for the SRS resources across the aggregated carriers for UL-TDOA and Multi-RTT positioning methods - Single UL RTOA or gNB Rx-Tx time difference is reported for the SRS resources across aggregated carriers - FFS: RSRP or RSRPP - FFS: SRS carrier aggregation indication is reported along with the measurement results to indicate whether/which carriers are aggregated for the joint SRS measurement - Support LMF to request gNB for the UL positioning measurement from aggregated SRS resources across multiple CCs

On the first agreement, it is mentioned that some indication per TRP or per PRS resource basis could be introduced in the future to indicate whether there is an aggregation across positioning frequency layers. If such indication is needed, it could be indicated in the NRPPA and F1AP PRS CONFIGURATION RESPONSE messages where both TRP IDs and PRS configuration are signalled to LMF.

Regarding the second set of agreements, the signaling in NRPPA and F1AP will need to be impacted to link the SRS that need to be aggregated in the measurement report. For instance, based on LMF request, an Aggregation ID could be added to the SRS configuration either per SRS resource set level or per SRS Resource level. By knowing if SRS aggregation is supported and has been configured by the gNB, the LMF can ask the TRP to report UL measurement using only the bandwidth associated with this aggregated SRS resources, e.g., by pointing to the aggregation ID in the MEASUREMENT REQUEST. See Fig. 2 below.

Fig.2 Signaling of SRS Bandwidth aggregation

In the measurement report, the gNB-DU can include in the TRP measurement report whether the positioning measurements have been done using the indicated SRS aggregated bandwidth, i.e. by indicating in the TRP report whether the measurement are associated with an aggregation ID (e.g., as new IE in the SRS Resource Type IE), or by listing the SRS resources used for this measurement reporting which are part of the aggregated bandwidth. If not, the gNB-DU reports the measurement as in legacy. However an indication in the SRS Resource Type IE will still allow LMF to know if measurements with aggregated SRS resources could be reported by the gNB.

Observation 3: Bandwidth aggregation for positioning measurements may have RAN3 impacts on the PRS Configuration procedure and in the SRS measurement report

Proposal 3: RAN3 to consider supporting SRS bandwidth aggregation in NRPPA/F1AP, by defining an Aggregation ID  the SRS configuration IE per SRS resource and SRS Resource set level, and an Aggregated SRS in the SRS Resource Type IE with the list of SRS resources when aggregated SRS is used for measurements reporting

# Conclusion

Observation 1: For Sidelink Positioning, LMF should be able to deliver SL Positioning information to measuring UEs in coverage after gNB configuration

Proposal 1: Take proposed NRPPA/F1AP design as baseline for signalling and send LS to RAN1/2 to prompt for SL PRS configuration parameters

Observation 2: To support CPP simultaneous measurements of target UE and PRU, new enhancements are needed in the F1AP/NRPPA Positioning Information Exchange and Measurement procedures.

Proposal 2: RAN3 to discuss the following changes to support CPP configuration and measurement reporting:

- Include a new Activation time IE in the POSITIONING INFORMATION REQUEST message. The gNB will configure UE and TRP to transmit UL SRS during the same time window indicated in the Activation time IE. FFS if the Activation time window is needed in the POSITIONING INFORMATION RESPONSE, or if the existing SFN Initialisation Time IE can be re-used.

- Include a new bit for simultaneous (e.g., UL RSCP with UL-RTOA and/or gNB Rx-Tx) measurements for CPP in the Measurement Characteristics Request Indicator IE (9.2.81) in the MEASUREMENT REQUEST message.

- Include a new UL CPP IE in the TRP Measurement Result IE (9.2.37), which contains the UL RSCP information (encoding FFS – pending RAN4).

- Adopt similar changes in corresponding F1AP messages and IEs

Observation 3: Bandwidth aggregation for positioning measurements may have RAN3 impacts on the PRS Configuration procedure and in the SRS measurement report

Proposal 3: RAN3 to consider supporting SRS bandwidth aggregation in NRPPA/F1AP, by defining an Aggregation ID  the SRS configuration IE per SRS resource and SRS Resource set level, and an Aggregated SRS in the SRS Resource Type IE with the list of SRS resources when aggregated SRS is used for measurements reporting

# References

- R3-233064, Discussion on RAN3 impacts to support Rel-18 positioning enhancements, Ericsson

# Annex 1 - LS to RAN1, RAN2

3GPP RAN WG3 Meeting #120	R3-23xxxx

Toulouse, France, 21 – 25 Aug, 2023

Title:	LS on SL-PRS resource allocation and configuration

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN3

To:	RAN1, RAN2

Cc:

Contact person:	Yazid Lyazidi

Yazid.Lyazidi(at)Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 have discussed how to support SL-PRS resource allocation in scheme 1 network-controlled mode.

To RAN3’s understanding, the SL-PRS resource allocation and configuration in scheme 1 will follow similar design as UL-SRS configuration, where the serving gNB configures the SL positioning transmission (e.g. SL bandwidth information) to the transmitting (source) UE, then informs the LMF, which will then signal the configuration to the listening (destination) UEs.

RAN3 kindly asks RAN2 and RAN1 to provide the SL PRS configuration information that needs to be signalled to LMF.

2. Actions:

To RAN1,RAN2

ACTION: 	RAN3 kindly asks RAN1 and RAN2 to feedback on the above.

3. Date of Next TSG-RAN3 Meetings:

3GPP RAN3#121bis                               09 – 13 October 2023			Xiamen, China

3GPP RAN3#122	                       13 – 17 November 2023	                          Chicago, US

# Annex 2 – TP to NRPPA BL CR addition of CPP

START OF CHANGES

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the activation time IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node shall, if supported, configure the UE to start transmitting its UL SRS transmission at the indicated time instance.

NEXT CHANGES

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Activation time | O |  | 9.2.x | Indicates the start time when the SRS activation is requested |  |  |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |

NEXT CHANGES

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Z-AoA | M |  | 9.2.67 |  | YES | reject |
| >>Multiple UL-AoA | M |  | 9.2.71 |  | YES | reject |
| >>UL SRS-RSRPP | M |  | 9.2.72 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |
| >SRS Resource type | O |  | 9.2.73 |  | YES | ignore |
| >ARP ID | O |  | 9.2.75 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.2.77 |  | YES | ignore |
| >UL RSCP | O |  | 9.2.y |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

NEXT CHANGES

### 9.2.81	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: Additional Path Power Fourth Bit: Multiple UL AoA of Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG-ID information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Eleventh bit: UL-RSCP simultaneously reported with RTOA and/or gNB Rx-Tx time difference measurements Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

NEXT CHANGES

### 9.2.x	Activation time

This IE Indicates the start time when the SRS activation is requested.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | 9.2.36 |  |
| Simultaneous SRS window start time | O |  | FFS |  |
| Simultaneous SRS window duration | O |  | FFS |  |
| Simultaneous SRS window periodicity | O |  | FFS |  |

### 9.2.y	UL RSCP

This IE Indicates the UL Reference signal carrier phase measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL RSCP | M |  | FFS – RAN4 |  |