3GPP RAN WG3 Meeting #120	R3-234270

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	23.2.2

Source:	Ericsson

Title:		Discussion on RAN3 impacts to support LPHAP with TP

Document for:	Discussion

# Introduction

During RAN3#120 meeting, the following conclusion was made for LPHAP [1]:

Call flow with details on how to determine SRS validating area and which node allocates SRS resources in validating area.

This contribution provides our view on supporting LPHAP in RAN3 specifications, specifically we address the following aspects:

- How LMF provides the recommended validity area and SRS Characteristics to gNB;

- How the LMF is aware when UE leaves VA;

- Whether there is need of LPHAP assistance information from CN.

# Discussion

## VA determination

A RAN2 LS was received in [2] with the following agreement:

Agreements: - RAN2 consider that the LMF should determine the area-specific SRS configuration. Details are up to RAN3. - LS to RAN3 to indicate this conclusion, including RAN1 to prompt them for parameters. To be included in the LS from [AT122][415].

Further, it has been agreed by RAN2 that there can be area specific TAT.

Agreements from RAN2#122: Define an SRS for positioning validity-area specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. - The UE starts/restarts the area-specific TA timer when it receives the TA command. - The UE stops the SRS transmission when the area-specific TA timer expires. - The UE stops the area-specific TA timer when it reselects to a cell out of the SRS validity area. - Other stop/restart conditions can be discussed

Some aspects need to be discussed in RAN3 based on the RAN2 LS and above agreements. Although the LMF is stated by RAN2 to determine the “area-specific SRS configuration”, but this refers to an initial set of area recommendation, which follows similar design for SRS configuration where the LMF signals, e.g., the Spatial Relation Information and Pathloss Reference Information as recommendation, and leaves the final decision of the SRS configuration to the gNB. In fact, the LMF is not a radio entity, so it can only indicate what it considers, from its point of view, to be a candidate area of validity for the LPHAP. But the determination of the validity area and the SRS configuration associated to it needs to be acknowledged by the gNB.

Observation 1: The LMF signals VA recommendation to the serving gNB, the LMF is not a radio entity so it cannot force cells to gNB that it has no control of.

Therefore, the LMF can suggest a validity area to the serving gNB, e.g., it may be cells {A,B,C,D,E,F}. The serving gNB may coordinate with other gNBs, e.g., over Xn or OAM, and realize that it is only possible to include {A,B,C,D} in the VA. The serving gNB then informs the LMF of the final VA content. We provide below a signalling flow to illustrate the end-to-end signalling:

Fig.1 : Signalling and configuration of VA and SRS configuration for LPHAP

Proposal 1: Similarly to SRS configuration principle over NRPPA, the LMF sends a suggested VA (list of cells) to serving gNB, and serving gNB takes final decision of VA content according to OAM or Xn coordination.

The LMF may also suggest a VA timer so that serving gNB includes this value in the TA command sent to UE during step 6 in Fig. 1. But the serving gNB may end up deciding on another value of the TA timer for VA, based on the final VA size, available SRS resource pool, and positioning requirements to mitigate interference. Therefore, the serving gNB should also inform of final VA specific TA timer value to LMF together with the content of the determined VA. This is needed so that LMF knows for how long the VA LPHAP resources are valid before it can send a NRPPA POSITIONING DEACTIVATION message to end the LPHAP session.

Observation 2: the VA specific TA timer value is tied, among others, to VA size, available SRS resource pool, and positioning requirements to mitigate interference.

Proposal 2: In order for LMF to know the validity of the LPHAP session, the serving gNB must signal the final VA specific TA timer value to LMF together with the final VA size

Once the serving gNB sends the final VA cell content and VA-specific TA timer value to LMF, the LMF needs to further forward the information to other nodes in the VA via a new non-UE associated positioning message to notify the other gNBs of the VA’s SRS configuration and for how long they should reserve the resources. Since this is a notification, which can be updated if the serving gNB later signals a change of SRS configuration to LMF (in POSITIONING INFORMAITON UPDATE message), the new procedure from LMF to other nodes in the area should be called e.g., SRS Validity Area Update Notification procedure, since LMF notifies other gNBs to update the VA-specific SRS configuration.

Observation 3: The LMF needs to signal the VA-dedicated SRS information to other nodes in the VA via a new non-UE associated positioning message to notify the other gNBs of the VA’s SRS configuration and for how long resources should be reserved.

Proposal 3: Define a new SRS Validity Area Update Notification procedure from LMF to gNB that carries VA-related information. This notification does not force anything on gNBs.

This mechanism is applicable for both initial configuration of the validity-area specific TA timer or when UE reset VA specific TA timer.  LMF having the knowledge of VA specific TA timer can ensure that the nodes in VA will all know for how long the SRS configuration will be valid for the VA and when UE should start transmission (SFN start time).

Proposal 4: Define a new VA SRS Information IE in the POSITIONING INFORMATION RESPONSE and POSITIONING INFORMATION UPDATE messages that contains:

- The VA size (list of cells)

- The SRS configuration associated to this VA

- The VA-specific timer

## Case of UE leaving VA

In the below, we provide our view on the network behaviour in case UE resumes in a new node outside the VA. We also compare it with the legacy case:

Fig.2 : Signalling and configuration of VA and SRS configuration for LPHAP when UE resumes out of VA

In legacy case, the new gNB reconfigures the UE with a new SRS configuration, based on what old gNB signals in the XnAP RETRIEVE UE CONTEXT RESPONSE message. The LMF (and CN) is informed that UE has moved to another gNB after the Path Switch Request Procedure.

In case of LPHAP, when the UE resumes in a new gNB, the LMF must be notified first, so that LMF takes decision to either update the VA to include the new gNB in the VA, or to immediately start positioning. After the UE context retrieval in step 3 Fig. 2, the new gNB can send the NRPPA message including its local cell ID information to the LMF (step 8). At step 9, if LMF decides to update the VA, then it sends the POSITIONING INFORMATION REQUEST message to new gNB to reconfigure the UE with a new SRS configuration associated to an updated VA.

Otherwise, if LMF decides to not continue LPHAP, the LMF can ask the old node to release all the UL SRS resources using the legacy NRPPA POSITIONING DEACTIVATION message. It can also send a new POSITIONING INFORMATION REQUEST message to new gNB, as in legacy, to start positioning.

Observation 4: When UE resumes out of VA, the LMF must be timely informed of UE’s new cell ID to make decision. Whether the decision is to update LPHAP VA or to terminate LPHAP and start legacy positioning is up to LMF implementation

Proposal 4: The new gNB sends a POSITIONING INFORMAITON UPDATE message with new Cell ID IE to the LMF

## LPHAP Indication and Assistance Information

In TS 23.273, it is specified that LMF signals an LPHAP indication to NG-RAN in the positioning message if received from AMF:

### 5.15 Support of Low Power and High Accuracy Positioning Service requirements for low power and high accuracy positioning (LPHAP) is defined in TS 22.261 [3] and TS 22.104 [39]. Support of low power and high accuracy positioning is optional in this release of specification. Low power and high accuracy positioning is supported via subscription and in the LCS related subscriber data in the UDM, an LPHAP indication may be included. During the positioning procedure, AMF provides the LPHAP indication to the LMF. The LPHAP indication is either obtained from the GMLC, or stored in the UE LCS context received during UE registration procedure. If LMF receives from AMF of the LPHAP indication in the location request, LMF determines appropriate positioning method, e.g. network-based positioning method, or may determine to trigger the low power periodic and triggered 5GC-MT-LR procedures in clause 6.7 by taking into account the LPHAP indication. In addition, LMF also sends LPHAP indication to RAN in the positioning procedure, as defined in clause 6.11.2. Editor's note: whether Low power and/or High Accuracy capability/preference can be indicated by the UE to the network is FFS.

The details of the LPHAP indication are not specified, it can be a simple flag to indicate that the UE will use a LPHAP session. It is however unclear what is gNB behavior upon receiving the IE, i.e., whether deciding for UE RRC state decision or another purpose. Since we specify the receiver’s behavior in RAN3, we cannot add this IE without knowing its purpose.

Observation 5: It is unclear what is the purpose of the LPHAP indication and how should gNB act upon

Proposal 5: Do not add the LPHAP indication and send a LS to SA2.

Besides, the new LPHAP feature allows the UE to have the SRS configuration valid across multiple cells and UE will decide when to activate the SRS based upon the events configured. However, it is only LMF or UE which are currently aware of certain events, whereas the gNB which (pre)configures the SRS along with validity area or has to reserve the resources for it is not aware of the events type. Hence, the current mechanism is not efficient.

Observation 6: Currently the gNB does not know the events that causes the UE to request for UL SRS transmission.

Since it is gNB that configures the SRS and controls the radio resources used by UE, the gNB should be made aware of events so that gNB knows what configuration the UE needs in advance. If any configured event needs to be updated or modified, it is rather simpler if LMF updates to gNB using NRPPa Positioning Information Exchange procedures, and gNB provides the updated event reconfiguration to UE using RRC (Fig .3). This would enable gNB to have better control on the utilization of radio resources; example the SRS request trigger events can be preconfigured whereas the needed radio resource is only allocated when the request is sent by the UE.

Fig. 3: Signalling of Event type in NRPPA message.

The Event Type Information IE can contain the following information:

Whether the event type is a:

- Location Deferred Request (LDR)

- Location Immediate Request (LIR)

The Location Deferred Request indicates the following event types:

- UE available event information

- Periodic event information

- Entering area event information

- Leaving area event information

- Being inside area event information

- Motion event information

Proposal 6: The gNB is made aware of events type so that gNB knows what configuration the UE needs in advance for LPHAP. The LMF signals the Event type information to gNB in the POSITIONING INFORMATION REQUEST message.

We provide in Annex of this document a TP to NRPPA BLCR adding the Event Type Information IE

# Conclusions and Proposals

Our observations and proposals are summarized below.

Observation 1: The LMF signals VA recommendation to the serving gNB, the LMF is not a radio entity so it cannot force cells to gNB that it has no control of.

Proposal 1: Similarly to SRS configuration principle over NRPPA, the LMF sends a suggested VA (list of cells) to serving gNB, and serving gNB takes final decision of VA content according to OAM or Xn coordination.

Observation 2: the VA specific TA timer value is tied, among others, to VA size, available SRS resource pool, and positioning requirements to mitigate interference.

Proposal 2: In order for LMF to know the validity of the LPHAP session, the serving gNB must signal the final VA specific TA timer value to LMF together with the final VA size

Observation 3: The LMF needs to signal the VA-dedicated SRS information to other nodes in the VA via a new non-UE associated positioning message to notify the other gNBs of the VA’s SRS configuration and for how long resources should be reserved.

Proposal 3: Define a new SRS Validity Area Update Notification procedure from LMF to gNB that carries VA-related information. This notification does not force anything on gNBs.

Proposal 4: Define a new VA SRS Information IE in the POSITIONING INFORMATION RESPONSE and POSITIONING INFORMATION UPDATE messages that contains:

- The VA size (list of cells)

- The SRS configuration associated to this VA

- The VA-specific timer

Observation 4: When UE resumes out of VA, the LMF must be timely informed of UE’s new cell ID to make decision. Whether the decision is to update LPHAP VA or to terminate LPHAP and start legacy positioning is up to LMF implementation

Proposal 4: The new gNB sends a POSITIONING INFORMAITON UPDATE message with new Cell ID IE to the LMF

Observation 5: It is unclear what is the purpose of the LPHAP indication and how should gNB act upon

Proposal 5: Do not add the LPHAP indication and send a LS to SA2.

Observation 6: Currently the gNB does not know the events that causes the UE to request for UL SRS transmission.

Proposal 6: The gNB is made aware of events type so that gNB knows what configuration the UE needs in advance for LPHAP. The LMF signals the Event type information to gNB in the POSITIONING INFORMATION REQUEST message.

# References

- RAN2-122 Chair’s Notes

- R3-226165, S2-2209591, LS on LPHAP information delivery to RAN

# TP to NRPPA BL CR

START OF CHANGES

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the NG-RAN node shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account for allocating proper CG-SDT resources when positioning a UE.

If the UE TEG Information Request IE is included in the POSITIONING INFORMATION REQUEST message and set to "onDemand", the NG-RAN node shall, if supported, provide the UE Tx TEG association in the POSITIONING INFORMATION RESPONSE message.

If the UE TEG Information Request IE is set to "periodic", the NG-RAN node shall, if supported, reply with the POSITIONING INFORMATION RESPONSE message without including any UE Tx TEG association in this message. The NG-RAN node shall then take the UE TEG Reporting Periodicity IE into account when configuring the UE’s periodic UE Tx TEG association reporting and initiate the Positioning Information Update procedure for reporting the UE Tx TEG association received from the UE, if any.

If the Event Area Information IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE for LPHAP.

If the List of recommended cells for VA determination IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when determining the Validity Area for the UE’s SRS transmission, and it shall include the List of cells included in the VA IE and the VA Specific Timer IE in the POSITIONING INFORMATION RESPONSE message.

NEXT CHANGES

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by the LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| UE Reporting Information | O |  | 9.2.70 |  | YES | ignore |
| UE TEG Information Request | O |  | ENUMERATED(onDemand, periodic, stop, …) |  | YES | ignore |
| UE TEG Reporting Periodicity | C-ifUeTegInfoReqPeriodic |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, …) |  | YES | reject |
| Event Type Information | O |  | 9.2.X |  | Yes | ignore |
| List of recommended cells for VA determination |  | 1 .. <maxnoOfCells> |  |  | EACH | reject |
| >NR CGI | M |  | 9.2.9 |  | - | reject |
| VA specific timer | O |  | ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity} | suggested VA specific TA timer value | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifUeTegInfoReqPeriodic | This IE shall be present if the UE TEG Information Request IE is set to the value "periodic". |
| maxnoOfCells | MAXIMUM NUMBER OF CELLS IN ng-ran 16384 |

#### 9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by the NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| UE Tx TEG Association List | O |  | 9.2.78 |  | YES | ignore |
| List of cells included in the VA |  | 1 .. <maxnoOfCells> |  | Validity Area | EACH | reject |
| >NR CGI | M |  | 9.2.9 |  | - | reject |
| VA specific timer | O |  | ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity} | Determined VA specific TA timer value | YES | ignore |

| Condition | Explanation |
| --- | --- |
| maxnoOfCells | MAXIMUM NUMBER OF CELLS IN ng-ran 16384 |

NEXT CHANGES

#### 9.2.X Event Type Information

The IE contains the area event type information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Area Event Type | M |  |  |  |
| >Location Immediate Request |  |  |  |  |
| >>Location Immediate Request | M |  | OCTET STRING | LIR reference as defined in TS 29.572 [X] |
| >Location Deferred Request |  |  |  |  |
| >>Location Deferred Request | M |  | OCTET STRING | LDR reference as defined in TS 29.572 [X] |
| >>LDR type | M |  | ENUMERATED(UE available event, Periodic event, Entering area event, Leaving area event, Being inside area event, Motion event, …) |  |

NEXT CHANGES

ASN.1 changed to be fixed during the meeting pending convergence