3GPP TSG-RAN WG3 Meeting #121	R3-234291

21st – 25th August 2023

Toulouse, France

Title: 	Open points on requested prediction time

Source: 	Ericsson, InterDigital, Deutsche Telekom

Agenda item:	12.2.1

Document Type:	Discussion and Decision

# Introduction

In the RAN3#120 meeting, the following agreements and FFS were captured in the meeting minutes:

The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed.

FFS on the encoding and exact values of the specific time point. Down-select the options:

- INTEGER TYPE

- ENUMERATED TYPE

FFS on the benefits of the predicted information over a period of time.

The specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future, can be configured for both one-time reporting and periodic reporting.

In the following, we discuss the open points and put forward our proposals.

# Discussion on requested predicted time

## RAN3 agreements

During the last meeting, RAN3 agreed to introduce the requested prediction time, for both one-time and periodic reporting, as a distinct IE different from the Reporting Periodicity IE. Furthermore, RAN3 agreed that the requested prediction time should refer to a point in time in the reasonable future. This is an important remark since, currently, only three prediction quantities have been agreed by RAN3, where a prediction time would be applicable; these are “Predicted Radio Resource Status”, “Predicted Number of Active UEs”, and “Predicted RRC connections”. These predictions are proposed to support load balancing operations for which actions are expected in the immediate future; hence, there is no clear need or benefit to introduce predictions times for a distant time in the future.

The current RAN3 understanding of requested prediction time is depicted in Figure 1 for the case of periodic reporting. In that example, NG-RAN node 1 requests a prediction, e.g., “Predicted Number of Active UEs”, to NG-RAN node 2 with a requested prediction time of 20 seconds and a reporting periodicity of 10 seconds. This means that, every 10 seconds, NG-RAN node 2 will send a prediction that refers to 20 seconds into the future (i.e., the predicted value of the requested quantity in that time in the future).

Although not explicitly shown in Figure 1, there could be an initial delay after the AI/ML INFORMATION RESPONSE message where NG-RAN node 2 prepares the prediction framework, for example, by loading needed AI/ML software libraries and models. After that, NG-RAN node 2 sends an AI/ML INFORMATION UPDATE message every 10 seconds.

The periodic reporting procedure is finished when NG-RAN node 1 signals the AI/ML INFORMATION REQUEST message with the Registration Request IE set to “stop”.

Figure 1: Example flowchart for periodic reporting.

## Open points

### Encoding of requested prediction time

The first open point captured during the RAN3#120 meeting relates to the encoding of the requested prediction time, where two options were selected: integer vs enumerated type. We analyze both options below.

Integer encoding is the most flexible of both options since it allows any possible value in the defined range. One option described in the SoD in R3-233341 proposed an integer encoding with a range between 0 and 4095 seconds. A first question regarding this proposal is whether 4095 seconds, which correspond to around 68 minutes, can be understood as a “point in time in the reasonable future” in terms of load balancing operations.

Normal load balancing actions are expected to happen at most in a matter of minutes after analyzing the present system state. The predicted load situation in one hour in the future would most likely be a seasonal average, e.g., depending on the time of the day and week, and would not depend significantly on the present load situation. Therefore, focusing on  such predictions that are far away in time an uncorrelated with current conditions and events seems inappropriate.

A second question is whether there would be a significant difference between a prediction with a requested predicted time of e.g. 59 minutes in the future versus 60 minutes in the future. Probably not much, whereas a prediction with a requested predicted time of 1 minute in the future would likely differ from one with a requested prediction time of 2 minutes in the future. The same 1-minute difference has different effects depending on how close to the present time the prediction refers to.

Observation 1:  Encoding the requested prediction time with an integer type results in high flexibility, but most values far from the present time will generate similar predictions. Such predictions far away in time are of reduced value and questionable accuracy.

A third, and final question is whether the flexibility provided by the INTEGER type justifies the complexity in the AI/ML model development. One high-level principle agreed by RAN3 for AI-enabled RAN intelligence is:

An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment.

The complexity derived from encoding the Prediction Time as an INTEGER comes from the need to properly train, validate and test an AI/ML model for inferring a prediction at each and every possible requested prediction times, even if many different requested prediction times return similar prediction outputs. As an example, if the Prediction Time is an INTEGER (1, ..100), a Model Training Function would need to be trained with the following training datasets:

- One data set including, for each training data sample, the set of inputs at the time of inference plus the ground truth at Prediction Time = 1

- One data set including, for each training data sample, the set of inputs at the time of inference plus the ground truth at Prediction Time = 2

- …

- One data set including, for each training data sample, the set of inputs at the time of inference plus the ground truth at Prediction Time = 100

A similar process would need to be run for model validation and testing.

It is immediately evident that, for each value of the Prediction Time, a different training, validation and testing process needs to be taken. This makes the complexity and cost of developing a trained, validated and tested model linearly dependent on the number of values in the Prediction Time range.

The latter is exactly why the set of values in the Prediction Time range should be limited. This point is even more valid if predictions derived for Prediction Times that are close to each other are similar, i.e. predictions for nearby prediction times do not reveal any information that would add information on how to make the system behaviour change.

Hence, defining the Prediction Time as an INTEGER brings a very high cost that does not result in a significant added benefit.

Observation 2:  Encoding the requested prediction time as an INTEGER type results in high cost and complexity in order to properly train, validate, and test the AI/ML models, while not creating additional benefits.

To overcome the problem of complexity and cost, when using an INTEGER type, some companies argued during the RAN3#120 meeting, that the requested prediction time could be encoded using an INTEGER type while, at the same time, the AI/ML models could be trained with only a subset of possible values of the Prediction Time. According to such views, when a node requests a prediction with a certain requested Prediction Time, the reporting node (providing the prediction) could reply with the prediction derived for the closes prediction time the model was trained for.

The example used during RAN3-120 was the following:

- Assume that Prediction Time is e.g. an INTEGER (1, ..100)

- A requesting node asks for a prediction at Prediction Time = 20

- The AI/ML model at the reporting node has been trained for Prediction Time = 5, 15, 30, 60, 100

- Then the reporting node would provide a prediction for Prediction Time = 15, since it is the closest available one to the requested Prediction Time

In other words, according to the logic presented during RAN3-120, the actual Prediction Time for which a prediction is provided would be unknown to the requesting node and only an approximation of the requested Prediction Time.

We consider that this approach would lead to problems of accuracy, reliability and interpretability, where the node receiving the prediction cannot be sure if the prediction received is for the requested Prediction Time or any other time. It would be totally up to the node providing the prediction to determine which Prediction Time to use in case its model is not trained for the requested Prediction Time. This approach contradicts the principle that the selection of input parameters for the inference (i.e. the Prediction Time in this case) shall be done by the requesting node, which will use the prediction, and not modified by the reporting node.

One immediate issue we would have with the approach above is that the requesting node may compare the received prediction with the values measured later on at the time indicated by the Prediction Time. Such measured values might be different from the predictions, making the requesting node deduce that the predictions generated by the reporting node are inaccurate. This is incorrect, as the predictions may not be inaccurate but only calculated for a different Prediction Time. In conclusion, if a prediction is requested for a given Prediction Time, the reporting node shall respect the request and provide the appropriate prediction. If the reporting node cannot infer the requested prediction, the measurement configuration shall be failed.

The alternative to encoding the Prediction Time as an INTEGER is to encode it as an ENUMERATED.

An ENUMERATED type can be defined to include the most valuable requested prediction times in order to reduce the cost and complexity of developing the AI/ML models.

As an example, if the Prediction Time is encoded as ENUMERATED (1, 2, 4, 8, 16, 32, 64, 128,…), where each value indicates an offset in seconds, then an AI/ML model would have to be trained, validated and tested only for the limited set of values in the ENUMERATED type range (i.e. 8 values in this case). By making the ENUMERATED extendible, more values can be added if it is identified that predictions for such Prediction Time values reveal important events.

Proposal 1:  The requested prediction time should be encoded as an ENUMERATED type.

The following tabular is an example of the Requested Prediction Time IE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time | C-ifPredictionRequested |  | ENUMERATED(1, 2, 4, 8, 16, 32, 64, 128, …) FFS on the values | Unit: seconds. This IE shall be present if there is a request for predictions. |

We note that this IE should be present whenever the procedure is used to request a prediction, i.e., “Predicted Radio Resource Status”, “Predicted Number of Active UEs”, or “Predicted RRC connections”. This is why the example above includes a condition for the IE presence.

Proposal 2:  RAN3 to take the following tabular as baseline for defining the Requested Prediction Time IE:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time | C-ifPredictionRequested |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, 128, …) FFS on the values | Unit: seconds. This IE shall be present if there is a request for predictions. |

### Requested prediction time as an interval

A second FFS is about the benefits of the requested prediction time being defined as a period of time instead of a point in time in the future. The first question to be asked is how to compute a prediction, e.g., “Predicted Number of Active UEs”, in a time interval. The simplest answer is that the prediction would be an average value during the future time interval. However, as we explain next, this results in a loss of information, and thus it is not beneficial.

For example, let us assume that NG-RAN node 1 subscribes to “Predicted Number of Active UEs” from NG-RAN node 2 for requested prediction times of 5 seconds and 10 seconds in the future. NG-RAN node 1 therefore receives the information that the “Predicted Number of Active UEs” is 200 in 5 seconds and 100 in 10 seconds. This information reflects that, in the time span of 10 seconds, there is a reduction in the load at the NG-RAN node 2, which could be used by NG-RAN node 1 to plan offloading of some UEs.

On the other hand, if NG-RAN node 1 subscribes to the same prediction but as an average in the time interval between 5 seconds and 10 seconds in the future, the prediction would indicate 150 active UEs in that interval. This information does not allow NG-RAN node 1 to know if it should wait for a certain period (e.g. 10 seconds) to perform a load balancing action or not, while with the predictions received at 5 and 10 seconds, this could be deduced.

It should be also noted that, once NG-RAN node 1 has the predictions for Prediction Time = 5 and Prediction Time = 10, it can itself easily derive the average between these two values.

As shown, we do not see any benefit in defining the requested prediction time as an interval. Averaging in an interval is, at most, an enhancement for a system that only produces pointwise predictions. An AI/ML model that needs to output both pointwise and average predictions is more costly to train, validate, and test that one that only produces pointwise predictions. And the added cost does not result in a clear benefit both because averaging implies loss of information and because averaging can be carried out by the node receiving pointwise predictions.

Proposal 3:  Requested prediction time should not be defined as a time period.

### Corrections to Reporting Periodicity semantics

In the agreed BLCR to TS 38.423, the semantics description of the Reporting Periodicity IE included in the AI/ML INFORMATION REQUEST has the following text (where the yellow highlight has been added here):

Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported.

The highlighted sentence was copy pasted from the Reporting Periodicity IE semantics in the legacy RESOURCE STATUS REQUEST message. 
However, while this sentence could be valid for measurements configured via the AI/ML INFORMATION REQUEST, it does not make sense for predictions carried out at specific prediction times. The semantics of the Reporting Periodicity IE included in the AI/ML INFORMATION REQUEST should therefore be corrected as follows:

| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for requested measurements (i.e. excluding predictions), if supported. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Proposal 4:  The sentence in thes emantics description of the Reporting Periodicity IE included in the AI/ML INFORMATION REQUEST stating that the Reporting Period can be also used as the averaging window length should apply only to requested measurements and not to requested predictions.

# Conclusion

In this contribution, we discussed the open issues regarding requested prediction time, and we made the following observations and proposals:

Observation 1:  Encoding the requested prediction time with an integer type results in high flexibility, but most values far from the present time will generate similar predictions. Such predictions far away in time are of reduced value and questionable accuracy.

Observation 2:  Encoding the requested prediction time as an INTEGER type results in high cost and complexity in order to properly train, validate, and test the AI/ML models, while not creating additional benefits.

Proposal 1:  The requested prediction time should be encoded as an ENUMERATED type.

Proposal 2:  RAN3 to take the following tabular as baseline for defining the Requested Prediction Time IE:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested prediction time | C-ifPredictionRequested |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, 128, …) FFS on the values | Unit: seconds. This IE shall be present if there is a request for predictions. |

Proposal 3:  Requested prediction time should not be defined as a time period.

Proposal 4:  The sentence in the semantics description of the Reporting Periodicity IE included in the AI/ML INFORMATION REQUEST stating that the Reporting Period can be also used as the averaging window length should apply only to requested measurements and not to requested predictions.

A TP mirroring the above proposal is available in R3-234292.