3GPP TSG-RAN3 Meeting #121	R3-234298

Toulouse, France - August 21st – August 25th 2023

Agenda Item:	12.3

Source:	Ericsson, AT&T, InterDigital, Deutsche Telekom

Title:	Continuous MDT proposition

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#120 meeting contributions on Continuous MDT have been discussed were the benefits of the solution on AI/ML based processes have been presented but questions on the correlation of the logged and immediate MDT have been raised.

In this paper we analyse the comments made during RAN3-120 and derive conclusions for an appropriate realisation of continuous MDT.

# 2	Discussion

During the RAN3#120 Continuous MDT has been discussed and a number of interesting questions, captured in the extract below from the chair’s notes, have been raised:

Common understanding in RAN3:

For signaling based MDT, TCE could identify the UE via UE identifier

For management based MDT, it is not possible for TCE to identify the UE with UE identifier that can lead to the unique UE identity since m-based MDT is intentionally made anonymous.

Further evaluate the gap of continuous MDT during idle to active transition for both management based MDT and signaling based MDT, e.g. how many milliseconds? What is the maximum acceptable gap for continuous MDT? e.g. consider the report periodic of UE measurement report.

Further check whether the management based MDT identifier e.g. IMEI-TAC, NR- CGI, TR/TRSR allows the TCE to correlate logged MDT and immediate MDT collected during continuous MDT.

If correlation of log MDT and immediate MDT is possible for management based MDT, FFS whether the current mechanism already support configuration of continuous MDT i.e. when UE connected to a new cell and report the availability of log MDT, the connected cell could configure the UE with immediate MDT immediately.

If correlation of log MDT and immediate MDT is possible for management based MDT, FFS on selection of group of UE when deciding to implement continuous MDT.

Continuous MDT has been discussed during a number of meetings and it has been recognized that it is beneficial for AI/ML to enable collection of continuous data sets via MDT, i.e., data sets that do not need to be normalised against UE capabilities. Based on the arguments presented during these meetings it has been proposed to agree to support continuous MDT tracing for a given UE and across various RRC states (RRC_Connected, RRC_Idle, RRC_Inactive).

We would like to first clarify that the the proposals for continuous MDT made so far should be based on Management Based MDT. The reason for such statement is that Management Based MDT does not require the storage of a per UE MDT context, which is required in the case of Signalling based MDT.

In fact, in signalling based MDT, a UE-dedicated procedure needs to be started from the OAM to the CN and from the CN to the NG-RAN for every UE that needs to be traced. With such UE dedicated procedure, the RAN needs to create and maintain for each UE an MDT context, which makes Signalling Based MDT extremely expensive, especially when applied to AI/ML where many UEs need to be selected for MDT data collection.

Another issue with Signalling Based MDT is that it is affected by the gaps in MDT data collection. 
If Signalling Based MDT is used, it would be possible to configure a UE with Immediate MDT to make it collect data during RRC_CONNECTED, as well as to configure the UE with Logged MDT to make it collect data in case the UE moves to RRC_IDLE/RRC_INACTIVE. Therefore, data collection continuity can be ensured for RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE transitions.

However, whenever a UE moves from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED, Signalling Based MDT is not able to provide data collection continuity. This is because for a UE that moves from RRC_IDLE to RRC_CONNECTED the UE/network need to perform the following tasks before the UE can be configured with Signalling Based MDT again:

- UE needs to perform access to the radio network (RACH access up to Msg5), including gNB-DU to gNB-CU signalling for RRC message transfers

- Triggering of NG: UE Context Setup procedures, involving: RAN to CN NG: Initial UE Message triggering; Intra CN signalling for retrieval of subscriber´s data from UDM (see TS23.502); Intra CN signalling for CN function selection, e.g. AMF re-allocation (see TS23.502)

- UE context Setup at NG-RAN, including bearer setup between gNB-CU-UP, gNB-CU-CP and gNB-DU; UE reconfiguration over RRC to establish security and DRBs

- Signaling form OAM to CN of a Signalling Based MDT configuration (unless this is done in advance)

- Signalling over the NGAP, from CN to RAN, of the received Signalling Based MDT configuration; Signalling of the Signalling Based MDT configuration over F1AP and E1AP to gNB-DU and gNB-CU-UP

- UE reconfiguration over RRC to configurer MDT measurement collection

The above procedures can easily amount to an overall time of seconds (for example think of the case of retrieving UE capabilities and retrieving subscriber´s data, which may take several hundreds of milliseconds if not seconds). This implies that achieving Continuous MDT via Signalling Based MDT is not viable because in cases of RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED transitions, the MDT data collection process would be affected by a data collection gap of up to seconds. In such interval of time many events can occur that an AI/ML training/re-training function should be informed about, such as RLF, connection establishment failures, connections to erroneous cells, poor UL converage etc.

Conclusion 1:  Achieving Continuous MDT via Signalling Based MDT is not viable because in cases of RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED transitions, the MDT data collection process would be affected by a data collection gap of up to seconds

Proposal 1: Agree to achieve Continuous MDT by means of enhancements to Management Based MDT

In order to achieve Continuous MDT via Management Based MDT, a number of challenges have been identified together with solutions to address them.

We start with the assumption that a UE selected for Management Based MDT can be configured at the same time with Immediate and Logged MDT, so to ensure continuous MDT data collection during RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE, as explained above. To achieve this, the RAN should configure the UE for immediate MDT (with a configuration associated to e.g. Trace Session 1) and before sending it to IDLE it should configure it again for logged MDT (with a configuration associated to e.g. Trace Session 2).
The challenge come for RRC state transitions from RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED. In this case a UE previously configured with Logged MDT should be identified as a “Continuous MDT UE, namely a UE that needs to be configured with immediate MDT measurements and therefore achieve continuous MDT reporting.

To overcome this challenge, it is proposed that the logged MDT configuration given to the UE also indicates that the UE is selected for continuous MDT reporting. Once the UE has this information, it can signal it to the NG-RAN when it moves to RRC Connected again, so to give the possibility to the RAN to configure the UE with Immediate MDT as soon as the UE moves to RRC_CONNECTED.

Proposal 2: Continuous MDT is based on signalling to the UE, as part of the Logged MDT configuration, that the UE is selected for Continuous MDT. This enables the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from RRC_Idle/RRC_Inactive. Hence the UE can be configured with Immediate MDT measurements.

A summary of the procedure can be provided by the following figure and steps

Figure 2: Example of “continuous MDT” signalling support

- The RAN receives a Management Based MDT configuration made of at least two configurations: an Immediate MDT configuration and a Logged MDT configuration. The two configurations have different Trace Session identifiers. This configuration contains a “Continuous MDT” flag. With this information, the RAN knows that it has to configure selected UEs with MDT measurements in RRC_CONNECTED and in RRC_IDLE/INACTIVE.

- The RAN may configure the UE with Immediate MDT and logged MDT as per the configuration details above (i.e. including a “continuous MDT” flag in the Logged MDT configuration pushed to the UE).

- The UE reports Immediate MDT measurements collected while in RRC_CONNECTED and according to the Immediate MDT configuration received.

- The UE moves to RRC_IDLE/INACTIVE. The UE will collect logged MDT measurements and store them as per received Logged MDT configuration.

- When the UE moves from RRC_IDLE/INACTIVE to RRC_CONNECTED, the UE indicates to the RAN, as part of the RRC Setup procedure, that it is configured with Continuous MDT. This allows the RAN to immediately configure the UE with Immediate MDT measurements, which can be added to the Continuous MDT trace.

- The RAN configures the UE with Immediate MDT measurements

- The UE reports Logged MDT results, which can be added to the Continuous MDT trace

- The RAN configures the UE with a Logged MDT configuration including a “Continuous MDT” flag

- To guarantee for continuous MDT measurement collection across connected mode mobility, an indication of “Continuous MDT” is included in the Handover Preparation signalling towards the target RAN. The target RAN is therefore able to appropriately configure MDT at the UE.

As it can be seen above, a UE can be configured with Management Based Immediate MDT already at Step 6. Therefore, the measurement collection gap during RRC_IDLE/RRC_INACTIVE to RRC_CONNECTED transitions when using Management Based MDT is at most of a few tens of milliseconds, namely it reduces to the latency in concluding the RRC Setup procedure.

Conclusion 2: Achieving Continuous MDT via Management Based MDT reduces the data collection gap at RRC_IDLE to RRC_CONNECTED transitions to the latency needed to complete the RRC Setup procedures, which is at worst of a few tens of milliseconds.

During the last meeting concerns regarding correlation of logged and intermediate MDT have been raised. In particular the following open point was raised:

Further check whether the management based MDT identifier e.g. IMEI-TAC, NR- CGI, TR/TRSR allows the TCE to correlate logged MDT and immediate MDT collected during continuous MDT.

We acknowledge that Management Based MDT does not offer the possibility to merge Immediate and Logged MDT data collected by the same UE across different RRC states. In order to do this an identifier would be needed that is consistently maintained across Management Based MDT data collection in different RRC states and that is associated to the collected MDT data.

We propose to raise the need of such identifier to SA5 as a requirement for the realization of Continuous MDT and therefore we propose the following:

Proposal 3: To achieve realization of Continuous MDT an identifier would be needed that is consistently maintained across RRC state changes and that is associated to the collected MDT data. It is proposed to LS SA5 and to ask on the possibility to define such identifier.

# 3	Conclusion

In this paper an analysis of how to achieve Continuous MDT has been made in light of recent discussions taken at RAN3-120. The following conclusions were derived:

Conclusion 1:  Achieving Continuous MDT via Signalling Based MDT is not viable because in cases of RRC_IDLE to RRC_CONNECTED transitions, the MDT data collection process would be affected by a data collection gap of up to seconds

Conclusion 2: Achieving Continuous MDT via Management Based MDT reduces the data collection gap at RRC_IDLE to RRC_CONNECTED transitions to the latency needed to complete the RRC Setup procedures, which is at worst of a few tens of milliseconds.

While the following was proposed:

Proposal 1: Agree to achieve Continuous MDT by means of enhancements to Management Based MDT

Proposal 2: Continuous MDT is based on signalling to the UE, as part of the Logged MDT configuration, that the UE is selected for Continuous MDT. This enables the UE to report a “Continuous MDT” flag as soon as the UE establishes an RRC connection from RRC_Idle/RRC_Inactive. Hence the UE can be configured with Immediate MDT measurements.

Proposal 3: To achieve realization of Continuous MDT an identifier would be needed that is consistently maintained across RRC state changes and that is associated to the collected MDT data. It is proposed to LS SA5 and to ask on the possibility to define such identifier.

It is proposed to agree to the LS to SA5 in R3-234299, which reflects the above conclusions and proposals.