3GPP TSG-RAN WG3 Meeting #121	R3-234326

Toulouse, France, 21 - 25 August 2023

Agenda item:	23.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Coordination of SRS resources within a validity area

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#120, there was discussion about how to coordinate SRS resources within an SRS validity area. Although no agreements were captured, the following next step was identified in the Chair’s Minutes:

RAN3#120 Chair’s Minutes: Call flow with details on how to determine SRS validating area and which node allocates SRS resources in validating area.

In this paper, we discuss coordination of SRS resources within an SRS positioning validity area and identify impacts to NRPPa signalling.

# 2	Discussion

When a UE reselects within the positioning validity area during SRS transmission, the UE should be able to continue the SRS transmission (subject to validation for SRS transmission which is outside RAN3 scope).  This requires the UE to be configured with one or multiple SRS configurations valid in a multi-cell validity area.

The legacy Positioning Information Exchange procedure already enables the LMF to request the serving gNB to configure the target UE with UL-SRS resource sets as follows:

Step 1:	The LMF sends the POSITIONING INFORMATION REQUEST message containing the Requested SRS Transmission Characteristics IE to the serving gNB.

Step 2:	The serving gNB determines the resources available for UL-SRS, and configures UL SRS transmissions for the target UE taking into account the information received from the LMF in the Requested SRS Transmission Characteristics IE.

Step 3:	The serving gNB provides the UL-SRS configuration to the LMF in the POSITIONING INFORMATION RESPONSE message.

The Positioning Information Exchange procedure can be enhanced to enable coordination of SRS configurations in a validity area. The LMF can also recommend a validity area, but the serving gNB should have final say on the validity area based on its knowledge of e.g. the resources available for UL-, RNA, etc.

This leads to the following new steps:

Step 1:	The LMF sends the POSITIONING INFORMATION REQUEST message containing the Requested SRS Transmission Characteristics IE(s) to the serving gNB. The LMF may optionally recommend a validity area, i.e., a list of cell IDs.

Step 2:	The serving gNB determines the resources available for UL-SRS, and configures UL SRS transmissions for the target UE taking into account the information received from the LMF in the Requested SRS Transmission Characteristics IE and the gNB’s knowledge of the SRS resources that have been allocated to other UEs in the validity area.

Step 3:	The serving gNB provides the UL-SRS configuration and associated validity area to the LMF in the POSITIONING INFORMATION RESPONSE message.

Based on the above, the following is proposed:

Proposal 1:	The serving gNB determines the SRS configuration and validity area, based on assistance information from the LMF.

To enable the target UE to use its configured SRS throughout the validity area, it is beneficial to coordinate among gNBs to minimize/avoid interference caused by UL SRS transmissions of UEs configured by different gNBs in the validity area. Since the LMF is able to recommend parameters of the UL-SRS configuration using the Requested SRS Transmission Characteristics IE, and has knowledge of e.g. SRS configurations in use at each cell, it seems reasonable for the LMF to play a role in coordinating the SRS configurations within the validity area.

At least the following parameters should be considered for resource coordination:

1.	Frequency domain resource allocation, including, e.g., SRS frequency domain start point, SRS bandwidth, comb information, etc.

2.	Time domain resource allocation, including e..g, periodicity, offset, number of symbols, etc.

3.	SRS sequence IDs

RAN1 has agreed that for SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, the SRS sequence ID in the SRS for positioning configuration is commonly configured across cells within the validity area. Then, next questions are how to determine the sequence ID for SRS resources in the validity area, and how to ensure that sequence ID(s) allocated to UE-1 by gNB-X is not concurrently allocated to any other UEs within the same validity area.

Observation:	For area-based SRS resource coordination, the LMF needs to consider SRS configuration parameters such as time/frequency resources and SRS sequence ID.

In order to minimize the interference caused by potential data transmissions and SRS transmissions from UEs and gNBs within the validity area, it is beneficial to indicate resource reservation information to gNBs within the physical coverage of the positioning area. The “reserved” resources are intended to be coordinated by the LMF, and should not be autonomously allocated by a gNB unless recommended by the LMF.

Proposal 2: 	The LMF should be able to indicate to gNBs that certain SRS resources are “reserved” for multi-cell SRS for positioning configurations (i.e. gNB should avoid autonomously allocating reserved SRS resources).

To determine in more detail the impacts to NRPPa, an important question is as follows: how does the gNB determine the resources available for UL-SRS in the validity area (i.e. resources that it should / should not use when configuring UL SRS transmission for the UE)?

For example, to enable the SRS sequence ID to be commonly configured across cells with the validity area, the LMF can first indicate to gNBs that certain SRS sequence ID values are “reserved”, i.e. to be used only by LPHAP UEs which require an SRS for positioning configuration in multiple cells. These “reserved” SRS sequence IDs should not be autonomously allocated by gNB, unless recommended for a specific UE by the LMF (e.g. in the Requested SRS Transmission Characteristics IE of the POSITIONING INFORMATION REQUEST message).

Alternatively, the LMF may be able to partition a set of SRS sequence IDs among cells/gNBs in a validity area, to allow each gNB more autonomy to allocate resources within its partition.

Proposal 3:	Introduce a new non-UE associated NRPPa message to indicate SRS resources that are “reserved” for SRS for positioning configurations across cells.

# 3	Conclusions

In this paper, we evaluated coordination of SRS resources within an SRS positioning validity area, leading to the following observation and proposals:

Observation:	For area-based SRS resource coordination, the LMF needs to consider SRS configuration parameters such as time/frequency resources and SRS sequence ID.

Proposal 1:	The serving gNB determines the SRS configuration and validity area, based on assistance information from the LMF.

Proposal 2: 	The LMF should be able to indicate to gNBs that certain SRS resources are “reserved” for multi-cell SRS for positioning configurations (i.e. gNB should avoid autonomously allocating reserved SRS resources).

Proposal 3:	Introduce a new non-UE associated NRPPa message to indicate SRS resources that are “reserved” for SRS for positioning configurations across cells.

# References

- RP-230328 Revised WID on Expanded and Improved NR Positioning, Intel Corporation