3GPP TSG-RAN3 #121	R3-234329

Toulouse, France – August 21st – August 25th 2023

Title: 	Partial reporting of AI-ML assistance data

Source: 	Ericsson

Agenda item:	12.2.2.1

Document Type:	Discussion and Decision

# Introduction

At RAN3#120 the following agreements and open points related to requested measurements and predictions were captured:

The Failed Report Characteristics shall be split into two fields:

- Failed Report Characteristics for per cell measurements, reported with per cell granularity

- Failed Report Characteristics for per node measurements, reported with per node granularity

Introduce a Cause IE for the Measurement Failed Report Characteristics per cell and Measurement Failed Report Characteristics per node.

The group discussed and had the common understanding as below:

Cause value measurement not supported -> is covered by legacy cause “Measurement not Supported For The Object,”

Cause value measurement temporarily not available -> is covered by legacy cause “Measurement Temporarily not Available,”

It is FFS whether any more cause values should be added to the legacy list of causes.

There is consensus on the benefits of enabling the requesting node to optimize the measurement request by indicating whether requested measurements can be reported in full or in part. It needs to be further discussed whether such enhancements can be introduced in Rel18.

The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed.

The specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future, can be configured for both one-time reporting and periodic reporting.

In this paper, we focus on the open points regarding the partial reporting of AI/ML assistance data.

# Cause values for partial reporting

As indicated above, last RAN3 meeting agreed to introduce an information on failed Report Characteristics (per cell measurements and per node measurements) in the AI/ML INFORMATION RESPONSE message (FFS on the name). With this, the reporting node informs the requesting node about which measurements failed to be configured. It has also been agreed to introduce a Cause IE for the Measurement Failed Report Characteristics per cell and for the Measurement Failed Report Characteristics per node.

The point that remains to clarify is whether the existing values of the Cause IE are sufficient or new values are needed.

At last meeting, RAN3 also agreed that:

The requested prediction time includes the specific point in time for which the prediction information is being requested with the assumption that this specific point in time is in the reasonable future. The encoding needs to be further discussed.

According to this agreement, the requesting node may ask for predictions for a certain specific point in time in the future, and (optionally) to provide updates with a certain reporting periodicity. The requested node may be able to provide the requested data, but not necessarily at the requested specific point in time in the future, or with the requested periodicity. In some situation, we can face an inherent inability of the reporting node to provide the requested data, e.g., the reporting node has not been trained to provide predictions X seconds ahead from current time. Note that this inability can be more pronounced if “X” can be picked in a continuous interval of values. In another situation, the inability of the reporting node is only temporary, e.g., the reporting node has the capability to provide the measurements or predictions, but the measurements or predictions cannot be provided at that particular point in time due to node internal limitations such as shortage of processing/memory resources.

Knowing that a failure happens due to such causes is useful to the requesting node. For instance, if the requesting node is aware that the reporting node cannot provide (temporarily or not): 1) predictions for certain prediction times, or 2) predictions for certain periodicities, or 3) certain combinations of prediction times and periodicities, then the requesting node can avoid indicating them in the future. If this information is not revealed, the mere listing of the failed measurement will not be of much use.

Conclusion 1: Failing to provide the requested information can be due to one of many causes which would require different handling from the requesting node.

In general, a requested measurement can fail to be reported either because it is not supported or because it is not available at the time of requesting. Hence, supporting the following failure causes per measurement is beneficial,

- measurement not supported: this can be signalled reusing the existing cause value “Measurement not Supported For The Object”

- measurement temporarily not available: this can be signalled reusing the existing cause value “Measurement Temporarily not Available”

- measurement not supported with requested reporting periodicity,

- measurement temporarily not available with requested reporting periodicity,

- measurement not supported with current combination of requested information,

- measurement temporarily not available with current combination of requested information,

At the last RAN3 meeting it was commented that the list of additional cause values above seems too granular. We believe that the cause values above offer a detailed understanding of the type of error that occurred, however, one possible way to reduce the number of cause values to be introduces could be to limit the additional new cause values to:

- measurement not supported with current combination of requested information and periodicity,

- measurement temporarily not available with current combination of requested information and periodicity,

Based on the above we propose to add the failure cause per failed measurement in the AI/ML INFORMATION RESPONSE message.

Proposal 1:  Add in the list of failed measurements the corresponding cause indicating the reason for failure per measurement in the AI/ML INFORMATION RESPONSE message. The following failure causes should be supported:

- measurement not supported, reusing “Measurement not Supported For The Object”

- measurement temporarily not available, reusing “Measurement Temporarily not Available”

- measurement not supported with requested reporting periodicity,

- measurement temporarily not available with requested reporting periodicity,

- measurement not supported with current combination of requested information,

- measurement temporarily not available with current combination of requested information,

Proposal 2:  An alternative to reduce the number of new cause values to introduce is to support the following list of cause values:

- measurement not supported, reusing “Measurement not Supported For The Object”

- measurement temporarily not available, reusing “Measurement Temporarily not Available”

- measurement not supported with current combination of requested information and periodicity,

- measurement temporarily not available with current combination of requested information and periodicity,

# Support of partial reporting

There might be AI/ML models that would be able to infer only if all the requested data indicated in the AI/ML INFORMATION REQUEST is available. For example, an AI/ML algorithm may only be able to produce inference outputs if both Predicted Resource Status and Energy Efficiency measurements are provided. Other AI/ML model implementations instead, could work even if some of the requested data are not available.

The objective of the partial reporting mechanism is to provide the possibility of a successful procedure in cases where the reporting node is not able to provide all requested information or if it is not able to fulfil the requested configuration. Let’s consider the two scenarios below:

- If the AI/ML model needs all the requested data, sending ONLY a part of them is a waste of signalling and processing, since the received data do not allow the model to infer.

- if the AI/ML model could work even if some the requested data is missing, it would be a waste to NOT collect such data.

We therefore propose that the requesting node indicates whether partial reporting is possible, i.e., whether it is possible or not - for the AI/ML model(s) at the reporting node – to properly function if only receiving some of the requested data, or if instead, all the data requested shall be reported. This can be achieved by including a flag directly in the request (i.e., in the AI/ML INFORMATION REQUEST message (FFS on the name)).

Proposal 3:  Add a flag in the AI/ML INFORMATION REQUEST message to indicate if partial acceptance of the reporting configuration is acceptable.

There might be cases also where, while the reporting node successfully configured the requested measurements and it was able to report a number of requested information for a period of time, at some point it is not able to continue with the reporting, either in part or in full. For example, the reporting node might not be able to report at the agreed periodicity, or it might not be able to report some measurements due to processing overload. The most efficient way forward in that case would be that the reporting node could indicate unavailability of measurements in the AI/ML INFORMATION UPDATE message, to inform the requesting node of the data that will not be available.

Proposal 4:  The reporting node can use the AI/ML INFORMATION UPDATE messages to indicate potential issues preventing the reporting.

# Conclusion

In this contribution, we have discussed the agreement regarding partial reporting mechanism support for the agreed AI/ML information procedures. Corresponding proposals are reported below.

Proposal 1:  The reporting node can use the AI/ML INFORMATION RESPONSE messages to explicitly indicate the measurements that can be provided and the list of failed measurements together with the corresponding cause indicating the reason for failure per measurement. The following failure causes are supported:

- measurement not supported, reusing “Measurement not Supported For The Object”

- measurement temporarily not available, reusing “Measurement Temporarily not Available”

- measurement not supported with requested reporting periodicity,

- measurement temporarily not available with requested reporting periodicity,

- measurement not supported with current combination of requested information,

- measurement temporarily not available with current combination of requested information,

Proposal 2:  An alternative to reduce the number of new cause values to introduce is to support the following list of cause values:

- measurement not supported, reusing “Measurement not Supported For The Object”

- measurement temporarily not available, reusing “Measurement Temporarily not Available”

- measurement not supported with current combination of requested information and periodicity,

- measurement temporarily not available with current combination of requested information and periodicity,

Proposal 3:  Add a flag in the AI/ML INFORMATION REQUEST message to indicate if partial acceptance of the reporting configuration is acceptable.

Proposal 3:  The reporting node can use the AI/ML INFORMATION UPDATE messages to indicate potential issues preventing the reporting.

A TP mirroring the proposals above is shown in the R3-234294