3GPP TSG-RAN WG3 Meeting #121															 R3-234380

Toulouse, France, 21st – 25th August, 2023

Agenda item:		12.2.2.4

Source:	ZTE

Title:	(TP to 38.473&37.483) Consideration on impact of F1/E1 impacts

Document for:		Others

1	Introduction

The normative work on AIRAN is nearing its final stages, with three more meetings remaining. Given the progressing maturity of Xn interface, the deliberation on the E1F1 interface can commence in this meeting.

FFS on detailed E1/F1 impact for the exchange of current/predicted UE traffic after the sufficient work for non-split architecture.

FFS on detailed E1/F1 impact for the exchange of current/predicted UE trajectory after the sufficient work for non-split architecture.

FFS on detailed energy efficiency metric and corresponding E1/F1 impact after the sufficient work for non-split architecture.

In this contribution, we articulate our views on the interface impacts on the current three cases on the E1&F1 interface, along with the corresponding TP in the Annex.

- Discussion

Based on the progress over Xn interface, following information are conveyed between NG-RAN nodes:

- - Predicted Resource Status

- Energy Cost

Firstly, according to the deployment of AI/ML function described in the TR37.817 [1] as shown below, we discuss whether there is any impacts over E1/F1 interface.

In case of CU-DU split architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU. - AI/ML Model Training and Model Inference are both located in the gNB-CU.

We can see that in Rel-18, the AI/ML function is located in the gNB-CU, which means the gNB-DU has no ability to perform AI/ML function. Hence, predicted information conveyed from gNB-DU to gNB-CU is not possible in Rel-18.

- Predicted information conveyed from gNB-DU to gNB-CU is not pursed in Rel-18.

With respect to the legacy information, e.g., resource status, following the principles over Xn, the legacy information can be conveyed from gNB-DU to gNB-CU via existing procedure.

- Legacy information over F1/E1 interface that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

With respect to the Energy Cost, the measured EC now is agreed to transferred between NG-RAN nodes. In the case of CU/DU split architecture, DU can also report its energy cost to CU for energy decision. The new class1&class2 AI/ML related information procedure to convey EC, which is the same reporting mechanisms as new procedure over Xn interface.

- Introduce the new class1&class2 procedure over F1, as the same reporting mechanisms of resource status procedure, to support transferring energy cost from DU to CU.

- The corresponding TP to 38.473 is provided in the Annex-A.

- Besides, in the previous meeting, it was agreed that UE traffic metric takes the data volume for a UE as the starting point. Considering the gNB-CU-UP knowing the UE traffic related information, and CU-CP has the ability to perform the AI/ML function according to the TR37.817. One way to support generating predicted UE traffic is that CU-CP needs to collect the current/historical UE traffic over E1 interface.

- However, in the current E1 spec[2],  the existing DATA USAGE REPORT message can not satisfy the requirement of data collection for AI/ML function, because the existing procedure is a class2 procedure without the corresponding class1 procedure. There is no request message for requesting entity to use to trigger the data collection, and set the configuration, e.g., periodicity.

- Introduce the new class1&class2 procedure over E1, as the same reporting mechanisms of resource status procedure, to support transferring UE traffic from gNB-CU-UP to gNB-CU-CP.

- The corresponding TP to 37.483 is provided in the Annex-B.

- Conclusion

- Predicted information conveyed from gNB-DU to gNB-CU is not pursed in Rel-18.

- Legacy information over F1/E1 interface that are used to support AI/ML are transferred via existing legacy procedures (no need to signal them via other procedures)

- Introduce the new class1&class2 procedure over F1, as the same reporting mechanisms of resource status procedure, to support transferring energy cost from DU to CU.

- Introduce the new class1&class2 procedure over E1, as the same reporting mechanisms of resource status procedure, to support transferring UE traffic from gNB-CU-UP to gNB-CU-CP.

- Reference

- TR37.817

- TS37.483

- Annex TP to 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |
| AI/ML Information Reporting Initiation | AI/ML INFORMATION REQUEST | AI/ML INFORMATION RESPONSE | AI/ML INFORAMTION FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |
| AI/ML Information Reporting | AI/ML INFORMATION UPDATE |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.AA	AI/ML Information Reporting Initiation

8.2.AA.1	General

This procedure is used by an gNB-CU to request the reporting of AI/ML related information to gNB-DU.

The procedure uses non UE-associated signalling.

8.2.AA.2	Successful Operation

Figure 8.2.AA.2-1: AI/ML Information Reporting Initiation, successful operation

The gNB-CU initiates the procedure by sending the AI/ML INFORMATION REQUEST message to gNB-DU to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, gNB-DU:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects gNB-DU shall perform measurements or prediction on. gNB-DU shall include in the AI/ML INFORMATION UPDATE message:

-	the Energy Cost IE, if the first bit, “Energy Cost” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

8.2.AA.3	Unsuccessful Operation

Figure 8.2.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting cannot be initiated, gNB-DU shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

8.2.AA.4	Abnormal Conditions

Void

8.2.BB	AI/ML Information Reporting

8.2.BB.1	General

This procedure is initiated by gNB-DU to report AI/ML related information to gNB-CU following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

8.2.BB.2	Successful Operation

Figure 8.2.11.2-1: AI/ML Information Reporting, successful operation

gNB-DU shall report the accepted AI/ML related information in AI/ML INFORMATION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

8.2.BB.3	Unsuccessful Operation

Not applicable.

8.2.BB.4	Abnormal Conditions

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.CC	AI/ML INFORMATION REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. First Bit = Energy Cost | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

9.1.3.DD	AI/ML INFORMATION RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested AI/ML related information, for all the measurement objects included in the reporting is successfully initiated.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.EE	AI/ML INFORMATION FAILURE

This message is sent by the gNB-DU to gNB-CU to indicate that the requested objects the reporting cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CUMeasurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.FF	AI/ML INFORMATION UPDATE

This message is sent by gNB-DU to gNB-CU to report the requested AI/ML related information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | reject |
| Energy Cost | O |  | INTEGER (1..1000,...) | It represents the actual measurement of the Energy Cost | YES | reject |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- Annex TP to 37.483

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| BC Bearer Context Setup |  | BC BEARER CONTEXT SETUP REQUEST |  | BC BEARER CONTEXT SETUP RESPONSE |  | BC BEARER CONTEXT SETUP FAILURE |  |
| BC Bearer Context Modification (gNB-CU-CP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUEST |  | BC BEARER CONTEXT MODIFICATION RESPONSE |  | BC BEARER CONTEXT MODIFICATION FAILURE |  |
| BC Bearer Context Modification Required (gNB-CU-UP initiated) |  | BC BEARER CONTEXT MODIFICATION REQUIRED |  | BC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| BC Bearer Context Release (gNB-CU-CP initiated) |  | BC BEARER CONTEXT RELEASE COMMAND |  | BC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| MC Bearer Context Setup |  | MC BEARER CONTEXT SETUP REQUEST |  | MC BEARER CONTEXT SETUP RESPONSE |  | MC BEARER CONTEXT SETUP FAILURE |  |
| MC Bearer Context Modification (gNB-CU-CP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUEST |  | MC BEARER CONTEXT MODIFICATION RESPONSE |  | MC BEARER CONTEXT MODIFICATION FAILURE |  |
| MC Bearer Context Modification Required (gNB-CU-UP initiated) |  | MC BEARER CONTEXT MODIFICATION REQUIRED |  | MC BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| MC Bearer Context Release (gNB-CU-CP initiated) |  | MC BEARER CONTEXT RELEASE COMMAND |  | MC BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| AI/ML Information Reporting Initiation |  | AI/ML Information Reporting Initiation |  | AI/ML Information Reporting Initiation |  | AI/ML Information Reporting Initiation |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| Bearer Context Release Request (gNB-CU-UP initiated) | BEARER CONTEXT RELEASE REQUEST |
| Bearer Context Inactivity Notification | BEARER CONTEXT INACTIVITY NOTIFICATION |
| DL Data Notification | DL DATA NOTIFICATION |
| UL Data Notification | UL DATA NOTIFICATION |
| Data Usage Report | DATA USAGE REPORT |
| gNB-CU-UP Counter Check | GNB-CU-UP COUNTER CHECK |
| gNB-CU-UP Status Indication | GNB-CU-UP STATUS INDICATION |
| MR-DC Data Usage Report | MR-DC DATA USAGE REPORT |
| Trace Start |  | TRACE START |
| Deactivate Trace |  | DEACTIVATE TRACE |
| Resource Status Reporting |  | RESOURCE STATUS UPDATE |
| Early Forwarding SN Transfer |  | EARLY FORWARDING SN TRANSFER |
| GNB-CU-CP Measurement Results Information |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |
| IAB PSK Notification |  | IAB PSK NOTIFICATION |
| BC Bearer Context Release (gNB-CU-UP initiated) |  | BC BEARER CONTEXT RELEASE REQUEST |
| BC Bearer Context Release (gNB-CU-UP initiated) |  | BC BEARER CONTEXT RELEASE REQUEST |
| AI/ML Information Reporting (FFS on the name) |  | AI/ML INFORMATION UPDATE (FFS on the name) |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.2.AA	AI/ML Information Reporting Initiation

8.2.AA.1	General

This procedure is used by an gNB-CU-CP to request the reporting of AI/ML related information to gNB-CU-UP.

The procedure uses non UE-associated signalling.

8.2.AA.2	Successful Operation

Figure 8.2.AA.2-1: AI/ML Information Reporting Initiation, successful operation

The gNB-CU-CP initiates the procedure by sending the AI/ML INFORMATION REQUEST message to gNB-CU-UP to start AI/ML related information reporting and stop AI/ML related information reporting. Upon receipt, gNB-CU-UP:

-	shall initiate the requested AI/ML related information reporting according to the parameters given in the request in case the Registration Request IE is set to “start”; or

-	shall stop all cells AI/ML related information reporting and terminate the reporting in case the Registration Request IE is set to “stop”; or

If the Registration Request IE is set to “start” in the AI/ML INFORMATION REQUEST message and the Report Characteristics IE indicates cell specific AI/ML related information reporting, the Cell To Report List IE shall be included.

If the Reporting Periodicity IE in the AI/ML INFORMATION REQUEST is present, this indicates the periodicity for the reporting of periodic AI/ML related information. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity IE.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the AI/ML INFORMATION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements or prediction on.gNB-CU-UP shall include in the AI/ML INFORMATION UPDATE message:

-	the Energy Cost IE, if the first bit, “Energy Cost” of the Report Characteristics IE included in the AI/ML INFORMATION REQUEST message is set to “1”.

8.2.AA.3	Unsuccessful Operation

Figure 8.2.AA.3-1: AI/ML Information Reporting Initiation, unsuccessful operation

If none of the requested AI/ML related information reporting cannot be initiated, gNB-CU-UP shall send the AI/ML INFORMATION FAILURE message with an appropriate cause value.

8.2.AA.4	Abnormal Conditions

Void

8.2.BB	AI/ML Information Reporting

8.2.BB.1	General

This procedure is initiated by gNB-CU-UP to report AI/ML related information to gNB-CU-CP following a successful AI/ML Information Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

8.2.BB.2	Successful Operation

Figure 8.2.11.2-1: AI/ML Information Reporting, successful operation

gNB-CU-UP shall report the accepted AI/ML related information in AI/ML INFORMATION UPDATE message. The accepted AI/ML related information is the information that was successfully initiated during the preceding AI/ML Information Reporting Initiation procedure.

8.2.BB.3	Unsuccessful Operation

Not applicable.

8.2.BB.4	Abnormal Conditions

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.CC	AI/ML INFORMATION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested AI/ML related information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. First Bit = Energy Cost | YES | reject |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

9.1.3.DD	AI/ML INFORMATION RESPONSE

This message is sent by gNB-CU-UP to gNB-CU-CP to indicate that the requested AI/ML related information, for all or part of the measurement objects included in the reporting is successfully initiated.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.EE	AI/ML INFORMATION FAILURE

This message is sent by the gNB-CU-UP to gNB-CU-CP to indicate that the requested objects the reporting cannot be initiated.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| gNB-CU-CPMeasurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.3.FF	AI/ML INFORMATION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the requested AI/ML related information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | reject |
| DATA USAGE REPORT | O |  | 9.3.1.44 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>