3GPP TSG-RAN WG3 #121                                               R3-234392

21th – 25th Aug 2023 Toulouse, France

Agenda item:		11.3

Source:				CATT

Title:	Discussion on Support for RV-QoE in NR-DC

Document for:		Discussion and Decision

1		Introduction

In RAN3#120 meeting, the NR QoE supporting in NR-DC was discussed. The RVQoE related agreements and open issues are captured in chair Note[1] as below:

When the RVQoE-configuring node receives an RVQoE report and determines that the non-RVQoE-configuring node provides the bearer(s) for the application session, the RVQoE-configuring node indicates that to the non-RVQoE-configuring node. FFS whether the indication is explicit or implicit.

In the response, the non-RVQoE-configuring node:

- Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

- Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

- Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

Mobility

In line with the Rel-17 principles, if the MN configured the UE with QoE measurements, every subsequent MN serving the UE can configure and release the RVQoE measurements.

PSCell change

At SN change without MN change, the QoE configuration information is passed from the old/source SN to the new/target SN via the MN.

SN release

If the SN configured a UE with QoE measurements, at SN release, the QoE/RVQoE configuration can be released. Whether the SN-configured QoE/RVQoE configuration information can be passed to the MN in case of SN release needs to be further discussed.

Proposal 3-3: When the non-RVQoE-configuring node receives an RVQoE report and determines that the RVQoE-configuring node provides the bearer(s) for the application session, the non-RVQoE-configuring node indicates that to the RVQoE-configuring node. Whether in an explicit way or implicit way needs further discussion.

Proposal 3-4: The non-RVQoE-configuring node can update its preferred RVQoE configuration parameters and reporting leg preference, and it can request from the RVQoE-configuring node to update these parameters at the UE.

This contribution will further discuss the supporting on the RV QoE in NR-DC based on the agreements.

2		Discussion

2.1 RVQoE Reporting

In last meeting, we discussed the report received by the RVQoE-configuring node but the service bearer provided by non-RVQoE-configuring node. We agree another the RVQoE-configuring node should indicate to  non-RVQoE-configuring node. But whether the indication is explicit or implicit is FFS. to be simple, the RVQoE-configuring node may forward the report to non-RVQoE-configuring node. If the non-RVQoE-configuring node doesn’t want to have it, the non-RVQoE-configuring node may discard it.

Proposal 1: RVQoE-configuring node implicitly indicate non-RVQoE-configuring node via forwarding the received report

We also agreed the non-RVQoE-configuring node may response:

•	Can indicate to the RVQoE-configuring node that it does not want to receive the RVQoE reports.

•	Can indicate whether it prefers to receive the RVQoE reports directly from the UE.

•	Can indicate to the RVQoE-configuring node its preferred RVQoE configuration parameters, based on which the RVQoE-configuring node should modify the RVQoE configuration.

Another scenario may be simpler and directly except above agreed response. The non-RVQoE-configuring node indicates RVQoE-configuring node to release its configured RVQoE and then non-RVQoE-configuring node configure the RVQoE by itself.

Proposal 2: The non-RVQoE-configuring node can indicate RVQoE-configuring node to release the configured RVQoE and then non-RVQoE-configuring node configure the RVQoE by itself if needed.

In last meeting, we also discussed the report received by the non-RVQoE-configuring node but the service bearer provided by RVQoE-configuring node. But there are no agreements on this topic.

Both RVQoE-configuring node and non-RVQoE-configuring node can receive the report. So the UE need not to differentiate the initial node. The UE APP layer will send the RV QoE report with PDU session ID plus QoS flow ID to AS layer and then the AS layer send the report to the node which the QoS flow is located. The report receiving node may compare the report metrics with its configured RVQoE. There are three possibilities:

- If the reporting metrics is aligned with its configuration, this node is RVQoE-configuring node. It may store and use this report as normal.

- If the reporting metrics is not aligned with its configuration, this node is non-RVQoE-configuring node. It may discard this report and indicate RVQoE-configuring node to release the RVQoE configuration if it does not want it.

- If the reporting metrics is not aligned with its configuration, this node is non-RVQoE-configuring node. It also may store the report if it is interested it, and indicate RVQoE-configuring node to release the RVQoE configuration and configure the RVQoE by itself as above Proposal 2.

Proposal 3: UE may directly send the RV-QoE report to service bearer provided node according QoS flow information

Proposal 4: if non-RVQoE-configuring receive report, the node can discard the report if it doesn’t want it, and indicate RVQoE-configuring node to release the configured RVQoE

If we cannot agree above Proposal 3, the UE need not to differentiate the initial node if we don’t agree the UE send the report to RAN blindly. So then the report can be sent to RVQoE-configuring node. For above discussed case, the report received by the non-RVQoE-configuring node may come from that RVQoE-configuring node indicate offload the report due to overload.

The UE receive the configuration and perform the RV-QoE related measurement. And send the report to RAN node. Which node is the report sent? To be reasonable, it is better that the UE sends the report to the RVQoE-configuring node.

The UE app layer and AS layer should be aware of RVQoE-configuring node ID. So the RV-QoE configuration and report should carry the indication of the node. And the UE app layer may be informed RVQoE-configuring node and the APP layer send the report to AS layer carrying the indication of the node. So then the report can be correctly sent to the RVQoE-configuring node.

Proposal 5: UE may differentiate the RV-QoE configuration and report according node indication if the report cannot be sent to RAN according QoS flow information.

2.2 RVQoE Configuring

If we support the RVQoE configuring coordination between MN and SN before sending to UE, the RVQoE of SN and MN configuration exchange is needed. So we may let the MN generate the final configuration and send to UE. The SN may send RVQoE wanted configuration to MN. The MN considers its wanted RVQoE parameters and the SN wanted parameters to generate the final one RVQoE configuration. And then MN sends to UE.

Proposal 6: MN generates one final RVQoE configuration covers both MN and SN and sends to UE

For these SN wanted parameters, we don’t think it is necessary for MN to modify the reporting metrics. But the report periodicity may be change or not to align the two nodes if MN also configures the RVQoE. If both MN node and SN configure the RV-QoE for the same service type and QoE reference, we need more thinking. In R17, we just specified two parameters for RV-QoE. In the future release, there may be more parameters introduced. If both MN and SN configure the RV-QoE for same service type and QoE reference, the value of the parameters may be difference if there is no any coordination between two nodes. If like this, it introduces more complex in UE app layer and in report sending. So we propose the MN and SN may select same value for a parameter.

Proposal 7: MN can modify the SN RVQoE wanted configuration if One RVQoE configuration supported

Proposal 8: MN and SN may aligned the value of parameters if both MN and SN configure same RV-QoE parameter

The proposed completed RVQoE configuring and reporting work flow refers to the section Annex

3		Conclusion

In the present contribution we make the following observations and proposal:

Proposal 1: RVQoE-configuring node implicit indicate non-RVQoE-configuring node via forwarding the received report

Proposal 2: The non-RVQoE-configuring node can indicate RVQoE-configuring node to release the configured RVQoE and then non-RVQoE-configuring node configure the RVQoE by itself if needed.

Proposal 3: UE may directly send the RV-QoE report to service bearer provided node according QoS flow information

Proposal 4: if non-RVQoE-configuring receive report, the node can discard the report if it does want it, and indicate RVQoE-configuring node to release the configured RVQoE

Proposal 5: UE may differentiate the RV-QoE configuration and report according node indication if the report cannot be sent to RAN according QoS flow information.

Proposal 6: MN generates one final RVQoE configuration covers both MN and SN and sends to UE

Proposal 7: MN can modify the SN RVQoE wanted configuration if One RVQoE configuration supported

Proposal 8: MN and SN may aligned the value of parameters if both MN and SN configure same RV-QoE parameter

4		Reference

- RAN3_120_agenda_20230526_EOM

5		Annex

- Proposed RVQoE configuring and reporting work flow

The MN is in charge of the RVQoE configuration. It generates the RVQoE configuration and sends to UE. UE sends the report to the node which provides service bearers according QFI. The report received node may modify the RVQoE configuration via MN after it receive the report.

- SN sends the wanted RVQoE configuration to MN

- MN consider its wanted RVQoE configuration

- MN generates the RVQoE configuration, combine the MN and SN RVQoE configuration

- MN sends the RVQoE configuration to UE

- UE generate the RVQoE report for RVQoE

- UE sends RVQoE report to service bearer provided node according the QFI, either to MN or to SN.

- The SN may modify the RVQoE configuration based on the received report due to the SN may not want have the reporting metrics

- MN sends the updated RVQoE configuration to UE for SN or MN required. The MN may also modify the RVQoE configuration based on the received report due to the MN may not want have the reporting metrics.

Note: step 7/8 is optional. The report received node may not want to modify the RVQoE configuration.