3GPP TSG-RAN WG3 Meeting #121	R3-234424

Toulouse, France, 21 – 25 Aug 2023

Agenda Item:	11.4

Source:	ZTE, China Unicom, China Telecom, CMCC

Title:	Discussion on other issues in NR QoE

Document for:	Discussions & Approval

1	Introduction

This paper provides discussion on the other issues of this WI.

2	Discussion

2.1 Threshold-based trigger

During last meeting, there is an agreement in RAN3 on threshold-based trigger:

The network can configure UE with two types of events for threshold based RVQoE reporting, the RVQoE reporting is triggered when the buffer level is below a threshold or above a threshold. Whether these two types of events for threshold based RVQoE reporting can be configured simultaneously needs to be further discussed.

Besides, SA4 has sent a reply LS [1] on the buffer level threshold, which states that SA4 does not want to confirm the threshold trigger to be applied in APP layer:

Hence based on this, SA4 would like to update the reply sent in S4-230684, and does not confirm RAN2 preference that application layer triggering of buffer level threshold-based RVQoE reporting can be supported in Rel-18 based on the corresponding QoE configuration received from the AS layer.

With this reply LS, the threshold trigger could only be settled in AS layer, as described in the LS previously triggered by RAN2:

- With AS layer triggering, APP layer provides RVQoE measurements reports to AS layer according to the configured reporting periodicity (i.e. Rel-17 RVQoE reporting mechanism is reused), and the AS layer reports to gNB the RVQoE measurements when the received buffer level measurements satisfy a buffer level threshold.

So, the buffer level should be checked by the AS layer. And considering the previous open issue:

RAN3 should discuss how the UE should send the RVQoE reports after the threshold is met, e.g., the following options:

Option 1: Just once (after receiving this RVQoE report, gNB might reconfigure this threshold value to get additional reports)

Option 2: Periodically based on a gNB configured reporting periodicity

Option 3: A certain number of times based on gNB configured report amount

How the RVQoE results could be reported after the threshold is triggered is still not confirmed yet. Considering it is the AS layer to check the threshold, we provide our anaylsis on the three options on the table:

- Option 1: Just one RVQoE report is almost of no use to the network for analysis.

- Option 2: Now that SA4 would not check the threshold, which means it would directly send the RVQoE reports to the AS layer according to the AS layer, and AS layer is the entity to check threshold, it would be hard for the AS layer to periodically send the reports to the network, say, if some of the RVQoE reports does not satisfy the threshold. But it depends on RAN2 on how the RVQoE reporting the forwarded to the network after the measurement checking.

- Option 3: This would introduce more complexity for the network, which is not preferred.

Based on current situation, it seems the key of solution for the RVQoE reporting is in RAN2 side. How the RVQoE measurement should be checked and how the AS layer forward the RVQoE reports to the network after network is depended on RAN2.

Proposal 1: How the RVQoE measurement is checked and how the RVQoE reports are forwarded to the network is depended on RAN2.

Further discuss whether to introduce TTT (time to trigger) for threshold-based triggers.

Also, with the fact that UE AS layer performs the RVQoE measurement checking, there is no significat benefit for the introduction of TTT in the configuration. There are some discussion in RAN2 about the to filter the RVQoE reports according to the threholds, for example, to compare the average of buffer level of all entries (even the average after removing the maximum and the minimum) with the threhosld. The TTT here seems of not much use for the checking mechanism in the AS layer. And considering there is not much time left, we would prefer to keep it simple in this release.

Proposal 2: In this release, there is no need to introduce TTT (time to trigger) for threshold-based triggers.

2.2 DU participation

Turn WA to agreement:

WA: A class-2 procedure is used for DU to deactivate the RVQoE reporting over F1AP.

UE associated or non-UE associated?

Based on the agreement at last meeting, a new procedure for DU to deactivate RVQoE reporting over F1AP, since there is no existing class-2 procedure which could be used.

For the granularity of deactivation, we think it should be per node. We do not agree to deactivating RVQoE reporting over F1 per RVQoE configuration for the following reasons:

- The RVQoE reporting over F1 is not even per RVQoE configuration.

- The gNB-DU is not aware of the QoE Reference.

Since the gNB-DU is not aware of the QoE Reference or the start/end of application session, the RVQoE reporting can only be deactivated in a per-node basis. And to differentiate with the pause/resume mechanism that is not supported in R18, we prefer to clarify that once deactivation of RVQoE reporting from DU is triggered, the CU should stop sending RVQoE reporting to that DU, and not restart sending reports to that DU again.

Proposal 3: The deactivation of RVQoE reporting over F1 is per node.

Proposal 4: In R18, once the deactivation of RVQoE reporting is triggered by DU, the CU should stop sending RVQoE reports to that DU and not restart sending RVQoE reports to that DU again.

Another left issue is whether DU could provide RAN visible QoE configuration to the CU. Although we think it feasible somehow, considering limited time left for this release, we would suggest we do not further discuss this issue in this release. In future releases, this point could be taken as an enhancement.

Proposal 5: Do not continue to discuss the DU participation in RVQoE configuration in Release 18.

2.3 Intra-5GC inter RAT handover

Option 3: NR to LTE/5GC

WA: From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep.

Multiple QoE measurements can be supported in NR, but only one QoE measurement is supported in LTE. In our view, it should be the source node to decide which configuration to keep. Since the QoE measurements are hold be the source node, it is the source knows better about which configuration is more important and it can make the best decision on which configuration deserves to be kept.

Proposal 6: Turn the WA that says ‘From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep’ into agreement.

Option 4: LTE/5GC to NR

When the UE handover from LTE to NR, one problem is that the RRC id should be allocated for the QoE reference. The target node could allocate a new RRC id for the corresponding LTE configuration and notify the UE by sending a new RRC message. There seems no RAN3 impact.

Proposal 7: Confirm the WA that says For HO from LTE/5GC to NR, there is no impacts to RAN3.

At previous meetings, RAN3 did not send the reply to the LS（R3-231110）from RAN2, in which RAN3 tries to check RAN’3 opinion for their progress, especially whether option 3 and option 4 could be supported and further discussed. It is proposed that at this meeting, a reply LS is sent to RAN2 to confirm the support for option 3 and option 4 without impact on LTE specifications.

Proposal 8: Send a reply LS to RAN2, to provide RAN3 feedback on intra-5GC inter-RAT handover.

3	Conclusions

Proposal 1: How the RVQoE measurement is checked and how the RVQoE reports are forwarded to the network is depended on RAN2.

Proposal 2: In this release, there is no need to introduce TTT (time to trigger) for threshold-based triggers.

Proposal 3: The deactivation of RVQoE reporting over F1 is per node.

Proposal 4: In R18, once the deactivation of RVQoE reporting is triggered by DU, the CU should stop sending RVQoE reports to that DU and not restart sending RVQoE reports to that DU again.

Proposal 5: Do not continue to discuss the DU participation in RVQoE configuration in Release 18.

Proposal 6: Turn the WA that says ‘From NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep’ into agreement.

Proposal 7: Confirm the WA that says For HO from LTE/5GC to NR, there is no impacts to RAN3.

Proposal 8: Send a reply LS to RAN2, to provide RAN3 feedback on intra-5GC inter-RAT handover.

4	Reference

[1] R3-233813 Reply LS on buffer level threshold-based RVQoE reporting

5	LS to RAN2

Title:	[draft]Reply LS on buffer level threshold-based RVQoE reporting

Response to:

Release:	Release 18

Work Item:	NR_QoE_enh-Core

Source:	ZTE [to be RAN3]

To:	RAN2

Cc:	SA4

Contact Person:

#### Name:	Man Zhang

E-mail Address:	zhang.man4@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks the LS from RAN2 to share agreements on intra-5GC inter-RAT HO.

It has been discussed in RAN3 whether the option 3 and option 4 could be supported and whether there are any RAN3 impact. This reply LS is sent to provide RAN3 understanding:

- RAN3 would like to confirm option 3 and option 4 without impact on LTE specifications

- For HO from LTE/5GC to NR, there is no impacts to RAN3.

- For HO from NR to LTE, the source node adapts to the target node on deciding which one of the QoE configuration to keep

RAN3 kindly asks RAN2 to condsider the above feedback and continue the discussion.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 kindly asks RAN2 to consider the above feedback and continue the discussion.

3. Date of Next TSG-RAN3 Meetings:

3GPP RAN3#121-bis	from 2023-10-09	to 2023-10-13		Xiamen, CN

3GPP RAN3#122	from 2023-11-13	to 2023-11-17		Chicago, US