3GPP TSG-RAN WG3 Meeting #121	R3-234425

Toulouse, France, 21 – 25 Aug, 2023

Agenda Item:	11.4

Source:	ZTE, Nokia, Nokia Shanghai Bell,  China Unicom, China Telecom

Title:	(TPs for BL CR of TS38.401&38.470) Deactivation of RVQoE reporting over F1AP

Document for:	Discussions & Approval

1	Introduction

This paper provides the text proposal based on the discussion in R3-234424.

2	Text Proposal for 38.401

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

7.10	Support of RAN visible QoE measurement

The RAN visible QoE measurement function is specified in TS 38.300 [2].

In split gNB architecture, upon the reception of the RAN visible QoE measurement report from the UE, the gNB-CU may forward the corresponding QoE information to the gNB-DU. The QoE information transferred to the gNB-DU may include the RAN visible QoE metrics in the RAN visible QoE measurement result from the UE, along with the corresponding DRB ID(s).

The gNG-DU may deactivate the transmission of RAN visible QoE measurements from the gNB-CU, by sending a deactivation message to the gNB-CU. Upon receiving the deactivation message, the gNB-CU stops forwarding the RAN visible QoE reports to the gNB-DU.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

- Text Proposal for 38.470

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.1.15	QMC procedures

The QMC procedures are listed below:

-	QoE Information Transfer procedure;

-	QoE Information Transfer Deactivation procedure.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>