3GPP TSG-RAN WG3 Meeting #121	R3-234453

Toulouse, France, 21st – 25th August 2023

Agenda Item:	16.3

Source: 				LG Electronics Inc.

Title:			Remaining open issues for service continuity enhancement

Document for:		Discussion and Decision

1.	Introduction

There are two remaining open issues for service continuity enhancement of the U2N Relay UE. In this contribution, we examine these issues and then provide our view on it.

2.	Discussion

2.1 Remaining FFS in Stage 2

It is still FFS on whether to move Step 7 before Step 5 in inter-gNB d2i/i2i path switch procedures in the BL CR to TS 38.300 [1]. In our understanding, when the target relay UE is in the RRC_CONNECTED, the target gNB can perform the RRC Reconfiguration procedure to the target relay UE after the admission control (i.e., Step 4). That is, the order of Step 5 and 7 is up to the gNB implementation.

Proposal 1: In inter-gNB d2i/i2i path switch procedures, the order of Step 5 and 7 is up to the gNB implementation.

However, according to the figure for the intra-gNB d2i path switch procedure in current TS 38.300, the RRCReconfiguration message to the target relay UE may be happened after the gNB’s decision of switching to a target relay UE. In the figure for the inter-gNB d2i path switch procedure of current BL CR to TS 38.401 [2], the reconfiguration to target U2N Relay UE is also performed after the gNB’s decision of switching to a target relay UE. In order to align with other figures for the d2i path switch procedure in different specification, therefore, we propose to update the figures for inter-gNB d2i/i2i path switch procedures by moving Step 7 before Step 5, and add a NOTE regarding the order between the reconfiguration to the target relay UE and the HANDOVER REQUEST ACK message.

Proposal 2: It is proposed to move Step 7 before Step 5 in the figures for inter-gNB d2i/i2i path switch procedures and add the NOTE “The order of Step 5 and 6 is up to the gNB implementation.”

2.2 DL Lossless delivery

For lossless delivery, RAN2 made the following agreement in last meeting:

Agreements:

RAN2 will not specify any Rel-18 enhancement for lossless behaviour for uplink service continuity in L2 U2N relay.

RAN2 will not specify any Rel-18 enhancement from UE perspective for lossless behaviour for downlink service continuity in L2 U2N relay.

As shown in RAN2 agreements, any Rel-18 enhancement for DL lossless behaviour is not specified from the UE point of view. Also, in last meeting, RAN3 has agreed to exclude the solution-D3 for DL lossless delivery. For now, there are two possible solutions to support the DL lossless delivery from the network perspective.

- Solution-D4: Enhanced Data forwarding from source gNB to target gNB per target gNB request (legacy PDCP status report based)

- Solution-D5: Proactive Data forwarding from source gNB to target gNB

In the solution-D4, the target gNB is aware of the missing DL packets based on the PDCP status report from the remote UE. Then, the target gNB requests to forward the missing DL packets to the source gNB. Upon the reception of the request from the target gNB, the source gNB initiates the additional data forwarding for the missing DL packets to the target gNB. Therefore, the target gNB can retransmit the missing DL packets to the remote UE.

However, this solution has a XnAP impact because a new XnAP message should be defined to request the forwarding of the missing DL packets. In addition, since the additional data forwarding is triggered, it causes the unnecessary delay. From the source gNB point of view, even if some DL packets are successfully delivered to the relay UE, it may need to store these DL packets because the target gNB may request the additional forwarding for theses DL packets.

In the solution-D5 as well, the source gNB needs to buffer some DL packets even if these are successfully delivered to the relay UE. Then, the source gNB forwards all or some of the buffered DL packets to the target gNB based on the source gNB’s implementation. Upon the reception of the forwarded DL packets, the target gNB retransmits the missing DL packets to the remote UE based on the PDCP status report. In this solution, there is no XnAP impact. Also, this solution does not have additional delay.

However, the solution-D5 causes the redundant data forwarding over Xn because the source gNB may forward to the target gNB the DL packets which are successfully transmitted to the remote UE. In addition, when the source gNB does not forward enough data to the target gNB, it may be difficult to ensure the DL lossless delivery for the remote UE.

We think that the solution-D4 has much less XnAP impact and delay than the solution-D4. Therefore, we prefer the solution-D5 to support the DL lossless delivery. Also, in the solution-D5, good gNB implementation can reduce the redundant data forwarding over Xn and ensure the DL lossless delivery for the remote UE.

Proposal 3: It is proposed to adopt the solution-D5 for the DL lossless delivery.

Proposal 4: It is proposed to add a following NOTE for the i2d/i2i path switching procedure.

- In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data may be acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB. The target gNB retransmits unsuccessfully delivered DL data to the L2 U2N Remote UE.

In order to capture the above proposals, the following proposal is also suggested to RAN3:

Proposal 5: It is proposed to agree the corresponding TP in Appendix as a BL CR to TS 38.300.

3.	Conclusion

In this contribution, we focused on the open issues of the Rel-18 U2N service continuity enhancement and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: In inter-gNB d2i/i2i path switch procedures, the order of Step 5 and 7 is up to the gNB implementation.

Proposal 2: It is proposed to move Step 7 before Step 5 in the figures for inter-gNB d2i/i2i path switch procedures and add the NOTE “The order of Step 5 and 6 is up to the gNB implementation.”

Proposal 3: It is proposed to adopt the solution-D5 for the DL lossless delivery.

Proposal 4: It is proposed to add a following NOTE for the i2d/i2i path switching procedure.

- In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data may be acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB. The target gNB retransmits unsuccessfully delivered DL data to the L2 U2N Remote UE.

Proposal 5: It is proposed to agree the corresponding TP in Appendix as a BL CR to TS 38.300.

4.	Reference

- R3-233787, “Support of SL relay enhancement in TS38.300,” CMCC, CATT, Samsung, China Telecom, Nokia, Nokia Shanghai Bell, Ericsson, LG Electronics, ZTE, August, 2023.

- R3-233745, “(BLCR to 38.401) Introduction of NR SL relay enhancements,” LG Electronics, August, 2023.

5. Appendix: TP for TS 38.300

This appendix provides the Text proposal for TS 38.300 based on the proposals of this contribution.

----------------Start of the First Change---------------

### 16.12.6	Service Continuity for L2 U2N relay

#### 16.12.6.0	General

The service continuity procedure is applicable for the mobility cases of path switch from indirect to direct path, and from direct to indirect path when the L2 U2N Remote UE and L2 U2N Relay UE belong to the same gNB or different gNB. This procedure is also applicable for the mobility cases of path switch from indirect to indirect path when the two L2 U2N Relay UEs belong to the same gNB or different gNBs. For inter-gNB path switching, the source gNB decides to trigger path switching and the path switch type, i.e. direct or indirect path.

#### 16.12.6.1 Switching from indirect to direct

For service continuity of L2 U2N Relay, the following procedure is used, in case of L2 U2N Remote UE switching from indirect to direct path under the same gNB:

Figure 16.12.6.1-1: Procedure for L2 U2N Remote UE intra-gNB indirect to direct path Switching

1.	The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from L2 U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least L2 U2N Relay UE's source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity result. The sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

2.	The gNB decides to switch the L2 U2N Remote UE onto direct Uu path.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The L2 U2N Remote UE stops UP and CP transmission via the L2 U2N Relay UE after reception of the RRCReconfiguration message with the path switch configuration.

4.	The L2 U2N Remote UE synchronizes with the gNB and performs Random Access.

5.	The UE (i.e., L2 U2N Remote UE in previous steps) sends the RRCReconfigurationComplete message to the gNB via the direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., L2 U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

6.	The gNB sends the RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the gNB. The RRCReconfiguration message to the L2 U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration related to the L2 U2N Remote UE).

7.	Either L2 U2N Relay UE or L2 U2N Remote UE's AS layer can release PC5-RRC connection and indicates upper layers to release PC5 unicast link after receiving the RRCReconfiguration message from the gNB. The timing to execute link release is up to UE implementation.

8.	The data path is switched from indirect path to direct path between the UE (i.e., previous L2 U2N Remote UE) and the gNB. The PDCP re-establishment or PDCP data recovery in uplink is performed by the UE (i.e., previous L2 U2N Remote UE) for lossless delivery during path switch if gNB configures it.

NOTE:	Step 8 can be executed any time after step 4. Step 8 is independent of step 6 and step 7.

For service continuity of L2 U2N Relay, the following procedure is used, in case of L2 U2N Remote UE switching from indirect to direct path under another gNB:

Figure 16.12.6.1-2: Procedure for inter-gNB indirect to direct path switching

1.	The source gNB configures the UE measurement procedures and the L2 U2N Remote UE reports according to the measurement configuration.

2.	The source gNB decides to trigger path switch for the L2 U2N Remote UE, based on MeasurementReport and RRM information.

3.	The source gNB sends HANDOVER REQUEST message to the target gNB with necessary information to prepare the handover at the target side.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB prepares the path switch and sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB, which contains new RRC configuration for remote UE.

6.  The source gNB triggers the handover by sending an RRCReconfiguration message to the L2 U2N Remote UE, containing at least cell ID and the information required to access the target cell.

7.  The L2 U2N Remote UE synchronizes with the target gNB and performs Random Access.

8.  The L2 U2N Remote UE sends RRCReconfigurationComplete message to target gNB.

9.	The target gNB sends the UE CONTEXT RELEASE to inform the source gNB about the success of the path switch.

10. The gNB sends RRCReconfiguration message to the L2 U2N Relay UE to reconfigure the connection between the L2 U2N Relay UE and the source gNB.

11. Either L2 U2N Relay UE or L2 U2N Remote UE’s AS layer can release PC5-RRC connection and indicates upper layer to release PC5 unicast link after receiving the RRCReconfiguration message from the source gNB. The timing to execute link release is up to UE implementation.

#### 16.12.6.2 Switching from direct to indirect

The gNB can select a L2 U2N Relay UE in any RRC state i.e., RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED, as a target L2 U2N Relay UE for direct to indirect path switch.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from direct to indirect path under the same gNB via a L2 U2N Relay UE in RRC_CONNECTED:

Figure 16.12.6.2-1: Procedure for L2 U2N Remote intra-gNB direct to indirect path switching via a L2 U2N Relay UE in RRC_CONNECTED

1.	The L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) and Uu measurements, after it measures/discovers the candidate L2 U2N Relay UE(s):

- -	The L2 U2N Remote UE filters the appropriate L2 U2N Relay UE(s) according to relay selection criteria before reporting. The L2 U2N Remote UE shall report only the L2 U2N Relay UE candidate(s) that fulfil the higher layer criteria;

- -	The reporting includes at least a L2 U2N Relay UE ID, a L2 U2N Relay UE' s serving cell ID, and a sidelink measurement quantity information. SD-RSRP is used as sidelink measurement quantity.

2.	The gNB decides to switch the L2 U2N Remote UE to a target L2 U2N Relay UE. Then the gNB sends an RRCReconfiguration message to the target L2 U2N Relay UE, which includes at least the L2 U2N Remote UE's local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

3.	The gNB sends the RRCReconfiguration message to the L2 U2N Remote UE. The RRCReconfiguration message includes at least the L2 U2N Relay UE ID, Remote UE's local ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). The L2 U2N Remote UE stops UP and CP transmission over the direct path after reception of the RRCReconfiguration message from the gNB.

4.	The L2 U2N Remote UE establishes PC5 RRC connection with target L2 U2N Relay UE.

5.	The L2 U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the L2 U2N Relay UE.

6.	The data path is switched from direct path to indirect path between the L2 U2N Remote UE and the gNB.

In case the selected L2 U2N Relay UE for direct to indirect path switch is in RRC_IDLE or RRC_INACTIVE, after receiving the path switch command, the L2 U2N Remote UE establishes a PC5 link with the L2 U2N Relay UE and sends the RRCReconfigurationComplete message via the L2 U2N Relay UE, which triggers the L2 U2N Relay UE to enter RRC_CONNECTED state. The procedure for L2 U2N Remote UE switching to indirect path in Figure 16.12.6.2-1 can be also applied for the case that the selected L2 U2N Relay UE for direct to indirect path switch is in RRC_IDLE or RRC_INACTIVE with the exception that the RRCReconfiguration message is sent from the gNB to the L2 U2N Relay UE after the L2 U2N Relay UE enters RRC_CONNECTED state, which happens between step 4 and step 5.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from dirct to indirect path under another gNB via a L2 U2N Relay UE in RRC_CONNECTED:

Figure 16.12.6.2-2: Procedure for inter-gNB switching from direct to indirect path

1.	The source gNB configures the UE measurement procedures and the L2 U2N Remote UE reports according to the measurement configuration.

2.	The source gNB trigger path switch for the L2 U2N Remote UE from direct to indirect path of another gNB.

3.	The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to one cell.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB sends the RRCReconfiguration message to L2 U2N Relay UE for relaying configuration.

6.  The source gNB sends the RRCReconfiguration message to the L2 U2N Remote UE.The target gNB prepares the path switch and sends the HANDOVER REQUEST ACKNOWLEDGE message to the source gNB.

NOTE 1:	The order of Step 5 and 6 is up to the gNB implementation.

NOTE 2:		In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data may be acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB. The target gNB retransmits unsuccessfully delivered DL data to the L2 U2N Remote UE.

7.  The target gNB sends the RRCReconfiguration message to L2 U2N Relay UE for relaying configuration. The source gNB sends the RRCReconfiguration message to the L2 U2N Remote UE.

8.  The L2 U2N Remote UE establishes PC5 connection with L2 U2N Relay UE, if not exist.

9.  The L2 U2N Remote UE sends the RRCReconfigurationComplete message to target gNB via the L2 U2N Relay UE.

#### 16.12.6.x Switching from indirect to indirect

Notes: RAN2 can put the description of intra-gNB i2i path-switching procedures in the 38.300 draft spec.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching from indirect to indirect path under another gNB via a L2 U2N Relay UE in RRC_CONNECTED:

Figure 16.12.6.x-2: Procedure for L2 U2N Remote UE inter-gNB indirect to indirect path switching

The signalling flow for U2N Remote UE switch from indirect path to indirect path under another gNB is shown as follows,

1.	The source gNB configures the UE measurement procedures and the L2 U2N Remote UE reports according to the measurement configuration.

2.	The source gNB decides to trigger the L2 U2N Remote UE to switch to an indirect path of another gNB.

3.	The source gNB sends a HANDOVER REQUEST message to the target gNB to prepare the path switch at the target side. The HANDOVER REQUEST message includes Remote UE L2 ID and a list of candidate target relay UE IDs belonging to one cell.

4.	Admission Control may be performed by the target gNB.

5.	The target gNB sends the RRCReconfiguration message to the L2 U2N Relay UE for relaying configuration.

6. 	The target gNB prepares the path switch and sends the HANDOVER REQUEST ACKNOWLEDGE to the source gNB.

NOTE 1:	The order of Step 5 and 6 is up to the gNB implementation.

NOTE 2:		In order to support the DL lossless handover for the L2 U2N Remote UE, the source gNB may not discard the DL data even though the delivery of the data may be acknowledged by the L2 U2N Relay UE based on the gNB implementation. Then, the source gNB forwards the buffered DL data to the target gNB. The target gNB retransmits unsuccessfully delivered DL data to the L2 U2N Remote UE.

The source gNB sends the RRCReconfiguration message to the the L2 U2N Remote UE.

7.  The target gNB sends the RRCReconfiguration message to the L2 U2N Relay UE for relaying configuration.The source gNB sends the RRCReconfiguration message to the the L2 U2N Remote UE.

- The L2 U2N Remote UE establishes PC5 connection to target the L2 U2N Relay UE, if not exist.

- The L2 U2N Remote UE sends the RRCReconfigurationComplete message to the target gNB via target relay UE.

- The source gNB sends the RRCReconfiguration message to the source L2 U2N Relay UE to reconfigure the connection between the source L2 U2N Relay UE and the source gNB.

- Either L2 U2N Relay UE or L2 U2N Remote UE’s AS layer can release PC5-RRC connection and indicates upper layer to release PC5 unicast link after receiving the RRCReconfiguration message from the source gNB. The timing to execute link release is up to UE implementation.

-----------------End of the First Change---------------