# 3GPP TSG-RAN WG3 Meeting RAN3#121	R3-234471

# Toulouse, FR, Aug 21 – Aug 25 2023

# Agenda Item:	9.1

# Source:	ZTE, China Telecom, China Unicom, Ericsson

# Title:	Further discussion on the presence of QoE area scope

# Document for:	Discussions & Approval

1	Introduction

Companies have discussed the issue on QoE Area Scope IE in RAN3#120. At the end of previous meeting, the presence of the Area Scope IE of QoE has been corrected to optional in NGAP. In addition, during the CB discussion(R3-233315), more than half involved companies believed that the misalignment on the definition of the QoE Area Scope IE exists between SA5 and RAN3 specs.

In this tdoc, we will further discuss the left issue and new detected ones on the QoE Area Scope IE.

2	Background

The basic definition of the area scope for QoE in TS28.405 has not been changed in recent years. This definition is first time introduced in TS28.405-040 and the up-to-date version is in TS28.405-i20. Detail is shown below:

==== description in TS 28.405-040 ====

- 5.4 Area scope (CM)

The area scope parameter defines the area in terms or cells or Tracking Area/Routing Area/Location Area where the QMC shall take place. If the parameter is not present the QMC shall be done throughout the PLMN specified in PLMN target.

The area scope parameter in UMTS is either:

-	List of cells, identified by CGI. Maximum 32 CGI can be defined.

-	List of Routing Area, identified by RAI. Maximum of 8 RAIs can be defined.

-	List of Location Area, identified by LAI. Maximum of 8 LAIs can be defined.

The area scope parameter in LTE is either:

-	list of cells, identified by E-UTRAN-CGI. Maximum 32 CGI can be defined.

-	List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

The parameter is mandatory if area based QMC is requested.

==== description in TS 28.405-040 ====

==== description in TS 28.405-i20 ====

- 5.4	Area scope (CM)

The area scope parameter defines the area in terms or cells or Tracking Area/Routing Area/Location Area where the QMC shall take place. If the parameter is not present the QMC shall be done throughout the PLMN specified in PLMN target.

The area scope parameter in UMTS is either:

-	List of cells, identified by CGI. Maximum 32 CGI can be defined.

-	List of Routing Area, identified by RAI. Maximum of 8 RAIs can be defined.

-	List of Location Area, identified by LAI. Maximum of 8 LAIs can be defined.

The area scope parameter in LTE is either:

-	list of cells, identified by E-UTRAN-CGI. Maximum 32 CGI can be defined.

-	List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

The area scope parameter in NR is either:

-	list of cells, identified by N-CGI. Maximum 32 NCGI can be defined.

-	List of Tracking Area, identified by TAC. Maximum of 8 TAC can be defined.

The parameter is mandatory if area based QMC is requested.

==== description in TS 28.405-i20 ====

It is clear that from SA5 point of view, the area scope of QoE shares the same mechanism and may not always presence in both LTE and NR.

Observation 1: Based on SA5 specification TS 28.405, the area scope of QoE shares the same mechanism and may not always presence in both LTE and NR.

Meanwhile from RAN3 point of view, the presence of the Area Scope IE in both NGAP and XnAP is Optional after RAN3#120(R3-233430). But in LTE, this IE is defined as a mandatory IE in both S1AP and X2AP which is not as same as the presence of Area Scope IE RAN3 defined for NR QoE. The tabular in TS36.413-h40 is shown below:

==== content in TS 36.413-h40 ====

9.2.1.128	UE Application layer measurement configuration

The IE defines configuration information for the QoE Measurement Collection (QMC) function.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Container for application layer measurement configuration | M |  | Octet string (1..1000) | Indicates application layer measurement configuration, see Annex L in [43]. | - | - |
| CHOICE Area Scope of QMC | M |  |  |  | - | - |
| >Cell based |  |  |  |  |  | - |
| >>Cell ID List for QMC |  | 1 .. <maxnoofCellIDforQMC> |  |  |  | - |
| >>>E-CGI | M |  | 9.2.1.38 |  | - | - |
| >TA based |  |  |  |  |  | - |
| >>TA List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  |  | - |
| >>>TAC | M |  | 9.2.3.7 | The TAI is derived using the current serving PLMN. | - | - |
| >TAI based |  |  |  |  | - | - |
| >>TAI List for QMC |  | 1 .. <maxnoofTAforQMC> |  |  | - | - |
| >>>TAI | M |  | 9.2.3.16 |  | - | - |
| >PLMN area based |  |  |  |  |  | - |
| >>PLMN List for QMC |  | 1 .. <maxnoofPLMNforQMC> |  |  |  | - |
| >>>PLMN Identity | M |  | 9.2.3.8 |  | - | - |
| Service Type | M |  | ENUMERATED (QMC for streaming service, QMC for MTSI service, ...) | This IE indicates the service type of UE application layer measurements. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforQMC | Maximum no. of Cell ID subject for QMC scope. Value is 32. |
| maxnoofTAforQMC | Maximum no. of TA subject for QMC scope. Value is 8. |
| maxnoofPLMNforQMC | Maximum no. of PLMNs in the PLMN list for QMC scope. Value is 16. |

==== content in TS 36.413-h40 ====

Observation 2: Based on RAN3 definition, the presences of the Area Scope IE in LTE QoE and NR QoE are not same.

Companies has discussed the issue on the misalignment  between RAN spec(e.g. TS36.413, TS36.423) and SA5 spec(e.g. TS28.405). The CB summary of this discussion in previous meeting can be found in R3-233315. During that CB discussion, 9 companies involved and 6 of them thought the misalignment exists and preferred to fix by using various alternatives.

Observation 3: Though majority companies preferred to fix the misalignment on the definition of Area Scope in LTE QoE, there is no consensus on whether/how to fix this issue after previous meeting.

3	Discussion

As explained in the background section, there is no different definitions for LTE area scope or NR area scope for QoE in SA5 specification TS28.405. Meanwhile, at RAN side, the presence of the Area Scope IE in LTE QoE and NR QoE are not same. Hence,  it is proposed for RAN3 to believe that the misalignment exists on the definition of area scope between SA5 and RAN3 in LTE QoE.

Proposal 1: Misalignment exists on the definition of area scope between SA5 and RAN3 in LTE QoE.

Based on the comments from companies, various alternatives may be used to fix the detected misalignment. As example, possible corrections on the TS36.413 are listed below:

- Revise “PLMN area based” to “PLMN based” in the CHOICE branch of the Area Scope IE in S1AP.

- Add a new branch named(e.g. PLMN wide, PLMN UE based, etc) in Area Scope IE to handle the QoE configuration without area scope.

- Add a new IE called “PLMN target” and refer to TS28.405 in S1AP messages accordingly.

- Correct the presence of Area Scope IE from mandatory to optional in S1AP.

To make the correction simple and sync up the QoE mechanism in NR and LTE, we prefer to change the presence of Area Scope IE to optional for LTE which is as same as what RAN3 has done in NR field.

Though this correction is a NBC one, considering the QoE function(either NR or LTE) has not been widely deployed in the market, we prefer to unify the Area Scope IE in both LTE and NR to sync up the definition between SA5 and RAN3 and avoid any potential issues between LTE and NR QoE system(e.g. handover).

Proposal 2: The presence of the Area Scope IE for LTE QoE shall be corrected to optional in both S1AP and X2AP.

Detail corrections can be found in our contributions.

In addition, based on the so far definition in TS36.423, the TAI tabular  design in QoE area scope is not match the ASN.1 description.  The reference of some IEs in the QoE  area scope is not correct. We also fix the tabular design issue  by using the MDT mechanism. Detail can also be checked in our 423 related CRs.

Proposal 3: RAN3 shall discuss how to handle the tabular issues in the QoE Area Scope IE in TS 36.423.

3	Conclusion

Observation 1: Based on SA5 specification TS 28.405, the area scope of QoE shares the same mechanism and may not always presence in both LTE and NR.

Observation 2: Based on RAN3 definition, the presences of the Area Scope IE in LTE QoE and NR QoE are not same.

Observation 3: Though majority companies preferred to fix the misalignment on the definition of Area Scope in LTE QoE, there is no consensus on whether/how to fix this issue after previous meeting.

Proposal 1: Misalignment exists on the definition of area scope between SA5 and RAN3 in LTE QoE.

Proposal 2: The presence of the Area Scope IE for LTE QoE shall be corrected to optional in both S1AP and X2AP.

Proposal 3: RAN3 shall discuss how to handle the tabular issues in the QoE Area Scope IE in TS 36.423.