3GPP TSG-RAN WG3 Meeting #121	R3-234482

Toulouse, France, 21 – 25 August 2023

Agenda Item:	25.2

Source:	ZTE

Title:	(TP to TS 38.413) Discussion on support of XR enhancements

Document for:	Discussions & Approval

1	Introduction

3GPP TSG RAN has approved a new work item aimed at “XR Enhancements for NR”. Detailed work item description (WID) can be found in RP-230786. The objectives of XR enhancements are shown as follows:

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3); Specify the enhancements for XR Awareness: - Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2); - Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2); - Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3); - Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);

This paper further discusses how to support the XR enhancements.

# 2	Discussion

## 2.1	PDU Set based QoS handling

2.1.1	PDU set information

According to the following contents captured from TS 26.522, the PDU set information, including the End PDU of the PDU Set (E), End of Data Burst (EDB), PDU Set Importance (PSI), PDU Set Sequence Number (PSSN), PDU Sequence Number within a PDU Set (PSN) and PDU Set Size (PSSize) have been defined in the RTP header extension. According to the following contents captured from TS 23.501, the PDU set information also need to be introduced in the NG-U GTP-U header, i.e. DL PDU session information frame, to let the NG-RAN node to support PDU set based QoS handling.

TS 23.501 To support PDU Set based QoS handling, the PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the NG-RAN in the GTP-U header. The PDU Set information is used by the NG-RAN for PDU Set based QoS handling as described above. The PDU Set Information comprises: - PDU Set Sequence Number. - Indication of End PDU of the PDU Set. - PDU Sequence Number within a PDU Set. - PDU Set Size in bytes. - PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow. TS 26.522 - End PDU of the PDU Set [E] (1 bit): This field is a flag that shall be set to 1 for the last PDU of the PDU Set and set to 0 for all other PDUs of the PDU Set. - End of Data Burst [EDB] (3 bits): The EDB field is 3 bits in length and indicates the end of a Data Burst. The 3 bits encode the End of Data Burst indication as per the encoding and guidelines provided in Clause 4.4.2.6.1. - PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same QoS flow. Lower values shall indicate a higher importance PDU Set with the highest importance PDU Set indicated by 0 and the lowest importance PDU Set indicated by 15. NOTE 1: A complete set of guidelines for setting the PSI field for various audio/video codecs are provided in Clause 4.4.2.6.2 Editor’s Note: AS/UE are unaware of QoS flows and so the above text needs to be revised to remove the term. The PSI value needs to be set considering one or more RTP streams (depending on multiplexing and other aspects that are under discussion in reference to the guidelines). Once the guidelines are finalized, the text above will be revised possibly with the introduction of a new term in place for QoS flow that consists of one or more RTP streams. - PDU Set Sequence Number [PSSN] (10 bits): The field encodes the sequence number of the PDU Set to which the current PDU belongs acting as a 10-bit numerical identifier for the PDU Set. NOTE 2: This value wraps around at 1023, however, using the RTP packet sequence number and PSSN pair a receiver may uniquely distinguish between any PDU Sets. - PDU Sequence Number within a PDU Set [PSN] (6 bits): The sequence number of the current PDU within the PDU Set. The PSN shall be set to 0 for the first PDU in the PDU Set and incremented monotonically for every PDU in the PDU set in order of transmission from the sender. NOTE 3: A receiver may use the RTP packet sequence number together with the PSN to distinguish between PDUs within a PDU Set that contains more than 64 PDUs. - PDU Set Size [PSSize] (24 bits): The PDU Set Size indicates the total size of all PDUs of the PDU Set to which this PDU belongs. This field is optional and subject to an SDP signaling offer/answer negotiation, where the Application Server may indicate whether it will be able to provide the size of the PDU Set for that RTP stream. If not enabled, the field should not be present. If enabled, but the Application Server is not able to determine the PDU Size for a particular PDU Set, it should set the value to 0 in all PDUs of that PDU Set. The PSSize shall indicate the size of a PDU Set including RTP/UDP/IP header encapsulation overhead of its corresponding PDUs. The PSSize is expressed in bytes. NOTE 4: This field may be optionally present given the signaling of the “pdu-set-size” extension attribute in the SDP offer/answer negotiation as per Clause 4.4.2.5. NOTE 5: Guidelines to set the PDU Set Size in bytes by an Application Server are provided in Clause 4.4.2.6.3.

Observation 1: According to TS 23.501, the PDU set information needs to be added in the NG-U GTP-U header.

Proposal 1: Introduce the PDU set information, including the End PDU of the PDU Set (EPDU), End of Data Burst (EDB), PDU Set Importance (PSI), PDU Set Sequence Number (PSSN), PDU Sequence Number within a PDU Set (PSN) and PDU Set Size (PSSize) in the DL PDU session information frame.

As for whether to add the PDU set information in the Xn-U or F1-U GTP-U header needs to be further discussed since the benefits for adding the PDU set information are still not clear.

Observation 2: The benefits of adding the PDU set information in the Xn-U or F1-U GTP-U header are not clear.

Proposal 2: RAN3 discusses whether to add the PDU set information in the Xn-U or F1-U GTP-U header.

2.1.2	PDU set QoS parameters

According to TS 23.501, PDU set QoS parameters, i.e. PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) are introduced to support PDU set based QoS handling in NG-RAN. Therefore, the PDU set QoS parameters need to be added in the QoS Flow Level QoS Parameters IE in NGAP to support PDU set based QoS handling.

TS 23.501 PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN. At least one PDU Set QoS Parameter shall be sent to the NG-RAN to enable PDU Set based QoS handling. The following PDU Set QoS Parameters are specified: 1. PDU Set Delay Budget (PSDB). 2. PDU Set Error Rate (PSER). 3. PDU Set Integrated Handling Information (PSIHI).

Observation 3: According to TS 23.501, the PDU set QoS parameters needs to be introduced in the NGAP.

Proposal 3: Add the PDU set QoS parameters, i.e. PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) in the QoS Flow Level QoS Parameters IE in NGAP to support PDU set based QoS handling.

Since the QoS Flow Level QoS Parameters IE is also included in the XnAP, F1AP and E1AP, the PDU set QoS parameters also need to be added in the XnAP, F1AP and E1AP to support PDU set based QoS handling.

Proposal 4: Add the PDU set QoS parameters, i.e. PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) in the QoS Flow Level QoS Parameters IE in XnAP, F1AP and E1AP to support PDU set based QoS handling.

The non-homogenous support of PDU set based handling in NG-RAN is also specified in TS 23.501 as below.

TS 23.501 By sending at least one PDU Set QoS parameter to the NG-RAN, the SMF requests the NG-RAN to activate PDU Set QoS handling for a given QoS flow and the NG-RAN provides the SMF with an indication of whether the PDU Set QoS parameter is accepted or not. At NG-RAN Xn handover and N2 handover, the target NG-RAN provides to the SMF with an indication of whether the target NG-RAN node supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the NG-RAN indications, the SMF configures the PSA UPF to activate/deactivate the PDU Set identification and marking. In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header it shall start doing so from a complete PDU Set.

If the SMF sends the PDU set QoS parameter to the NG-RAN node to activate the PDU set QoS handling for a given QoS flow, the NG-RAN node needs to provide an indication of whether the PDU set QoS parameter is accepted or not. Therefore, an indication of whether the PDU set QoS parameter is accepted or not needs to be added in the PDU session resource setup response message and the PDU session resource modify response message.

Observation 4: If the SMF sends the PDU set QoS parameter to the NG-RAN node to activate the PDU set QoS handling for a given QoS flow, the NG-RAN node needs to provide an indication of whether the PDU set QoS parameter is accepted or not.

Proposal 5: Add an indication of whether the PDU set QoS parameter is accepted or not in the PDU session resource setup response message and the PDU session resource modify response message.

During Xn based handover and NG based handover, the target NG-RAN node needs to provide an indication of whether the target NG-RAN node supports PDU set based handling. Therefore, an indication of whether the target NG-RAN node supports PDU set based handling needs to be added in the handover request acknowledge message and the path switch request message.

Observation 5: During Xn based handover and NG based handover, the target NG-RAN node needs to provide an indication of whether the target NG-RAN node supports PDU set based handling.

Proposal 6: Add an indication of whether the target NG-RAN node supports PDU set based handling in the handover request acknowledge message and the path switch request message.

2.2	UE power saving management

TS 23.501 has defined that the traffic assistance information, i.e. UL and/or DL Periodicity, N6 Jitter Information associated with the DL Periodicity and the Indication of End of Data Burst may be provide to the NG-RAN node to configure UE power saving management scheme for connected mode DRX.

TS 23.501 The following traffic assistance information may be provided by the CN to NG-RAN in order to configure UE power saving management scheme for connected mode DRX: - UL and/or DL Periodicity; - N6 Jitter Information associated with the DL Periodicity; - Indication of End of Data Burst. The UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity are provided by the CN to NG RAN via TSCAI.

Since the UL and DL Periodicity is already included in TSCAI, only N6 Jitter Information associated with the DL Periodicity is required to be added in the TSCAI in the NGAP. In addition, this enhancement is also needed in XnAP, F1AP and E1AP.

Proposal 7: Add the N6 Jitter Information associated with the DL Periodicity in the TSCAI in the NGAP, XnAP, F1AP and E1AP.

In addition, it is answered that TSCAI can be provided to the RAN for either GBR or non-GBR QoS flow in the LS R3-233731 from SA2.

TS 38.413 Question: RAN2 has discussed the provision of assistance information from CN to RAN for XR services and is wondering whether TSCAI can be provided for both GBR and non-GBR QoS flows in case of XR. SA2 Answer: for either GBR or non-GBR QoS flow, TSCAI can be provided to the RAN.

However, according to the current TS 38.413 as below, TSCAI can only be provided to the RAN for GBR QoS flow.

| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Observation 6: According to the current TS 38.413 as below, TSCAI can only be provided to the RAN for either GBR QoS flow.

Proposal 8: Extend the usage of TSCAI for either GBR or non-GBR QoS flow.

As for the indication of End of Data Burst, it needs to be added in the NG-U GTP-U header.

Proposal 9: Add the indication of End of Data Burst in the NG-U GTP-U header.

2.3	Congestion information signalling

At RAN3#118 meeting, SA2 send an LS to RAN3 to provide feedback whether it is feasible for RAN to estimate congestion information per QoS flow, per DRB in downlink and uplink directions. RAN3 replied that it is feasible to support congestion information estimation on a per DRB and/or per QoS Flow level in downlink and uplink directions. Since no further LS has received regarding the congestion information from SA2, RAN3 should postpone the discussion regarding this issue.

Proposal 10: RAN3 postpones the discussion regarding the congestion information signalling design until receiving an LS from SA2.

# 3	Conclusion

Based on above analysis, we provide the following observations and proposals.

Observation 1: According to TS 23.501, the PDU set information needs to be added in the NG-U GTP-U header.

Proposal 1: Introduce the PDU set information, including the End PDU of the PDU Set (EPDU), End of Data Burst (EDB), PDU Set Importance (PSI), PDU Set Sequence Number (PSSN), PDU Sequence Number within a PDU Set (PSN) and PDU Set Size (PSSize) in the DL PDU session information frame.

Observation 2: The benefits of adding the PDU set information in the Xn-U or F1-U GTP-U header are not clear.

Proposal 2: RAN3 discusses whether to add the PDU set information in the Xn-U or F1-U GTP-U header.

Observation 3: According to TS 23.501, the PDU set QoS parameters needs to be introduced in the NGAP.

Proposal 3: Add the PDU set QoS parameters, i.e. PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) in the QoS Flow Level QoS Parameters IE in NGAP to support PDU set based QoS handling.

Proposal 4: Add the PDU set QoS parameters, i.e. PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER) and PDU Set Integrated Handling Information (PSIHI) in the QoS Flow Level QoS Parameters IE in XnAP, F1AP and E1AP to support PDU set based QoS handling.

Observation 4: If the SMF sends the PDU set QoS parameter to the NG-RAN node to activate the PDU set QoS handling for a given QoS flow, the NG-RAN node needs to provide an indication of whether the PDU set QoS parameter is accepted or not.

Proposal 5: Add an indication of whether the PDU set QoS parameter is accepted or not in the PDU session resource setup response message and the PDU session resource modify response message.

Observation 5: During Xn based handover and NG based handover, the target NG-RAN node needs to provide an indication of whether the target NG-RAN node supports PDU set based handling.

Proposal 6: Add an indication of whether the target NG-RAN node supports PDU set based handling in the handover request acknowledge message and the path switch request message.

Proposal 7: Add the N6 Jitter Information associated with the DL Periodicity in the TSCAI in the NGAP, XnAP, F1AP and E1AP.

Observation 6: According to the current TS 38.413 as below, TSCAI can only be provided to the RAN for either GBR QoS flow.

Proposal 8: Extend the usage of TSCAI for either GBR or non-GBR QoS flow.

Proposal 9: Add the indication of End of Data Burst in the NG-U GTP-U header.

Proposal 10: RAN3 postpones the discussion regarding the congestion information signalling design until receiving an LS from SA2.

4	Text Proposal for BL CR TS 38.413

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.2	PDU Session Management Procedures

### 8.2.1	PDU Session Resource Setup

#### 8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node.

*// skip unchanged part //*

If the UE Slice Maximum Bit Rate List IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, store the UE Slice Maximum Bit Rate List in the UE context, and use it for each S-NSSAI for the concerned UE as specified in TS 23.501 [9].

The NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE SETUP RESPONSE message location information of the UE in the User Location Information IE.

If the User Location Information IE is included in the PDU SESSION RESOURCE SETUP RESPONSE message, the AMF shall handle this information as specified in TS 23.501 [9].

If the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall consider it for PDU Set based QoS handling as specified in TS 23.501 [9], and shall, if supported, include the PDU Set QoS parameter Indicator IE in the PDU SESSION RESOURCE SETUP RESPONSE message.

Interactions with Handover Preparation procedure:

If a handover becomes necessary during the PDU Session Resource Setup procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Setup procedure and initiate the Handover Preparation procedure as follows:

1.	The NG-RAN node shall send the PDU SESSION RESOURCE SETUP RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU session resources which failed to be setup with an appropriate cause value, e.g. "NG intra-system handover triggered", "NG inter-system handover triggered" or "Xn handover triggered".

2.	The NG-RAN node shall trigger the handover procedure.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.3	PDU Session Resource Modify

#### 8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE MODIFY REQUEST message to the NG-RAN node.

*// skip unchanged part //*

For each PDU session, if the PDU Session Expected UE Activity Behaviour IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, handle this information as specified in TS 23.501 [9].

For each PDU session for which the Secondary RAT Usage Information IE is included in the PDU Session Resource Modify Response Transfer IE, the SMF shall handle this information as specified in TS 23.502 [10].

If the PDU Set QoS Parameters IE is included in the QoS Flow Level QoS Parameters IE in the PDU Session Resource Modify Request Transfer IE of the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall consider it for PDU Set based QoS handling as specified in TS 23.501 [9], and shall, if supported, include the PDU Set QoS parameter Indicator IE in the PDU SESSION RESOURCE MODIFY RESPONSE message.

Interactions with Handover Preparation procedure:

If a handover becomes necessary during the PDU Session Resource Modify procedure, the NG-RAN node may interrupt the ongoing PDU Session Resource Modify procedure and initiate the Handover Preparation procedure as follows:

1.	The NG-RAN node shall send the PDU SESSION RESOURCE MODIFY RESPONSE message in which the NG-RAN node shall indicate, if necessary, all the PDU sessions failed with an appropriate cause value, e.g. "NG intra-system handover triggered", "NG inter-system handover triggered" or "Xn handover triggered".

2.	The NG-RAN node shall trigger the handover procedure.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.4.2	Handover Resource Allocation

8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

*// skip unchanged part //*

If the HANDOVER REQUEST message contains within the Source NG-RAN Node to Target NG-RAN Node Transparent Container IE the NGAP IE Support Information Request List IE, the target NG-RAN node shall, if supported and the target NG-RAN node accepts the request for handover, for each included NGAP Protocol IE-Id provided within the Target NG-RAN Node to Source NG-RAN Node Transparent Container IE in the HANDOVER REQUEST ACKNOWLEDGE message

-	set the NGAP Protocol IE Support Information IE to "supported" if the target NG-RAN node has information that the functionality associated with the indicated IE is supported

-	set the NGAP Protocol IE Support Information IE to "not-supported" if the target NG-RAN node has information that the functionality associated with the indicated IE is not supported

on the interface instance via which the HANDOVER REQUEST message has been received, and

-	set the NGAP Protocol IE Presence Information IE to "present" if the target NG-RAN node has received the respective NGAP Protocol IE-Id in the HANDOVER REQUEST message, and “not-present” otherwise.

If the PDU Set QoS Handling Indicator IE is included in the HANDOVER REQUEST ACKNOWLEDGE message, the AMF shall, if supported, use the IE according to TS 23.501 [9].

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the HANDOVER REQUEST message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send the RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

The NG-RAN node initiates the procedure by sending the PATH SWITCH REQUEST message to the AMF. Upon reception of the PATH SWITCH REQUEST message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transparently transfer the Path Switch Request Transfer IE to the SMF associated with the concerned PDU session.

*// skip unchanged part //*

If the 5G ProSe Authorized IE is contained in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, update its ProSe authorization information for the UE accordingly. If the 5G ProSe Authorized IE includes one and more IEs set to "not authorized", the NG-RAN node shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant 5G ProSe service(s).

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported:

-	replace the previously provided 5G ProSe UE PC5 Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 QoS Parameters IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, use it as defined in TS 23.304 [47].

If the PDU Set QoS Handling Indicator IE is included in the PATH SWITCH REQUEST message, the AMF shall, if supported, use the IE according to TS 23.501 [9].

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "single RRC connected state report" and the UE is in RRC_CONNECTED state, the NG-RAN node shall, if supported, send one RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE.

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "single RRC connected state report" and the UE is in RRC_INACTIVE state, the NG-RAN node shall, if supported, send to the AMF one RRC INACTIVE TRANSITION REPORT message plus one subsequent RRC INACTIVE TRANSITION REPORT message when the RRC state transitions to RRC_CONNECTED state.

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "subsequent state transition report", the NG-RAN node shall, if supported, send one RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE and subsequent RRC INACTIVE TRANSITION REPORT messages to report the RRC state of the UE when the UE enters or leaves RRC_INACTIVE state.

Interactions with PDU Session Resource Notify procedure:

If the QoS related parameters (e.g. the CN Packet Delay Budget Downlink IE or the CN Packet Delay Budget Uplink IE) are included in the Path Switch Request Acknowledge Transfer IE of the PATH SWITCH REQUEST ACKNOWLEDGE message, but can not be succesfully accepted by the NG-RAN node, the NG-RAN node should continue to use the old values received from the source NG-RAN node, if any. The NG-RAN node shall, if supported, send the PDU SESSION RESOURCE NOTIFY message to notify the AMF.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.19 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS flows and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [9]. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR QoS flows and is ignored otherwise. | - |  |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| QoS Monitoring Reporting Frequency | O |  | INTEGER (1.. 1800, …) | Indicates the reporting frequency for RAN part delay for QoS monitoring. Units: second | YES | ignore |
| PDU Set QoS Parameters | O |  | 9.3.1.xx1 | This IE indicates if the PDU Set QoS Parameter is accepted for a QoS flow. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.131	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.132 |  | - |  |
| Burst Arrival Time | O |  | 9.3.1.133 |  | - |  |
| Survival Time | O |  | 9.3.1.221 |  | YES | ignore |
| Jitter | O |  | FFS | FFS | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xx1	PDU Set QoS Parameters

This IE defines the PDU set QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | O |  | 9.3.1.xx2 | PDU Set Delay Budget is specified in TS 23.501 [9]. |
| PDU Set Error Rate | O |  | 9.3.1.xx3 | PDU Set Error Rate is specified in TS 23.501 [9]. |
| PDU Set Integrated Handling Information | O |  | 9.3.1.xx4 | PDU Set Integrated Handling Information is specified in TS 23.501 [9]. |

#### 9.3.1.xx2	PDU Set Delay Budget

This IE indicates the PDU Set Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Delay Budget | M |  | FFS | FFS |

#### 9.3.1.xx3	PDU Set Error Rate

This IE indicates the PDU Set Error Rate for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Scalar | M |  | FFS | FFS |
| Exponent | M |  | FFS | FFS |

#### 9.3.1.xx4	PDU Set Integrated Handling Information

This IE indicates the PDU Set Integrated Handling Information for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set Integrated Handling Information | M |  | FFS | indicates whether all PDUs of the PDU Set are needed for the usage of the PDU Set by the application layer in the receiver side. |

9.3.1.xx5	PDU Set QoS Parameter Indicator

The PDU Set QoS parameter Indicator IE indicates if the PDU Set QoS Parameter is accepted for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set QoS Parameter Indicator | M |  | ENUMERATED (accepted, not accepted, …) | Indicates that PDU Set QoS parameter is accepted or not |

9.3.1.xx6	PDU Set QoS Handling Indicator

The PDU Set QoS Handling Indicator IE indicates if the PDU Set QoS Handling is supported for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Set QoS Handling Indicator | M |  | ENUMERATED (supported, not supported, …) | Indicates that PDU Set QoS handling is supported or not |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.2.13	QoS Flow List with Data Forwarding

This IE is used to provide a list of QoS flows with indication if forwarding is accepted.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flow Item with Data Forwarding |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >Data Forwarding Accepted | O |  | 9.3.1.62 | This IE is included for the QoS flows in the PDU session to be forwarded over the PDU session forwarding tunnel. It may be included for the QoS flows in the PDU session to be forwarded over the DRB forwarding tunnel(s). | - |  |
| >Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set | YES | ignore |
| >PDU Set QoS Handling Indicator | O |  | 9.3.1.xx6 | This IE indicates if the PDU Set QoS Handling is supported for a QoS flow | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.4.8	Path Switch Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of DL PDUs. | - |  |
| DL NG-U TNL Information Reused | O |  | ENUMERATED (true, …) | Indicates that DL NG-U TNL Information has been reused. | - |  |
| User Plane Security Information | O |  | 9.3.1.60 |  | - |  |
| QoS Flow Accepted List |  | 1 |  | QoS flows associated with the DL NG-U UP TNL Information IE. | - |  |
| >QoS Flow Accepted Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.152 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| >>PDU Set QoS Handling Indicator | O |  | 9.3.1.xx6 | This IE indicates if the PDU Set QoS Handling is supported for a QoS flow | YES | ignore |
| Additional DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Redundant DL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | NG-RAN node endpoint of the NG-U transport bearer, for delivery of redundant DL PDUs. | YES | ignore |
| Redundant DL NG-U TNL Information Reused | O |  | ENUMERATED (true, …) | Indicates that Redundant DL NG-U TNL Information has been reused. | YES | ignore |
| Additional Redundant DL QoS Flow per TNL Information | O |  | QoS Flow per TNL Information List 9.3.2.1 | NG-RAN node endpoint of the additional NG-U transport bearer(s) for delivery of Redundant DL PDUs for split PDU session, together with associated QoS flows. | YES | ignore |
| Used RSN Information | O |  | Redundant PDU Session Information 9.3.1.136 |  | YES | ignore |
| Global RAN Node ID of Secondary NG-RAN Node | O |  | Global RAN Node ID 9.3.1.5 |  | YES | ignore |
| MBS Support Indicator | O |  | 9.3.1.210 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>