3GPP TSG-RAN WG3 Meeting #121	R3-234587

Toulouse, France, August 21st – 25th, 2023

Agenda item:	20.2

Source: 	LG Electronics, ZTE, CATT, Nokia, Nokia Shanghai Bell, Ericsson,	Google, Huawei

Title: 	(TP to 38.300) Leftover issues for MT-SDT

Document for:	Other

# Introduction

This TP is to capture the agreements based on discussions [1] related to [2].

# Reference

[1] R3-234534, “Summary of Offline Discussion on CB: # R18SDT_Solution”, ZTE

[2] R3-234352, “Necessary corrections for stage-2 BL CRs (TPs for NR_MT_SDT BL CR for TS 38.300 and TS 38.401) (LG Electronics Inc.)”, LG Electronics

# TP for NR_MT_SDT for TS 38.300

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 18.X	MT-SDT with/without UE context relocation

The overall procedure for MT-SDT procedure with/without UE context relocation is illustrated in the figure 18.X-1.

Figure 18.X-1. MT-SDT with/without UE context relocation

1.  DL user data and/or DL NAS signalling are received at the Last Serving gNB for the UE in RRC_INACTIVE state.

2.  The Last Serving gNB may send MT-SDT information to the neighbour gNBs within the RNA, via XnAP RAN PAGING message.

3.  The gNB that receives MT-SDT information within the RNA, takes into account the MT-SDT information received in the XnAP RAN PAGING message to decide whether to trigger MT-SDT Paging. The gNB which ultimately reaches the UE via the Uu Paging becomes the Receiving gNB.

NOTE: In case that the Receiving gNB decides not to trigger MT-SDT paging, the above step 3 and subsequent steps are the same as Figure 9.2.2.4.2 from step 3.

4/5.  The UE may decide to initiate MT-SDT procedure and in this case sends an RRCResumeRequest message with a MT-SDT resume cause to the Receiving gNB.

6.  The following steps are the same as Figure 18.2-1/18.3-1 from step 2, except that the first SDT user data and/or NAS signalling is DL SDT data and/or DL SDT NAS signalling.

NOTE: In case DL non-SDT data or DL non-SDT signalling has arrived between step 2 and step 6, the Last Serving gNB should relocate the UE context to the Receiving gNB and forward the received data to the Receiving gNB.