3GPP TSG-RAN3 Meeting #121	R3-233713

Toulouse, France, 21th Aug - 25th Aug 2023

3GPP TSG-RAN WG2 Meeting #122	R2-2306817

Incheon, Korea, 22 – 26 May 2023

Title:	LS on UE RACH-less handover for mobile IAB

Response to:

Release:	Rel-18

Work Item:	NR_mobile_IAB-Core

Source:	RAN2

To:	RAN3

Cc:

Contact person:

Name:	Yulong Shi

Tel. Number:

E-mail Address:	shiyulong5@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	N/A

1. Overall Description:

RAN2 has discussed the UE RACH-less handover in mobile IAB and achieved the following agreements:

- RAN2#121bis meeting agreements:

Feasibility of beam handling during RACH-less HO in the mIAB WI is FFS (and this need to be addressed for RACH-less to be supported for mIAB).

RAN2 discuss further the following options to support beam operation for the first UL transmission/DL reception towards the target logical DU in RACH-less HO during DU migration:

- Option 1: (Explicit approach) Explicit beam information is included in HO command. FFS the details.

- Option 2: (Implicit approach) UE re-uses the same beam status as in the source cell (the beam information is not carried explicitly in HO command).

RACH-less HO with same TA with security key change is in scope for served UEs during mIAB DU migration. FFS UL grant and HO completion procedure in mIAB RACH-less HO.

- RAN2#122 meeting agreements:

RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured).

For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

for the UL grant and HO completion in RACH-less HO:

- 1. Both type-1 configured grant and dynamic grant are supported

- 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

Send LS to RAN3 to check whether there are issues / feasibility concerns

RAN2 would like to ask RAN3 to take those agreements into account and provide feedbacks if there are any issues or feasibility concerns.

2. Actions:

To RAN3.

ACTION: 	RAN2 respectively asks RAN3 to take the above agreements into account and provide feedbacks if there are any issues or feasibility concerns.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #123	August 21 – August 25, 2023		Toulouse, France

TSG RAN WG2 Meeting #123bis	October 9 – October 13, 2023		Xiamen, China