3GPP TSG-RAN WG3 #122	R3-237174

Chicago, USA, 13th – 17th Nov, 2023

Source:	CATT

Title:	Support of AI/ML based mobility optimization

Agenda Item:	12.2.2.2

Document for:	Discussion and decision

- Introduction

This discussion paper focuses on XnAP impacts of AI/ML for UE associated metrics.

- Discussion

Agreements on UE associated metrics achieved in last meeting are listed below:

Encode the Measured UE Trajectory with new IE with Global NG-RAN Cell Identity and Time UE Stays in Cell as mandatory.

Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

Define a new Number of Visited Cell IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

The maximum number of cells for predicted UE trajectory is 16.

The maximum number of cells for measured UE trajectory is 16.

2.1 triggering conditions

There are conditions A, B, C and D that may be used by target node to trigger UE trajectory feedback. The related open issue is listed below:

No consensus on signaling of an explicit indication of exit conditions for the collection of measured UE trajectory.

On the above open issue, first, we think the requesting node could be aware of condition C and condition D without any indication from the reporting node. For condition C, based on whether the sum of stay time in measured UE Trajectory collected in target node equals to the value of report time duration or not, source node can know whether condition C is satisfied. While for condition D, requesting node also can know how many times handover is implemented in the target cell via the number of visited cells in measured UE Trajectory. If the number of visited UE is the same as the configured number of handovers, then the requesting node could understand the trajectory report is triggered by condition D.

Then, for condition A and condition B, it is true that the requesting node could not differentiate the two conditions. However, the benefit to indicate to the requesting node on the trigger condition is negligible in these two conditions. For condition B, since measured UE Trajectory only includes cells in one-hop target node in R18, the request node could not have the ground truth on UE trajectory information even it knows that the trigger condition is B. Thereby, the measured UE trajectory is still not suitable for AI model retuning. And if multiple hop target node feedback could be supported in the future, source node can naturally know whether UE performs handover from target node to other node from measured UE Trajectory, i.e. the occurrence of condition B.

For condition A, i.e. UE moves to RRC_IDLE/RRC_INACTIVE mode, similar as condition B, even the requesting node is aware that state update happens on this UE, it still does not have the information of the visited cell of the UE after moving to RRC_IDLE/RRC_INACTIVE mode. Without ground truth from the reporting node, only indicator on trigger condition does not help.

Based on above analysis, we think it is not needed to include indication on condition A/B/C/D in request message.

Observation 1: In one hop scenario, the benefit is negligible to distinguish condition A and B due to lack of ground truth. In multiple hop scenario, it is naturally to distinguish condition A and B.

Proposal 1: No need to introduce an indication for signalled the requesting node about condition A/B/C/D.

For condition D, the number of intra-NG-RAN node inter-cell handovers is included in DATA COLLECTION REQUEST message, but the detail is FFS as below:

FFS on the details of the Number of Visited Cell IE

We think the Maximum number of Visited Cell may follow the number of visited cell in UE history information, i.e. 16.

Proposal 2: It is proposed to define the Number of Visited Cell IE in target NR-RAN node within the scope of (1..16).

2.2 The Time Duration

The time duration for the collection of measured UE trajectory may be longer than that for UE performance feedback, so a new time Duration IE is introduced in the DATA COLLECTION REQUEST message in last RAN3 meeting, the related agreement is listed below:

Define a new Time Duration IE for the collection of measured UE trajectory in the DATA COLLECTION REQUEST message.

The detail is still FFS as below:

FFS on the details of the Time Duration IE.

Here we continue discussing how to define the new Time Duration IE. When UE is handed over to target node, it may stay in multiply cells in target node. During this period of time, measured UE trajectory may be collected to include multiply cell information within target node. While UE performance feedback may only include one cell information, .i.e. only the information of the first accessed cell is included in UE performance feedback after handover. So, a longer time duration for the collection of measured UE trajectory may be needed. But source node have to keep UE context to wait for UE trajectory feedback during this period of time which means more storage resource in source node may be needed. To balance these two factors, we think it may be suitable to set the maximum Time Duration for measured UE trajectory as 4096s.

Proposal 3: It is proposed to define the new Time Duration IE within the scope of (1..4096)s.

As for the name of Report time duration in Additional UE Trajectory Reporting Conditions, we propose to rename Report time duration to collection time duration. The reason is that the measured UE trajectory collection will be terminated in case of condition A, B, C or D, but when to report it is still not clear and can be up to implement. For example, condition A i.e. UE moves to RRC_IDLE/RRC_INACTIVE mode is satisfied, the requesting node and the requested node may execute the periodicity report, the UE trajectory can be included in the next update message after collection terminated, which can reduce signalling load between two nodes. If there is no periodicity update between the two nodes, the UE trajectory can be sent once the collection is terminated. So, Report time duration IE cannot totally decide the time point when to trigger UE trajectory feedback. Then maybe the name of collection time duration is more suitable.

Proposal 4: It is proposed to rename Report time duration to collection time duration within Additional UE Trajectory Reporting Conditions.

- Conclusion

Observation 1: In one hop scenario, the benefit is negligible to distinguish condition A and B due to lack of grand truth. In multiple hop scenario, it is naturally to distinguish condition A and B.

Proposal 1: No need to introduce an indication for signalled the requesting node about condition A/B/C/D.

Proposal 2: It is proposed to define the Number of Visited Cell IE in target NR-RAN node within the scope of (1..16).

Proposal 3: It is proposed to define the new Time Duration IE within the scope of (1..4096)s.

Proposal 4: It is proposed to rename Report time duration to collection time duration within Additional UE Trajectory Reporting Conditions.