3GPP TSG-RAN WG3 Meeting #122	R3-237412

Chicago, USA, 13th – 17th November, 2023

Agenda Item:	13.2

Source: 	Lenovo

Title: 	(TP to TS 38.473 BL CR) Discussion on mIAB-DU migration

Document for:	Discussion and decision

- Introduction

A new WID for mobile IAB-node [1] has been approved to be discussed in Rel-18, and following detail objectives are identified mainly focus on RAN3 and RAN2.

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2] - The mobile IAB-node can connect to a stationary (intermediate) IAB-node. Optimizations specific to the scenarios, where the mobile IAB-node connects to a stationary (intermediate) IAB-node, or where it directly connects to an IAB-donor-DU are de-prioritized. - The mobility of dual-connected IAB-nodes is down-prioritized. - Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2] Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility. - Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

In this contribution, we continue to discuss the general principles of mobile IAB-node inter-donor topology adaptation, including mobile IAB-DU migration and mobile IAB-node integration.

- Discussion

As shown in Figure 1, before IAB-DU migration, IAB-MT of IAB-node connects to IAB-donor C and IAB-DU of the IAB-node connects to IAB-donor A. And IAB-DU performs migration from source IAB-donor A to target IAB-donor B.

Figure 1. Example of IAB-DU migration

IAB-DU migration tigger coordination

IAB-DU migration may be triggered by the IAB-node itself based on OAM or pre-configuration, alternatively, IAB-DU migration may also be triggered by the source F1-terminating IAB-donor-CU. Then, in some cases, both IAB-node and source F1-terminating IAB-donor-CU may trigger separate IAB-DU migration procedure. Moreover, IAB-node and source F1-terminating IAB-donor-CU may select different target IAB-donor-CU for IAB-DU migration. Therefore, source F1-termigation IAB-donor and IAB-node may need to coordinate with each other for which node to trigger IAB-DU migration, in order to avoid IAB-DU migration trigger collision.

Proposal 1: In order to avoid IAB-DU migration trigger collision, source F1-termigation IAB-donor and IAB-node may need to coordinate with each other for which node to trigger IAB-DU migration.

As an example, the IAB-donor-CU can determine the node to trigger IAB-DU migration and notify the result to the mIAB-node via F1AP message, e.g., F1 setup response message. Alternatively, the mIAB-node can also make the decision the send the request to the IAB-donor-CU.

F1 setup outcome notification

After F1 setup between second logical IAB-DU and target IAB-donor-CU, the IAB-node will send the outcome of F1 setup procedure to source IAB-donor-CU via F1AP. Then based on the outcome, the source CU can determine to perform handover for the served UEs. In case the IAB-DU migration is triggered by the source IAB-donor-CU and the F1 setup is failure with the target IAB-donor-CU, the source IAB-donor-CU is only aware of the failure outcome based on the latest running CR. However, as the triggering node, the source IAB-donor-CU has to be aware of the failure cause of the F1 setup and the time-to-wait for next triggering to the same target IAB-donor-CU, so it can perform smarter operation for the IAB-DU migration.

Proposal 2: In case CU-based triggering IAB-DU migration, source F1-terminating IAB-donor-CU may need to be aware of the failure cause and minimum waiting time for F1 setup procedure.

Sequence of UE HO and IAB TMM procedures

As the definition, the purpose of the IAB Transport Migration Management procedure is to exchange information between the F1-terminating IAB-donor-CU and the RRC-terminating IAB-donor-CU of an IAB-node, for the purpose of managing the migration of the traffic between the topologies managed by the two IAB-donor-CUs.

The IAB TMM procedure can be used to manage the F1 traffic of the IAB-node, and can also be used to manage the UP traffic of the served UEs. Then the IAB TMM procedure can be performed ahead of UE handover for managing the F1 traffic of the IAB-node, and the IAB TMM procedure needs to be performed after the UE handover.

Proposal 3: IAB TMM procedure can be performed before and/or after the UE handover.

# Conclusion

This contribution aims to discuss the general principles of mobile IAB-node inter-donor topology adaptation. And following observations and proposals are concluded.

Proposal 1: In order to avoid IAB-DU migration trigger collision, source F1-termigation IAB-donor and IAB-node may need to coordinate with each other for which node to trigger IAB-DU migration.

Proposal 2: In case CU-based triggering IAB-DU migration, source F1-terminating IAB-donor-CU may need to be aware of the failure cause and minimum waiting time for F1 setup procedure.

Proposal 3: IAB TMM procedure can be performed before and/or after the UE handover.

In addition, based on proposal 2, a TP to TS 38.473 BL CR is provided in Annex.

Proposal 4: Agree to the TP to TS 38.473 BL CR in Annex.

# Reference

- RP-222671. Revised WID for NR_mobile_IAB.

- Chairman notes of RAN3 118 – 121 meetings.

# Annex – TP to TS 38.473 BL CR

-------------------------------------------Start of Change------------------------------------------

### 8.10.Y	Mobile IAB F1 Setup Outcome Notification

#### 8.10.Y.1	General

The purpose of the Mobile IAB F1 Setup Outcome Notification procedure is to report the outcome of the F1 interface setup between a target logical gNB-DU and a target F1-terminating IAB-donor-CU. The target logical gNB-DU is co-located with the gNB-DU that sends the notification message and can be used to set up another F1 interface for supporting mobile IAB-DU migration. This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for mobile IAB-nodes, where the term "gNB-DU" applies to mobile IAB-DU, and the term "gNB-CU" applies to source F1-terminating IAB-donor-CU during mIAB-DU migration.

#### 8.10.Y.2	Successful Operation

Figure 8.10.Y.2-1: Mobile IAB F1 Setup Outcome Notification: Successful Operation

The gNB-DU initiates the procedure by sending the MIAB F1 SETUP OUTCOME NOTIFICATION message to the gNB-CU. After the target logical gNB-DU co-located with the gNB-DU has performed the F1 setup procedure with the target F1-terminating IAB-donor-CU, the gNB-DU sends the MIAB F1 SETUP OUTCOME NOTIFICATION message to the gNB-CU, to report the outcome of the F1 setup towards the target F1-terminating IAB-donor-CU.

If the Activated Cells Mapping List is included in the MIAB F1 SETUP OUTCOME NOTIFICATION message, the gNB-CU shall, if supported, take it into account when further performing handover of the connected UEs from this gNB-DU to its co-located target logical gNB-DU.

If the F1 Setup Outcome IE in the MIAB F1 SETUP OUTCOME NOTIFICATION message is set to “failure”, the MIAB F1 SETUP OUTCOME NOTIFICATION message may further include appropriate cause value and the Time To Wait IE,

#### 8.10.Y.3	Abnormal Conditions

Not applicable.

-------------------------------------------Next Change------------------------------------------

#### 9.2.9.X2	MIAB F1 SETUP OUTCOME NOTIFICATION

This message is sent by the gNB-DU to notify the gNB-CU about the outcome of F1 interface setup between the gNB-DU’s co-located target logical gNB-DU and a target F1-terminating IAB-donor-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| F1 Setup Outcome | M |  | ENUMERATED (success, failure, …) |  | YES | reject |
| Activated Cells Mapping List |  | 0..1 |  |  | YES | ignore |
| >Activated Cells List Mapping Item IEs |  | 1.. <maxCellingNBDU> |  | List of activated cells. | EACH | ignore |
| >>NR CGI for Target Logical gNB-DU | O |  | NR CGI 9.3.1.12 | The identity of an activated cell belonging to the target logical gNB-DU of the mobile IAB-node | - |  |
| >>NR CGI for Source Logical gNB-DU | O |  | NR CGI 9.3.1.12 | The identity of an activated cell belonging to the source logical gNB-DU of the mobile IAB-node | - |  |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

-------------------------------------------Next Change------------------------------------------

#### -- MIAB F1 SETUP OUTCOME NOTIFICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MIAB F1 SETUP OUTCOME NOTIFICATION

--

-- **************************************************************

MIABF1SetupOutcomeNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MIABF1SetupOutcomeNotificationIEs}},

...

}

MIABF1SetupOutcomeNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-F1SetupOutcome						CRITICALITY reject	TYPE F1SetupOutcome								PRESENCE mandatory	}|

{ ID id-Activated-Cells-Mapping-List		CRITICALITY ignore	TYPE Activated-Cells-Mapping-List			PRESENCE optional	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE optional	}|

{ ID id-TimeToWait							CRITICALITY ignore	TYPE TimeToWait									PRESENCE optional	},

...

}

F1SetupOutcome ::= ENUMERATED {success, failure,...}

Activated-Cells-Mapping-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Activated-Cells-Mapping-List-ItemIEs } }

Activated-Cells-Mapping-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Activated-Cells-Mapping-List-Item	CRITICALITY ignore	TYPE Activated-Cells-Mapping-List-Item PRESENCE mandatory },

...

}

-------------------------------------------End of Change-------------------------------------------