3GPP TSG-RAN WG3 Meeting #122	R3-237515

# Chicago, U.S , November 13th – 17th 2023

Agenda Item:	10.2.4

Source:	Ericsson

Title:	(TP for SON for BLCR to TS38.413 and TS38.423) SON enhancements for Non-public networks

Document for:	Discussion, Decision

# Introduction

Non-public networks were standardized by 3GPP in rel-16 and enhanced in rel-17 in order to support private 5G networks. These networks can be deployed either as integrated with a PLMN, or in a standalone method. In rel-18, during the previous RAN3 meetings, the following agreements were taken on support for SON enhancements for NPN networks,

Area Scope for PNI-NPNs: Introduce a CAG list for MDT area scope. The use cases RAN3 should support are: Use Case 1: Enhanced area scope information should allow collection of MDT measurements in specific PNI-NPNs, i.e. MDT measurements should be collected only within specific CAGs. Use Case 2: Enhanced area scope information should allow collection of MDT measurements both in specific PNI-NPNs (i.e. in specific CAGs) and in public network areas (e.g. specific PN cells, TAIs, etc.). Agree to the addition of a CAG list inside and outside the current choice structure for the MDT Area Scope. Further enhancements are FFS Set the range of maxnoofCAGforMDT to 256 FFS: Check the company's preference: in case of PNI-NPN area scope is not present, whether how to interpret the old area scope choices when it set as cell base, TA based, TAI based, and PLMN wide with following cases: -only represents the PN area, i.e, only including non-CAG cells -can represent both PN and NPN area, i.e, including both non-CAG cells and CAG cells. If the latter is preferred, whether further clarification is needed to avoid misunderstanding. If old area scope choices only represents the PN area , FFS whether there is a need to preserve the legacy functionality of the NGAP "PLMN wide" choice, namely, this choice is equivalent to absence of area scope over Xn and RRC, i.e. MDT is performed within the MDT PLMN List without any restriction. Whether and how to support PNI-NPN PLMN wide option for PNI-NPN area scope configuration? If there is no consensus achieved in this meeting, leave it out in R18. FFS on whether the PNI NPN Area Scope for MDT IE shall be ignored in absence fo the Area Scope for MDT IE over Xn. Area Scope for SNPNs: RAN3 to focus on the following use case for SNPN and to continue discussions on how to address MDT Area Scope for specific cells or TAs of an SNPN: - Use Case 3: Enable collection of MDT measurements in the SNPN where the UE is registered. Add separate SNPN Cell Based MDT and SNPN TAI Based MDT area scope Add multi-SNPN MDT area scope. The maximum number of SNPNs in the MDT SNPN list should be 16. No need for SNPN-wide area scope. Son Reports: How to introduce NID or PNI-NPN ID into SON/MDT reports depends on RAN2’s decision. UE History Information: It is up to configuration and operator’s policies whether PN and NPN information can be included in the UE History Information. The undisclosed cell information in UHI for NPN is not pursued in R18. Other issues: Wait SA3's response for NPN user consent to check if any RAN3 impact. No need to introduce user consent of SNPN over interfaces.

In this paper, we discuss the possible NPN related enhancements and suggestions for future discussions. We also provide a TP to TS38.413 and TS38.423

# Discussion

## Support for MDT for NPN

2.1.1 PNI-NPN Wide Area Scope

Some companies brought up the need for PNI-NPN wide Area Scope during the previous meeting with the idea that this would be similar to the PLMN wide option in the legacy area scope.

In legacy, the PLMN wide option however indicates to the UE that it is allowed to collect MDT in all cells it has access to within the PLMN. The PLMN wide option is not forwarded over Xn as this is not needed, instead the Area Scope of MDT-NR is an optional IE and an absence of this over Xn indicates PLMN wide MDT collection.

This would not work for PNI-NPN wide as considering that the UE would have access to both PN and PNI-NPN cells, there would be no way of restricting the UE to collect MDT only in PNI-NPN cells. Such a functionality can be accomplished by listing all the CAG IDs supported by the PLMN in the Area Scope IE.

We believe the current functionality agreed by RAN3 with the introduction of a CAG ID list in and out of the the Area Scope for MDT structure is the best option and no PNI-NPN area scope is needed. The reason for that is that we do not see the relevance of a use case where an operator wants to collect MDT from all PNI-NPN networks, but not from the public network. We do see the usefulness of MDT data collection from some PNI-NPNs, e.g. to troubleshoot issues specific to a customer associated to the PNI-NPN, but in case of wide collection of data, we believe the PLMN wide option is the right functionality to support as it enables collection in and out of PNI-NPN cells.

PNI-NPN wide area scope is not necessary.

In the previous meeting, it was agreed to clarify semantics in the Area Scope of MDT-NR IE to ensure that the MDT area scope for non-public networks works as intended. There is still some clarification needed over XnAP. Consider the case that the CHOICE Area Scope of MDT-NR is not sent in the MDT Configuration-NR IE over XnAP after HO of a UE. This would indicate that the area of MDT collection is PLMN wide. The PNI-NPN Area Scope of MDT, that is present outside of the Choice can still be sent leading to a wrong configuration.

A wrong configuration can occur if the Choice Area Scope of MDT-NR is absent and the PNI-NPN Area Scope of MDT is present over Xn in the MDT Configuration-NR.

An abnormal condition is needed in the procedural text to make sure that this wrong configuration does not occur or that if it occurs, the PNI-PN Area Scope of MDT IE is ignored. This is shown in the TP to TS 38.423 in the Appendix of this paper.

Introduce an abnormal condition for the Xn, according to which if the Area Scope of MDT-NR IE is not present (meaning PLMN wide) and if the PNI-PN Area Scope of MDT IE is present, the PNI-PN Area Scope of MDT IE is ignored.

2.1.2 Area Scope of MDT for SNPNs

In the previous meetings RAN3 has agreed to support collection of MDT in SNPNs through configuration of the CHOICE Area Scope of MDT to denote Areas in three different ways (Cell ID based, TAI based, and NID based). Two of these options for SNPNs, namely the Cell ID based and the TAI based are redundant as these parameters are unique in a PLMN.

Namely, the Cell ID based collection for SNPNs consists of NR CGI and a NID. The NR CGI is however unique in a PLMN and considering that the SNPN is deployed together within a PLMN ID, the UE does not need the additional information of the NID to understand where MDT collection should occur. Similarly, the TAI is also a unique parameter in a PLMN and denotes a tracking area in that PLMN. In order to configure the UE with the tracking areas where MDT should be collected for an SNPN, thre is no need to signal to the UE the TAI plus the NID, the TAI is sufficient.

Thus the operator can already use the legacy structure consisting of a list of CGIs and a list of TAIs for these two SNPN based area scopes and specification changes can be minimized. Moreover, the configuration towards the UE can be simplified, avoid duplication of configurations that would not bring any benefit.

Remove the SNPN TAI Based MDT and SNPN Cell Based MDT from the Choice Area Scope of MDT in TS 38.413 and TS 38.423 agreed in this release.

# Conclusion

In the previous sections we made the following observations:

Observation 1	PNI-NPN wide area scope is not necessary.

Observation 2	A wrong configuration can occur if the Choice Area Scope of MDT-NR is absent and the PNI-NPN Area Scope of MDT is present over Xn in the MDT Configuration-NR.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce an abnormal condition for the Xn, according to which if the Area Scope of MDT-NR IE is not present (meaning PLMN wide) and if the PNI-PN Area Scope of MDT IE is present, the PNI-PN Area Scope of MDT IE is ignored.

Proposal 2	Remove the SNPN TAI Based MDT and SNPN Cell Based MDT from the Choice Area Scope of MDT in TS 38.413 and TS 38.423 agreed in this release.

# References

[1] RP-221825 : Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR and EN-DC

Appendix A: TP for TS 38.413

#### Start of Changes

- 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDT Activation |  | M |  |  |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  |  |  | - |  |  |  |
| CHOICE Area Scope of MDT |  | M |  |  |  |  |  |  |  | - |  |  |  |
| >Cell based |  |  |  |  |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |  |  |  |
| >>Cell ID List for MDT |  |  |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |  |  |  |  |  |
| >>>NR CGI |  | M |  |  |  | 9.3.1.7 |  |  |  | - |  |  |  |
| >TA based |  |  |  |  |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |  |  |  |
| >>TA List for MDT |  |  |  | 1..<maxnoofTAforMDT> |  |  |  |  |  |  |  |  |  |
| >>>TAC |  | M |  |  |  | 9.3.3.10 |  | The TAI is derived using the current serving PLMN. |  | - |  |  |  |
| >PLMN wide |  |  |  |  |  | NULL |  |  |  |  |  |  |  |
| >TAI based |  |  |  |  |  |  |  | If PNI-NPN Area Scope of MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |  |  |  |
| >>TAI List for MDT |  |  |  | 1..<maxnoofTAforMDT> |  |  |  |  |  |  |  |  |  |
| >>>TAI |  | M |  |  |  |  |  |  |  | - |  |  |  |
| >PNI-NPN Based MDT |  |  |  |  |  |  |  |  |  | YES |  | Ignore |  |
| >>CAG List for MDT |  |  |  | 1..<maxnoofCAGforMDT> |  |  |  |  |  |  |  |  |  |
| >>>PLMN ID |  |  |  |  |  | 9.3.3.5 |  |  |  |  |  |  |  |
| >>>CAG ID |  | M |  |  |  | 9.3.3.43 |  |  |  |  |  |  |  |
| >SNPN Based MDT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>MDT SNPN List |  |  |  | 1..<maxnoofMDTSNPNs> |  |  |  |  |  |  |  |  |  |
| >>>PLMN Identity |  | M |  |  |  | 9.3.3.5 |  |  |  |  |  |  |  |
| >>>NID |  | M |  |  |  | 9.3.3.42 |  | Identifies an SNPN together with the PLMN Identity IE. |  |  |  |  |  |
| CHOICE MDT Mode |  | M |  |  |  |  |  |  |  | - |  |  |  |
| >Immediate MDT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Measurements to Activate |  | M |  |  |  | BITSTRING (SIZE(8)) |  | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. |  | - |  |  |  |
| >>M1 Configuration |  | C-ifM1 |  |  |  | 9.3.1.171 |  |  |  | - |  |  |  |
| >>M4 Configuration |  | C-ifM4 |  |  |  | 9.3.1.172 |  |  |  | - |  |  |  |
| >>M5 Configuration |  | C-ifM5 |  |  |  | 9.3.1.173 |  |  |  | - |  |  |  |
| >>M6 Configuration |  | C-ifM6 |  |  |  | 9.3.1.174 |  |  |  | - |  |  |  |
| >>M7 Configuration |  | C-ifM7 |  |  |  | 9.3.1.175 |  |  |  | - |  |  |  |
| >>Bluetooth Measurement Configuration |  | O |  |  |  | 9.3.1.177 |  |  |  | - |  |  |  |
| >>WLAN Measurement Configuration |  | O |  |  |  | 9.3.1.178 |  |  |  | - |  |  |  |
| >>MDT Location Information |  | O |  |  |  | 9.3.1.176 |  |  |  | - |  |  |  |
| >>Sensor Measurement Configuration |  | O |  |  |  | 9.3.1.179 |  |  |  | - |  |  |  |
| >Logged MDT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Logging interval |  | M |  |  |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) |  | This IE is defined in TS 38.331 [18]. |  | - |  |  |  |
| >>Logging duration |  | M |  |  |  | ENUMERATED (10, 20, 40, 60, 90,120, …) |  | This IE is defined in TS 38.331 [18]. Unit: [minute]. |  | - |  |  |  |
| >>CHOICE Report Type |  | M |  |  |  |  |  |  |  | - |  |  |  |
| >>>Periodical |  |  |  |  |  | NULL |  |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration |  | M |  |  |  | 9.3.1.180 |  |  |  | - |  |  |  |
| >>Bluetooth Measurement Configuration |  | O |  |  |  | 9.3.1.177 |  |  |  | - |  |  |  |
| >>WLAN Measurement Configuration |  | O |  |  |  | 9.3.1.178 |  |  |  | - |  |  |  |
| >>Sensor Measurement Configuration |  | O |  |  |  | 9.3.1.179 |  |  |  | - |  |  |  |
| >>Area Scope of Neighbour Cells |  | O |  |  |  | 9.3.1.182 |  |  |  | - |  |  |  |
| >>Early Measurement |  | O |  |  |  | ENUMERATED (true, ...) |  | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. |  | YES |  | ignore |  |
| Signalling Based MDT PLMN List |  | O |  |  |  | MDT PLMN List 9.3.1.168 |  |  |  | - |  |  |  |
| PNI-NPN Area Scope of MDT |  | O |  |  |  | 9.3.3.x |  | This IE is ignored if the PLMN Wide IE is present |  | YES |  | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT scope. Value is 256. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-CommonNetworkInstance,

id-ConfiguredTACIndication,

id-CurrentQoSParaSetIndex,

id-DAPSRequestInfo,

id-DAPSResponseInfoList,

id-DataForwardingNotPossible,

id-DataForwardingResponseERABList,

id-DirectForwardingPathAvailability,

id-DL-NGU-UP-TNLInformation,

id-EndpointIPAddressAndPort,

id-EnergySavingIndication,

id-ExtendedPacketDelayBudget,

id-ExtendedRATRestrictionInformation,

id-ExtendedReportIntervalMDT,

id-ExtendedSliceSupportList,

id-ExtendedTAISliceSupportList,

id-ExtendedUEIdentityIndexValue,

id-EUTRA-PagingeDRXInformation,

id-GlobalCable-ID,

id-GlobalRANNodeID,

id-GlobalTNGF-ID,

id-GlobalTWIF-ID,

id-GlobalW-AGF-ID,

id-GUAMIType,

id-IncludeBeamMeasurementsIndication,

id-IntersystemSONInformationRequest,

id-IntersystemSONInformationReply,

id-IntersystemResourceStatusUpdate,

id-LastEUTRAN-PLMNIdentity,

id-LastVisitedPSCellList,

id-LocationReportingAdditionalInfo,

id-M4ReportAmount,

id-M5ReportAmount,

id-M6ReportAmount,

id-ExcessPacketDelayThresholdConfiguration,

id-M7ReportAmount,

id-MaximumIntegrityProtectedDataRate-DL,

id-MBS-AreaSessionID,

id-MBS-QoSFlowsToBeSetupList,

id-MBS-QoSFlowsToBeSetupModList,

id-MBS-QoSFlowToReleaseList,

id-MBS-ServiceArea,

id-MBS-SessionFSAIDList,

id-MBS-SessionID,

id-MBS-ActiveSessionInformation-SourcetoTargetList,

id-MBS-ActiveSessionInformation-TargettoSourceList,

id-MBS-SessionTNLInfo5GC,

id-MBS-SupportIndicator,

id-MBSSessionFailedtoSetupList,

id-MBSSessionFailedtoSetuporModifyList,

id-MBSSessionSetupResponseList,

id-MBSSessionSetuporModifyResponseList,

id-MBSSessionToReleaseList,

id-MBSSessionSetupRequestList,

id-MBSSessionSetuporModifyRequestList,

id-MDTConfiguration,

id-MicoAllPLMN,

id-NetworkInstance,

id-NGAPIESupportInformationRequestList,

id-NGAPIESupportInformationResponseList,

id-NID,

id-NR-CGI,

id-NRNTNTAIInformation,

id-NPN-MobilityInformation,

id-NPN-PagingAssistanceInformation,

id-NPN-Support,

id-NR-PagingeDRXInformation,

id-OldAssociatedQosFlowList-ULendmarkerexpected,

id-OnboardingSupport,

id-PagingAssisDataforCEcapabUE,

id-PagingCauseIndicationForVoiceService,

id-PDUSessionAggregateMaximumBitRate,

id-PduSessionExpectedUEActivityBehaviour,

id-PDUSessionPairID,

id-PDUSessionResourceFailedToSetupListCxtFail,

id-PDUSessionResourceReleaseResponseTransfer,

id-PDUSessionType,

id-PEIPSassistanceInformation,

id-PSCellInformation,

id-PNI-NPNBasedMDT,

id-PNI-NPN-AreaScopeofMDT,

id-QMCConfigInfo,

id-QosFlowAddOrModifyRequestList,

id-QosFlowFailedToSetupList,

id-QosFlowFeedbackList,

id-QosFlowParametersList,

id-QosFlowSetupRequestList,

id-QosFlowToReleaseList,

id-QosMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-SNPN-BasedMDT,

id-SuccessfulHandoverReportList,

id-UEContextReferenceAtSource,

id-RAT-Information,

id-RedundantCommonNetworkInstance,

id-RedundantDL-NGU-TNLInformationReused,

id-RedundantDL-NGU-UP-TNLInformation,

id-RedundantDLQosFlowPerTNLInformation,

id-RedundantPDUSessionInformation,

id-RedundantQosFlowIndicator,

id-RedundantUL-NGU-UP-TNLInformation,

id-SCTP-TLAs,

id-SecondaryRATUsageInformation,

id-SecurityIndication,

id-SecurityResult,

id-SgNB-UE-X2AP-ID,

id-S-NSSAI,

id-SONInformationReport,

id-SourceNodeID,

id-SourceNodeTNLAddrInfo,

id-SourceTNLAddrInfo,

id-SurvivalTime,

id-TNLAssociationTransportLayerAddressNGRAN,

id-TAINSAGSupportList,

id-TargetHomeENB-ID,

id-TargetRNC-ID,

id-TraceCollectionEntityURI,

id-TSCTrafficCharacteristics,

id-UEHistoryInformationFromTheUE,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityForPagingOfNB-IoT,

id-UL-NGU-UP-TNLInformation,

id-UL-NGU-UP-TNLModifyList,

id-ULForwarding,

id-ULForwardingUP-TNLInformation,

id-UsedRSNInformation,

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

id-EarlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-TAI,

id-HFCNode-ID-new,

id-GlobalCable-ID-new,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofBluetoothName,

maxnoofBPLMNs,

maxnoofCAGforMDT,

maxnoofCAGSperCell,

maxnoofCandidateCells,

maxnoofCellIDforMDT,

maxnoofCellIDforQMC,

maxnoofCellIDforWarning,

maxnoofCellinAoI,

maxnoofCellinEAI,

maxnoofCellsforMBS,

maxnoofCellsingNB,

maxnoofCellsinngeNB,

maxnoofCellsinNGRANNode,

maxnoofCellinTAI,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEmergencyAreaID,

maxnoofEAIforRestart,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofE-RABs,

maxnoofErrors,

maxnoofExtSliceItems,

maxnoofForbTACs,

maxnoofFreqforMDT,

maxnoofMBSFSAs,

maxnoofMBSQoSFlows,

maxnoofMBSServiceAreaInformation,

maxnoofMBSAreaSessionIDs,

maxnoofMBSSessions,

maxnoofMBSSessionsofUE,

maxnoofMDTPLMNs,

maxnoofMRBs,

maxnoofMultiConnectivity,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighPCIforMDT,

maxnoofNGAPIESupportInfo,

maxnoofNGConnectionsToReset,

maxNRARFCN,

maxnoofNRCellBands,

maxnoofNSAGs,

maxnoofPagingAreas,

maxnoofPC5QoSFlows,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofPLMNforQMC,

maxnoofQosFlows,

maxnoofQosParaSets,

maxnoofRANNodeinAoI,

maxnoofRecommendedCells,

maxnoofRecommendedRANNodes,

maxnoofAoI,

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo,

maxnoofReportedCells,

maxnoofSensorName,

maxnoofServedGUAMIs,

maxnoofSliceItems,

maxnoofMDTSNPNs,

maxnoofSNSSAIforQMC,

maxnoofSuccessfulHOReports,

maxnoofTACs,

maxnoofTACsinNTN,

maxnoofTAforMDT,

maxnoofTAforQMC,

maxnoofTAIforInactive,

maxnoofTAIforMBS,

maxnoofTAIforPaging,

maxnoofTAIforRestart,

maxnoofTAIforWarning,

maxnoofTAIinAoI,

maxnoofTargetS-NSSAIs,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofUEAppLayerMeas,

maxnoofUEsforPaging,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofThresholdsForExcessPacketDelay

FROM NGAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM NGAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Container{},

NGAP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

NGAP-PROTOCOL-IES

FROM NGAP-Containers;

#### -- A

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

pLMNWide					NULL,

tAIBased					TAIBasedMDT,

choice-Extensions		ProtocolIE-SingleContainer { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs NGAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT		CRITICALITY ignore	TYPE PNI-NPNBasedMDT		PRESENCE mandatory }|

{ ID id-SNPN-BasedMDT		CRITICALITY ignore	TYPE SNPN-BasedMDT			PRESENCE mandatory },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### -- S

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SNPN-BasedMDT::= SEQUENCE {

sNPNListforMDT		SNPNListforMDT,

iE-Extensions		ProtocolExtensionContainer { {SNPN-BasedMDT-ExtIEs} } OPTIONAL,

...

}

SNPN-BasedMDT-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

SNPNListforMDT ::= SEQUENCE (SIZE(1.. maxnoofMDTSNPNs)) OF SNPNListforMDTItem

SNPNListforMDTItem ::= SEQUENCE {

pLMNIdentity						PLMNIdentity,

nID									NID,

iE-Extensions		ProtocolExtensionContainer {{ SNPNListforMDTItem-ExtIEs}}		OPTIONAL,

...

}

SNPNListforMDTItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-ManagementBasedMDTPLMNModificationList				ProtocolIE-ID ::= 359

id-EarlyMeasurement										ProtocolIE-ID ::= 360

id-BeamMeasurementsReportConfiguration					ProtocolIE-ID ::= 361

id-HFCNode-ID-new										ProtocolIE-ID ::= 362

id-GlobalCable-ID-new									ProtocolIE-ID ::= 363

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-PNI-NPN-AreaScopeofMDT								ProtocolIE-ID ::= XXX

id-PNI-NPNBasedMDT										ProtocolIE-ID ::= YYY

id-SNPN-BasedMDT										ProtocolIE-ID ::= KKK

END

-- ASN1STOP

End of Changes

Appendix B: TP for TS 38.423

#### Start of Changes

## 8.2	Basic mobility procedures

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 8.2.1.4	Abnormal Conditions

If the supported algorithms for encryption defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EEA0 and NEA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed encryption algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the supported algorithms for integrity defined in the UE Security Capabilities IE in the UE Context Information IE, plus the mandated support of the EIA0 and NIA0 algorithms in all UEs (TS 33.501 [28]), do not match any allowed algorithms defined in the configured list of allowed integrity protection algorithms in the NG-RAN node (TS 33.501 [28]), the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the CHO trigger IE is set to "CHO-replace" in the HANDOVER REQUEST message, but there is no CHO prepared for the included Target NG-RAN node UE XnAP ID, or the candidate cell in the Target Cell ID IE was not prepared using the same UE-associated signaling connection, the NG-RAN node shall reject the procedure using the HANDOVER PREPARATION FAILURE message.

If the HANDOVER REQUEST message includes information for a PLMN not serving the UE in the target NG-RAN node in the Management Based MDT PLMN List IE, the target NG-RAN node shall ignore information for that PLMN within the Management Based MDT PLMN List.

If the MDT Configuration NR IE includes PNI-NPN Area Scope of MDT IE but does not include the CHOICE Area Scope of MDT-NR IE, the target NG-RAN node shall ignore the MDT configuration present in the PNI-NPN Area Scope of MDT IE.

Next Change

### 8.2.4	Retrieve UE Context

#### 8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer, or to request for small data transmission.

The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 8.2.4.4	Abnormal Conditions

If both the PNI-NPN Area Scope of MDT IE and the Area Scope of MDT IE are included in the MDT Configuration-NR IE in the RETRIEVE UE CONTEXT RESPONSE message, and the Area Scope of MDT IE is set to "PNI-NPN based", the new NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN areas, and ignore the PNI-NPN Area Scope of MDT IE.

If the MDT Configuration NR IE includes PNI-NPN Area Scope of MDT IE but does not include the CHOICE Area Scope of MDT-NR IE, the target NG-RAN node shall ignore the MDT area configuration present in the PNI-NPN Area Scope of MDT IE.

Next Change

## 8.3	Procedures for Dual Connectivity

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 8.3.1.4	Abnormal Conditions

If the S-NG-RAN node receives an S-NODE ADDITION REQUEST message containing in a PDU Session Resource To Be Added Item IE neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE, the S-NG-RAN node shall fail the S-NG-RAN node Addition Preparation procedure indicating an appropriate cause.

If the supported algorithms for encryption defined in the NR Encryption Algorithms IE in the NR UE Security Capabilities IE, plus the mandated support of NEA0 in all UEs (TS 33.501 [28]), do not match any algorithms defined in the configured list of allowed encryption algorithms in the S-NG-RAN node (TS 33.501 [28]), the S-NG-RAN node shall reject the procedure using the S-NODE ADDITION REQUEST REJECT message.

If the supported algorithms for integrity defined in the NR Integrity Protection Algorithms IE in the NR UE Security Capabilities IE do not match any algorithms defined in the configured list of allowed integrity protection algorithms in the S-NG-RAN node (TS 33.501 [28]), the S-NG-RAN node shall reject the procedure using the S-NODE ADDITION REQUEST REJECT message.

If the S-NG-RAN node receives an S-NODE ADDITION REQUEST message containing a S-NG-RAN node UE XnAP ID IE that does not match any existing UE Context that has such ID, the S-NG-RAN node shall reject the procedure using the S-NODE ADDITION REQUEST REJECT message.

If the M-NG-RAN node receives an S-NODE ADDITION REQUEST ACKNOWLEGE message containing a value for PDU Session ID in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE, the M-NG-RAN node shall regard setup of S-NG-RAN node resources of that PDU Session as being failed.

If the S-NG-RAN node receives an S-NODE ADDITION REQUEST message containing, for a PDU session, a PDU Session Resource Setup Info – SN terminated IE for which the Split Session Indicator IE is included and set to "split", the Security Result IE is not included, and either the Integrity Protection Indication IE or the Confidentiality Protection Indication IE is set to "preferred", it shall reject the PDU session.

If the MDT Configuration NR IE includes PNI-NPN Area Scope of MDT IE but does not include the CHOICE Area Scope of MDT-NR IE, the target NG-RAN node shall ignore the MDT area configuration present in the PNI-NPN Area Scope of MDT IE.

Interaction with the M-NG-RAN node initiated S-NG-RAN node Release procedure:

If the M-NG-RAN node receives an S-NODE ADDITION REQUEST ACKNOWLEDGE message containing in a PDU Session Resource Admitted To Be Added Item IE neither the PDU Session Resource Setup Response Info – SN terminated IE nor the PDU Session Resource Setup Response Info – MN terminated IE, the M-NG-RAN node shall trigger the M-NG-RAN node initiated S-NG-RAN node Release procedure indicating an appropriate cause.

If the timer TXnDCprep expires before the M-NG-RAN node has received the S-NODE ADDITION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall regard the S-NG-RAN node Addition Preparation procedure as being failed and shall trigger the M-NG-RAN node initiated S-NG-RAN node Release procedure.

##### Interactions with the S-NG-RAN node Reconfiguration Completion and S-NG-RAN node initiated S-NG-RAN node Release procedure:

If the timer TXnDCoverall expires before the S-NG-RAN node has received the S-NODE RECONFIGURATION COMPLETE or the S-NODE RELEASE REQUEST message, the S-NG-RAN node shall regard the requested RRC connection reconfiguration as being not applied by the UE and shall trigger the S-NG-RAN node initiated S-NG-RAN node Release procedure.

Next Change

### 8.3.14	Trace Start

#### 8.3.14.1	General

The purpose of the Trace Start procedure is to allow the M-NG-RAN node to request the S-NG-RAN node to initiate a trace session for a UE. The procedure uses UE-associated signalling.

#### 8.3.14.2	Successful Operation

Figure 8.3.14.2-1: Trace Start, successful operation

The Trace Start procedure is initiated by the M-NG-RAN sending the TRACE START message to the S-NG-RAN for that specific UE. Upon reception of the TRACE START message, the S-NG-RAN node shall initiate the requested trace session as described in TS 32.422 [23].

If the Trace Activation IE includes

-	the MDT Activation IE set to "Immediate MDT and Trace", and if the S-NG-RAN node is a gNB, it shall, if supported, initiate the requested trace session and MDT session as described in TS 32.422[23].

-	the MDT Activation IE set to "Immediate MDT Only"or "Logged MDT only", and if the S-NG-RAN node is a gNB, it shall, if supported, initiate the requested MDT session as described in TS 32.422[23] and the S-NG-RAN node shall ignore the Interfaces To Trace IE and the Trace Depth IE.

-	the MDT Location Information IE, within the MDT Configuration IE, and if the S-NG-RAN node is a gNB, it shall, if supported, store this information and take it into account in the requested MDT session.

-	the MDT Activation IE set to "Immediate MDT Only" or "Logged MDT only", and if the Signalling based MDT PLMN List IE is included in the MDT Configuration IE, and if the S-NG-RAN node is gNB, it may use it to propagate the MDT Configuration as described in TS 37.320 [43].

-	the Bluetooth Measurement Configuration IE, within the MDT Configuration IE, and if the S-NG-RAN node is a gNB, it shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the WLAN Measurement Configuration IE, within the MDT Configuration IE, and if the S-NG-RAN node is a gNB, it shall, if supported, take it into account for MDT Configuration as described in TS 37.320 [43].

-	the Sensor Measurement Configuration IE, within the MDT Configuration IE, the S-NG-RAN node shall take it into account for MDT Configuration as described in TS 37.320 [43].

-	the MDT Configuration IE, and if the S-NG-RAN Node is a gNB at least the MDT Configuration-NR IE shall be present, while if the S-NG-RAN Node is an ng-eNB at least the MDT Configuration-EUTRA IE shall be present.

If the Area Scope IE is not present in the MDT Configuration IE, the S-NG-RAN node shall consider that the MDT Configuration is applied to all PLMNs indicated in the MDT PLMN List, as described in TS 32.422 [23].

If the PNI-NPN Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the TRACE START message, the S-NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN. Upon reception of the PNI-NPN Area Scope of MDT IE, the S-NG-RAN node shall consider that the area scope for MDT measurement collection of PNI-NPN areas is defined only by the areas included in the PNI-NPN Area Scope of MDT IE.

#### 8.3.14.3	Abnormal Conditions

If the Trace Activation IE is not included in the TRACE START message, the S-NG-RAN node shall ignore the message.

If both the PNI-NPN Area Scope of MDT IE and the Area Scope of MDT IE are included in the MDT Configuration-NR IE in the TRACE START message, and the Area Scope of MDT IE is set to "PNI-NPN based", the S-NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection in PNI-NPN areas, and ignore the PNI-NPN Area Scope of MDT IE.

If the MDT Configuration NR IE includes PNI-NPN Area Scope of MDT IE but does not include the CHOICE Area Scope of MDT-NR IE, the target NG-RAN node shall ignore the MDT area configuration present in the PNI-NPN Area Scope of MDT IE.

Next Change

- 9.2.3.126	MDT Configuration-NR

The IE defines the MDT configuration parameters of NR.

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MDT Activation |  | M |  |  |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…) |  |  |  | – |  |  |  |
| CHOICE Area Scope of MDT-NR |  | O |  |  |  |  |  |  |  | – |  |  |  |
| >Cell based |  |  |  |  |  |  |  | If PNI-NPN Area Scope for MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |  |  |  |
| >>Cell ID List for MDT-NR |  |  |  | 1 .. <maxnoofCellIDforMDT> |  |  |  |  |  | – |  |  |  |
| >>>NR CGI |  | M |  |  |  | 9.2.2.7 |  |  |  | – |  |  |  |
| >TA based |  |  |  |  |  |  |  | If PNI-NPN Area Scope for MDT IE is present, this IE covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |  |  |  |
| >>TA List for MDT |  |  |  | 1 .. <maxnoofTAforMDT> |  |  |  |  |  | – |  |  |  |
| >>>TAC |  | M |  |  |  | OCTET STRING (SIZE (3)) |  | The TAI is derived using the current serving PLMN. |  | – |  |  |  |
| >TAI based |  |  |  |  |  |  |  | If PNI-NPN Area Scope for MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |  |  |  |
| >>TAI List for MDT |  |  |  | 1 .. <maxnoofTAforMDT> |  |  |  |  |  | – |  |  |  |
| >>>TAI List for MDT Item |  |  |  | 1 .. <maxnoofTAforMDT> |  |  |  |  |  | – |  |  |  |
| >>>>PLMN Identity |  | M |  |  |  | 9.2.2.4 |  |  |  | – |  |  |  |
| >>>>TAC |  | M |  |  |  | 9.2.2.5 |  |  |  | – |  |  |  |
| >PNI-NPN based |  |  |  |  |  |  |  |  |  | YES |  | ignore |  |
| >>CAG List for MDT |  |  |  | 1..<maxnoofCAGforMDT> |  |  |  |  |  | – |  |  |  |
| >>>PLMN ID |  | M |  |  |  | 9.2.2.4 |  |  |  | – |  |  |  |
| >>>CAG-Identifier |  | M |  |  |  | 9.2.2.66 |  |  |  | – |  |  |  |
| >SNPN-ID Based MDT |  |  |  |  |  |  |  |  |  | YES |  | ignore |  |
| >>MDT SNPN List |  |  |  | 1..<maxnoofMDTSNPNs> |  |  |  |  |  | – |  | - |  |
| >>>PLMN Identity |  | M |  |  |  | 9.2.2.4 |  |  |  | – |  | - |  |
| >>>NID |  | M |  |  |  | 9.2.2.65 |  | Identifies an SNPN together with the PLMN Identity IE. |  | – |  | - |  |
| CHOICE MDT Mode |  | M |  |  |  |  |  |  |  | – |  |  |  |
| >Immediate MDT-NR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Measurements to Activate |  | M |  |  |  | BITSTRING (SIZE(8)) |  | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [43]. First Bit = M1, Second Bit= M2, Fourth Bit = M4, Fifth Bit = M5, Sixth Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 3. |  | – |  |  |  |
| >>M1 Configuration |  | C-ifM1 |  |  |  | 9.2.3.128 |  |  |  | – |  |  |  |
| >>M4 Configuration |  | C-ifM4 |  |  |  | 9.2.3.129 |  |  |  | – |  |  |  |
| >>M5 Configuration |  | C-ifM5 |  |  |  | 9.2.3.130 |  |  |  | – |  |  |  |
| >>MDT Location Information |  | O |  |  |  | BITSTRING(SIZE(8)) |  | Each position in the bitmap represents requested location information as defined in TS 37.320 [43]. First Bit = GNSS Other bits are reserved for future use and are ignored if received. Value "1" indicates "activate" and value "0" indicates "do not activate". The eNB shall ignore the first bit unless the Measurements to Activate IE has the first bit or the sixth bit set to "1". |  | – |  |  |  |
| >>M6 Configuration |  | C-ifM6 |  |  |  | 9.2.3.131 |  |  |  | – |  |  |  |
| >>M7 Configuration |  | C-ifM7 |  |  |  | 9.2.3.132 |  |  |  | – |  |  |  |
| >>Bluetooth Measurement Configuration |  | O |  |  |  | 9.2.3.11 |  |  |  | – |  |  |  |
| >>WLAN Measurement Configuration |  | O |  |  |  | 9.2.3.12 |  |  |  | – |  |  |  |
| >>Sensor Measurement Configuration |  | O |  |  |  | 9.2.3.136 |  |  |  | – |  |  |  |
| >Logged MDT-NR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>Logging interval |  | M |  |  |  | ENUMERATED (ms320, ms640, ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960 and ms61440, infinity) |  | This IE is defined in TS 38.331 [10]. The value "infinity" represents one shot logging, i.e., only one log per event in the logged MDT report. |  | – |  |  |  |
| >>Logging duration |  | M |  |  |  | ENUMERATED (10, 20, 40, 60, 90, 120) |  | This IE is defined in TS 38.331 [10]. Unit: [minute]. |  | – |  |  |  |
| >>CHOICE Report Type |  | M |  |  |  |  |  |  |  | – |  |  |  |
| >>>Periodical |  |  |  |  |  |  |  |  |  |  |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |  |  |  | – |  |  |  |
| >>>>Logged Event Trigger Config |  | M |  |  |  | 9.2.3.137 |  |  |  | – |  |  |  |
| >>Bluetooth Measurement Configuration |  | O |  |  |  | 9.2.3.134 |  |  |  | – |  |  |  |
| >>WLAN Measurement Configuration |  | O |  |  |  | 9.2.3.135 |  |  |  | – |  |  |  |
| >>Sensor Measurement Configuration |  | O |  |  |  | 9.2.3.136 |  |  |  | – |  |  |  |
| >>Area Scope of Neighbour Cells |  | O |  |  |  | 9.2.3.140 |  |  |  | – |  |  |  |
| >>Early Measurement |  | O |  |  |  | ENUMERATED (true, ...) |  | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [10]. |  | – |  |  |  |
| Signalling based MDT PLMN List |  | O |  |  |  | MDT PLMN List 9.2.3.133 |  |  |  | – |  |  |  |
| PNI-NPN Area Scope of MDT |  | O |  |  |  | 9.2.3.x |  |  |  | YES |  | Ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT scope. Value is 256. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is 16. |

| Condition | Explanation |
| --- | --- |
| ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to "1". |
| ifM4 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to "1". |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1". |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1". |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1". |

Next Change

### 9.3.5	Information Element definitions

<Unchanged part omitted>

id-TAINSAGSupportList,

id-earlyMeasurement,

id-BeamMeasurementsReportConfiguration,

id-CoverageModificationCause,

id-UERLFReportContainerLTEExtension,

id-ExcessPacketDelayThresholdConfiguration,

id-Full-and-Short-I-RNTI-Profile-List,

id-QosFlowMappingIndication,

id-PNI-NPNBasedMDT,

id-PNI-NPN-AreaScopeofMDT,

id-SNPN-IDBasedMDT,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

<Unchanged part omitted>

maxnoofSRBs,

maxnoofSMBR,

maxnoofNSAGs,

maxnoofRBsetsPerCell1,

maxnoofTargetSNsMinusOne,

maxnoofThresholdsForExcessPacketDelay,

maxnoofCAGforMDT,

maxnoofMDTSNPNs

<Unchanged part omitted>

AreaScopeOfMDT-NR ::= CHOICE {

cellBased					CellBasedMDT-NR,

tABased						TABasedMDT,

tAIBased					TAIBasedMDT,

...,

choice-extension		ProtocolIE-Single-Container { {AreaScopeOfMDT-NR-ExtIEs} }

}

AreaScopeOfMDT-NR-ExtIEs XNAP-PROTOCOL-IES ::= {

{ ID id-PNI-NPNBasedMDT			CRITICALITY ignore	TYPE PNI-NPNBasedMDT		PRESENCE mandatory}|

{ ID id-SNPN-IDBasedMDT		CRITICALITY ignore	TYPE SNPN-IDBasedMDT			PRESENCE mandatory},

...

}

<Unchanged part omitted>

SymbolAllocation-in-Slot-BothDLandUL ::= SEQUENCE {

numberofDLSymbols	INTEGER (0..13),

numberofULSymbols	INTEGER (0..13),

iE-Extension		ProtocolExtensionContainer { {SymbolAllocation-in-Slot-BothDLandUL-ExtIEs} }	OPTIONAL,

...

}

SymbolAllocation-in-Slot-BothDLandUL-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-permutation		CRITICALITY ignore	EXTENSION Permutation	PRESENCE optional },

...

}

SNPN-IDBasedMDT ::= SEQUENCE (SIZE(1.. maxnoofMDTSNPNs)) OF SNPN-IDBasedMDT-Item

SNPN-IDBasedMDT-Item ::= SEQUENCE {

plmn-ID				PLMN-Identity,

nID					NID,

iE-Extensions		ProtocolExtensionContainer { {SNPN-IDBasedMDT-Item-ExtIEs} }	OPTIONAL,

...

}

SNPN-IDBasedMDT-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<Unchanged part omitted>

### 9.3.7	Constant definitions

<Unchanged part omitted>

id-AdditionalListofPDUSessionResourceChangeConfirmInfo-SNterminated									ProtocolIE-ID ::= 369

id-UERLFReportContainerLTEExtension																ProtocolIE-ID ::= 370

id-ExcessPacketDelayThresholdConfiguration															ProtocolIE-ID ::= 371

id-HashedUEIdentityIndexValue																		ProtocolIE-ID ::= 372

id-QosFlowMappingIndication																			ProtocolIE-ID ::= 373

id-Full-and-Short-I-RNTI-Profile-List																ProtocolIE-ID ::= 374

id-PNI-NPN-AreaScopeofMDT																			ProtocolIE-ID ::= XXX

id-PNI-NPNBasedMDT																					ProtocolIE-ID ::= YYY

id-SNPN-IDBasedMDT																					ProtocolIE-ID ::= xx3

END

-- ASN1STOP

End of Changes