3GPP TSG-RAN WG3 Meeting #122	R3-237526

# Chicago, USA, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1240 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on LCS reporting for UL-AoA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current F1AP signalling of UL-AoA, if Local Coordinate System (LCS) is used for UL Angle of Arrival, the gNB-CU receives the LCS to global coordinate system (GCS) translation information. If the LCS to GCS translation information is absent from the F1AP signalling, it is interpreted by the gNB-CU the azimuth and zenith are provided in the GCS. An issue that can happen is if the UL AoA angle info needs to be reported in LCS, but the LCS to GCS Translation IE is not provided. For example, the gNB -DU may not be configured with LCS to GCS translation information or not support such functionality. In that case, it is not clear how does the gNB-CU know whether the azimuth/zenith is provided in LCS and not GCS (there would be unresolvable ambiguity). To address this amboguity, when LCS to GCS translation is not configured in gNB-DU and gNB-CU requests UL AoA, then reporting in LCS is the only option for gNB-DU. It is proposed to add a new IE in the angular reporting to indicate that the UL-AoA measurements are in LCS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new LCS indicator that indicates to gNB-CU that the UL-AoA measurement are reported following LCS - Revise the semantics of the LCS to GCS Translation IE to mention that if absent the azimuth and zenith are provided in GCS, unless the LCS indicator is present. Impact analysis: Impact assessment towards the previous version of the specification (same release): This CR does not have impact from protocol point of view. This CR may have an impact from functional point of view because the UL-AoA IE may be reported in LCS but gNB-CU does not know. The impact can be considered isolated because it impacts only UL-AoA measurement reporting in the Measurement procedure . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if LCS to GCS translation is not configured in gNB-DU and gNB-CU requests UL AoA, then gNB-DU cannot indicate that UL-AoA are reported in LCS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.167, 9.3.1.198, 9.3.1.239, 9.3.1,256, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.455 CR 0123 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | n/a |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 9.3.1.167	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [38] |  |  |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [38] |  |  |
| LCS to GCS Translation AoA | O |  | 9.3.1.241 | If absent, the azimuth and zenith are provided in GCS, unless the LCS indicator is present. |  |  |
| LCS Indicator | O |  | ENUMERATED (true,...) | If present, azimuth and zenith angles are provided in LCS | YES | ignore |

NEXT CHANGE

#### 9.3.1.198	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PRS Beam Information List |  | 1 |  |  |  |  |
| >NR-PRS Beam Information Item |  | 1 .. < maxnoofPRS-ResourceSets > |  |  |  |  |
| >>PRS Resource Set ID | M |  | INTEGER (0..7) | The resource set in which the resources are associated with the angle. |  |  |
| >>PRS Angle List |  | 1 |  |  |  |  |
| >>>PRS Angle Item |  | 1..< maxnoofPRS-ResourcesPerSet > |  |  |  |  |
| >>>>NR PRS Azimuth | M |  | INTEGER (0..359) |  |  |  |
| >>>>NR PRS Azimuth fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >>>>NR PRS Elevation | O |  | INTEGER (0..180) |  |  |  |
| >>>>NR PRS Elevation fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >>>PRS Resource ID | O |  | INTEGER(0..63) |  | YES | ignore |
| LCS to GCS Translation List |  | 0..1 |  | If absent, the azimuth and elevation are provided in GCS, unless the LCS indicator is present. |  |  |
| >LCS to GCS Translation |  | 1 .. <maxnooflcs-gcs-translation> |  |  |  |  |
| >>Alpha | M |  | INTEGER (0..359) |  |  |  |
| >>Alpha-fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >>Beta | M |  | INTEGER (0..359) |  |  |  |
| >>Beta-fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| >>Gamma | M |  | INTEGER (0..359) |  |  |  |
| >>Gamma-fine | O |  | INTEGER (0..9) | Fine angles |  |  |
| LCS Indicator | O |  | ENUMERATED (true,...) | If present, azimuth and zenith angles are provided in LCS | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxnoofPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |
| maxnooflcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 3. The current version of the specification supports 1. |

NEXT CHANGE

#### 9.3.1.239	Zenith Angle of Arrival Information

This information element contains the Zenith Angle of Arrival, which can correspond to linear array measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Zenith Angle of Arrival | M |  | INTEGER(0..1799) | TS 38.133 [38] |  |  |
| LCS to GCS Translation | O |  | 9.3.1.241 | If absent, the zenith is provided in GCS unless the LCS indicator is present. the z-axis of LCS is defined along the linear array axis |  |  |
| LCS Indicator | O |  | ENUMERATED (true,...) | If present, azimuth and zenith angles are provided in LCS | YES | ignore |

NEXT CHANGE

#### 9.3.1.256	TRP Beam Antenna Information

The IE provides the beam antenna information of the TRP. It includes either the explicit beam antenna information, or a reference to another TRP’s signalled configuration, or the indication that no change has occurred with respect to previously signalled configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP Beam Antenna Info Item | M |  |  |  |  |  |
| >Reference |  |  |  |  |  |  |
| >>Associated TRP ID | M |  | TRP ID 9.3.1.197 | This IE specifies the TRP ID of the associated TRP from which the beam information parameters are adopted in Local Coordinate System (LCS). |  |  |
| >Explicit |  |  |  |  |  |  |
| >>TRP Beam Antenna Angles | M |  | 9.3.1.257 |  |  |  |
| >>LCS to GCS Translation | O |  | 9.3.1.241 | Included if the azimuth and elevation are not provided in GCS, unless the LCS indicator is present. |  |  |
| >>LCS Indicator | O |  | ENUMERATED (true,...) | If present, azimuth and zenith angles are provided in LCS | YES | ignore |
| >No Change |  |  | NULL | No change compared to the previously signalled configuration for this TRP. |  |  |

NEXT CHANGE

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NEXT CHANGE

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-LCSIndicator,

maxNRARFCN,

NEXT CHANGE

LCS-to-GCS-TranslationAoA::= SEQUENCE {

alpha				INTEGER (0..3599),

beta				INTEGER (0..3599),

gamma				INTEGER (0..3599),

iE-Extensions		ProtocolExtensionContainer { { LCS-to-GCS-TranslationAoA-ExtIEs} } OPTIONAL,

...

}

LCS-to-GCS-TranslationAoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LCStoGCSTranslationList ::= SEQUENCE (SIZE (1.. maxnooflcs-gcs-translation)) OF LCStoGCSTranslation

LCStoGCSTranslation ::= SEQUENCE {

alpha			INTEGER (0..359),

alpha-fine		INTEGER (0..9)		OPTIONAL,

beta			INTEGER (0..359),

beta-fine		INTEGER (0..9)		OPTIONAL,

gamma			INTEGER (0..359),

gamma-fine		INTEGER (0..9)		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {LCStoGCSTranslation-ExtIEs} } OPTIONAL

}

LCStoGCSTranslation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LCSIndicator ::= ENUMERATED {true, ...)

NEXT CHANGE

NR-PRSBeamInformation ::= SEQUENCE {

nR-PRSBeamInformationList		NR-PRSBeamInformationList,

lCStoGCSTranslationList 		LCStoGCSTranslationList		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NR-PRSBeamInformation-ExtIEs } } OPTIONAL

}

NR-PRSBeamInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-LCSIndicator		CRITICALITY ignore EXTENSION LCSIndicator		PRESENCE optional },

...

}

NEXT CHANGE

TRP-BeamAntennaExplicitInformation ::= SEQUENCE {

trp-BeamAntennaAngles					TRP-BeamAntennaAngles,

lcs-to-gcs-translation	 				LCS-to-GCS-Translation										OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaExplicitInformation-ExtIEs}}				OPTIONAL,

...

}

TRP-BeamAntennaExplicitInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-LCSIndicator		CRITICALITY ignore EXTENSION LCSIndicator		PRESENCE optional },

...

}

NEXT CHANGE

UL-AoA ::= SEQUENCE {

azimuthAoA					INTEGER (0..3599),

zenithAoA					INTEGER (0..1799)	OPTIONAL,

lCS-to-GCS-TranslationAoA	LCS-to-GCS-TranslationAoA		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-AoA-ExtIEs } }	OPTIONAL,

...

}

UL-AoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-LCSIndicator		CRITICALITY ignore EXTENSION LCSIndicator		PRESENCE optional },

...

}

NEXT CHANGE

#### -- Z

ZoAInformation ::= SEQUENCE {

zenithAoA					INTEGER (0..1799),

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { ZoAInformation-ExtIEs } }	OPTIONAL,

...

}

ZoAInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-LCSIndicator		CRITICALITY ignore EXTENSION LCSIndicator		PRESENCE optional },

...

}

NEXT CHANGE

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NEXT CHANGE

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList                                 ProtocolIE-ID ::= 707

id-LCSIndicator										ProtocolIE-ID ::= xxx

END

-- ASN1STOP