3GPP TSG-RAN3 Meeting #122	R3-237531

# Chicago, USA, 13th November – 17th November 2023

Agenda Item:	21.2

Source:	Ericsson, Xiaomi, Qualcomm Inc.

Title:	(TP to TS 38.413 BL CR): Addition of data size information options for MT-SDT paging with long enhanced eDRX

Document for:	Other

# Introduction

RAN3 is expecting the reply LS from SA2/CT4 on feasibility of signalling a list of buffered data sizes per QFI over the NGAP RAN Paging Request to enable accurate decision of NG-RAN during paging decision.

Another option that was considered last meeting is to include the size of the first data packet as feasible solution recommended by SA2.

This TP captures both options of data size information can be signalled over NGAP, waiting for final decision at RAN3#122 meeting

# TP to TS 38.413 BLCR

START OF CHANGES

### 8.3.Y	RAN Paging Request

#### 8.3.Y.1	General

This procedure is initiated by the AMF to indicate that there is DL data buffered in 5GC or DL signalling arrives for the UE.

The procedure uses UE associated signalling.

#### 8.3.Y.2	Successful Operation

Figure 8.3.Y.2-1: RAN Paging Request

The AMF initiates the RAN Paging Request procedure by sending the RAN PAGING REQUEST message to the NG-RAN node.

Upon reception of the RAN PAGING REQUEST message, the NG-RAN node shall perform RAN Paging for the UE in RRC_INACTIVE state.

If the Paging Policy Differentiation IE is included in the RAN PAGING REQUEST message, the NG-RAN shall, if supported, take it into account when performing RAN Paging for the UE in RRC_INACTIVE state.

If the DL Signalling IE is included in the RAN PAGING REQUEST message, the NG-RAN shall, if supported, take it into account when performing RAN Paging for the UE in RRC_INACTIVE state.

If the Data Size Information IE is included in the RAN PAGING REQUEST, the NG-RAN shall is supported use it to decide whether to initiate Mobile Terminated Small Data Terminated procedures towards the UE as described in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.X4	RAN PAGING REQUEST

This message is sent by the AMF to request RAN Paging due to MT data buffered in 5GC or the arrival of DL signalling for the UE.

Direction: AMF  NG-RAN

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| Paging Policy Differentiation | O |  | 9.3.3.X |  | YES | ignore |
| DL Signalling | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Data Size Information | O |  | 9.3.3.Z |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.Z	Data Size Information (option 1) - FFS

This IE contains a list of QFI(s) and for each QFI the associated buffered data size.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Data Size Item |  | 1..<maxnoofQoSFlows> |  |  |
| >QoS Flow Identifier | M |  | 9.3.1.51 |  |
| >DL Data Size | M |  | INTEGER (0..65535,…) | Unit: Byte. |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.3.3.Z	Data Size Information (option 2) - FFS

This IE indicates the data size of the first DL packet.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Data size | O |  | INTEGER(0..65535,…) | Unit: Byte. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

WarningMessageContents,

WarningSecurityInfo,

WarningType,

WUS-Assistance-Information,

RIMInformationTransfer,

FiveGCAction,

PagingPolicyDifferentiation,

DL-Signalling,

DataSizeInformation

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

id-WarningSecurityInfo,

id-WarningType,

id-WUS-Assistance-Information,

id-RIMInformationTransfer,

id-FiveGCAction,

id-PagingPolicyDifferentiation,

id-DL-Signalling,

id-DataSizeInformation

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

-- RAN Paging Request procedure

--

-- **************************************************************

-- **************************************************************

--

-- RAN PAGING REQUEST

--

-- **************************************************************

RANPagingRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {  RANPagingRequestIEs} },

...

}

RANPagingRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY ignore	TYPE AMF-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY ignore	TYPE RAN-UE-NGAP-ID		PRESENCE mandatory	}|

{ ID id-PagingPolicyDifferentiation		CRITICALITY ignore	TYPE PagingPolicyDifferentiation		PRESENCE optional	}|

{ ID id-DL-Signalling					CRITICALITY ignore	TYPE DL-Signalling		PRESENCE optional}|

{ ID id-DataSizeInformation				CRITICALITY ignore	TYPE DataSizeInformation		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- D

DataCodingScheme ::= BIT STRING (SIZE(8))

DataForwardingAccepted ::= ENUMERATED {

data-forwarding-accepted,

...

}

DataForwardingNotPossible ::= ENUMERATED {

data-forwarding-not-possible,

...

}

DataForwardingResponseDRBList ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF DataForwardingResponseDRBItem

DataForwardingResponseDRBItem ::= SEQUENCE {

dRB-ID								DRB-ID,

dLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

uLForwardingUP-TNLInformation		UPTransportLayerInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer {{DataForwardingResponseDRBItem-ExtIEs}}		OPTIONAL,

...

}

DataForwardingResponseDRBItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

--option 1

DataSizeInformation ::= SEQUENCE (SIZE(1..maxnoofMBSQoSFlows)) OF DL-DataSizeItem

DL-DataSizeItem ::= SEQUENCE {

qosFlowIdentifier				QosFlowIdentifier,

dL-DataSize						DL-DataSize,

iE-Extensions					ProtocolExtensionContainer { { DL-DataSizeItem-ExtIEs} }	OPTIONAL,

...

}

DL-DataSizeItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

DL-DataSize ::= INTEGER (0..65535, ...)

--option 2

DataSizeInformation ::= INTEGER (0..65535, ...)

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

id-TargetHomeENB-ID										ProtocolIE-ID ::= 364

id-HashedUEIdentityIndexValue							ProtocolIE-ID ::= 365

id-ExtendedMobilityInformation							ProtocolIE-ID ::= 366

id-CN-MT-communication-handling							ProtocolIE-ID ::= xx1

id-FiveGCAction											ProtocolIE-ID ::= xx2

id-PagingPolicyDifferentiation							ProtocolIE-ID ::= xx3

id-DL-Signalling										ProtocolIE-ID ::= xx4

id-DataSizeInformation									ProtocolIE-ID ::= xx5