3GPP TSG-RAN WG3 Meeting #122	R3-237579

Chicago, USA, 13 – 17 November, 2023

Agenda item:	12.2.2.1

Source:	Nokia, Nokia Shanghai Bell, BT, Orange

Title:	Discussion on Predicted Radio Resource Status

Document for:	Discussion and Decision

# 1	Introduction

One of the open issues that remain in the Data Collection Reporting Initiation procedure is related to the details of the Predicted Radio Resource Status IE. Currently we have used a reference to Radio Resource Status IE (Clause 9.2.2.50) which, however, considers aspects, currently not covered by Rel.18. Such aspects include for example radio resources per SSB and per slice. Furthermore, in RAN3 #121bis we made the following agreement:

Support of the AI/ML function does not apply to ng-eNBs (E-UTRA nodes connected to 5GC).

In this contribution, we provide our views on how to address this open issue.

# 2	Discussion

The current Radio Resource Status IE indicates the usage of the PRBs per cell for MIMO, per SSB area and per slice for all traffic in Downlink or Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling. Even though we provided reference to this clause in the BL CR, this would imply considerations that we haven’t made in Rel.18. For instance, introducing load predictions per SSB area granularity requires means for intra-cell trajectory prediction, which we haven’t considered in Rel.18.

Observation 1: Introducing load predictions per SSB area granularity requires means for intra-cell trajectory prediction, that is not considered in Rel.18.

Also, according to an agreement from the previous meeting ng-eNBs do not support AI/ML function, which means that they cannot predict radio resources.

Observation 2: In Rel.18, ng-eNBs cannot provide predicted radio resources.

Given that this is the last meeting for Rel.18, we propose to introduce a new IE for Predicted Radio Resource Status.

Proposal 1: Introduce a new IE to capture Predicted Radio Resource Status.

The new IE should indicate predicted PRB usage per cell for MIMO and Predicted Slice Radio Resource Status for all traffic in Downlink and Uplink, and predicted usage of PDCCH CCEs for Downlink and Uplink scheduling.

Proposal 2: We propose to introduce a new Predicted Radio Resource Status IE, that indicates predicted PRB usage per cell for MIMO and Predicted Slice Radio Resource Status for all traffic in Downlink and Uplink.

# 3	Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Introducing load predictions per SSB area granularity requires means for intra-cell trajectory prediction, that is not considered in Rel.18.

Observation 2: In Rel.18, ng-eNBs cannot provide predicted radio resources.

Proposal 1: Introduce a new IE to capture Predicted Radio Resource Status.

Proposal 2: We propose to introduce a new Predicted Radio Resource Status IE, that indicates predicted PRB usage per cell for MIMO and Predicted Slice Radio Resource Status for all traffic in Downlink and Uplink.