3GPP TSG-RAN WG3 #122                                                    R3-237588

Chicago, USA, 13th – 17th Nov, 2023

Agenda Item:	10.2.5

Source:	CATT

Title:	Discussion on SON enhancement for NR-U

Document for:	Discussion

Introduction

In last RAN3 meeting, the agreements on MRO and MLB for NR-U are collected as below:

MRO:

WA: RAN3 agrees to enable reporting of number of DL LBT failures from the target node to the source node in case of failed HO attempt (assuming the UE can be identified). Details are FFS.

In the document, we provide some analysis on the open issue on MRO and MLB for NR-U.

MRO

FFS if reporting of the number of UL LBT failures in case of non-HOF RLF can be addressed in Rel.18.

FFS whether in case of DL LBT problems at the last serving node that led to the TEH / HWC, the last serving node skips informing the source node or informs it with additional information.

MLB:

FFS on whether to add a load metrics on the number of UL consistent LBT failures in the Resource Status Update.

Discussion

  - MRO for NR-U

In last RAN3 meeting, we identified the following issue on MRO for NR-U:

Following LBT-related issues shall be eliminated from the MRO analysis:

- HOF due to UL consistent LBT failure;

- HOF due to DL consistent LBT failure;

- RLF (TLH / TEH / HWC) due to UL LBT delay;

- RLF (TLH / TEH / HWC) due to DL LBT delay;

Point (1) is addressed with the “consistent LBT failure” flag in the RLF Report.

Point (2) may be addressed with a report from the target node to the source node, but the target must be able to identify the UE.

Point (3) may be addressed with the number of transmission attempts being reported in RLF report also in case of RLF (i.e. when the cause is not HOF).

Point (4) may be addressed by implementation: when the last serving node receives the RLF information it can identify the UE context and find out retrieve DL LBT situation. Based on this, the lat serving node may decide if it triggers MRO optimisation (TLH) or if it informs the source node (TEH / HWC).

Here we continue discussing these issues.

For point (2), we achieved the following agreement:

WA: RAN3 agrees to enable reporting of number of DL LBT failures from the target node to the source node in case of failed HO attempt (assuming the UE can be identified). Details are FFS.

We do not support report the number of DL LBT failures from the target node to the source node for the following reasons:

1. Handover target cell is selected based on RRM measurement. After receiving measurement report, network may trigger handover. At that time point target cell is not occupied by shared network and DL LBT failure would not be happened. Given the time period from measurement report to handover execution is very short, it is impossible for the shared network being occupied during this period of time because the PRB usage will not change as dramatic as signal quality. In other words, at the time point when handover measurement report is sent to network the handover target cell is free. We can believe it is still free when handover is executed.

2. Handover target cell knows well about which part of DL PRB resources can be used because target cell has to send SSB and service many other UEs when receiving Handover Request message. If available PRB resource is not sufficient to serve the handed-over UE, target cell would reject handover by Handover Preparation Failure message. Otherwise, if target cell can accept the UE, Handover Request Ack message which includes the available PRB resources which have be checked by target cell. So, it is rare case that DL PRB resources are not available when UE access the target cell.

3. Target cell may be not able to identify UE for MSG1/MSG2 in the procedure of contention-based Random Access.

Therefore, we think during handover execution period, DL LBT failure is corner case and we may not consider it.

Proposal 1: It is proposed not to consider the case that DL LBT failure occurs during handover execution period.

For point (3), the following FFS is captured:

FFS if reporting of the number of UL LBT failures in case of non-HOF RLF can be addressed in Rel.18

The related scenario may be the measurement report should be sent to network to trigger handover when UE move to the edge of the serving cell, but UL LBT failures is detected and handover is delayed. RLF Report is generated with the cause of t310-expiry or t312-expiry. Network may detect too late handover failure type according to the RLF Report, but the true reason is UL LBT failure. Including the number of UL LBT failure in RLF Report may be helpful for this case, but how to record the number of UL LBT failure when UE want to send measurement report to network needs further discussion. We propose to postpone this issue to Rel-19.

Proposal 2: How to report the number of UL LBT failures in case of non-HOF RLF needs further discussion, the other WG should be involved in, it is proposed to postpone this issue to Rel-19.

Point (4) may be addressed by implementation: when the last serving node receives the RLF information it can identify the UE context and find out retrieve DL LBT situation. Based on this, the lat serving node may decide if it triggers MRO optimisation (TLH) or if it informs the source node (TEH / HWC).

FFS whether in case of DL LBT problems at the last serving node that led to the TEH / HWC, the last serving node skips informing the source node or informs it with additional information.

The related scenario may be handover command message should be sent to UE to trigger handover, but DL LBT failures is detected and handover is delayed. RLF Report is generated with the cause of t310-expiry or t312-expiry. Network may detect too late handover failure type if only referring to the RLF Report. However, network can store the UE context and DL LBT situation, and then analyzes RLF report together with UE context and DL LBT situation to find the true reason. So, Point (4) can be addressed by implementation.

Proposal 3: last serving node can analyses RLF report together with UE context and DL LBT situation to address Point (4) by implementation.

  - MRO for MLB

FFS on whether to add a load metrics on the number of UL consistent LBT failures in the Resource Status Update.

GNB can know COT in UL and PRB usage in UL per NR-U in R18. Based on such information, a node is able to know which neighbour cell is a suitable offload cell e.g., a cell with high COT and PRB usage per NR-U which is not a good offload cell. An example is considered that gNB may not detect the accurate COT in UL and PRB usage of NR-U because the interface is close to UE instead of gNB then gNB may need some information report from UE.

If the times of low layer UL LBT failure over threshold, UE may report UL consistent LBT failure to DU via MAC CE. It requires DU to calculate the number of UL consistent LBT failures for a period. For example, if the reporting periodicity of cell 1 is 500ms and DU receives 2 times UL consistent LBT failure during 500ms. For cell 2 the reporting periodicity is 1000ms and DU receives 3 times UL consistent LBT failure. It is difficult to say that cell 1 is better than cell 2 because it may only 10 RACH procedure occurs in cell 1 while 100 RACH procedures occurs in cell 2. Also, the periodicity are different i.e., the periodicity of cell 1 is smaller than cell 2.

Observation 1: It is difficult to CU to compare the number of the number of UL consistent LBT failures between neighbour cells. The neighbour cell may have different reporting periodicity and different number RACH procedures during the periodicity.

From other point view, we can consider reusing the UL consistent LBT failures reported in RLF report. However, RLF report does not an in-time report i.e., up to 48h but the resource status update procedure is more like an in-time procedure i.e., Reporting Periodicity (500ms, 1000ms, 2000ms, 5000ms,10000ms, …). When CU making a decision of offloading, the UL consistent LBT failures per BWP reported from UE may reflect the NR-U occupied situation of potential offload cell 48h ago.

Observation 2: UL consistent LBT failures reported by UE in RLF report cannot work in this case because RLF report is not an “in-time” report while resource status report is a second level report.

Proposal 4: Do not add load metrics on the number of UL consistent LBT failures in the Resource Status Update until we clear how it works. We are open to discuss how to address the inaccurate UL COT/PRB usage of NR-U detection in gNB.

Conclusions

Proposal 1: It is proposed not to consider the case that DL LBT failure occurs during handover execution period.

Proposal 2: How to report the number of UL LBT failures in case of non-HOF RLF needs further discussion, it is proposed to postpone this issue to Rel-19.

Proposal 3: last serving node can retrieve UE context and DL LBT situation to address Point (4) by implementation.

Observation 1: It is difficult to CU to compare the number of the number of UL consistent LBT failures between neighbour cells. The neighbour cell may have different reporting periodicity and different number RACH procedures during the periodicity.

Observation 2: UL consistent LBT failures reported by UE in RLF report cannot work in this case because RLF report is not an “in-time” report while resource status report is a second level report.

Proposal 4: Do not add load metrics on the number of UL consistent LBT failures in the Resource Status Update until we clear how it works. We are open to discuss how to address the inaccurate UL COT/PRB usage of NR-U detection in gNB.