3GPP TSG-RAN WG3 Meeting #122	R3-237658

Chicago, USA,13th – 17th November 2023

Agenda Item:	11.4

Source:	ZTE, China Telecom, China Unicom

Title:	Further discussion on other issues

Document for:	Discussions & Approval

Introduction

This paper provides the discussion on the left issues of QoE Others.

Discussion

RVQoE deactivation over F1

It was agreed at last meeting to use a non-UE associated class-2 procedure to deactivate RVQoE reporting over F1, and the corresponding TPs were agreed, with an optional UE-id list included in the message.

However, there is only some general description in stage2, without specifying the reactions of the CU, after receiving the deactivation message from the DU. In our mind, at least the two cases below should be clearly specified in 38.401:

- Case 1: The gNB-DU sends the QoE Information Transfer Control message, with the deactivation list included.

- Case 2: The gNB-DU sends the QoE Information Transfer Control message, without including the deactivation list.

For case 1, after receiving the deactivation message, the gNB-CU could directly stop sending the RVQoE reports to the UE(s) indicated by the deactivation list. While, for case 2, if no deactivation list, we believe it could be taken as an implicit indication for the CU to deactivate the RVQoE reporting to the whole gNB-DU. With the above understanding, we provided our TPs to F1AP and 38.401[1].

Proposal 1: Clarify in stage 2 about the how the gNB-CU deactivate RVQoE reporting over F1, depended on different cases of control information from the gNB-DU.

Inter-RAT mobility

The stage3 impact needs to be further checked, e.g., clarify in the semantics description of UE Application Layer Measurement Information List IE in TS 38.423 that, in case of intra-system inter-RAT HO, there can be only one item in the list? clarify in the semantics description of Container for Application Layer Measurement Configuration IE in TS 38.423 that, for intra-system inter-RAT HO, the maximum size of the container is 1000 bytes?

The proposal above about adding claridication on the number of item sin the configuration list makes some sense to us. For LTE QoE, only one QoE measurement session could be supported, so it is reasonable to regulate that, there can be only one item in the QMC configuration list, in case of intra-system inter-RAT HO.

Similarly, since the maximum size of QoE configuration container is LTE is 1000 bytes, it would be better to add corresponding semantic descriptions in NG specifications, for clarification in intra-system inter RAT HO. With this understanding, the gNB would not select the QMC configuration whose configuration container is larger than 1000 bytes to the target ng-eNB.

Proposal 2: Add semantic descriptions that there can only be one item in the QMC configuration list during intra-system inter-RAT HO.

Proposal 3: Add semantic descriptions that the maximum size of QoE configuration container is 1000 bytes during intra-system inter-RAT HO.

Conclusions

Proposal 1: Clarify in stage 2 about the how the gNB-CU deactivate RVQoE reporting over F1, depended on different cases of control information from the gNB-DU.

Proposal 2: Add semantic descriptions that there can only be one item in the QMC configuration list during intra-system inter-RAT HO.

Proposal 3: Add semantic descriptions that the maximum size of QoE configuration container is 1000 bytes during intra-system inter-RAT HO.

References

[1] R3-237659 (TPs for BL CR of TS38.401&38.473) Deactivation of RVQoE reporting over F1AP