3GPP TSG-RAN WG3 Meeting #122                 	                                             R3-237680

Chicago, USA, Nov 13- Nov 17, 2023

Agenda Item:	25.2.2

Source:	CMCC

Title:	Discussion on ECN marking and congestion exposure

Document for:	Discussion

Introduction

During the last meeting, the conclusions and remaining issues of ECN marking and congestion exposure are listed below:

RAN3#121bis： - Add the ECN Marking Request indicator or the information used to request congestion monitoring over NGAP, XnAP, and E1AP. The granularity over interfaces can be further checked. - Add the information used to request congestion monitoring over F1AP. - NG-RAN node provides the indication whether the QoS Flow is established with ECN marking request or congestion monitoring request activated or not activated over NGAP, F1AP and E1AP. FFS on XnAP. RAN3 agrees to address SA case with the first priority. For TS 38.415 (PDU TYPE 1) and TS 38.425 (PDU TYPE 2), two new presence flags are needed. FFS on the details (names and descriptions). There is a single piece of information produced by NG-RAN to address ECN marking for L4S at UPF and congestion information exposure. Agree the basic structure in the SoD for the request IE for XnAP, NGAP and E1AP to transfer the request. For F1AP, only one information request IE is needed. FFS on the IE name. Add the request IE in the same level of the QoS Flow Level QoS Parameters IE. Agree the basic structure in the SoD for the feedback IE. FFS whether separate feedback IEs are needed for uplink and downlink.

In this contribution, we will continue discussing ECN marking and congestion monitoring.

Discussion

Feedback IE for Uplink and Downlink

During last meeting, RAN3 reached the agreement that the ECN Marking Request indicator or the information used to request congestion monitoring should be introduced. Once receiving the indication from SMF, the NG-RAN node needs to provide feedback IE to show the activation status of the QoS flow. Regard to the feedback IE, we do not think it is necessary to introduce two separate feedback IEs for uplink and downlink. SMF provides a new indicator to activate ECN marking for L4S or congestion monitoring. It is not a common case to perform ECN marking or congestion monitoring only for UL or DL, hence, one feedback IE on ECN marking or congestion monitoring request for uplink and downlink is enough.

Proposal 1: One feedback IE on ECN marking or congestion monitoring request for uplink and downlink is enough.

Assistance information to be reported over the user plane

For information to be reported over the user plane, two options are listed as follows:

- Option 1: make the contents of these IE to be the percentage of IP packets that should be ECN marked in uplink and downlink

- Option 2: make the contents of these IE to be the percentage of congestion level in uplink and downlink

Both descriptions are found in TS 23.501 described as follows:

#### 5.37.3.3 Support of ECN marking for L4S in PSA UPF To enable ECN marking for L4S by a PSA UPF, a QoS Flow level ECN marking for L4S indicator may be sent by SMF to PSA UPF over N4. SMF also indicates to NG-RAN to report the congestion information (i.e. a percentage of packets that UPF uses for ECN marking for L4S) of the QoS Flow on UL and/or DL directions via GTP-U header extension to PSA UPF. If there is no UL packet when report for DL and/or UL needs to be provided, NG-RAN may generate an UL Dummy GTP-U Packet for such a reporting. …. - 5.37.4 Network Exposure of 5GS information 5GS and XR/media services cooperate to provide a better user experience using External Network Exposure. Based on the AF request, the 5GS can expose the following information based on the QoS Monitoring as defined in clause 5.33.3 and/or clause 5.45: - For the UL and/or DL congestion information monitoring (see clause 5.45.3), based on the PCC rule from PCF, the SMF requests the NG-RAN to report the information via GTP-U header to PSA UPF This NG-RAN reported information is common to support congestion information exposure and to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3. In the case of congestion information exposure, the PSA UPF exposes the UL and/or DL congestion information via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18. It can be applied to a Non-GBR or GBR QoS flow. …. - 5.45.3 Congestion information monitoring The NG-RAN may be required to provide the UL and/or DL QoS Flow congestion information (i.e. a percentage of congestion level for exposure). The UPF may be required to monitor the UL and/or DL QoS Flow congestion information reported from the NG-RAN. QoS monitoring request to the NG-RAN and NG-RAN reporting for UL and/or DL QoS Flow congestion information to PSA UPF is as defined in 5.37.3. The PSA UPF reports the received UL and/or DL QoS Flow congestion information to the target NF as instructed by the QoS Monitoring request from the SMF. Only one of ECN marking for L4S (in the case of ECN marking for L4S in RAN as described in clause 5.37.3) or QoS monitoring of congestion information may be requested to NG-RAN for a QoS Flow. They are mutually exclusive, therefore, measurements of Congestion information on a QoS Flow are not provided in QoS Monitoring reports if SMF enables ECN marking for L4S in RAN (see clause 5.37.3).

From the highlight part, it is noted that reported information is common to support congestion information exposure and to support ECN marking for L4S in UPF. This congestion information can either by represented as a percentage of packets that UPF uses for ECN marking for L4S or a percentage of congestion level for exposure.

Observation 1: Reported information is common to support congestion information exposure and to support ECN marking for L4S in UPF.

Actually, from our view, we have no strong preference to adopt which option. We can only provide the difference is that option 1 is more direct way and easy to implement by representing the congestion information as a percentage of packets that should be ECN marked. For example, if the percentage of packets to be ECN marked in UL/DL is set to 50%, the UPF should interpret this information as a recommendation to mark half of the UL/DL IP packets with the ECN flag. Regard to option 2, congestion level should be uniformly defined, companies may have different views on the definition of congestion level.

Observation 2: Option 1 is more direct way and easy to implement by representing the congestion information as a percentage of packets that should be ECN marked.

Observation 3: Regard to option 2, congestion level should be uniformly defined, companies may have different views on the definition of congestion level.

Proposal 2: No strong preference, slight inclined towards option 1: make the contents of these IE to be the percentage of IP packets that should be ECN marked in uplink and downlink.

Reporting Frequency and Threshold

From SA2 specification, it is specified that QoS monitoring and reporting mechanism described as follows:

#### 5.8.2.18 QoS Flow related QoS monitoring and reporting The SMF may configure the UPF to perform QoS monitoring for a QoS Flow and to report the monitoring results with the help of the following parameters provided in the Session Reporting Rule (SRR) described in clause 5.8.5.11: - QoS parameter(s) to be measured indicating the subject of the QoS monitoring as defined in clause 5.45; - Reporting period indicating the time interval in which a new measurement result and a potential report has to be available. If no measurement result is available to the UPF within the Reporting period, the UPF shall report a measurement failure; - Reporting frequency indicating the type of the reporting as "periodic" or "event triggered": - If the Reporting frequency indicates "periodic", the UPF shall send a report each time the reporting period is over. - If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS parameter(s) to be measured and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports shall not be sent by the UPF during the Minimum waiting time. If measurement results are received during the Minimum waiting time, the UPF shall report the minimum and the maximum measurement result when the Minimum waiting time is over.

From the yellow highlight part, it is noted that SMF may configure the UPF to perform QoS monitoring for a QoS flow with the parameters including the reporting frequency. The reporting frequency indicates the type of the reporting as “periodic” or “event triggered”. If the type indicates “periodic”, a reporting period shall be introduced. If the type indicates “event triggered”, a reporting threshold shall be introduced.

Likewise, same reporting frequency should be configured from SMF to NG-RAN. It is known that NG-RAN node can monitor congestion information, so that it can report by the means of “periodic” or “event triggered”. If adopting the type of “event triggered”, NG-RAN node only reports the congestion information as long as the results matches or exceeds the preconfigured threshold. Introducing a report threshold can reduce the frequency of reporting and coordinates the behavior between UPF and NG-RAN node. If adopting the type of “periodic”, UPF can periodically sense changes in congestion status according to the congestion information reported periodically from NG-RAN node.

Observation 4: Introducing a report threshold can reduce the frequency of reporting and coordinates the behavior between UPF and NG-RAN node.

Observation 5: Introducing the reporting period can make UPF periodically sense changes in congestion status according to the congestion information reported periodically from NG-RAN node.

Proposal 3: It is proposed to support SMF to configure RAN with the reporting frequency, which can be periodic or event triggered.

Conclusions

In this contribution, related to ECN marking and congestion exposure, the following observations or proposals are listed below:

Observation 1: Reported information is common to support congestion information exposure and to support ECN marking for L4S in UPF.

Observation 2: Option 1 is more direct way and easy to implement by representing the congestion information as a percentage of packets that should be ECN marked.

Observation 3: Regard to option 2, congestion level should be uniformly defined, companies may have different views on the definition of congestion level.

Observation 4: Introducing a report threshold can reduce the frequency of reporting and coordinates the behavior between UPF and NG-RAN node.

Observation 5: Introducing the reporting period can make UPF periodically sense changes in congestion status according to the congestion information reported periodically from NG-RAN node.

Proposal 1: One feedback IE on ECN marking or congestion monitoring request for uplink and downlink is enough.

Proposal 2: No strong preference, slight inclined towards option 1: make the contents of these IE to be the percentage of IP packets that should be ECN marked in uplink and downlink.

Proposal 3: It is proposed to support SMF to configure RAN with the reporting frequency, which can be periodic or event triggered.

References

- TS 23.501;