3GPP TSG-RAN WG3 Meeting #122	R3-237851

Chicago, US, 13th – 17th  November, 2023

Agenda item:	25.2.1

Source: 	Huawei, Lenovo, ZTE, Nokia, Nokia Shanghai Bell

Title: 	(TP to BL CR TS 38.473) Support for N6 jitter

Document for:	Discussion and Decision

1. Introduction

This is to produce F1AP TP to capture the N6 jitter design according to the summary of following CB:

CB: # R18XR1_PDUSet

- Discuss the open issues above

- Provide TPs if agreeable

(moderator - Nok)

Summary of offline disc R3-237778

TP for NR_XR_enh BL CR of TS 38.473

Start of Changes

#### 9.3.1.y	N6 Jitter Information

This IE indicates the N6 jitter information associated with the Periodicity in downlink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| N6 Jitter Lower Bound | M |  | INTEGER (-127..127) | Indicates the lower bound of the N6 jitter. The unit is: 0.5ms. |
| N6 Jitter Upper Bound | M |  | INTEGER (-127..127) | Indicates the upper bound of the N6 jitter. The unit is: 0.5ms. |

Next Change

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

** Unchanged text skipped **

#### -- N

** Unchanged text skipped **

NRUESidelinkAggregateMaximumBitrate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NZP-CSI-RS-ResourceID::= INTEGER  (0..191)

N6JitterInformation ::= 	SEQUENCE {

n6JitterLowerBound		INTEGER (-127..127),

n6JitterUpperBound		INTEGER (-127..127),

iE-Extensions    ProtocolExtensionContainer { { N6JitterInformationExtIEs } }    OPTIONAL,

...}

N6JitterInformationExtIEs   F1AP-PROTOCOL-EXTENSION ::= {

...

}

** Unchanged text skipped **

End of Changes