3GPP TSG-RAN WG3 Meeting #122	R3-237994

Chicago, USA, 13 – 17 November 2023

Agenda item:	14.4

Source:	Nokia, Nokia Shanghai Bell, Lenovo, ZTE, Google, Huawei, Ericsson, LG Electronics, NEC, CATT

Title:	[TP to BL CR to TS 37.340, S-CPAC] Complete RAN3 part of the S-CPAC solution

Document for:	Discussion

# 1	Introduction

At RAN3 meeting #122, the complete solution for S-CPAC was reviewed and corrected where necessary. As a result, it was decided to create a new stage-2 description for TS 37.340.

# Annex 1: Text proposal to TS 37.340

First change, ommited text not changed

## 10.X	Subsequent Conditional PSCell Addition or Change

A Subsequent Conditional PSCell Addition or Change (subsequent CPAC) is defined as a conditional PSCell change procedure that is executed after a PSCell addition or PSCell change based on pre-configured CPA or CPC configuration of candidate PSCells without reconfiguration and re-initiation of CPC/CPA. The UE keeps configured candidate PSCell configurations and evaluates the execution conditions of candidate PSCells after a PSCell addition or a PSCell change is triggered. SN initiated intra-SN subsequent CPAC, inter-SN subsequent CPAC initiated either by MN or SN are supported.

Editor’s note: FFS whether to support subsequent CPA, e.g. maintaining candidate PSCell configurations for subsequent CPA after SCG release.

Editor’s note: FFS how many subsequent conditional PSCell changes are targeted, and potential impacts.

The following principles apply to subsequent CPAC:

-	The subsequent CPAC configuration may contain a reference SCG configuration, the SCG configuration(s) of candidate PSCell(s) and execution condition(s).

Editor’s note: FFS if the reference configuration is optional in subsequent CPAC. FFS whether MCG configuration is included in the reference configuration. FFS RRC model for the reference configuration.

Editor’s note: FFS whether the MCG configuration associated with the SCG configuration of a candidate PSCell is included in subsequent CPAC.

-	Each candidate PSCell configuration can be provided as delta configuration on top of a reference configuration, which is used to form a complete candidate cell configuration. The reference configuration can be managed separately, and the UE stores the reference configuration as a separate configuration. Only one reference configuration is supported.

-	The network explicitly configures a conditional reconfiguration for the current serving PSCell if the network wants to use that PSCell as a candidate PSCell for subsequent CPAC.

-	The network explicitly releases all conditional reconfigurations for candidate PSCells after a PCell change (at least for inter-MN).

Editor’s note: FFS if and how to release the conditional reconfigurations in other cases, e.g. at intra-MN PCell change, SCG release, etc.

-	A candidate PSCell configuration for CPA can be used for subsequent CPC, but with different execution conditions of the candidate PSCell.

Editor’s note: FFS how to handle the security issue in the subsequent CPC, i.e. the same sk-counter/ S-KgNB is used while connected to SN #1 before and after being connected to SN #2 [pending to SA3].

MN initiated subsequent CPAC

The subsequent CPAC procedure is initiated by the MN for inter-SN subsequent CPAC configuration and inter-SN CPC execution.

Figure 10.X-1: Inter-SN subsequent CPAC - MN initiated

Figure 10.X-1 shows an example signalling flow for the inter-SN subsequent CPAC initiated by the MN:

1/2/3/4.	The MN initiates the inter-SN subsequent CPAC by requesting the candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure, indicating that the request is for subsequent CPAC. The MN also provides the candidate cells recommended by MN via the latest measurement results for the candidate SN(s) to choose and configure the SCG cell(s), provides the lists of candidate PSCells proposed to other candidate SN(s) and provides the upper limit for the number of PSCells that can be prepared by the candidate SN. Within the list of cells as indicated within the measurement results indicated by the MN, the candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). For each prepared PSCell, the candidate SN also decides the execution conditions for the following execution of the subsequent CPAC within the lists of candidate PSCells proposed to other candidate SN(s). If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN may also propose data forwarding to the MN or other candidate SN(s) for subsequent CPAC. The candidate SN includes the indication of the full or delta RRC configuration. The candidate SN can either accept or reject each of the candidate cells listed within the measurement results indicated by the MN, i.e. it cannot configure any alternative candidates.

The MN may select one of the candidate SN(s) and requests providing the reference configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

NOTE 1:	If the UE is configured with SN-1 in Dual Connectivity operation, then the MN starts the subsequent CPAC operation with MN-initiated SN Modification procedure instead of the SN Addition procedure.

NOTE 2:	If the UE is configured with SN-1 in Dual Connectivity operation, then the MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration or request a reference configuration for the subsequent CPAC, and to allow provision of data forwarding related information before step 1.

NOTE 3:	If applicable, the MN stores the data forwarding addresses and data forwarding proposals provided from all the candidate SN(s).

5.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

6/7.	If the lists of prepared PSCells received from the candidate SN(s) in steps 2 and 4 are different than the lists of proposed PSCells, the MN initiates the SN Modification procedures towards all candidate SN(s) to inform them about the updated lists of prepared PSCells in other candidate SN(s).

8.	The MN sends to the UE an RRCReconfiguration message including the subsequent CPAC configuration, i.e. a list of RRCReconfiguration* messages and associated execution conditions for the subsequent CPAC, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from one of the candidate SN(s) in steps 2 and 4, and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, e.g., to configure the required conditional measurements.

9.	The UE applies the RRCReconfiguration message received in step 3, stores the subsequent CPAC configuration and replies to the MN with an RRCReconfigurationComplete message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

11.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell. The RRCReconfigurationComplete* message may also include the sk-Counter value associated with the selected candidate PSCell if a new sk-Counter is selected.

12.	The MN informs the SN of the selected candidate PSCell that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message.

13.	The UE performs synchronisation towards the PSCell indicated in the RRCReconfiguration* message applied in step 12. The order the UE sends the MN RRCReconfigurationComplete* message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

14.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer message.

15.	For SN terminated bearers or QoS flows moved from the MN, dependent on the characteristics of the respective bearer or QoS flow, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

16.	If data forwarding is needed, the MN may send the Xn-U Address Indication message to the selected candidate SN. The SN may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 4:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared subsequent CPAC. In this case, it is up to the MN and the candidate SN implementations to make sure that the Early Status Transfer message(s) from the selected SN, if any, is forwarded to the right other candidate SN.

18.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCReconfiguration* message corresponding to the selected candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the selected candidate PSCell, and information enabling the MN to identify the SN of the selected candidate PSCell. The RRCReconfigurationComplete* message may also include a sk-Counter value associated with the selected candidate PSCell if a new sk-Counter is selected.

19.	The MN informs the SN of the selected candidate PSCell that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message.

20.	The UE performs synchronisation towards the PSCell indicated in the RRCReconfiguration* message applied in step 19. The order the UE sends the MN RRCReconfigurationComplete* message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

21/22/23.	The MN triggers the MN initiated SN Modification procedure to inform the last serving SN to stop providing user data to the UE, to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses. If applicable, the MN triggers the Xn-U Address Indication procedure to inform the last serving SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

24/25.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the selected candidate PSCell, if needed.

26.	If applicable, data forwarding from the last serving SN takes place. It may be initiated as early as the the last serving SN receives the early data forwarding address in step 4a.

27.	If data forwarding is needed, the MN may send the Xn-U Address Indication message to the selected candidate SN. The SN may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

NOTE 5:	Separate Xn-U Address Indication procedures may be initiated to provide different forwarding addresses of the prepared subsequent CPAC. In this case, it is up to the MN and selected SN implementations to make sure that the Early Status Transfer message(s) from the selected SN, if any, is forwarded to the right other candidate SN.

SN initiated subsequent CPAC

The subsequent CPAC procedure is initiated by the SN for inter-SN subsequent CPAC configuration and inter-SN CPC execution. The procedure is initiated with SN Change Required procedure, which is then followed with the steps as presented above for the MN-initiated subsequent CPAC.

Remaining text not changed