3GPP TSG-RAN WG3 Meeting #123bis	R3-241820

Changsha, China, 15th – 19th April 2024

Agenda Item:	13.2

Source:	Ericsson

Title:	High level design principle for inter-CU LTM

Document for:	Approval

# 1	Introduction

Rel-19 Mobility WI has been agreed in [1] with the following objectives for inter-CU LTM.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - As secondary priority, support the case when NR-DC is configured and CU is acting as SN and MCG is unchanged

  - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released

    - Note: The case that LTM is configured in both MCG and SCG is excluded

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

Given the substantial impact of network signalings, it is important to identify the primary principles in high-level design.

# 2	Discussion

There are several phases for LTM as being depicted in Rel-18, i.e., LTM preparation, Early synchronization, LTM execution. In this contribution, we focus on the case that the CU is acting as MN without configured DC as a starting point.

## 2.1	LTM Preparation

It is essential to figure out how to configure LTM between the source NG-RAN node and the target NG-RAN node. The involved nodes are different compared to the intra-CU case, while the gNB-CU makes the decision of LTM configuration based on L3 measurement report from the UE.

In inter-CU LTM scenarios, similar to legacy mobility procedures, the source node initiates the request to the target node, after which the target node decides to prepare for LTM configuration.

Figure 1. Example signaling flows for Inter-CU LTM Configuration

The main issue is whether to configure LTM candidate cells individually for each cell, like the existing handling for mobility. The following options are given:

- The configuration procedure is triggered per cell; That is aligned with the cell specific information provided in the handover messages.

- The configuration procedure is per node basis; That means the procedure will include a list of candidates cells within the same target NG-RAN node.

The granularity of network signaling during the inter-CU LTM configuration phase needs to be evaluated.

Legacy discussions have taken place regarding other topics, and currently, the approach remains focused on a per-cell basis. Regarding LTM, we suggest the group taking a step forward towards a node-based solution. A key reason to consider configuring multiple cells in one procedure is to reduce signaling overload. When a single candidate gNB-CU is connected to hundreds of gNB-DUs, handling a large volume of signaling over the Xn interface becomes a major challenge.

Accordingly, three possible options should be considered when go to details for signaling design.

- Option 1: Enhance the existing handover procedure to include single cell in each message.

- Option 2: Enhance the existing handover procedure to include a list of candidate cells within one NG-RAN node.

- Option 3: Introduce a new procedure for LTM configuration purpose, which includes a list of candidate cells per node.

Before selecting any option, it's important to establish the high-level principle. The most efficient approach could be using a procedure that configures multiple cells within one candidate gNB, serving as a solid foundation for Rel-19.

Agree to use a procedure that handles multiple candidate cells for inter-CU LTM configuration.

## 2.2	Early Synchronization

Prior to executing LTM, the network may prompt the UE to perform uplink and downlink pre-synchronization, also referred to as early UL/DL sync, with one or more candidate cells for LTM. This would reduce mobility interruption time during cell changes.

Figure 2. Early UL/DL pre-sync

To facilitate downlink pre-synchronization for inter-CU LTM, the serving gNB needs to receive the TCI states configuration from the candidate gNB(s). As for uplink pre-synchronization, the primary steps involve obtaining the UE's preamble index and then acquiring the Timing Advance (TA) value to avoid random access during the cell switch. Consequently, the signaling design should take at least the following goals into account to support early synchronization.

- RACH configurations retrieval

- TCI states configuration transferring

- Preamble index allocation

- TA information transferring

RACH configuration:

To enable early uplink synchronization, RACH configurations are necessary for inter-CU LTM. This information, like in intra-CU LTM, can be obtained during the preparation phase.

Current Rel-18 intra-CU design for configuration of UL pre-sync can be reused for inter-CU LTM.

RACH configuration is obtained during the LTM configuration phase, for example, within the existing handover procedure, to support UL pre-sync.

TCI states configuration:

A simple approach involves adopting the intra-CU design, wherein the configuration of transferring TCI states for DL pre-sync occurs concurrently with the LTM configuration request procedure.

Current Rel-18 intra-CU design for DL pre-sync can be reused for inter-CU LTM.

For specific UE, the TCI states configuration is obtained in the LTM configuration phase for DL pre-sync.

Preamble index allocation:

In Release 18 intra-CU LTM, the allocation of preamble indexes takes place during the UE Context Setup and UE Context Modification procedures, while the pool of preamble indexes is actually shared by multiple UEs. These indexes are provided in a list format on a per-cell basis within the Early UL Sync Configuration information. However, shortly after such allocation happens across the gNBs, the preamble index becomes depleted rapidly. It's preferable to have a more efficient procedure for requesting a preamble index for the UE to avoid scalability issue considering the preamble index limits to 64 values.

Therefore, triggering preamble index allocation based on a request per UE is preferred.

The existing design of preamble index allocation in Release 18 for intra-CU case would lead to scalability issues.

Preamble index needs to be allocated upon a request from the source node as per UE basis in the new signalings.

TA information transfer:

F1AP introduces two new procedures for transferring a list of Timing Advance (TA) values along with the allocated preamble index. This principle can also be applied for signaling exchange over Xn.

The same way of transferring TA values can be repurposed for the design of Xn interface.

## 2.3	LTM Execution

In Rel-18, the DU-CU/CU-DU Cell Switch Notification procedures are initiated as UE-associated signaling when a LTM cell change is triggered from the source cell to the UE. This same principle applies to inter-CU LTM scenarios.

Figure 3. Example signaling flows for Inter-CU LTM Execution

Early data forwarding is a topic worth discussing regarding inter-CU LTM. Unlike the intra-CU scenario, sending user data to the target node earlier could be beneficial, even if duplication happens.

Introduce a new procedure, e.g., Cell Switch Notification, for each UE over the Xn interface, following the same structure as intra-CU LTM.

RAN3 discusses whether and how to support early data forwarding for inter-CU LTM.

# 3	Conclusion

In the previous sections we made the following observation:

Observation 1	The granularity of network signaling during the inter-CU LTM configuration phase needs to be evaluated.

Observation 2	Current Rel-18 intra-CU design for configuration of UL pre-sync can be reused for inter-CU LTM.

Observation 3	Current Rel-18 intra-CU design for DL pre-sync can be reused for inter-CU LTM.

Observation 4	The existing design of preamble index allocation in Release 18 for intra-CU case would lead to scalability issues.

In this paper we propose:

Proposal 1:	Agree to use a procedure that handles multiple candidate cells for inter-CU LTM configuration.

Proposal 2:	RACH configuration is obtained during the LTM configuration phase, for example, within the existing handover procedure, to support UL pre-sync.

Proposal 3:	For specific UE, the TCI states configuration is obtained in the LTM configuration phase for DL pre-sync.

Proposal 4:	Preamble index needs to be allocated upon a request from the source node as per UE basis in the new signalings.

Proposal 5:	The same way of transferring TA values can be repurposed for the design of Xn interface.

Proposal 6:	Introduce a new procedure, e.g., Cell Switch Notification, for each UE over the Xn interface, following the same structure as intra-CU LTM.

Proposal 7:	RAN3 discusses whether and how to support early data forwarding for inter-CU LTM.

# 4	References

[1] RP-240299, R19 Mobility WID

[2] R3-241821, Details of XnAP signaling design, Ericsson