3GPP TSG-RAN WG3 Meeting #123-bis	R3-241900

Changsha, China, 15 - 19 April 2024

Agenda Item:	13.2

Source:	ZTE

Title:	(TP to TS 38.423) Support of inter-CU LTM

Document for:	Discussions & Approval

1	Introduction

This paper provides the corresponding TP for TS 38.423 to introduce new Xn TA information transfer function and Cell Switch Notification function for inter-CU LTM.

4	Text Proposal for BL CR TS 38.423

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.2.X	Cell Switch Notification

#### 8.2.X.1	General

The purpose of the Cell Switch Notification procedure is to enable the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the source NG-RAN node to the target NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.X.2	Successful Operation

Figure 8.4.X.2-1: Cell Switch Notification, successful operation

The source NG-RAN node initiates the procedure by sending a CELL SWITCH NOTIFICATION message.

Upon reception of the CELL SWITCH NOTIFICATION message, the target NG-RAN node shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Target Cell ID IE.

#### 8.2.X.3	Unsuccessful Operation

Not applicable.

#### 8.2.X.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.4.X	TA Information Transfer

#### 8.4.X.1	General

The purpose of the TA Information Transfer procedure is to enable the target NG-RAN node to transfer TA related information to the source NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.X.2	Successful Operation

Figure 8.4.X.2-1: TA Information Transfer, successful operation

The target NG-RAN node initiates the procedure by sending a TA Information Transfer message to source NG-RAN node.

Upon reception of the TA Information Transfer message, the source NG-RAN node shall, if supported, consider that the received information is the TA information from the candidate cell(s) that is indicated by the included Candidate Cell ID IE.

#### 8.4.X.3	Unsuccessful Operation

Not applicable.

#### 8.4.X.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.X	CELL SWITCH NOTIFICATION

This message is sent by the source NG-RAN node to inform the target NG-RAN node about the initiation of the cell switch  command to the UE.

Direction: source NG-RAN node  target NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | YES | reject |
| Target Cell ID | M |  | Target Cell Global ID 9.2.3.25 |  | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | O |  | INTEGER (0..127) | Corresponds to the TCI-StateId IE as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | INTEGER (0..63) | Corresponds to the TCI-UL-StateId IE as defined in TS 38.331 [8]. |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.3.X	TA INFORMATION TRANSFER

This message is sent by the target NG-RAN node to inform the source NG-RAN node about TA information.

Direction: target NG-RAN node    source NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.2.3.25 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>