| # 3GPP TSG-RAN WG3 Meeting #123-bis R3-241904 # Changsha, China, 15th Apr – 19th Apr 2024 CR-Form-v12.2. CHANGE REQUEST. 38.473; CR; -; rev; -; Current version:; 18.1.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; Radio Access Network; X; Core Network. |
| --- |
| Title: | Correction to 38.473 on NES CHO awareness at source gNB-DU |  |  |  |  |  |  |  |  |  |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-Core |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Only a subset of UEs in a cell might be configured with NES-specific CHO events. Currently, the source gNB-DU is not aware which UE(s) were configured with NES specific CHO event and therefore can’t decide which UE(s) to send DCI 2_9 (group DCI) to trigger the evaluation of NES specific CHO execution condition(s). - In the worst case, none of the UEs in a cell might be configured with NES-specific CHO and the source gNB-DU may unnecessarily send the DCI 2_9 to trigger NES CHO wasting both network and UE energy and causing unnecessary signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Source gNB-CU should further inform the source gNB-DU the UE(s) which were configured with NES specific CHO event so that it can decide which UE(s) to include while sending the DCI 2_9 to trigger the evaluation of NES specific CHO execution condition(s). - A NES CHO List IE is added in gNB-CU CONFIGURATION UPDATE with the list of IEs that are configured with NES specific CHO event. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Source gNB-DU will have no idea which UE(s) to include when sending DCI 2_9 to trigger the NES CHO evaluation, thereby causing the NES CHO feature very inefficient. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/CR ...CR... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.2.5       gNB-CU Configuration Update

#### 8.2.5.1            General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2            Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

…..

…..

If the NES CHO List IE is included in the GNB-CU CONFIGURATION UPDATE, the gNB-DU shall consider the UEs in the list to be configured with NES specific CHO configuration(s).

#### 9.2.1.10          GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:      If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU ® gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| .. |  |  |  |  |  |  |
| . |  |  |  |  |  |  |
| NES CHO List |  | 0..1 |  |  | YES | ignore |
| >NES CHO Item |  | 1 .. <maxnoofUEforNESCHO> |  |  | - |  |
| >UE Configured with NES CHO | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |

### 9.4.5       Information Element Definitions

maxnoofUEforNESCHO                  INTEGER ::= 4096