3GPP TSG-RAN WG3 Meeting #123bis 	R3-241917

# Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | draft | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Handover Cancel in CHO with SN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It was agreed that the target SN can release the resource of the other candidate PSCell associated with the other candidate MN(s) upon receiving the SN Release Request message sent from the other candidate MN(s), and the corresponding correction to TS 37.340 for MR-DC with 5GC given in R3-241110 was endorsed in RAN3#123. The corresponding content in EN-DC also needs to be modified, and the other candidate MN should send the SN Release Request message(s) to all the associated candidate SN(s). The description of step12d/e in 10.19.1 should be update, and the arrows between the other potential target MN and the target SN on step12d and 12e should be added in the Figure 10.19.1-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description of step12d/e in 10.19.1 - Update the Figure 10.19.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target SN cannot release the resource of the other candidate PSCell associated with the other candidate MN(s). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.19.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of the Change >>>>>>>>>>>>>>>>>>>>

10.19	Conditional Handover with Secondary Node

10.19.1	EN-DC

The Conditional Handover with Secondary Node procedure is used for configuration and execution of CHO with SN. This procedure includes the cases where the SN is kept, changed or added. If the SN is kept, the UE context at the SN is kept. If the SN is changed, the UE context at the source SN is moved to the target SN.

Figure 10.19.1-1: Conditional Handover with Secondary Node procedure

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

12c.	The source MN sends the Handover Cancel message toward the other signalling connections or other candidate MNs, if any, to cancel CHO for the UE.

12d/e.	If the target MN is configured with other candidate PCell(s) associated with other candidate SN(s) than the target SN, the target MN sends the SgNB Release Request message(s) to the corresponding candidate SN(s). Other candidate MN(s) send(s) the SgNB Release Request message(s) to all the associated candidate SN(s), if configured. The associated candidate SN(s) acknowledges the release request.

<<<<<<<<<<<<<<<<<<<< End of the change >>>>>>>>>>>>>>>>>>>>