3GPP TSG-RAN WG3 Meeting #123-bis	R3-242049

Changsha, China, 15 – 19 April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1275 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting IE referenced in Coverage Modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Tabular refers to Global NG-RAN Cell Identity, but ASN.1 refers to GlobalCell-ID |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since the exchange of coverage modifications is only related to NG-RAN cells, it would be possible to change ASN.1 but this change may cause transfer syntax error to the important NG-RAN NODE CONFIGURATION UPDATE message. Therefore, it is proposed to align the tabular with global Cell ID. The semantic is also reworded to clarify that only NG-RAN cell identity can be included. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it restricts the values of an IE This CR has no impact (nether functional nor protocol) for nodes that are using the specification as intended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned ASN.1/tabular and the spec would allow to send an IE that is not intended. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Unchanged text omitted |  |  |  |  |  |  |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global Cell Identity 9.2.2.73 | Cell Global Identifier of the cell to be modified. In this version of the specification, only a NG-RAN cell identifier can be included. | – |  |
| Unchanged text omitted |  |  |  |  |  |  |