3GPP TSG-RAN WG3 #123bis	R3-242067

Changsha, China, 15-19 April 2024

Agenda Item:	11.4

Source:	Ericsson

Title:	TP on multi-hops UE Trajectory

Document for:	Discussion, Agreement

# TP to TR38.743

This TP mirrors the discussions and proposals in R3-242068.

<<<<<< Start of CHANGES >>>>>

## 5.5	Multiple-hop UE trajectory across gNBs

### 5.5.1 Use case description

A multi-hop UE trajectory consists of a list of cells in which the UE will be served in RRC_CONNECTED mode. The cells are listed in chronological order and belong to at least two different gNBs.

A gNB with a trained and capable AI/ML model, upon inferring a multi-hop UE trajectory prediction, may decide to send the prediction to the gNBs to which the cells which are part of the prediction belong to.

A gNB may decide to request for a UE measured UE trajectories that span across multiple gNBs.

### 5.5.2 Solutions and standard impacts

When a gNB, e, g., gNB0, infers a multi-hop trajectory prediction, it sends the prediction to the next “hop” node, e.g., gNB1, which, upon receiving the prediction, may decide either to send the prediction to the next hop, or to discard it and possibly infer a new trajectory. This process is repeated by each subsequent “hop” node receiving the prediction.

When a gNB, e.g. gNB0, requests a multi-hop UE trajectory that spans across multiple gNBs, such request is made to the next hop gNB, e.g. gNB1. gNB1 will teport to gNB0 the UE trajectory while the UE was in coverage of gNB1 and until the point when the UE moved to a different gNB, e.g. gNB2. gNB1 requests to gNB2 to signal back the measured UE trajectory while th eUe is in coverage of gNB2. Once gNB1 receives this information from gNB2 is signals it top gNB0. The process is repeated for further following hops.

### In this way gNB0 is able to receive, along the time, measured UE trajectories for the same UE, which come to gNB0 in chronological order from the gNBs that served the UE. This enables gNB0 to collate an overall multi-hop measured trajectory for the UE.

<<<<<< End of CHANGES >>>>>