3GPP TSG-RAN WG3 Meeting #123-bis	R3-242074

Changsha, China, 15th Apr – 19th Apr, 2024

Title: 	TP on AIML support for CCO

Source: 	Ericsson, Deutsche Telekom, Charter, Telecom Italia

Agenda item:	11.3

Document Type:	Discussion and decision

# TP to TR38.743

This TP mirrors the discussions and proposals in R3-242075.

-------------------------------------------Change 1-------------------------------------------

## 4.2	AI/ML based Coverage and Capacity Optimization

### 4.1.1	Use case description

Editor Note: Capture the description of use case

The Coverage and Capacity Optimization (CCO) function in NG-RAN supports the detection and resolution, or mitigation of CCO issues, e.g., coverage and cell edge interference issues.

In the legacy CCO solution, a reactive approach is used: the gNB-CU detects a CCO issue after it has already occurred, and the gNB-DU attempts to resolve it.

With an AI/ML based CCO, a more proactive approach is used, and overall network performance can be improved by preventing (or limiting at an early stage) the degradation of network (and UE) performance.

### 4.1.2	Solutions and standard impacts

Editor Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

In a gNB in split deployment, the gNB-CU predicts a CCO issue and sends it to the gNB-DU, together with the predicted affected cells and beams. If the gNB-CU indicates that the prediction relates to a reference time in the future, the gNB-DU may not react immediately, and the gNB-CU may provide updates of the prediction. When the gNB-DU applies the coverage modification, the newly adopted coverage states are sent to neighbouring nodes, where the CCO function can be executed following the legacy solution.

-------------------------------------------End of changes-------------------------------------------