3GPP TSG-RAN WG3 Meeting #123-bis	R3-242087

Changsha, China, 15 – 19 April, 2024

Title: 	Discussion on RAN architecture for Ambient-IOT

Source: 	ZTE

Agenda item:	16.2

Document for:	Discussion and Decision

- Introduction

The SID (RP-224058): Study on solutions for Ambient IoT (Internet of Things) has updated to RP-240826.

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination, which is the determination of whether BS or intermediate UE and ambient IoT device are near each other or not (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

This paper addresses to discuss RAN architecture for Ambient-IOT.

- Discussion

  - Topology

Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor

Topology 1: BS ↔ Ambient IoT device

Figure 1: Topology 1

Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

Figure 2: Topology 2

  - RAN architecture

Figure 3: Overall architecture for Ambient IoT (left is Topology 1 and right is Topology 2)

Proposal 1: RAN3 agrees to capture figure 3 as overall architecture.

It is possible that Data transport is via CP CIOT like procedure, other than UP CIoT procedure, and Session management is not needed. So that, CU-CP/ CU-UP split may be not needed.

Proposal 2: CU-CP/ CU-UP split gNB is not supported.

Since A-IoT cell and normal cell are co-site, it seems CU/DU split gNB shall be supported. However, before protocol stack is designed, RAN3 will no start CU/DU split work.

Proposal 3: CU/DU split gNB is supported.

For Topology 2, after RAN receives inventory/command message from CN, it shall forward it to UE. One question is, in this topology, whether the RAN can be legacy RAN. In other word, whether the message can be transparent for RAN node.

Another question is, for the RAN supporting Ambient-IoT signalling/procedure, whether it is allowed to not support the new air interface between A-IoT device and reader (e.g., UA-IoT).

Proposal 4: RAN discusses whether legacy gNB can be used in Topology 2.

Proposal 5: RAN discusses whether gNB supports Ambient-IoT signalling/procedure but does not support the new air interface between Ambient IoT device and reader.

- Conclusion

After the above analysis, we provide the following observations and proposals.

Proposal 1: RAN3 agrees to capture figure 3 as overall architecture.

Figure 3: Overall architecture for Ambient IoT (left is Topology 1 and right is Topology 2)

Proposal 2: CU-CP/ CU-UP split gNB is not supported.

Proposal 3: CU/DU split gNB is supported.

Proposal 4: RAN discusses whether legacy gNB can be used in Topology 2.

Proposal 5: RAN discusses whether gNB supports Ambient-IoT signalling/procedure but does not support the new air interface between Ambient IoT device and reader.

- Text Proposal for TR38.769

================================Start of change===============================

6.4	RAN architecture aspects

Editor’s note: Corresponds to the second RAN3 objective in the SID.

6.4.x	Overall Architecture for Ambient IoT

The NG-RAN architecture for Ambient IoT is illustrated in Figure 6.4.1-1 below.

Figure 6.4.1-1: Overall architecture for Ambient IoT (left is Topology 1 and right is Topology 2)

6.4.y Split gNB Architecture for Ambient IoT

================================End of change===============================